Use WordPad or word to view this document.

Talk PCR Version B6.1 (Still beta).

29 Jan 1999

What’s new!

EEPROM functions now disabled in demo mode.

This cures a potential problem, when: -

If you first ran the programme in demo mode, and did an EEPROM backup whilst in demo mode, the backup file produced was full of zeros.  If later, with the radio connected, you then did a Verify or a Write to the radio it used the blank backup file.  The Verify would fail, and a write would corrupt the EEPROM data.

File change flags.

The search form and also the spectrum analyser now monitor for file changes.  If the setup changes then a * will appear after the file name in the application title.  If the * is there you will be prompted for file saves, otherwise you wont.

Wave Control file and Search forms.

The columns can now be resized, the new size will be remembered between sessions.  Also the comment fields are now correctly save to disk.  In the Wave control file, the lock field now works.  If you lock a recording file, TalkPCR will not delete that file.

Spectrum Analyser.

A few changes here to stop a lock up if the step was incorrectly set.  Also there is now a peak trace.  Lots more work done in this area, the calibration looks very good so far, -110dBm to –10dBm, but not yet ready to put out.

Take Current Freq details back to main form.

There is now an exit option, ctrl Z, which will exit the spectrum analyser and also the search form, taking the current radio settings back to the main radio form.  This can be found in the file menu, normal exit takes you back to the last used main form setting.

First click on volume control from outside programme.

There was a problem if you clicked on the volume control, when moving from another application, the volume control would jump to the mouse position.  This is now fixed.

Serial port selection.

The serial port selection method has been changed.  You will now get a new form with your options if the radio response has not been received.  The ini file still holds the setting.

Registration key.

The programme now looks for a registration key.  I am not yet ready to take any registrations, but if you would like to remove the splash screen and help me test the key system, please email me with your name and you windows computer user name, and I will send you a pre-registration key.

End of issue Beta 6.1 notes, the following are old notes.

Talk PCR Version B6.0 (Still beta).

11 Jan 1999

What’s new!

Major work on mouse wheel support.

I have found problems with various mouse wheel drivers.  In particular, the Logitech ver 7.51 and MS Intellimouse ver 2.1, when installed in Win98 cause the native Win98 mouse driver to fail.  I think I have overcome these problems, but if you are using these drivers with Win98, I recommend that you update your driver.

Hopefully the mouse wheel should also now work under NT.

If you are not using a mouse with a wheel you are missing out!

Spectrum analyser.

The spectrum analyser has been extended to enable saving and loading of the settings.  The details are save as a ‘.spt’ file.  I have included a set of ‘.spt’ files in the zip that covers some of the amateur and broadcast bands.

S Meter Calibration.

There is now an option to have an S meter calibration as close to the Icom meter as I could manage.  You can select the main S meter mode either by double clicking on the S meter or by the options panel.  I have also spent a great deal of time on alternate S meter calibration methods, this will appear at a later time, it looks very promising but needs to be verified on other PCR1000’s.

Auto Freq Mode.

There is now an option, under the option panel, that will cause the programme to automatically select Freq mode, on selection/focus of the tuning knob.  The tuning knob has little use in channel mode, changing to frequency mode seems a natural choice.  It allows the user to quickly tune up or down from a channel frequency using the stored channel step and modulation mode.  To this end, I have included ‘Wave Bands.FQL’ within the zip, which is the start frequency for most of the shortwave broadcast and amateur bands.

Tuning knob.

The first mouse movement after selection of the tuning knob is now ignored.  This stops wild tuning on selection of the knob.  You still have to keep the mouse still though.  The other control knobs now also work on this principal, no longer using a delay.

Comm Port Priority.

If you select the Hog Processor Option, Talk PCR will push the read/write threads of the serial comm port to the highest priority.  This helps loss of data, particularly when the bandscope or scanning is active and the programme is not primary programme running on the PC.

Bug Fixes.

Many minor fixes.

Major fixes as follows:-

FQL Bank overwrite.

There was a nasty error, that would overwrite data banks, when changing banks via the bank button, when the current bank was not saved.  There is now a forced save.

Comm port timeout.

The programme did not wait properly for the first response from the radio.

Expiry / Registration.

Version B6.0 is set to expire on 27th Feb 1999.  Hopefully there will be updates well before then.  I am intending that the programme will be shareware, with a registration fee of £25 UK pounds, this will remove all timeout locks etc, and entitle the user for all updates.

However registration will only be available after first issue, ie not beta, so until then enjoy, and I am sorry that you have to keep downloading new versions.  Registration date is in sight now!  I apologise to those who are fed up with the date limitation, but I do not consider the programme finished as yet!

New areas of work, to appear soon.

Timer recording.

This will enabled timed recordings, ie I want to record channel x at 12:30 to 13:00.  (Use of a large hard disk or CD writer recommended.)

Channel Monitor.

This will monitor the signal strengths of selected channels over a period of time and produce a log file.

Help File.

The Help file at this point has not been updated, work is in progress.

DDE interface.

There is a DDE interface to Talk PCR, this allows full external control, if any one is interested, I will finalise it and publish the interface details.

Known Problems.

NT.

There is a problem in opening communications via the comm port.  It did used to work, but apparently does not now.  Until I have access to NT to debug the problem I can’t solve it.

Talk PCR Stays on top.

There seems to be times when Talk PCR gets stuck on top, still investigating.

End of issue Beta 6.0 notes, the following are old notes.

Talk PCR Version B5.0 (Still beta).

28 Nov 1998

What's new!

Variable Channel bank lengths.

TalkPCR now allows banks to contain 0-1000 channels per bank.

Spectrum analyser.

After much experimentation, have got a 3rd version of the spectrum analyser together.  This one I think might even be useful.  It does not use the normal signal strength meter response from the radio but polls for the raw A>D reading.  This enables the noise floor to be seen, even those signals that just open the noise squelch to be seen.  It is by no means finished, but feel now it is worth continuing with!  Have dropped the BandMonitor, similar idea but lost where I was going!

Channel Edit Form.

Now allows Delete and Insert.  (Right click over the channel number cell).

Hog Processor Option.

Under Options, there is now a Hog processor check Box, (it defaults to hog!).  Selecting this will speed up scanning, but the programme will appear to use all the processor time if viewed under window system monitor.  In fact it only uses up the processor free time, and I can't see it slow down other tasks like Word etc, but if you don't like it you can switch it off in the options menu, (it is held in the ini file).

Help File.

An F1, context sensitive help file has been started, it is only about 30% done but it is included in this issue.  The next version of the help file may well be available on the web page before the next TalkPCR issue.  The help file is hard work!  Please look at the signal meter calibration curves, is this typical of a PCR1000?

Bug Fixes.

Default file extensions, in several file dialogue boxes seemed to have disappeared, now back.

The deletion of WAV file stopped working, now fixed.

Volume, Squelch controls now have a delay before actioning, this is to stop accidental adjustment of the volume control.

Expiry / Registration.

Version B5.0 is set to expire on 14th Jan 1999.  Hopefully there will be updates well before then.  I am intending that the programme will be shareware, with a registration fee of £25 UK pounds, this will remove all timeout locks etc, and entitle the user for all updates.

However registration will only be available after first issue, ie not beta, so until then enjoy, and I am sorry that you have to keep downloading new versions.

End of issue Beta 5.0 notes, the following are old notes.

Talk PCR Version B4.0 (Still beta).

New Features.

1.  	Signal strength meter.

This is a completely new component.  It also now appears on the search form. The dBm calibration is only accurate for frequencies up to 50MHz.  Note that this dBm figure is now the figure held in the search form signal strength field.

2.	New Knobs for volume, squelch and IF shift.

These controls are activated by left click on them, they then obtain focus, as indicated by the dotted square around them.  To change their position move the mouse whilst the left mouse button is down.  The knob will always point to the mouse.  When the control has focus you can also use the left right cursor keys, or the mouse wheel.

Double clicking the control will either mute, disable squelch or centre the IF shift.

3.	  Tuning knob.

The tuning knob is derived from the above knob.  Again left click to set the focus on the knob, move the mouse whilst the left button is down and the knob will rotate to always point to the mouse.  You can take the mouse right off the form with the left button down, this allows fine tuning.

If your finger gets tired of holding down the left button, press the right button once only, this locks the mouse to the control, but you have to stay within the form boundary, otherwise the knob wont follow it.  Any mouse click will release this mode.

In freq mode, with a small tuning step say 50Hz, this gives the feel of a tuning a real radio!

4.	 Repeater shifts.

The channels now have a repeater duplex field.  Enter a repeater shift in this field, then on the main form the repeater shift buttons will be enabled, allowing you to tune + or - the shift.  This can be carried over to the freq mode or even the quick mem buttons, but must be set up from wither the channel edit form or from the main form in channel mode.

5.	  New text fields.

Two new text fields have been added to a channel record, Srv and Location, you can use them as you wish.  Later there might be some functionality to scanning modes, searches etc.

6.	  Pop up controls in the channel edit form.

The method of editing data in the channel edit form has been improved.  To edit a field, double click.  A slider will appear for volume, squelch and IF shift.  Those fields that have two states will toggle.  Mode, bandwidth and step will show a drop down box.  To change frequency, repeater offset and the text fields just type,  Esc should undo typing.  The old edit method is still available if you press shift with the mouse buttons.  If you have a mouse wheel you can still use that if you press the shift key first.

Other Changes.

1.	Wav recorder.

There is now no problem in having several Wave control files in one directory.  The wave files themselves take on the name of the control file as a prefix.  I am not sure how many files you can have in a single directory, but I believe there is no limit if you have FAT32.  I have had over 1000 without any problem.

2.	Bank Association Form.

Use right click to bring up the option menu.

3.	Tuning steps.

This now uses a drop down list box.  You can still add a custom value by typing in the box rather than dropping it.  Also now supports frequencies like 8 1/3 rd.  This is entered as ‘8k3r’, r is for recurring.  Eg 1M35r = 1.355555 MHz steps.  The r must be the last character in the number.

4.	Priority Channels.

Some remaining problems in the priority channels have been fixed.

5.	 BandScope.

Drop down list boxes now control the display, rather than the left right click buttons.

6.	Tone decoder.

This has moved into the control menu, hope to expand this at some time, to launch app etc as per the Icom programme.

7.	More stable.

Have tracked down two causes of programme crashes.  One showed when the bandscope was running and the bandscope title bar was clicked on.  The other was a range check error which showed up when a channel step was not available in the bandscope.

8.	Channel data save to file.

Changes to the channel data are now tracked a little better, if the title bar shows a ‘*’ then it means the file has been altered.  You will be prompted before saving the file.  There is still work in progress in this area.

Notes.

How to install the programme.

This programme is self-contained, no dlls or helpers, there is no real installation, just ensure all files from the zip are in the same directory, preferable one of its own.  The programme will make its own .ini file, and recording directory.  The supplied bank.fql file contains a few sample frequencies, mostly rubbish.  To run the programme use windows explorer and double click on the talkpcr.exe file.  You can also add a short cut to you desktop, or use the startbutton->settings->taskbar&Startmenu to add a short cut to your taskbar.  Later versions will include an installer, to do this for you.

How to store channels from the main radio screen.

Select Frequency Mode.  Use the controls to select the frequency, mode bandwidth, squelch, If shift etc, when happy with the settings go to channel mode.  Don’t worry that your the settings appear to be lost.  In channel mode select the channel that you wish to programme, then press the ‘T’ for transfer button, the tuning setting from the last Freq mode will be transferred.  Now add the text fields, repeater duplex offset and any other final tweaks, when happy press the ‘S’ for store button.  Note you cannot alter the frequency without going back into freq mode.

At any time if you wish to adjust the details of any channel, with that channel selected press the ‘U’ unlock button, then adjust the text, or controls when happy press the store button.

The idea of this method is to provide a method of saving to file, a frequency found whilst tuning the radio.  For normal data input use the improved channel edit screen, this screen is live and changes made can be heard.

S meter Calibration.

I have spent a lot of time looking at the S meter calibration of the PCR1000.  The final display on Talk PCR, shows a calibration of dBm ranging from -100dBm to -50dBm.  On my radio this is quite accurate up to 50MHz, ie +-3dB at room temperature.  However above 50MHz the accuracy is poor, not only does the starting point change but the gain of the meter changes.  Above 50MHz the meter is more sensitive, starting to move at -110dBm, and has a greater range even as far as -20dBm at 60MHz.  The starting point then falls again as the frequency increases as the range

I believe the reason for this is that up to 50MHz the PCR does not use any RF pre-amplifier, it certainly does not have a tracking filter below 50MHz.  Above 50MHz there are 5 bands of tracking filters to provide the image rejection.  There is also an extra stage of gain, this stage also has AGC, this explains the extended range of the S meter, it also explains the variation of range with frequency as at the higher frequencies the AGC range is not so great.

If anyone is interested I can post some calibration curves of my PCR1000.

Expiry.

There is no longer a day count of use of the programme, the programme is still date limited.  This version is coded to expire on the 1st of Dec 1998, by which time a new version should be available on the web page.  Hopefully the programme will go to first issue at the start of the new year.

Old Readmes.

Talk PCR Version 3.1

New Features.

1.  The DSP unit is now supported, (for what it's worth).

2.  Support for com ports 1-8, can someone let me know if it works above 4?

Other Changes.

Many Bug fixes.

1.  Priority channels, fixed crash, also much faster now.  All three done at same time, also shows count down to events.  The check box 'do priority in scan' now works.

2.  Wav recorder, fixed loss of end of recording, added variable limit on short file length auto deletion.

3.  EEPROM verify is much quicker.

4.  Work on problems in loss contact with radio, should not see that error message so often.

5.  General improvements most areas, more wheely mouse and a few more hints.

Have spent about two weeks working on the GSM6.1 audio compression codec, have got it to work,  record anyway, but not impressed, have abandoned this area for the moment, going back to concentrate on the other areas.

Have extended the expiry date to 1st Nov 98.

Talk PCR version 3.0

13 Sept 1998.

This version has only one new major function, but it is a very important one.

1. The programme now has an integrated wav recorder and multiple file player.

IMPORTANT when using this utility you must understand what it is doing, if mis-used you will fill your hard disk very very quickly, and windows does not like a full hard disk System crashes are to be expected, if you let the hard disk get too full.

I recommend that if possible you run the programme in its own disk partition, this will limit file damage if your operating system cannot cope!

I have put into the programme two very important, recording restrictions.  The first is a simple time limit for each recorded wav.  The second is a limit on the minimum amount of hard disk space left, on the disk that the recording is being placed.  These can be set by the user, but be warned that you should leave at least 50MB on your system disk, you can have less if using another partition or disk.  These will be set to default values of 120Sec and 47Mb respectively.

The AutoRec button, in the recorder toolbar, will start a new .wav recording each time the squelch opens.  With this clicked you can go back to scanning etc, each time a signal is found it will be recorded, together with a time, frequency and file length stamp.

To view/play all recordings open the 'Wave Control Form'.  Here you will see a list of all the recordings.  To play one, click on the line.  Use the right mouse click to see more options, in particular the tag functions will allow multiple files to be played in the order of recording.

At present 'Comments' and 'Lock' fields are not saved to disk.

When the 'Format', or the 'Wave Control File' screen is opened the audio from the PCR1000 is muted, and all recording is stopped.  It would be possible on most audio cards to run duplex, but I could not face the complication at present!

Other Notes.

1.  The EERPOM functions are now enabled, take care.

2.  The line number system in the search form does not correctly.

3.  There is now better checking on the RS232 communication.

4.  Many other bugs have been taken out.

5.  An unlock button has been added to the main form to help with the storing to channel.

6.  There still needs to be some work on the 'fql' and 'srh' file save functions ,  they are not always saved when they should be.

Important note.

The programme uses a Wave control file, default is PCR.WCF.  this is a text file containing the details about the recordings, do not edit this file.  Also you can only have one WCF file per directory, you can change the working directory within the 'Format' and the 'Wave Control File' Screens.  The prefix of the WCF file is up to you, but only one per directory please, the programme will not check this!

Next areas of work,

Spectrum Analyser.

The Help file!, (this might be the web page stuff that I am now writing.)  Keep looking at the web page for more help pages, hopefully they will be added soon.

73's de Pete, G4NIS.