E L N E C

Version 2.20 and later                10 November 1991

1

C O N T E N T S

I N T R O D U C T I O N  . . . . . . . . . . . . . . . . . . . . . .    4

QUALITY   . . . . . . . . . . . . . . . . . . . . . . . . . . .    6

ABOUT THIS MANUAL   . . . . . . . . . . . . . . . . . . . . . .    7

DESCRIPTION   . . . . . . . . . . . . . . . . . . . . . . . . .    7

HARDWARE REQUIREMENTS   . . . . . . . . . . . . . . . . . . . .    8

MEMORY CONSIDERATIONS   . . . . . . . . . . . . . . . . . . . .    9

THE MAXIMUM PULSE OPTION (MAXP)   . . . . . . . . . . . . . . .   10

G E T T I N G    S T A R T E D   . . . . . . . . . . . . . . . . . .   11

INSTALLATION  . . . . . . . . . . . . . . . . . . . . . . . . .   12

RUNNING ENSETUP   . . . . . . . . . . . . . . . . . . . . . . .   13

U P G R A D I N G    F R O M    V. 2 . 0 X    O R   2 . 1 X  . . . .   16

NEW FEATURES  . . . . . . . . . . . . . . . . . . . . . . . . .   17

UPGRADING   . . . . . . . . . . . . . . . . . . . . . . . . . .   17

M O D E L I N G    W I T H    E L N E C  . . . . . . . . . . . . . .   18

INTRODUCTION TO MODELING  . . . . . . . . . . . . . . . . . . .   19

MODELING THE ANTENNA STRUCTURE  . . . . . . . . . . . . . . . .   19

CONSIDERATIONS FOR MODELING WIRES   . . . . . . . . . . . . . .   20

Wires Joining at an Angle  . . . . . . . . . . . . . . . .   20

Closely Spaced Wires   . . . . . . . . . . . . . . . . . .   21

Quads  . . . . . . . . . . . . . . . . . . . . . . . . . .   21

Log Periodic Antennas  . . . . . . . . . . . . . . . . . .   21

Multiband Antennas   . . . . . . . . . . . . . . . . . . .   21

USING SOURCES   . . . . . . . . . . . . . . . . . . . . . . . .   22

Using Multiple Sources   . . . . . . . . . . . . . . . . .   22

USING LOADS   . . . . . . . . . . . . . . . . . . . . . . . . .   23

MODELING GROUND   . . . . . . . . . . . . . . . . . . . . . . .   23

INTERPRETING THE RESULTS  . . . . . . . . . . . . . . . . . . .   24

Patterns   . . . . . . . . . . . . . . . . . . . . . . . .   24

Source (Feedpoint) Impedance and SWR   . . . . . . . . . .   25

Currents   . . . . . . . . . . . . . . . . . . . . . . . .   25

Load Data  . . . . . . . . . . . . . . . . . . . . . . . .   26

TIPS  . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   26

Doubling the Number of Available "Pulses"  . . . . . . . .   26

Segment Tapering for Wires Joining at an Angle   . . . . .   26

Sources and Loads at Junctions of More Than Two Wires  . .   27

Scaling  . . . . . . . . . . . . . . . . . . . . . . . . .   27

Conductivity and Scaling   . . . . . . . . . . . . . . . .   27

Using Templates  . . . . . . . . . . . . . . . . . . . . .   28

Modeling Complex Structures  . . . . . . . . . . . . . . .   28

WHY NOT DBD?  . . . . . . . . . . . . . . . . . . . . . . . . .   29

R U N N I N G    E L N E C   . . . . . . . . . . . . . . . . . . . .   30

STARTING ELNEC  . . . . . . . . . . . . . . . . . . . . . . . .   31

STARTING ELNEC IN TRACEVIEW MODE  . . . . . . . . . . . . . . .   31

2

TEST DRIVE  . . . . . . . . . . . . . . . . . . . . . . . . . .   32

Along the Straightaway   . . . . . . . . . . . . . . . . .   32

Through the Curves   . . . . . . . . . . . . . . . . . . .   35

R E F E R E N C E    M A N U A L   . . . . . . . . . . . . . . . . .   39

GENERAL INFORMATION AND CONVENTIONS   . . . . . . . . . . . . .   40

THE MENUS   . . . . . . . . . . . . . . . . . . . . . . . . . .   41

The Main Menu  . . . . . . . . . . . . . . . . . . . . . .   41

The Options Menu   . . . . . . . . . . . . . . . . . . . .   46

The Wires Menu   . . . . . . . . . . . . . . . . . . . . .   48

The Sources Menu   . . . . . . . . . . . . . . . . . . . .   52

The Loads Menu   . . . . . . . . . . . . . . . . . . . . .   54

The Media Menu   . . . . . . . . . . . . . . . . . . . . .   55

The Plot Menu  . . . . . . . . . . . . . . . . . . . . . .   57

SAVING AND RECALLING ANTENNA DESCRIPTIONS   . . . . . . . . . .   58

TRACEVIEW   . . . . . . . . . . . . . . . . . . . . . . . . . .   59

Starting TraceView   . . . . . . . . . . . . . . . . . . .   59

ANALYZE and TraceView  . . . . . . . . . . . . . . . . . .   59

Changing the Primary Trace   . . . . . . . . . . . . . . .   59

Ending TraceView   . . . . . . . . . . . . . . . . . . . .   60

ELNEC FILES   . . . . . . . . . . . . . . . . . . . . . . . . .   60

Files Created by ELNEC and/or ENSETUP  . . . . . . . . . .   61

PRINTERS  . . . . . . . . . . . . . . . . . . . . . . . . . . .   61

PLOTTERS  . . . . . . . . . . . . . . . . . . . . . . . . . . .   62

PROBLEMS  . . . . . . . . . . . . . . . . . . . . . . . . . . .   62

ERROR MESSAGES  . . . . . . . . . . . . . . . . . . . . . . . .   64

PULSES  . . . . . . . . . . . . . . . . . . . . . . . . . . . .   68

NOTES FOR EXPERIENCED MININEC USERS   . . . . . . . . . . . . .   69

MORE MININEC INFORMATION  . . . . . . . . . . . . . . . . . . .   69

HELP!   . . . . . . . . . . . . . . . . . . . . . . . . . . . .   70

3

E L N E C

I N T R O D U C T I O N

4

ACKNOWLEDGEMENTS

The fundamental computation portion of this program is that of MININEC

(Version 3) by J.C. Logan and J.W. Rockway of the Naval Ocean Systems

Center.

ELNEC version 2 was under development since the first release of

version 1.  During this interval of several months, three people have

tested numerous prototypes, evaluated features, and given many, many

suggestions for improvements.  On behalf of all ELNEC version 2 users,

I want to particularly thank Dick Gardner, N1AYW; Ernie Guerri, W6MGI;

and Dick Kiefer, K0DK for their extended effort.  Testing of version 2

was done by these gentlemen and by John Brosnahan, W0UN; Paul Carr,

N4PC;  Bill Clarke, WA4BLC; and Tony DeBiasi, K2SG.  Special thanks to

Paul for a critical and excellent proofreading of the manual and to

Wes Hayward, W7ZOI for spotting and reporting several bugs in various

prototypes.  Version 2.2 was tested by Jim Bromley, W5GYJ; Paul Carr,

N4PC; Ed Hanlon; Doug McGarrett, WA2SAY; Bob Rullman, K7MSH; and Roger

Steyaert, K7RXV.  Special thanks to Bob and Paul for giving version

2.2 a hard workout.  ELNEC is a much more bug-free program due to all

these people.

Thanks to all the ELNEC users who took the time to send in their

suggestions.  Finally, but foremost, thanks to my family for being

understanding and supportive during the many, many hours I've spent

away from them working on this program.

QUALITY

ELNEC is a program I personally take pride in.  The software is the

best I know how to make.  The physical embodiment of ELNEC is, of

course simple, but I've always used quality, name-brand disks

(primarily Memorex or Maxell) for this purpose.  Now, however, most

ELNEC customers will receive the program on unbranded disks.  This is

because the burden of formatting store-bought disks has become too

great, so disks are now purchased pre-formatted from a company which

specializes in formatting disks and copying programs.  They aren't

"generic" disks but OEM (Original Equipment Manufacturer) grade disks,

very tightly specified for high-speed formatting.  They carry no brand

name so they can be printed and labeled to suit the software vendor.

The first ones purchased are BASF and Sony brand, and ELNEC will

continue to be supplied only on high-quality, name brand disks.

6

ABOUT THIS MANUAL

A glance at this manual might give the impression that ELNEC is a

complex, difficult-to-use program.  Nothing is farther from the truth!

In all likelihood, you'll be able to begin analyzing antennas with

just a few minutes' familiarization (Try the TEST DRIVE).  If you've

used MININEC or other MININEC-based programs, you'll find ELNEC to be

worlds easier and friendlier.  But inevitably you'll want to learn all

the shortcuts built into ELNEC, or will have a question about the

operation or meaning of some function or menu selection.  This manual

is written so you'll have the answers at your fingertips, whenever you

need them.

DESCRIPTION

ELNEC is a powerful but easy-to-use program for modeling and analyzing

antennas.  A wide variety of antenna types and parasitic structures

may be modeled.

The far-field pattern of an antenna, including gain, can be plotted on

an ARRL-type (logarithmic-dB) or linear-dB polar plot, or presented in

tabular form.  All outputs, including plots, can be printed on a

standard dot-matrix printer or HP LaserJet or DeskJet printer.  A

special ANALYZE feature tells you the forward gain, front-to-back or

front-to-side ratio, beamwidth, angles of 3-dB pattern points, major

sidelobe level, and front-to-sidelobe ratio.  The points which ANALYZE

finds can be included on the plot to verify that ANALYZE has measured

what you thought it had.  In addition, you can display or print the

voltage, current, impedance, and SWR (for a 50- or 75-ohm system) at

each excitation source, the voltage, current, impedance, and power

loss of each load, and the current distribution on each wire.  The

antenna description may also be printed.

ELNEC offers an easy, menu-based system for describing and modifying

the antenna.  Unlike MININEC, ELNEC doesn't require a tedious and

error-prone counting of "pulses" to determine where a source or load

is placed.  And once placed, the source or load stays where it belongs

when the antenna is modified.  (This feature is described in more

detail in the REFERENCE chapter.)  Many special features are included

to make modifications fast and simple; for example, wires can be

added, deleted, or tilted, or wire lengths or antenna height changed,

with a few keystrokes.

Antenna descriptions and patterns are easily saved and recalled from

disk files.

ELNEC is substantially faster than MININEC3, and is several times

faster yet if a coprocessor is available (see HARDWARE REQUIREMENTS).

ELNEC has many other features, just a very few of which are:

- Inclusion of true current, as well as voltage, sources.

7

- View Antenna feature graphically shows what the antenna looks

like.

- Ability to superimpose several patterns on a single grid to

compare antennas and see the effect of changes.

- Inclusion of wire loss if desired.

- Ability to save patterns.

- Ability to save computed arrays so that later you can begin where

you left off.

- Many shortcuts for entering the antenna description, including

tilting wires and changing wire length.

- Far-field (pattern) analysis and plots/tables can be restricted

to any range of angles to speed computation when only one portion

of the pattern is of interest.

- ANALYZE features (beamwidth, sidelobe level, etc.) can be

calculated with greater angular resolution than the resolution of

the far-field analysis.

- All choices are made within ELNEC; it's not necessary to leave

the program to change features or descriptions. (Exception: printer

type, default file subdirectory, and screen colors are defined with

a setup program.)

- ELNEC is "lazy" -- it calculates only what it has to, so no

calculation is unnecessarily repeated.  Many antenna changes don't

require complete recalculation, so results are fast after making

modifications.

- Ability to handle very complex antennas with the addition of the

Maximum Pulse Option.

HARDWARE REQUIREMENTS

ELNEC is designed to run on all IBM-PC compatible computers running

under DOS 2.0 or higher.  Unlike MININEC, ELNEC doesn't have a fixed

limit on the number of wires, sources, loads, etc. (Exception: total

number of "pulses" is limited to 127 unless used with the Maximum

Pulse Option, separately purchased -- see PULSES in the REFERENCE

MANUAL chapter).  As a result, the memory (RAM) requirements depend on

how elaborate an antenna you wish to model.  At least 512k of RAM is

recommended, with 640k being sufficient for the most elaborate

antenna.  If you don't have sufficient RAM for a particular antenna,

ELNEC won't crash; it'll notify you before attempting to do the

analysis.  See MEMORY CONSIDERATIONS, below.

All ELNEC functions and features except plotting and Antenna View are

available on non-graphic systems.  Plotting requires a CGA, EGA, MCGA,

8

VGA, or Hercules graphics system.  If your system doesn't have

graphics capability, far-field (pattern) data are presented in tabular

form.  NOTE: To provide the highest possible resolution, CGA plots are

monochrome.

Text printing can be done with any printer connected to the normal

parallel printer port (LPT1:).  An Epson MX or FX 8/9-pin, IBM

Proprinter 8/9 pin, Epson LQ 24-pin, HP LaserJet or DeskJet, or

compatible printer is required for printing of the graphics plot.

Most modern printers are compatible with one of these types.  Plotters

are not supported.

Two types of ELNEC are available: STD (Coprocessor), and N (No

coprocessor).  The N type runs substantially faster than MININEC3,

but isn't speeded up if a coprocessor is present.  The STD type

requires a coprocessor and is 5-6 times faster than the N type.  I

highly recommend that you consider investing in a coprocessor if you

use calculation-intensive programs like ELNEC, CAD or graphics

programs, or spreadsheets.  The cost is moderate, installation is

simple, and the performance increase is striking.

ELNEC is not copy-protected so can be installed on a hard disk simply

by copying the files.

MEMORY CONSIDERATIONS

The amount of memory ELNEC requires is determined by the complexity of

the antenna being modeled, the most significant factor being the total

number of wire segments.  (See "Modeling The Antenna Structure" in

MODELING WITH ELNEC for more information on segments.)  Due to the

limitations of the compiler used, ELNEC accesses only the bottom 640k

of RAM, or "conventional" RAM.  With most systems having at least 512k

RAM, adequate RAM will be available for analyzing nearly any antenna.

However, you might encounter occasions when ELNEC reports insufficient

available RAM.

Conventional RAM is used by DOS and several other things such as

"resident" (sometimes called "TSR" for Terminate-Stay-Resident)

programs.  These are programs which can be called up with a "hot key";

Sidekick and PCTools (when installed as resident) are examples.

Spoolers, disk caches, virtual (RAM) disks, expanded memory managers,

and device drivers (e.g., HPIB drivers) can also occupy conventional

RAM.  These can be unloaded or moved to free more RAM.  Products such

a QEMM, 386Max, and DOS v. 5 permit moving some or all of these out of

the conventional memory area in machines equipped with 80386 or higher

processors and >640k RAM.  DOS v. 5 can also be made to move itself

out of conventional RAM in machines with 80286 or higher processors

and >640k RAM.  This frees a significant amount of RAM.

Before ELNEC begins calculations, it checks to see if enough RAM is

available.  If not, it reports the amount which must be freed.  If

this occurs, the antenna complexity must be reduced or the program

must be exited and RAM-consuming programs or drivers must be unloaded.

9

(If this is done, the program will remember the antenna configuration

and restore it when the program is restarted.)  If you have 640k of

RAM, you should never see the insufficient memory message unless a

significant amount of RAM is occupied by other programs or drivers.

ELNEC uses only the lower 640k of RAM; it does not access expanded or

extended memory.  This limitation is imposed by the compiler used.

THE MAXIMUM PULSE OPTION (MAXP)

An auxiliary program is available to increase the number of total

pulses ELNEC can handle.  This is the Maximum Pulse Option (MaxP).  It

is sold separately.  Currently, it's available only in a coprocessor

version and requires a hard disk, at least 640k RAM, and ELNEC version

2.20 or higher.  (It doesn't use extended or expanded memory.)  With

MaxP installed, ELNEC operates normally with antennas having 127 or

fewer pulses.  When more pulses are specified, MaxP is automatically

called.  The calculation speed of MaxP is about 2-4 times slower than

that of ELNEC, so computation time can be lengthy.  Bear in mind also

that the calculation time increases approximately as the square of the

total number of pulses.  These reasons are why MaxP is offered only in

a coprocessor version.  If you're interested in MaxP, consult current

advertisements or contact me for information.

10

E L N E C

G E T T I N G    S T A R T E D

11

INSTALLATION

If your program was supplied on two disks, the files containing this

manual (ELNEC.DOC) and the example antenna notes (ANTNOTES.DOC) are on

the one marked "MANUAL".  The other disk (called the "program disk"

below) contains all other files.  If your program was furnished on a

3.5 inch disk or high density (1.2 Meg) 5.25 inch disk, all files are

on the single disk.

The first step for all users is to copy the distribution disk(s) and

put the distribution disk(s) away.  If your program was furnished on

two disks, you won't need the copy of the MANUAL disk for the

remainder of this procedure.  Use the copy of the program disk for the

remainder of the installation procedure.  This copy will be called

"the working copy".

At some point you may want to print ANTNOTES.DOC.  It contains

information about the example antennas and their models.

SYSTEMS WITH A SINGLE DISK DRIVE:

Copy all the files from the working copy to another formatted disk.

Delete the READ.ME file from the new disk.  If your program was

furnished on a single disk, also delete ELNEC.DOC (this file) and

ANTNOTES.DOC from the new disk.  If you don't have a Hercules graphics

system, you can also delete MSHERC.COM to free additional space.

Label the new disk "ELNEC Program/Antenna File Disk", and installation

is complete.  Note: With a single drive system, the antenna (.EN)

files and program files can't be put on separate disks as they can

with a dual drive system.

More space can be made available by moving ENSETUP.EXE to another disk

(the "ENSETUP Disk").  If you do so, however, you must follow this

procedure whenever you want to run ENSETUP:  1. Copy ELNEC.CFG from

the Program/Antenna File Disk to the ENSETUP disk.  2. Run ENSETUP.

3. Copy ELNEC.CFG from the ENSETUP disk to the Program/Antenna File

Disk.

SYSTEMS WITH A DUAL DISK DRIVE:

ELNEC saves and recalls antenna descriptions in "antenna files" which

have the extension ".EN".  Traces (patterns) are saved in "trace

files" with the extension ".ENT".  You can keep these files on the

same disk or on a separate disk.  (You can always change your mind

later.)  The antenna files are quite small, so roughly 50-100 will fit

on the same disk as the program files.  Trace files are a bit larger,

so fewer will fit.  If you choose to put the antenna files on the same

disk as the program files, follow the procedure given for a single

drive system.  If you want to keep the antenna files on a separate

disk, you'll need two formatted disks.  Copy the following files from

the working copy to the first formatted disk:

ELNEC.EXE

ENSETUP.EXE

12

If you have a Hercules graphics system, also copy:

MSHERC.COM

Label this diskette "ELNEC Program Disk".  Then copy all the ".EN"

files (LAST.EN, 4SQUARE.EN, etc.) from the working copy to the second

formatted diskette.  Label this diskette "ELNEC Antenna File Disk".

You now need to tell ELNEC that they'll be in drive B:.  See RUNNING

ENSETUP for instructions.

SYSTEMS WITH A HARD DISK:

To install ELNEC on a hard disk, create a subdirectory named ELNEC and

copy all the files from the working copy to the subdirectory.  You can

delete ELNEC.DOC (this file), ANTNOTES.DOC and READ.ME to free disk

space, and you can also delete MSHERC.COM if you don't have a Hercules

graphics system.  If you want to keep the antenna files (extension

".EN") and trace files (extension ".ENT") in a separate subdirectory,

move them to the other subdirectory then see RUNNING ENSETUP for

instructions on how to notify ELNEC where to find them.

RUNNING ENSETUP

ENSETUP changes the values stored in file ELNEC.CFG, which ELNEC reads

each time it's started (see FILES in the REFERENCE MANUAL chapter).

Parameters which you can modify are:

Printer type (8/9- or 24-pin dot-matrix, or HP DeskJet or LaserJet

printers)

Path for antenna (.EN) and trace (.ENT) files  (tells ELNEC where

to find them)

Date format (mm-dd-yyyy, dd-mm-yyyy, or yyyy-mm-dd)

Background color (background color for text displays on color

monitors)

Plot colors (effective only with EGA/VGA systems -- CGA plots are

monochrome)

Recalled trace colors for the first five recalled traces (effective

only with EGA/VGA systems)

Monitor type (required only if you have a monochrome or LCD monitor

connected to a color adapter)

The default values are:

8/9-pin Epson FX type dot-matrix printer

Antenna (.EN) files and trace (.ENT) files on the current drive, in

current subdirectory

U.S. convention date format (mm-dd-yyyy)

Black background

Plot grid, total field plots white

Horizontal, vertical field plots white

Recalled traces white

Monitor type automatically determined by ELNEC

13

If the default values are satisfactory, you don't need to run ENSETUP.

FOR SYSTEMS WITHOUT A HARD DISK:

Boot the system with a DOS disk.  Remove the DOS disk and insert the

ELNEC Program Disk or ELNEC Program/Antenna Files Disk.  Type

'ENSETUP'.

FOR SYSTEMS WITH A HARD DISK:

Go to the ELNEC subdirectory by typing 'CD ELNEC'.  Then type

'ENSETUP'.

Printer type

Five choices are currently available.  If you don't know which type of

printer yours will emulate, choose 8/9 pin Epson FX type.  Most modern

8/9-pin printers will emulate the Epson FX, most 24-pin printers the

Epson LQ, and most laser printers the HP LaserJet.  You can't damage

your printer by selecting the wrong type, but you might get some very

strange-looking plots!

Dot Matrix Printers: Use the Epson LQ (24-pin) driver if you are using

a 24-pin printer.  There are two choices of drivers for 8/9 pin

printers.  The Epson MX driver uses fewer graphics density options

than the FX.  Therefore it will work with a wider variety of printers,

including IBM 8/9 pin printers.  The plot it generates is somewhat

smaller than FX driver plots, but you may get a faster plot with this

driver on FX-compatible printers depending on your printer and

computer hardware.  Note that there are some dot-matrix printers which

don't imitate the Epson printers.  See PROBLEMS in the REFERENCE

MANUAL chapter.

Laser Printers: If you are using an HP LaserJet III printer or

compatible, you can use either the LaserJet or DeskJet/LaserJet III

drivers.  The two drivers use different formats to send data to the

printer, so one may be faster than the other depending on your

computer hardware.  If you have a LaserJet, LaserJet II(), LaserJet

2000 or compatible, select the LaserJet driver.

DeskJet Printers: Select the DeskJet/LaserJet III driver if you are

using an HP DeskJet printer.

14

Path for antenna (.EN) files

If you want to keep the antenna (.EN) and trace (.ENT) files on a

different drive or subdirectory than the program file (ELNEC.EXE),

you'll need to enter their location.  If you have a dual floppy drive

system and the antenna files will be on the B: drive, simply enter

'B:' as the path.  On a hard disk drive, as an example suppose that

ELNEC.EXE is in subdirectory ELNEC and the antenna files are in a

subdirectory of ELNEC named ANTFILES.  The most foolproof way to enter

the path is to specify the full path from the root directory

(C:\ELNEC\ANTFILES for the example).

Background color

This parameter will change the background color of the text display on

color monitors.  It does not change the background color of the plot.

If you have a monochrome monitor, select Black.

Other colors

These select colors for the various parts of the plot.  CGA PLOTS ARE

MONOCHROME, so color choices are useful only for EGA/VGA monitors.

Hercules and some other monochrome systems won't support bright white,

and will display in standard intensity even if bright white is chosen.

Monitor type

If "Automatically determined" (the default) is selected, ELNEC will

assume that the monitor is color if a CGA, EGA, or VGA adapter is

present or monochrome if the adapter is Hercules.  If you're using a

monochrome monitor with a color adapter, some colors may not appear on

the monitor.  In this case, make the "Monochrome" selection.  The

"LCD" selection, at present, does the same thing as "Monochrome" and

modifies the calculation progress "thermometer" display which may

otherwise not reproduce properly.

Save choices and exit

If you make this selection, the choices you've made will be saved in

ELNEC.CFG.

Exit without saving changes

If you make this selection, ELNEC.CFG won't be changed.

15

E L N E C

U P G R A D I N G    F R O M    V. 2 . 0 X    O R   2 . 1 X

16

NEW FEATURES

ELNEC version 2.2 includes several new features not present in earlier

versions.   The new features are:

Selective listing of files when saving or recalling antenna

descriptions.  This and the following feature make it easier to

organize antenna and trace files.

Ability to change the default path to the antenna (.EN) and trace

(.ENT) files from the Options Menu.

True VGA support.  Previous versions generated EGA graphics when a

VGA adapter was present, causing plot distortion on some VGA

monitors.

Sources and loads shown on antenna diagram.

User-defined impedance for SWR indications.  The program now gives

SWR for 50-ohm systems and systems with the impedance of your

choice.

The ability to change the diameter of a number of wires at one

time.

Special screen modes for monochrome or LCD monitors connected to a

color adapter.

Support for MaxP, the Maximum Pulse Option (available separately).

UPGRADING

All you need to do to update ELNEC is to replace your older ELNEC.EXE

and ENSETUP.EXE files with the files of the same names from version

2.2.  Begin, as always, by making a copy of your distribution disk(s)

and use the copy for all operations.  It's not necessary to run

ENSETUP unless you've been having difficulty with a monochrome or LCD

monitor.  All version 1, 2, and 2.2 programs and files are completely

compatible; no translation of any kind is required.  Be sure to read

the rest of this manual for information about how to take full

advantage of the new features.

17

E L N E C

M O D E L I N G    W I T H    E L N E C

18

INTRODUCTION TO MODELING

You may already use modeling as a tool without realizing it.  When you

draw plans for a room for your house, the lines are straight and the

dimensions exact.  When the room is finished, none of the features is

exactly like the plans -- but the plans were close enough for you to

plan what materials to get and how to put them together, and whether

the room would suit the desired purpose.  Likewise, a schedule of the

day's activities is an idealized model of what will really come to

pass (and sometimes it's a pretty poor one, too!)  ELNEC is a tool for

antenna modeling -- building a model of antenna which will imitate the

real thing.

Is it really possible to predict how an antenna will behave by

building a model and analyzing it?  You bet!  But HOW ACCURATELY can

we predict its behavior?  That depends on the kit of tools that's

provided and the skill of the person making the model.  In this

chapter I'll discuss some of the strengths and limitations of ELNEC's

"tool kit" and give some guidance to help you build your modeling

skill.

The limitations of the MININEC modeling code (which are shared by

ELNEC) are discussed at length in "MININEC: The Other Edge of the

Sword", in February, 1991 QST.

You may want to take the "Test Drive" in the next chapter before or

during the reading of this chapter.

MODELING THE ANTENNA STRUCTURE

ELNEC models every antenna as a collection of straight WIRES.  I'll

emphasize STRAIGHT, again; a round loop must be built out of short,

straight pieces of wire.  You're free to choose the diameter of each

wire, and the program will give accurate results with diameters from

as small as you wish up to at least 0.02 wavelengths (that's about 3

feet at 40 meters).  With some imagination, nearly any type of

conducting structure can be modeled as wires.  For example, a wall can

be modeled as a grid of wires with a mesh on the order of 0.1

wavelength or less.

You tell ELNEC where the wires are placed in space by giving their x,

y, and z coordinates relative to a universal origin, or 0,0,0 point.

(See the first part of the REFERENCE MANUAL chapter for an

illustration of the axis system.)  Until you get used to this

description method, a sketch may help.  ELNEC has several features to

make this job as easy as possible; they're described in the REFERENCE

MANUAL chapter.  Wires can be connected only at their ends; crossing

wires won't connect them.  Modeling an "X" - shaped structure requires

four wires if the crossmembers are connected at the center of the "X".

Each wire is divided into SEGMENTS for analysis purposes.  ELNEC

assumes that the current is constant from the center of one segment to

the center of the adjacent segment.  This makes the problem one of

19

finding a finite number of impedances, currents, and field strength

contributions.  You, the user, must choose the number of segments for

each wire, and this is where some of the skill part of modeling comes

in.  Although accuracy improves when more segments are specified,

computation time increases approximately as the square of the number

of segments.  A useful rule of thumb is 10 segments per half

wavelength for pattern/gain analysis, and perhaps twice that number if

really accurate impedance values are required.  Closely-spaced wires

or wires joining at acute angles may require many more segments (see

the next paragraph).  If in doubt, a straightforward way of telling

whether enough have been specified is to increase the number and see

if the results change.  When analyzing antennas with wires which are

closely spaced or which join at acute angles, you might also look at

the currents on the wires.  Abrupt current changes may indicate an

insufficient number of pulses (but note that apparently abrupt phase

reversals at junctions may be due to internal conventions of assigning

current direction -- see INTERPRETING THE RESULTS, below).

Determining a reasonable number of segments isn't as hard as it

sounds.  You'll soon get a good feel of about how many you need to

find out the shape of a pattern or a feedpoint impedance with the

accuracy you need.

MININEC documentation specifies that the segment length should be

greater than 1.25 times the wire diameter, and greater than .0001

wavelength.  Another rule which should be observed for best accuracy

is that the segment lengths of connecting wires shouldn't differ by

more than a factor of about 2.

CONSIDERATIONS FOR MODELING WIRES

Wires Joining at an Angle

It's best to have approximately equal segment lengths in wires joining

at an angle.  Wires joining at a right angle require slightly more

segments than a single straight wire or wires joining in a straight

line.  A good working number for single square one-wavelength (quad)

loops is six segments per side.  Four may be used without noticeably

changing the pattern.  More than six are required if more than one

element is involved or if highly accurate feedpoint impedance results

are required.  See "Quads", below.

Wires joining at an acute angle require many more segments.  This is

because of the method MININEC uses to handle wire connections.  A

method to reduce the total number of required segments is described in

the TIPS section below.

20

Closely Spaced Wires

Use care when modeling very closely-spaced parallel wires such as

open-wire transmission line.  Although the wires themselves are

handled quite well, difficulties can arise at the point where they're

connected.  The best results occur if the connecting wire has only one

segment, and even in this case the segment length of the parallel

wires has to be no longer than about three times the wire spacing.

This means that transmission-line models usually require a large

number of segments.  A method to reduce this number is described in

the TIPS section below.

Quads

Experiments indicate that quads require quite a large number of

segments (perhaps 12 or more per loop side) to give a reasonable

representation of front-to-back ratio.  When dealing with wires

connected at an angle, MININEC (and ELNEC) "cuts the corner" by half a

segment length.  This doesn't cause much error with single loops but

apparently causes enough change in the relative currents in multiple

loops to quite noticeably affect indicated front-to-back ratio.  The

forward pattern and gain are fairly accurate with 6 segments/side or

so.  The segment-tapering method (see TIPS, below) is an effective but

rather tedious way of improving the accuracy of front-to-back

indications.  I'm investigating methods to improve quad modeling but

haven't made any breakthroughs yet.

Log Periodic Antennas

An integral part of log periodic antennas is the transmission line

connecting the elements.  The restrictions on closely spaced wires and

wires joining at an angle, explained above, make analysis of this

class of antennas difficult.  Probably the best approach is to make

the transmission line and element segment lengths equal and at the

feedpoint use the segment-tapering method described in TIPS.

Multiband Antennas

ELNEC is well-suited for modeling multiband antennas.  Don't forget to

pay attention to the segment length, however.  Remember that as you

increase the frequency the segment length increases in terms of

wavelength.  In general, you should double the number of segments each

time you double the frequency.  A bare minimum number of segments for

a square (quad) loop is about four per quarter wavelength.  This means

four segments per side at the frequency at which the loop is a full

wavelength in circumference, eight per side at twice the frequency,

etc.  Consequently, a quad loop which is one wavelength at 3.5 MHz

can't accurately be modeled at 28 MHz or above because of ELNEC's

limit of 127 pulses unless you use segment tapering, (p. 26, 51), the

pulse-doubling technique (p. 26), or the Maximum Pulse Option

(available separately).  (A loop has the same number of pulses as

segments.)

21

USING SOURCES

Because of the large number of segments required, it's usually not

practical to model transmission lines with ELNEC (see "Closely Spaced

Wires", above).  Therefore it's not practical to evaluate a phased

array feed system with ELNEC.  However, ELNEC permits you force the

array elements to have any ratio of voltages or currents and will tell

you the resulting impedances.  This is the information required to

design a feed system to actually accomplish the assumed voltage or

current ratio.  Phased arrays generally require currents to be in a

specified ratio, and ELNEC provides true current sources.  If an

antenna is driven at only one point, it doesn't matter whether a

current or voltage source is chosen.

Unlike MININEC, source placement is easy with ELNEC (see the REFERENCE

MANUAL chapter for details).  Sources appear in series with the wire

at the specified point.

Using Multiple Sources

Some precautions must be taken when using multiple sources in an

antenna.  The phase of resulting currents can be 180 degrees from

where you intended.  This can be prevented in phased arrays of

parallel wires by making sure that end 1 of the wires are all facing

the same direction (for example, all wire end 1's of a vertical array

connected to ground).  Reversing a wire will reverse the effective

polarity of all sources in that wire.  An ambiguity can arise when

placing a source at a wire junction because the current polarity can

depend on which wire the source "belongs" to and which way this wire

is facing.  A source is always placed so the positive terminal (or

terminal of outward-flowing current) faces end 2 of the wire in which

it's placed.  A source placed on a junction of two wires "belongs" to

the higher-numbered wire, even if ELNEC reports it to be connected to

the specified wire.  Therefore, it will be placed with the positive

terminal facing end 2 of the higher-numbered wire.  Ambiguity can be

avoided by making the wires face the same direction by connecting them

end 1-to-end 2.  If more than two wires are connected at a common

point, the source always will "belong" to the specified wire and will

have its positive terminal facing end 2.  Whenever you use multiple

sources in any antenna other than simple, parallel wires, it's highly

recommended that you look at the currents in the wires and make sure

they're really flowing in the direction you thought.  See "Currents"

under INTERPRETING THE RESULTS, below.

22

USING LOADS

Lumped impedances can be inserted into the wires if desired.  Like

sources, these are easy to place with ELNEC, and appear in series with

the wire.  Loads are useful to simulate loading coils, capacitors in

capacitive-loaded antennas, traps, and losses.  They can be specified

as either an impedance in R + jX form or as a quotient of Laplace

transform polynomials.  Automatic conversion of series or parallel RLC

circuits to Laplace transform coefficients is provided.  More detail

is given in the REFERENCE MANUAL chapter.

MODELING GROUND

ELNEC provides free space, ideal ground, or "real" ground

environments.

You'll find the following statement repeated several times in this

manual:  If either a perfect or "real" ground is specified, ELNEC

assumes a perfect ground for impedance and current calculations.  The

"real" ground description is used only for determining the shape and

strength of the far field (pattern).  This means that you cannot

determine the efficiency of a ground radial system with ELNEC, and the

impedances given for low (lower than about 0.2 wavelengths) horizontal

antennas won't be correct.  (Impedances will be those seen over

perfect, not real, ground).  Specifically, the resistive part of the

impedance of low horizontal antennas will be unrealistically low,

which also results in incorrectly high reported gain.  ELNEC shares

this limitation with MININEC and all other MININEC-based programs.

The limitation was consciously made by the authors of MININEC to keep

the code size and computation time reasonable.  Perhaps the necessary

code can be included in future modeling programs as PC power continues

to increase.

Note that the pattern of a low horizontal antenna will have the

correct shape provided that all parts of the antenna are at the same

height.

What ELNEC will tell you is what happens to your signal as the result

of ground absorption or partial reflection due to finite ground

conductivity and permittivity.  This is valuable in determining the

best height for an antenna, and you'll learn a lot about the

limitations of vertical antennas unless they're near a hillside or

lake.

The ground may be broken into several "MEDIA", each having its own

conductivity and dielectric constant (relative permittivity).  Each

medium can be a different height, permitting modeling of hills and

other features.  However, ELNEC won't recognize any shielding caused

by ground features -- it looks for a point of reflection from the

ground and ignores any features in between the antenna and the

reflection point or beyond the reflection point.  The media can be

arranged in parallel slices or concentric rings.  If rings are chosen,

you can put radials in the innermost circle.  These radials modify the

23

ground conductivity in that region, again only for far field (pattern)

analysis.  See the REFERENCE MANUAL chapter for an example of the use

of several media.

A Wire is connected to ground by specifying a zero z-coordinate.

INTERPRETING THE RESULTS

Patterns

An antenna radiates in three dimensions but pattern plots have only

two.  This poses a problem not unlike that faced by cartographers

making flat maps of the spherical Earth.  ELNEC uses a standard method

of representing the pattern, with elevation and azimuth plots.  For

both types, the antenna is assumed to be so far away it's represented

only by a point at the center of the plot.

To see how an azimuth plot is generated, visualize one of those

conical paper cups with a pointed bottom.  Now imagine it being much,

much larger than the antenna.  With the tip of the cup bottom on the

antenna, place yourself on the rim of the cup with a field strength

meter.  As you traverse the rim of the cup, take readings and plot

them on a polar graph.  If you start directly over the +x axis, the

first reading is plotted at zero degrees.  The reading halfway around

the rim is 180 degrees, and so forth.  The "elevation angle" is the

angle the side of the cup makes with the ground.  To measure the

pattern at a lower elevation angle, the cup is made more squat.

For the elevation pattern, slice a flat plate in half, leaving a

semicircle.  Make the half-plate much larger than the antenna, and

place it on edge with the straight side on the ground and the antenna

at the center of the straight side.  Again you're on the rim taking

readings and plotting.  If the bottom of the half-plate is on the x-

axis and you're on the rim at the ground in the +x direction, the

reading is for zero degrees elevation.  The reading from the top of

the rim, directly over the antenna, is 90 degrees elevation.  The

"azimuth angle" is the angle the half-plate bottom makes with the x-

axis.  For example, if the plate, still on edge, is rotated so it

lines up with the y-axis, the azimuth angle is 90 degrees.

24

Source (Feedpoint) Impedance and SWR

The impedance, SWR, voltage, and current at each source can be viewed

or printed by selecting 'SD' (Source Data) from the Main Menu.  If

ELNEC reports a very low value of resistance at any source, be careful

-- this might indicate operation beyond ELNEC's limits.  For example,

ELNEC will report low resistances for low horizontal antennas over any

type of ground (See MODELING GROUND, above).  If two elements are

closely spaced and fed out of phase (W8JK-type antennas), the low

resistance is real but the real antenna might not work like ELNEC

predicts unless you have included wire loss (a Main Menu choice).

Losses become important when the resistance is low, so be sure wire

loss is included if a low resistance is indicated.  Negative

resistances sometimes are reported for multi-element arrays.  These

actually can occur but are subject to the same cautions as low

positive resistances.

The SWR shown is the SWR which would be present on a feedline

connected in place of the source.  Values are given for 50 ohm and

user-defined feedlines.  This is directly calculated from the

impedance of the source.

Currents

The currents at each pulse (segment junction) are seen by selecting

'CU' at the Main Menu.  The currents give some important clues about

the antenna.  One thing to look for is symmetry.  If the antenna is

symmetrical, the currents also should be -- if not, some error has

been made in wire definition, source placement, or other area.  The

current from the vertical part of a ground plane antenna should split

evenly among the radials if the radials are the same length and evenly

spaced.  Make it a habit to look for these symmetries and you'll spot

problems before bad results lead you astray.  Another thing to look

for is abrupt and unexplained current changes (instead of a smooth

change from one pulse to the next).  This usually is due to not having

enough segments.  Sudden current reversals might be due to the way

you've defined the wires.  Positive current flow always is defined as

being from end 1 to end 2, so if you've connected two end 1's or two

end 2's together you'll see a 180-degree shift in current direction at

the junction of the wires.  The current actually is continuous as it

should be, but the definition of direction changes from one wire to

the other.  There's nothing wrong with connecting wires in this

fashion but don't be confused by the currents ELNEC shows as a result.

Also, be careful if using multiple sources -- see "Using Multiple

Sources" under USING SOURCES, above.

The importance of currents is underscored by the fact that the field

generated by an antenna is proportional to the current flowing on it.

25

Load Data

Load data are shown by typing 'LD' at the Main Menu.  The voltage

across, current through, impedance of, and power loss of each load is

shown.  In addition, the total load loss is shown in watts and dB.

The loss figures are invaluable in determining the loss caused by

traps, loading coils, and the like.  You also can determine the

voltage across the load or current through it under actual operating

conditions.  Note the total applied power reported by ELNEC.  The

voltage across or current through the load at a given power input will

be the reported voltage or current multiplied by the square of the

ratio of the power you'll be applying to the total power ELNEC is

reporting.  If you do this calculation a lot, let me know -- it could

be included in future versions.

TIPS

Doubling the Number of Available "Pulses"

The limit of 127 "pulses" can be effectively doubled in some cases.

(See p.68 for more information on pulses.)  The requirements are that

the antenna (including sources and loads) is symmetrical, and that

only free-space analysis is desired.  If these are true, one-half the

antenna can be modeled over a perfect ground, and the result will be

the same as the whole antenna modeled in free space.  (Note, however,

that the reported gain will be 3 dB higher, since all the antenna's

power is concentrated in one hemisphere.)  A simple example of this

principle is a quarter-wavelength vertical over a ground plane, which

has the same pattern as a free-space dipole.

Segment Tapering for Wires Joining at an Angle

Wires joining at an angle must have shorter segments than single

straight wires or wires joining in a line.  This is because of the way

MININEC handles the currents at wire junctions.  The straightforward

solution in this situation is to increase the number of segments.

However, doing so increases computation time and may require more than

the permitted maximum of 127 total pulses.  The technique described

here provides high accuracy with a smaller total number of pulses or

segments.

Instead of making the entire wires out of short segments, the segments

can be made short near the junction, tapering to a longer length away

from the junction.  ELNEC automates this process but it's useful to

know how the procedure works so you can optimize it for your

particular purpose.  The basic procedure is to replace the original

wire with several wires of different lengths.  The new wire closest to

the junction is made very short and with one segment.  The second wire

is made twice the length of the first, also with one segment.  This

process is continued until the segment length becomes long enough

(say, 1/20 wavelength), and the remainder of the original wire is made

up of a multiple-segment wire of approximately this segment length.

In the automated process, you can choose the minimum and maximum

26

segment lengths or use the default values of 1/400 and 1/20

wavelength.  More information can be found in the REFERENCE MANUAL,

under "The Wires Menu".

Sources and Loads at Junctions of More Than Two Wires

If you want to place a source or load on a wire end which is connected

to more than one other wire, the wire containing the source or load

can't have the lowest number of the group.  For example, if you have

five wires arranged as a ground plane antenna with four radials and

want to place a source at the base of the vertical wire, the vertical

wire can't be wire number 1.  This is due to the way the basic MININEC

code assigns "pulse" numbers.  Unlike MININEC or other MININEC-based

programs, ELNEC will warn you if it's unable to place a source or load

where requested due to this cause.  Since there's no way within the

structure of MININEC or ELNEC to place a source or load at the end of

the first of a group of wires connected to a common point, the only

solution is to renumber the wire (see "The Wires Menu" in the

REFERENCE MANUAL chapter) or avoid making the wire containing the

source/load the lowest numbered of the group connected to the common

junction.  (No problem exists if only two wires are connected

together.)

Scaling

The antenna can be scaled for another frequency if some cautions are

observed.  The method is to go to the Main Menu, choose Wavelengths

for UNITS, and change the frequency as desired.  When the frequency is

chosen, ELNEC will ask whether you want the antenna to stay the same

in terms of meters or wavelengths.  Choose (w)avelengths.  This will

scale the antenna so that the wire end coordinates (which includes

scaling of height above ground) and media height and boundaries remain

the same in terms of wavelength at the new frequency.  The wire

diameter and radial wire diameter will be scaled ONLY IF THE DIAMETERS

AREN'T SPECIFIED AS WIRE GAUGE.  Wire and ground conductivity aren't

scaled by this process; see "Conductivity and Scaling", below.

Conductivity and Scaling

If you're modeling your antenna over "real" ground at other than the

actual frequency of use, the ground conductivity and wire loss (as

well as wire and radial length and diameter) must be scaled for

accurate results.  The conductivity of the ground or wire should be

changed in direct proportion to the frequency.  For example, if the

actual antenna operates at a frequency of 7 MHz over ground with .001

S/m conductivity, it can be accurately modeled at 299.8 MHz if the

ground conductivity is changed to 299.8 / 7 * .001 S/m.  Dielectric

constant should not be scaled.  Failure to correctly scale ground

conductivity generally will be evident only at very low angles.

27

Using Templates

It's usually faster to make modifications to an existing antenna than

to describe one from scratch.  Save some typical examples of the types

of antennas you frequently model and recall them as starting points

when you want to design a new antenna of the same general type.

Modeling Complex Structures

One project I undertook was the modeling of a group of towers, each of

which had a top hat.  First, I constructed a fairly elaborate model of

a single tower.  Next, I modeled a single, large diameter wire of the

same height, adjusting the diameter to obtain the same impedance as

the tower (the height needed only slight adjustment).  This simpler

tower model was used for the remaining steps.  A fairly elaborate top

hat model was placed on the simplified tower and analyzed.  Then the

top hat was simplified as much as possible while retaining

approximately the correct impedance at the tower feedpoint (a good

example of the usefulness of ELNEC's Delete Wires function).  Finally,

the group of towers was modeled, using the simplified models.  This

approach can be taken for a variety of complex structures.

28

WHY NOT DBD?

It makes sense to compare an antenna's gain with the gain of a known,

real antenna.  After all, the "isotropic radiator" doesn't exist.  A

dipole is a common and convenient reference.  So why not use the gain

of a dipole as a reference?  When a dipole is used as a reference, the

gain is measured in "dBd", or decibels gain relative to a dipole.

(Similarly, the universally accepted standard "dBi" is decibels gain

relative to an isotropic source -- one which truly radiates equally

[poorly] in all directions).  To use either term, a reference field

must be established.  That is, we need to know the field strength that

an isotropic source or dipole would produce for the same power input

as the antenna we're comparing.  The isotropic antenna has the great

advantage of being theoretical -- since it doesn't exist anyway, we

can precisely define its field strength under theoretical conditions.

This makes it constant and not subject to ground, orientation, or any

real considerations.  But what's the gain of a dipole?  Well, the gain

of an infinitely thin, half-wavelength dipole in free space is 2.15

dBi.  But that's no more "real" an antenna than the isotropic

radiator!  The danger in using 2.15 dBi as 0 "dBd" is that it's easy

to get the impression that it represents the gain of A REAL DIPOLE.

It does not!  To see just how bad an error this represents, model YOUR

reference antenna using ELNEC and see what the gain actually is.  The

gain of the "back yard dipole" modeled in TEST DRIVE has a gain of 6.8

dBi (or +4.65 "dBd") -- just by putting the dipole in the back yard,

we've picked up more than 4 dB gain "relative to a dipole"!  (One of

the factors increasing the gain of a dipole over ground is that all

its power is concentrated in one hemisphere -- above ground -- while

the isotropic radiator and free-space dipole spread power over both.)

For these reasons, ELNEC uses the universally accepted and unambiguous

standard: dBi.  ELNEC permits you to enter any other reference of your

choosing, but it's highly recommended that you use it for comparison

between models of REAL antennas.  Decide what reference antenna to use

and model it.  Find the gain at the elevation and azimuth angles of

interest (Who cares what the maximum gain of an antenna is if the

maximum is straight up?) and use that as a reference to compare other

antennas against.

29

E L N E C

R U N N I N G    E L N E C

30

STARTING ELNEC

SYSTEMS WITH A SINGLE DISK DRIVE:

Boot the computer with a disk containing DOS.  Remove the DOS disk and

insert the ELNEC Program/Antenna File Disk.  If you have a Hercules

graphics system, type 'MSHERC' (followed by a carriage return).  Type

'ELNEC' to start the program.

SYSTEMS WITH A DUAL DISK DRIVE:

If you've chosen to keep the antenna files on the same diskette as the

program files, follow the procedure for the single-drive system.  If

the antenna files are on a separate diskette, boot the computer with a

DOS disk.  Remove the DOS disk and insert the ELNEC Program Disk in

one drive and the ELNEC Antenna File Disk in the other.  The Antenna

File Disk MUST be inserted before starting the program.  If you have a

Hercules graphics system, type 'MSHERC' (followed by a carriage

return).  Type 'ELNEC' to start the program.

SYSTEMS WITH A HARD DISK:

Make the ELNEC subdirectory the current directory by typing 'CD

ELNEC'.  If you have a Hercules graphics system, type 'MSHERC'

(followed by a carriage return).  Type 'ELNEC' to start the program.

It is possible to start ELNEC from a different directory than the one

containing ELNEC.EXE.  However, attention must be paid to the location

of certain files, so this isn't recommended unless you're willing to

give the necessary attention to the file location.  Note that "current

directory" means the directory you were in when you started ELNEC; not

necessarily the directory containing ELNEC.EXE.  ELNEC looks for the

configuration file ELNEC.CFG in the current directory.  Also, ENSETUP

saves ELNEC.CFG in the current directory.  So you should always run

ENSETUP and ELNEC from the same directory.  If you are using MaxP,

MAXP.EXE also must be in the current directory, and EN.BAT must be

modified (see the MaxP manual).  These are the only restrictions which

apply.

STARTING ELNEC IN TRACEVIEW MODE

If you've saved traces and want to take another look at them, print

them, or compare them without doing a new far-field calculation, you

can use the TraceView mode.  This is done by typing 'ELNEC TV' at the

DOS prompt.  More information about TraceView is in the REFERENCE

MANUAL chapter.

31

TEST DRIVE

The best way to get familiar with ELNEC is to take it for a spin.

Let's analyze a 20-meter dipole hung 30 feet up in the back yard.  If

you've started ELNEC, you should see the Main Menu.  The file

DIPOLE1.EN is included on the disk and should be installed at the

proper location, so let's start with that antenna and modify it as

necessary.

(Note: <RET> means carriage return or Enter, <ESC> is the Escape key.

You may type any entry in uppercase, lowercase, or any combination.)

--------- Along the Straightaway ----------

From the Main Menu, type 'RE'.  You don't have to follow it with

<RET>.

You should see a list of all the antenna files in the default

directory (there should be several).  DIPOLE1.EN should be on the

list.  If not, the antenna (.EN) files are not installed where ELNEC

can find them (ELNEC is looking for them in the directory named near

the top of the screen).  See INSTALLATION and RUNNING ENSETUP if

DIPOLE1 doesn't appear.  Assuming DIPOLE1 is on the list,

Type 'DIPOLE1' <RET>

The TITLE shown near the top of the Main Menu should now read "Dipole

in free space" -- this is the title of the antenna description stored

in file DIPOLE1.  Let's enter a title for our back yard dipole.  As

you type, notice the cursor near the lower right corner of the screen.

The first letter you type will appear in this area.  When you type the

second, ELNEC will begin the desired action, terminate the entry, and

erase both letters.  If you type an unrecognized combination, the

program will ignore and erase them so you can start over.

Type 'TI'

This brings up an entry area near the bottom of the screen.

Type 'Back yard dipole' <RET>

Now the new title is entered and appears near the top of the menu.

Let's change the frequency to twenty meters --

Type 'FR', then '14' <RET>

You've now entered 14 MHz as the frequency.  Let's choose feet for a

convenient unit of measure.

Type 'UN', then 'F'

32

to select feet.  Presuming you don't have a perfect ground plane in

your back yard (and for many wavelengths in all directions), you'll

want to do the analysis over "real" ground.

Type 'GT', then 'R'

At this point you'll get a warning that the antenna is lying on or in

the ground plane.  This sort of warning is typical of ELNEC; they

usually tell you quite specifically what the problem is.  As you'll

see in a moment, the dipole we're starting with has z-coordinates of

zero.  This is fine for free space, but isn't ok now that we've

specified a ground plane since the ground plane is at z = 0.

Press any key

to clear the error message and return to the Main Menu.  (ELNEC

returns you to the Main Menu in case you want to solve the problem by

changing the ground type.  In this case, you don't.)

Now we're ready to describe the antenna itself.

Type 'WI'

which takes us to the Wires Menu.  Since we're starting from a dipole

description, the menu already shows one wire.  We'll modify it to suit

our circumstances.

Type '1'

Note the highlighted area which appeared at the wire 1, end 1

coordinate area, and the prompt just below the wire description.

Assuming our back yard dipole was designed using 468/f(MHz) to

determine the length, the length is 33.43 feet.  ELNEC doesn't require

any symmetry, so for convenience we'll put one end of the wire at x,y

= 0,0 and the other at x,y = 0,33.43.  Placing it along the y axis

makes the maximum lobes at zero and 180 degrees, in the direction of

the x axis.  Ground is always defined as being at z = 0 (for the

innermost medium).  Since the antenna is horizontal and up 30 feet,

the z coordinate of both ends is 30.  To enter the coordinates,

Type '0,0,30', then press the right cursor arrow ->

Pressing the arrow enters the value and moves the highlighted area, as

in spreadsheet entry.  Separate <RET> and -> keystrokes also could be

used.

Type '0,33.43,30', then press the right cursor arrow ->

Both end coordinates are now entered.  The next prompt is for wire

diameter (in inches or wire gauge).  Supposing that the antenna is

made from #12 wire, just

Type '#12' <RET>

33

and the correct diameter is entered into the program.  We could have

entered the diameter in inches if desired.  The last item on the line

is the number of segments.  10 is a reasonable number for pattern

analysis of a half-wave antenna, so we don't need to change it.

Type <ESC>

to leave the wire description area.  Before we leave the Wires Menu,

let's see what the antenna looks like

Type 'V'

and the screen changes to show a three-view display of the antenna.

When finished viewing the antenna,

Press any key

to exit the View Antenna screen and return to the Wires Menu.

Type <ESC>

to leave the Wires Menu and return to the Main Menu.  Now let's put a

source at the center of the dipole.

Type 'SO'

and note that we've gone to the Sources Menu.  The specified and

actual positions of source 1 are on wire 1, 50% of the way from end 1.

This is where we want it.  The amplitude, phase, and type of source

won't have any effect on a single-source antenna (as long as the

amplitude isn't zero), so there's no need to change them.

Type <ESC>

to return to the Main Menu.  Menu selection LO says that no loads are

specified.  Since our dipole doesn't have any networks inserted in it,

zero loads is what we want.  Now let's look at the ground description.

Type 'GD'

and note that we've gone to the Media Menu.  One medium is shown, with

values corresponding to average soil.  Suppose that our soil is very

good.

Type '1', then 'VG', <RET>

and note that the conductivity and dielectric constant have been

changed to appropriate values for very good ground.  To end the entry

and leave the menu,

Type <ESC>, <ESC>

We know that the dipole's maximum lobe will be at zero degrees, but at

what angle above the horizon will it be maximum?  We'll run an

34

elevation plot to find out.  Selection PT shows that an azimuth plot

is the current choice, so

Type 'PT'

Note that the plot type has changed to Elevation.  The azimuth angle

for the plot is zero degrees, which is where we'd like to look, and

all the other parameters look fine.  To plot the pattern,

Type <RET>

When the plot is finished, note the choices in the upper right corner

of the screen.

Type 'A'

to run an analysis of the plot.  The values appear on the screen, and

you can see the points on the pattern which ANALYZE found.  This

assures you that ANALYZE is measuring what you think it's measuring.

If desired, you can print the annotated plot at this point by typing

'P'.  This concludes the drive down the straightaway.  If you'd like

to try your hand at a little more complex maneuvering, try taking

ELNEC. . .

---------- Through the Curves ----------

If you haven't yet done so, take ELNEC "along the straightaway" above.

In this section you'll begin with the plot generated by the

"straightaway" drive and get introduced to a few of ELNEC's more

advanced features.  Several shortcuts are used in this section.

Remember that they don't have to be used -- when you're using the

program you can ignore them until you're more familiar with ordinary

entry methods.  And when you are ready to use them, there's usually a

prompt on the screen to remind you how.  Ready to go?  Let's see how

an inverted vee compares with our back yard dipole.  First we'll save

the dipole trace for future reference.  With the "straightaway" plot

on the screen,

Type 'S'

to save the trace.  Enter a file name for the saved trace:

Type 'BYDIPOLE' <RET>

then

Press any key

to return to the Main Menu.

Type 'WI'

35

to go to the Wires Menu.  An inverted vee can't be made from just one

wire since it's bent in the middle and all wires must be straight.  So

we'll have to add another wire and use each of the two wires for half

of the inverted vee.

Type 'A', then '1', <RET>

to add a wire.  Then

Type '1'

to select wire 1.  Let's put the center of the antenna at 0,0,30 (30

feet straight up from the origin).  Note that end 1 of wire 1 is

already at this point, so

Press the right cursor arrow ->

to move to the other end of the wire.  At this point, with other

modeling programs you would have to do some trigonometry or carefully

draw the inverted vee on graph paper to determine the coordinates of

the end.  But not with ELNEC.  We'll start with a dipole and use

ELNEC's Rotate feature to make it into an inverted vee.  To make the

inverted vee the same length as the back yard dipole, each wire needs

to be 16.715 feet long.  So

Type ',16.715,' (note the two commas), then press the right cursor

arrow ->.

Since the x and z coordinates were already what we wanted, we can use

an entry shortcut.  You only have to enter the coordinate(s) you want

to change, as you just did.  The coordinates of end 2 of wire 1 should

now be 0,16.715,30.  Since the wires are half as long as before, half

the number of segments should be adequate, so

Press the right cursor arrow -> again, then Type '5'.

Press the right cursor arrow ->, then the down cursor arrow

to highlight end 1 of wire 2.  Here's another entry shortcut: To

connect this end to end 1 of wire 1,

Type 'W1E1', then press the right cursor arrow ->

and note that the coordinates of wire 1 end 1 have been duplicated for

wire 2 end 1, and that the end 1 Conn column shows the connection.

For wire 2 end 2

Type ',-16.715,30' (don't forget the comma).

Press the right cursor arrow -> twice, then Type '5', <RET>

to select 5 segments for wire 2.  ELNEC automatically makes the

diameter of added wires the same as that of the wire just before the

36

new ones, so this doesn't have to be changed.  Let's take a look at

our antenna so far.

Type <ESC>, then 'V'

to view the antenna.  It should look like a straight wire.  On a color

display it will be apparent that it's actually two wires.

Press any key

to return to the Wires Menu.  Now let's bend the wires down.  The

Rotate and Length change features follow the rule that only the

selected end of the wire changes -- the other end stays put.  Since we

want the center of the antenna to stay put, this means we need to

operate on end 2 of both wires.

Type '1', then press the right cursor arrow ->

to highlight end 2 of wire 1.  Then

Type 'RE-45', <RET>.

This tells ELNEC to Rotate the wire 45 degrees in Elevation; the minus

sign means downward.  Note the changed end 2 coordinates.  To see what

happened,

Type <ESC>, then 'V'

to view the antenna.  In the right side view you can see that the wire

has been bent downward.  Return to the Wires Menu and bend the other

one down:

Press any key, then Type '2', press the right cursor arrow ->, then

Type 'RE-45', <RET>

to rotate the other wire downward.  Let's take a look:

Type <ESC>, then 'V'.

You should see an inverted vee.

Type <ESC>, <ESC>

to return to the Main Menu.  Remember that we had one source in the

center of wire 1 for our dipole.  We need to move it to end 1 of wire

1 or wire 2 to put it in the center of the inverted vee.

Type 'SO', '1', '1,0', <RET>

to put the source at end 1 of wire 1.  You also could have specified

'2,0' for 0% from end 1 of wire 2.  Or you could have entered 'W1E1'

just like you do for wire end connections -- ELNEC will recognize that

format, too.

37

Type <ESC>, <ESC>

to return to the Main Menu, then

Type <RET>

to generate the plot.  When the plot is finished, notice that the gain

is a bit lower than for the dipole.  But how do the patterns compare?

To find out,

Type 'R', then 'BYDIPOLE', <RET>

to recall the dipole trace and superimpose it on the inverted vee.  If

you wish, you can return to the Main Menu and choose a different plot

outer ring value (say, 6.8 dBi) to scale both traces together.  It can

be shown that the center of current for a sinusoidal distribution is

1/3 of the way from the current loop.  This means that the effective

radiation strength from the inner 1/3 of the inverted vee equals that

from the outer 2/3 (since the current is heavier toward the center).

If we raise the inverted vee by 3.94 feet, it will place this current

center at 30 feet, which was the height of the dipole's center of

current.  Let's try it and see what happens:

Press any key, then Type 'WI'

to return to the Main Menu and go to the Wires Menu.  To raise the

antenna 3.94 feet, just

Type 'H' then '3.94', <RET>

and all z coordinates are increased by 3.94 feet.

Type <ESC>, <RET>

to return to the Main Menu and generate the new plot.  When the plot

is finished, it can be compared with the dipole trace:

Type 'R', then 'BYDIPOLE', <RET>.

The higher inverted vee is closer to the dipole pattern but still has

slightly lower gain.  This shouldn't be surprising if you investigate

the patterns in more detail.  The inverted vee has more radiation off

the end, reducing the gain from the side.  But as you can see, the

gain difference between the dipole and inverted vee is only an

insignificant 0.5 dB (providing the centers of current are at the same

height).

Now you've taken ELNEC for a good run.  Go ahead and begin

experimenting with your own antenna creations.  The REFERENCE MANUAL

chapter contains complete information about each menu and its

features.

38

E L N E C

R E F E R E N C E    M A N U A L

39

GENERAL INFORMATION AND CONVENTIONS

No distinction is made between uppercase and lowercase letters.

Scientific notation may be used for any numerical input if desired

(e.g., '7.15E6' = 7,150,000; '4E-3' = 0.004; '.123E2' = 12.3).

Because of size restrictions, a limited number of significant digits

can be shown in some menus.  If fewer significant digits show than you

had entered, the program is still using the full-precision number (up

to seven significant digits) for calculations; the shortening only

affects the display.

A carriage return isn't necessary following entries requiring a single

keystroke.  When the number of wires, sources, loads, or media exceeds

9, the program requires a carriage return following entry of the

number; if the total number is 9 or fewer, it doesn't.  Keystrokes

made while ELNEC is busy printing, calculating, or plotting are not

remembered.  This is intentional to prevent you from having, say, a

plot which disappears as soon as it's complete due to an accidental

keystroke entered during the calculating/plotting process.

In the Wires, Sources, Loads, and Media Menus, the item highlighted

can be changed by using the arrow cursor keys and the PgUp and PgDn

keys.  If the number of items exceeds the number which can be shown on

screen at one time, these same keys scroll the display.  After

entering the desired value, the entry can be terminated using any of

the above keys or the carriage return.  <ESC> also will terminate the

entry but the value you entered won't be saved.

During the impedance and current calculations, text and a graph are

shown which give the approximate percentage complete and time

remaining.  These are only approximations, intended to give you a

rough idea of the time required to perform the calculations.  The

erratic impedance calculation speed is normal, as ELNEC calculates

some values but recognizes others as being identical to previously

calculated points and skips them.

It's helpful to envision the antenna as you see it in the elevation

plots: the +x axis is to the right, the +y axis is into the screen,

and the +z axis is up.

+z           +y (into the screen)

|          /

|        /

|      /

|    /

|  /

-x _____________|/_____________ +x

In azimuth plots you're viewing from the top of the antenna, so +x is

to the right, +y is up, and +z is out of the screen.

40

Azimuth angles are measured in the conventional mathematical sense --

counterclockwise from the +x axis: the +x axis is zero degrees, the +y

axis 90 degrees azimuth.  Elevation angles are measured upward from

the horizon: the +x axis is zero degrees, the +z axis 90 degrees

elevation.

THE MENUS

The Main Menu

The Main Menu is the gateway for all program operations.  From it, you

can change the antenna description and how the plot is done, save or

recall antenna descriptions, or do any other program function.  Note

the cursor near the lower right corner of the screen.  The first

letter you type will be shown.  The second letter will terminate the

entry, begin the desired action, and erase the letters in the corner.

If you type an unrecognized combination, the program will erase them

so you can start over.  Following is a description of each selection

and its operation.

TI -- TITLE

The title can contain up to 30 characters and may be any combination

of letters, numbers, symbols, punctuation, and spaces.  The title

appears at the top of printed outputs.

FR -- FREQUENCY

Frequency is entered in MHz.  The corresponding wavelength in current

units is shown beside the frequency.  If you enter '0', the frequency

will be set to 299.7925 MHz, the frequency at which the wavelength is

one meter.  At this frequency, dimensions and coordinates entered in

meters will also be in wavelengths.  If you change the frequency when

units are wavelengths, you can choose to have the antenna stay the

same physical size (and change size in wavelengths) by selecting 'm'

or to stay the same size in wavelengths (and change physical size) by

selecting 'w'.  If the latter choice is made, all wavelength-

dimensioned items are changed:  wire end coordinates; wire diameter if

not specified as wire gauge; media heights; media boundaries; and

radial wire diameter if not specified as wire gauge.  This can be used

with some caution to scale an antenna for another frequency -- see

"Scaling" under TIPS in the MODELING WITH ELNEC chapter.

WI -- WIRES

Making this selection will bring up the Wires Menu, described below.

SO -- SOURCES

Making this selection will bring up the Sources Menu, described below.

41

LO -- LOADS

Making this selection will bring up the Loads Menu, described below.

GT -- GND TYPE

Choose this to select perfect ground, real ground, or free space.

GD -- REAL GND DESCRIPTION

If real ground has been selected (GND TYPE, above), this selection

will bring up the Media Menu, described below.  If the ground type is

perfect ground or free space, this selection won't appear.

WL -- WIRE LOSS

This selection permits you to model the effect of wire loss, an

exclusive ELNEC feature.  ELNEC automatically calculates the skin

effect loss at the chosen frequency and inserts it into the model.

You can choose the built-in resistivities of aluminum alloy or copper,

or enter the resistivity and relative permeability of any other

material.  (Note: Pure aluminum is seldom encountered in antenna work,

and the resistivity of alloys varies considerably.  The built-in value

is for 6061-T6 alloy, commonly used for antennas.)  If the antenna is

made of more than one material, choose the material of the smallest-

diameter conductors or those carrying the most current.  With plated

wires enter the parameters of the outermost material.  Wire loss can

have a major impact on the performance of small or short antennas.

Always do an analysis including wire loss if a lossless antenna has

gain that seems too good to be true (because it probably is!)

UN -- UNITS

You can choose feet, inches, meters, millimeters, or wavelengths as

the units of measurement.  Note: When feet are chosen, wire and radial

diameter are in inches; when meters are chosen, wire and radial

diameter are in millimeters.  Headings and prompts tell you this fact

but it's easy to overlook.

PT -- PLOT TYPE

Entering 'PT' will toggle between Elevation and Azimuth plots.

PA -- AZIMUTH or ELEVATION ANGLE

This chooses the angle for the plot.  For example, to see what the

azimuth pattern of your array looks like at an angle 20 degrees above

the horizon, choose AZIMUTH with selection PT, and 20 for selection

PA.  See "Patterns" under INTERPRETING THE RESULTS in the MODELING

WITH ELNEC chapter for a more detailed description of the meaning of

the plots.  If you switch back and forth between Azimuth and Elevation

plots, the angle for each is remembered and recalled when you return.

42

PR -- PLOT/TABLE RANGE

To choose a plot/table range of 360 degrees (azimuth plots) or 180

degrees (elevation plots), enter 'F' (for Full range) when prompted.

Or you can save some calculation time by restricting the angles to a

region of interest.  Step size (selection SS) must always be positive

but you can obtain any portion of the plot desired by putting the

upper and lower limits in the correct order.  For example (assuming

step size = 5), lower limit = 10 and upper limit = 190 will plot 10,

15, 20, . . .,185, 190 degrees.  Lower limit = 190 and upper limit =

10 will plot 190, 195, 200,. . .355, 0, 5, 10 degrees.

SS -- STEP SIZE

This determines the spacing between points on the plot or table.

Since the far field (pattern) is calculated for each point, far field

calculation and plotting time will vary with step size.  5 degrees is

a recommended step size for general purpose plotting, with 2 degrees

if the pattern has sharp lobes.  Step size must be positive.

OR -- OUTER RING OF PLOT

If you enter 'A' for automatic scaling, the dBi value of the outer

ring of the plot will be chosen automatically.  The automatically-

chosen value will be either an integer or the value of the pattern

maximum, depending on choice OA in the Options Menu.  The outer ring

value can be fixed at any other value of your choice if desired.

FI -- FIELD(S) TO PLOT

If the plot is too cluttered with total, horizontal polarization, and

vertical polarization traces, you can force it to plot the total field

only.  In HF skip communications the polarization is randomized by the

ionosphere so the total pattern generally is of most interest for HF

antennas.  However, you might be interested in looking at the

horizontal and vertical polarization patterns to appreciate the amount

of fading which can occur due to the changing polarization.

AR -- ANAL RES

The ANALYZE features can be done with greater precision than the plot,

if desired.  For example, if ANAL RES is set to 1 degree, the

direction of maximum gain, sidelobe angles, 3-dB points, etc., will be

determined within 1 degree even if the step size is larger.  Making

the ANALYZE resolution smaller will slow ANALYZE calculation but not

any others.  If ANAL RES is greater than the plot step size (selection

SS), ANALYZE calculations will be done at a resolution equal to the

step size.  When in the Trace View mode, ANAL RES is fixed at the step

size and can't be changed.

43

RF -- REFERENCE

If zero is chosen all plots, tables, etc. will be in dBi.  The

REFERENCE value is useful if you want to use some other standard of

comparison such as a reference antenna.

SZ -- SWR Z0

Source data includes SWR for each source.  This is the SWR which would

appear if the source were connected to the antenna through a

transmission line of a specific impedance.  SWR values for a line

impedance of 50 ohms is always given, along with a second impedance

you can choose with this selection.  This is useful if you want to

feed an antenna with a line of other than 50 ohm impedance and want to

know the SWR which will result.

--------------------------------------

(DE)lete, (RE)call, (SA)ve description

These are used to delete, recall, and save antenna descriptions.  You

can specify any name and path/directory to save or recall the files.

(There are restrictions on the extension.)  If not specified, ELNEC

will assume the directory specified by ENSETUP or the Options Menu,

and the extension ".EN".  If desired, calculated arrays can be saved

and recalled along with the descriptions; see SAVING AND RECALLING

ANTENNA DESCRIPTIONS, below.

<RET> = Plot

A carriage return (Enter) generates a far-field plot (pattern).  If

your computer doesn't have a graphics adapter, the heading will read

"<RET> = Table" and the data will be presented in tabular form.

(AN)alyze

ANALYZE calculates and shows forward gain, angle of forward gain,

front/back or front/side ratio, beamwidth, 3-db beam angles, major

sidelobe level, major sidelobe angle, and front/sidelobe ratio.  These

are presented in tabular form.  ANALYZE cannot be done before a plot

or table has been done; if you try, a message will appear.  ANALYZE

also can be run while the plot is on the screen by typing 'A'

according to the prompt at the upper right corner of the plot (see The

Plot Menu, below).  If this is done, ANALYZE results will appear with

the plot, along with lines on the plot which show you exactly what

ANALYZE found.  ANALYZE regards the second-largest lobe to be the

"sidelobe" for calculation purposes.  If the front/back ratio is unity

(zero dB), ANALYZE will calculate the gain at an angle of 90 degrees

from the maximum direction and report the front/side ratio instead.

44

(CU)rrents

Prints (to the screen or printer) a list of currents at each segment

junction.  This is useful to observe the current distribution on an

antenna and to check for problems with the description or those caused

by specifying too few segments (see INTERPRETING THE RESULTS in the

MODELING WITH ELNEC chapter).

(Load D)ata

Prints (to the screen, printer, or file) the impedance, voltage,

current, and power consumption of each load.  This can be very useful

and enlightening when analyzing trapped or loaded antennas (see

INTERPRETING THE RESULTS in the MODELING WITH ELNEC chapter).

(OP)tions

Making this selection will bring up the Options Menu, described below.

(Print D)esc

Prints the antenna description on the printer.  This is useful for

documenting results.

(Src D)ata

Prints (to the screen, printer, or file) a list of the voltage,

current, impedance, power, and SWR (relative to 50-ohm or user-defined

impedance systems) at each source.  Sometimes you'll see a negative

resistance and power.  This isn't a flaw in the program; it actually

happens.  When an array element shows a negative resistance, it means

there's net power flowing from the element into the feed system rather

than the other way around.  The element gets this power by means of

mutual coupling.  See INTERPRETING THE RESULTS in the MODELING WITH

ELNEC chapter.

(TA)ble

Prints (to the screen, printer, or file) the far-field pattern data in

tabular form.

(View A)ntenna

Graphically shows you what the antenna looks like.  The antenna is

presented in a standard three-view format.  Note that the "Front",

"Right Side", and "Top" views are relative to the coordinate system,

not the direction the antenna is pointed.  The "Front" view is looking

in the same direction as for the elevation plots, as shown by the

coordinate system diagram on p. 40.  When in the view screen, press

'P' to print the screen or any other key to return to the Main Menu.

This selection doesn't appear if a graphics adapter isn't present.

45

(EX)it program without saving desc

Normally, the current antenna description is saved in file LAST.EN

when the program is exited.  If this selection is made, the antenna

description won't be saved but the program will be allowed to

terminate even if the current description is defective.  This is

intended as a way of "bailing out" if you've made changes which

prevent a normal exit.  See (QU)it, below.

(QU)it

This is the normal way of exiting ELNEC.  When ELNEC is exited, the

current antenna description is saved in the LAST.EN file for use the

next time the program is run.  To prevent a defective description from

being saved, ELNEC might require you to correct certain problems

before permitting the program to end.  If this happens and you don't

want to correct the problems, use 'EX' to exit.

The Options Menu

The Options Menu is entered by making selection 'OP' at the Main Menu.

Choices in the Options Menu remain in effect until changed or until

you end ELNEC.  If desired, the changes can be made permanent.  The

choices are read from file ELNEC.CFG each time the program is started,

and the current choices are stored in ELNEC.CFG if you elect to make

them permanent.

GC -- DEFAULT GND CONSTANTS

These are the conductivity and dielectric constant which will be

assigned to newly added media (for example, when you change from

perfect ground or free space to real ground).  You probably will want

to assign the values of your local ground to this selection.

Representative values for common ground types are given under "The

Media Menu", below.

LF -- LAST FILE

You can choose to save all calculated arrays along with the LAST file

each time you exit the program.  The advantage is that if you've run

the program then ended it, the pattern and calculated arrays will be

available when you start the program again.  The disadvantages are

that a substantial amount of disk space may be required to save the

arrays (depending on the complexity of the antenna) and it will slow

down program termination.  "Save description only" is the recommended

choice for floppy disk based systems and ones with limited hard disk

space.

46

TU -- TABLE UNITS

You can choose to have tabular pattern data presented in terms of dBi

(or dBref), V/m at 1 mile (1 kW ref), or V/m at 1 km (1 kW ref).  When

either of the latter two is chosen, the antenna is assumed to have an

input power of 1 kW for determining table V/m values.

GS -- GRID STYLE

You can choose between the ARRL-style logarithmic-dB scale or a 40 dB

linear-dB scale for the far-field pattern plots.

OA -- OUTER RING AUTO

When automatic scaling of the plot outer ring is chosen, you can

choose to make it equal to the pattern maximum or the next larger

integer dB value.  The former makes it easier to read values from the

plot relative to the pattern maximum; the latter makes it easier to

determine dBi values.

PS -- PLOT STYLE

You can choose between "standard" and "plain" style plots.  The former

includes information about the plot (time, date, maximum gain, etc.)

and the latter doesn't.

AD -- ABBREV DESC UNDER PLOT

If "yes" is chosen, ELNEC will print an abbreviated antenna

description below the plot (on the printer, not on screen).  ANALYZE

information is included if ANALYZE has been run.

PQ -- PRINT QUAL

This selection appears only if you've specified a 24-pin printer with

ENSETUP.  "Draft" causes plot prints with 8-pin quality.  "High Res."

plot prints use all 24 pins for higher quality but slower prints.

EN -- .EN FILE PATH

This selection defines the drive and directory where antenna (.EN) and

trace (.ENT) files will be stored.  Although you may specify other

drives and directories when saving, recalling, and deleting files, the

drive and directory shown here are the defaults used when not

otherwise specified.  You may wish to organize your .EN/.ENT files in

several directories and use this selection to move from one to

another.

MA -- MAX P TEMP FILES PATH

This selection appears only if the Maximum Pulse Option is installed.

It is used to choose the location of the MaxP temporary files.  See

your MaxP manual for more information.

47

The Wires Menu

The Wires Menu is entered by making selection 'WI' at the Main Menu.

At the upper portion of the display is a summary of the wires:

connections, end coordinates, diameter, and number of segments.  Wire

end coordinates are defined as x, y, z coordinates relative to the

origin (+z is up, the +x axis -- 0 degrees azimuth -- is to the right,

and the +y axis -- 90 degrees azimuth -- is into the screen).  The

"Conn." column shows connections to other wires; if a wire end is

connected to more than one other wire, only one connection will show

in this column.  Since defining the wires usually is the most time-

consuming part of describing the antenna, ELNEC includes several

shortcuts to make the task faster and easier.  Remember that you don't

have to use the shortcuts -- they're there only for your convenience

if and when you want to use them.  The apparent complexity of this

menu is mostly due to the number of shortcuts ELNEC provides.

Entering or changing wire coordinates

Note that wires are considered to be straight, extending from one end

coordinate to the other.  If two wires have the same end coordinates

they're considered to be connected.  If a ground is specified and a

wire end has a z coordinate of zero, the end is considered to be

connected to ground.  Crossing wires does NOT connect them; they may

be connected only at the ends.  If a ground is specified, wire ends

may not have a negative z coordinate and the ends both can't have zero

z coordinates.

To enter or change wire coordinates, simply type the wire number.

This will highlight end 1 of the selected wire.  To move to end 2,

press the right cursor arrow key.  Wire coordinates can be entered in

two different ways.  Either may be used as desired:

1. Conventional.  Just enter the x, y, and z coordinates when

prompted, separating them with commas.  IF YOU NEED TO CHANGE ONLY

ONE OR TWO COORDINATES, IT'S NOT NECESSARY TO REENTER ALL THREE.

Simply leave unchanged coordinates blank.  For example, to change

coordinates 3.75,4.95,.713 to 3.85,4.95,.713 you only need to type

'3.85,,'.

2. As connections to other wires.  To connect a wire end to another

wire end, type 'W#E#' for "Wire number End number".  For example,

to connect an end to wire 3, end 1, type 'W3E1' at the prompt for

coordinates.  The correct coordinates will be entered and the

connection shown in the "Conn." column.

Once entered, the end coordinates can be modified in several ways.

When using the Length and Rotate features it's helpful to keep in mind

the following rule ELNEC uses:  Only the coordinates of the selected

end will be changed; the other end won't change.

1. Changing the length.  Once wire coordinates are entered, the

length can be changed as follows: Select the end to be changed,

then type 'L###' where "###" is the new wire length.  The

48

coordinates of the selected end will change to make the wire the

new length, in the same direction as before.  This is a powerful

feature for studying things like the effect of changing the length

of drooping radials.  To illustrate another use, consider an

inverted vee antenna guyed to the corners of a lot from a single

tower.  First create a long inverted vee extending from the tower

to the lot corners by entering the coordinates of the tower top and

lot corners for the wire ends.  Then change the lengths of the

wires to their real values.  This eliminates the tedious

trigonometric problem of figuring out what the actual antenna end

coordinates are.

2. Rotating the wire.  The wire can be rotated by entering 'RA###'

(Rotate Azimuth) or 'RE###' (Rotate Elevation) where "###" is the

desired amount of rotation in degrees.  The rotation amount can be

positive or negative, with positive meaning counterclockwise

azimuth rotation or upward elevation rotation.  The highlighted

wire end will move.  The other end will be the center of rotation

and will stay put.  When performing elevation rotation you won't be

permitted to rotate a wire closer than about 1 degree to vertical,

and you won't be able to rotate a vertical wire in elevation.  This

is because ELNEC keeps the same azimuth direction for an elevation-

rotated wire, and it can't tell which direction to move a vertical

wire.  There's no similar restriction on azimuth-rotated wires.

The rotation ability is a powerful feature for defining vee-type

antennas and delta loops.  You can begin by defining a straight

antenna then rotating the wires to the desired angle.  An inverted

vee example appears in TEST DRIVE in the RUNNING ELNEC chapter.  A

process similar to the TEST DRIVE example can be used to make a

delta loop: Make a dipole with two wires, each 1/3 wavelength long

(use wavelengths for units if desired).  Rotate the wire ends down

60 degrees, making an inverted vee as in the example.  Then add a

third wire, specifying end connections to the ends of the "inverted

vee".  Now want to adjust the height in feet?  Just go back to the

Main Menu and change the units, then to the Wires Menu to adjust.

No fuss, no muss, no trigonometry.

Entering or changing wire diameter

Note that the wire diameter might be in different units than the wire

end coordinates.  Watch the heading of the "Dia" column and the

prompt.  The diameter can be entered directly or as a wire gauge

(AWG).  To enter it as a wire gauge, type '#--' where "--" is the

gauge, at the prompt.  Wire gauges larger than #0 (e.g., #00) aren't

permitted.  To enter the same diameter or wire gauge in a group of

wires, select 'G' (Group dia.) from the Wires menu.  You will get

prompts asking the wire diameter or gauge and the range of wire

numbers to change.

49

Entering or changing the number of segments

Entering the number of segments is simple; deciding how many to enter

can be more difficult.  See the MODELING WITH ELNEC chapter for more

information.  The total number of segments (approximately equal to the

total number of pulses) is shown at the bottom of the Segs column.

Connecting a wire to ground

If the ground type is Real or Perfect, a wire end is connected to

ground by specifying a z-coordinate of zero.

Adding wires

To add wires, choose 'A'.  At the prompt, enter the number of wires

you want to add.  They will be added after the presently defined

wires.  If you want to add wires anywhere except at the end of the

list, enter two numbers separated by a comma.  The first number is the

number of wires to add; the second is the wire number to place them

after.  To place new wires at the very front of the list, enter '0'

for the second number.  Don't worry about sources and loads; they'll

stay on the same wires they were on before, even if the wires change

number.

Deleting wires

Choose 'D' to delete wires.  If you enter a single number at the

prompt, the wire with that number will be deleted.  If you enter two

numbers separated by a comma, the two wires and all between will be

deleted.  If there are any sources or loads on wires to be deleted,

ELNEC will notify you and ask for confirmation before making the

deletion.  Sources and loads on other wires will stay on the same

wires they were on before, even if the wires change number.

Renumbering a wire

Selection 'R' permits you to renumber a wire.  This is most useful if

you desire to put a source on the lowest numbered wire in a group of

wires connected to a common point.  See "Sources and loads at

junctions of more than two wires" under TIPS in the MODELING WITH

ELNEC chapter.

Changing antenna height

You can change the height of the entire antenna by selecting H.

Viewing the antenna

Type 'V' to see what the antenna looks like.  This operates

identically to the 'VA' selection in the Main Menu except that you'll

be required to delete or modify any zero-length wires before viewing

the antenna.

50

Changing units without changing numbers

This feature (selection 'C') was added because of a trap which can

happen.  You carefully enter all your antenna wire end coordinates,

then discover the units are still meters, not feet like you wanted.

If you return to the Main Menu and change the units, the units change

but the wires stay the same (incorrect) length as before.  Instead of

returning to the Main Menu, choose 'C' and select the new units.  The

units will change but the numbers you entered for end coordinates and

wire diameter won't.

Automated segment tapering

When wires intersect at an angle, particularly acute angles, a larger

number of segments is required for accurate results.  Increasing the

number of segments the required amount may lead to undesirably slow

program execution or may cause you to exceed the maximum allowable

number of pulses.  An alternative is to break the joining wires into

several one-segment wires of  different lengths, short near the

junction and increasing as distance from the junction increases, to

achieve high accuracy with a moderate total number of segments.  (See

TIPS in the MODELING WITH ELNEC chapter.)  ELNEC provides an automated

method of doing this tapering.  It's intended as an advanced feature

for use in special cases only after you're completely familiar with

ELNEC.  Note that the tapering process permanently changes the antenna

description so it's very wise to save the description before you

start.  (ELNEC will ask you if you want to do this before you begin.)

When several wires are to be tapered, you also may want to save

intermediate descriptions in case you make a mistake.

After typing 'T' and responding to the question about saving the

description, you're asked which wire to taper and at which end(s) to

put the short segments.  Respond with the wire number and the end(s)

connected to other wires at an angle.  If both ends are connected to

other wires at an angle (as, for example, in a delta loop), enter 'B'

(both) for the end number.  ELNEC will then ask you for the minimum

and maximum permissible segment length.  You can use the default

values of 1/400 and 1/20 wavelength by pressing <RET>, or specify

different values.  It will make the shortest segment equal to the

minimum you specify and will not exceed the maximum with any segment.

The lengths are in current units, or in wavelengths if you follow the

number with 'w'.  For example, '.002w,.02w' will taper lengths from

.002 wavelength to .02 wavelength.  Don't forget to add the "w" if you

intend to specify in wavelengths.  The program then will respond with

the number of wires and segments it will use to replace the wire.  If

this is ok, press 'y' or <RET> and the wire will be replaced; if not,

press 'n' and you'll be able to specify new minimum and maximum

segment lengths.  Doubling the minimum length will reduce the number

of wires and segments by one (per tapered end).  The result of

choosing a different maximum will depend on several factors and may

substantially change the resulting number of segments.  Adequate

accuracy might result with quite a large maximum segment size; you may

want to experiment to investigate this possibility.

51

The new wires will replace the old one.  After you have tapered the

first wire, notice that some wires are brightened, and appear in color

on a color monitor.  These are the new wires.  If you want to taper

the next original wire, select 'T' again, and give the number of the

first non-brightened wire (which is the next original wire), followed

by the end(s) to have the short segment when tapered.  If there are

more total wires than the screen can display at once, this wire will

be the bottom one on the screen.  Continue the process until you have

tapered all the wires you wish to.  The brightening/coloring of

tapered wires will persist until the antenna description is saved and

recalled or until ELNEC is exited and restarted.

Sources and loads which are at the end of a wire, or at the center of

a wire which is being tapered at both ends, automatically will be

moved to the correct locations on the new wires.  Sources or loads at

other locations may not end up where desired.  It would be a good idea

always to check the placement of all sources and loads after doing a

taper.

Wire Coordinate Errors

Wires must have a non-zero length.  Also, if a ground plane (either

real or perfect) is specified, the wires can't lie on the ground plane

(zero z coordinates for both wire ends), or in the ground plane

(negative z coordinate).  If any of these errors occurs, the affected

wires are shown as bright text and a description of the error appears

in the "Conn" column.  Added wires are given end coordinates of 0,0,0

so they fall into this category.  If you attempt to leave the Wires

Menu while zero-length wires are present, you'll get an error message

and be returned to the Wires Menu until the problem is corrected.  The

error message includes a prompt which permits you to delete all zero-

length wires.  You can choose to do this or return to the Wires Menu

and fix or delete the defective wires.  If wires are in or on the

ground plane, an error message will appear.  You'll be sent to the

Main Menu where you can correct the problem by specifying Free Space

as the ground type or by returning to the Wires Menu to raise the

wires.  You won't be permitted to run or normally exit ELNEC with this

problem present.

The Sources Menu

To add or delete sources type 'A' or 'D'.  To modify a source, type

the number of the source.  Sources are placed in series with the

specified wire.  Sources can't be placed at an open wire end.  No more

than one source may be placed at a given position on a wire.  At least

one non-zero source always must be specified.

52

Specifying the source position

With MININEC, this can be the most tedious part of the antenna

description.  Here's how easy it is with ELNEC: If you want to put a

source at the middle of wire # 1, enter '1,50' at the prompt (for

"wire 1, 50% of the way from end 1").  That's it.  The source will

stay there, even if you add or delete wires, or change lengths or

numbers of segments.  (Possible exception: it might move slightly to

stay on the "pulse" closest to the specified position.  This is

explained next.)  ELNEC does, however, require that sources be placed

on "pulses" -- junctions between segments.  So when you specify a

position, ELNEC will place it at the nearest pulse and tell you where

its actual position is.  The specified position is saved, and each

time a change is made to the wires the source is moved as close as

possible to that position.  Both the actual and specified positions

are shown in the menu.  If you want to place a source at a particular

pulse number, enter 'p#' where # is the desired pulse number.  This

was included in case you want to duplicate MININEC descriptions with

ELNEC.  However, it's NOT RECOMMENDED for general use since sources

placed this way will be treated just as MININEC treats them: they stay

on the same pulse number and may move from wire to wire or off all

wires as the antenna description is changed.

The source position also may be entered as 'w#e!' where # and ! are

the wire and end number, respectively, or 'w#c' to place at the center

of wire number #.  For example, 'w5e2' will put the source at end 2 of

wire 5, 'w1c' at the center of wire 1.

Source amplitude and phase

There are no restrictions on amplitude or phase except that if a

single source is specified, it must have nonzero amplitude.  If only

one source is specified, the amplitude and phase will have no effect

on the results (as long as the amplitude isn't zero).

Type

ELNEC features both voltage and current sources.  If only one source

is used the choice won't make any difference.  For arrays driven with

multiple sources, the choice can make a significant difference in

array performance, and the choice should reflect the desired driving

conditions.  Most phased arrays are designed to be driven with a

particular CURRENT ratio, so current sources should be chosen to

simulate a correctly-fed array (See THE ARRL ANTENNA BOOK, Fifteenth

or Sixteenth Edition, Chapter 8).

Adding and deleting sources

Addition and deletion are done the same as with wires.  Refer to "The

Wires Menu" above for information.

53

Connecting a source to ground

Sources appear in series with the wire, so a grounded source is

specified by inserting the source at a grounded wire end.  If you want

to place a source between ground and a wire end which isn't at zero

height, you'll first need to add a wire connecting the end to ground.

This is the same procedure you would have to follow to make the

connection to a real antenna.

The Loads Menu

Load placement, connection, addition, and deletion operates exactly

like corresponding source operations (with one exception, explained

below).  Refer to "The Sources Menu", above, for information.  Loads

are placed in series with the wire.  The one difference between source

and load addition is that if no loads are specified, a single load can

simultaneously be added and selected by typing '1' from the Loads

Menu.  This choice appears at the bottom of the screen when no loads

are specified.

Specifying or changing impedance

Impedances may be entered either as R + jX or as a quotient of Laplace

transform polynomials of up to fifth order.  All loads must be entered

as the same type; if the type is changed, all existing loads will be

deleted (after notifying you and receiving confirmation).  The type

which is in effect is indicated by the heading of the right-hand

column or columns.  The type is changed by typing 'T' at the Loads

Menu prompt.

Using Laplace transforms

The advantage of using Laplace-transformed load impedances is that the

impedances will be automatically adjusted as the frequency is changed.

Although a detailed discussion of the use of Laplace transforms is

much too complex for this document, ELNEC will "translate" simple

series and parallel RLC circuits for you as explained below.

When the Laplace entry type is in effect, the heading will indicate

this fact, and "Select to show values" will appear in place of the

load impedances.  When the highlighted box is moved to that area,

another menu will appear near the bottom of the screen, permitting

entry of the s coefficients.  At least one denominator coefficient

must be nonzero.  To select other loads while in the Laplace transform

submenu, use the PgUp and PgDn keys.  Press <ESC> to return to the

Loads Menu.

At the prompt "Coefficient, 'S', or 'P'", enter the coefficient for

the highlighted area, the letter 'S' (for series RLC circuit), or 'P'

for parallel RLC.  If you type 'S' or 'P' you'll get another prompt,

for the values of R in ohms, L in henrys, and C in farads.  Three

values must be entered in the order R,L,C, separated by commas.  Enter

0 to indicate a missing component.  For example, a series or parallel

54

combination of 200 pF and 3 uH would be entered as '0,3E-6,200E-12'.

(Note the entry order: R first, then L, then C.  In this example, R is

missing so is entered as 0.)  The corresponding Laplace coefficients

will be entered for you.

The Media Menu

This menu is available when "real" ground is selected.  The ground

model can consist of any number of "media".  These media are shaped as

concentric rings or parallel "slices".  If concentric rings are chosen

(radial boundary), the innermost ring is at z coordinate (height) of

zero and is centered at the origin.  If "slices" (linear boundary) are

chosen, the boundaries are straight and parallel to the y axis.  The

first medium boundary is at x = 0.  If radial boundaries are chosen,

radial wires can be inserted in the innermost medium.  Their length

extends to the outer edge of the innermost medium.  The media may be

placed at various heights to simulate sloping ground if desired.

NOTE: The ground description affects only the far-field pattern.  When

currents and impedances are calculated, a perfect ground approximation

is used.  Therefore, the impedance of grounded vertical and low

horizontal antennas won't be accurate.  This is a limitation of the

basic MININEC computational code.

Entering the conductivity and dielectric constant

The conductivity and dielectric constant of the ground vary with the

type and dampness of the soil.  Typical values for several types of

soil, as well as a map of typical average soil conductivities for the

U.S., are given on page 3-3 of THE ARRL ANTENNA BOOK, 15th or 16th Ed.

(ARRL) and in other handbooks.  Values from THE ARRL ANTENNA BOOK for

some common ground types are:

Ground Type                   Quality       Conductivity  Diel.Const.

(Siemens/m)

Fresh water                                   .001          80

Salt water                                   5              81

Pastoral, rich soil        very good          .0303         20

Pastoral, heavy clay soil   average           .005          13

Rocky, typ. mountainous      poor             .002          13

Cities, industrial areas   very poor          .001           5

Cities, hvy industrial  extremely poor        .001           3

As a shortcut, values for very good, average (good), poor, or very

poor soil can be entered by typing 'VG', 'A' or 'G', 'P', or 'VP' when

either the conductivity or dielectric constant is highlighted.  When a

ground is first specified or when new media are added, values of

conductivity and dielectric constant are assigned according to the

DEFAULT GND CONSTANTS choice in the Options Menu.  These values can be

changed at any time by entering the new value while the value to be

changed is highlighted.

55

Entering the R or X boundary coordinate

The boundary type can be determined by observing the heading of the

next-to-right-hand column: "R" indicates radial boundary and "X",

linear.  This can be changed by selecting 'B' from the Media Menu when

no media are highlighted.  The boundary coordinate is the coordinate

where the boundary begins; the last medium extends to infinity.  THE

VALUE OF THE COORDINATE OF EACH MEDIUM SHOULD BE GREATER THAN THE

BOUNDARY OF THE PRECEDING MEDIUM.  If it isn't, results may not be

accurate.  This is one of the few places where ELNEC will permit you

to enter values which may lead to incorrect results.  If a radial

boundary is chosen and radial wires are specified, the boundary of the

second medium (where the first medium ends and the second begins) is

also the end of the radial wires.

As an example, suppose you want to model a lake surrounded by a

"doughnut" of rich soil, which in turn is surrounded by average soil.

Perhaps the lake is 2000 feet across.  You'll have to model the lake

as perfectly circular, and three media must be specified to imitate

the lake and two ground conditions.  The inner boundary of the first

medium (the lake) is defined to be at x,y,z coordinates 0,0,0.  Make

sure the boundary is radial (see the above paragraph).  Specify the

conductivity and dielectric constant of the first medium as .001 S/m

and 80, respectively, to model the fresh water of the lake.  The inner

boundary of the second medium is the outer boundary of the lake, so

specify 1000 feet for the second medium boundary.  The conductivity

and dielectric constant for rich soil are about .03 S/m and 20, so

specify these for the second medium.  Finally, the third medium has an

inner boundary which corresponds to the interface between the rich and

average soil, and conductivity and dielectric constant of .005 S/m and

13 to model average soil.  The conductivity and dielectric constant of

the last (third) medium are presumed to exist outward from its inner

boundary.  Place your antenna anywhere you wish relative to the lake;

it doesn't need to be at the center of the coordinate system.

Entering the medium height

There are no restrictions on medium height except that THE GROUND

UNDER THE ANTENNA ALWAYS SHOULD HAVE A HEIGHT OF ZERO for correct

results.  ELNEC will not recognize any shielding effects from "hills",

"cliffs" or similar features.  Like MININEC, it looks for a straight-

line path to a medium surface to determine the strength of the ray

reflected from that surface, and ignores features between the antenna

and the reflection point and beyond the reflection point.

Adding and deleting media

These functions are exactly the same as the corresponding functions in

the Wires Menu.  See "The Wires Menu" for details.

56

Changing the boundary type

Typing 'B' from the Media Menu toggles the boundary type between

radial and linear.  The boundary type can be determined by observing

the next-to-left-hand column heading ("R Coord" for radial, "X Coord"

for linear).

Adding or changing radials

Selecting 'R' brings up a small display near the bottom of the screen

showing the number of radials and the diameter of the radial wires.

The radials are considered to extend to the outer edge of the

innermost medium; if only one medium is specified, the radials are

infinite in length.  Radials are used only to modify the ground

conductivity for far-field (pattern) calculation; they will have no

effect on ELNEC's indication of impedance or currents.  THEY CANNOT BE

USED TO EVALUATE THE EFFICIENCY OF A GROUND SYSTEM.  The calculation

is the same as that used by MININEC.  MININEC's documentation cautions

that the calculation method is accurate only for large numbers of

radials.  Note that the wire diameter might be in different units than

coordinates and boundaries.  If desired, the diameter may be entered

as wire gauge (AWG) by typing '#--' where "--" represents the wire

gauge.  Gauges of wires larger than #0 (i.e., #00) are not permitted.

When you are finished with radial entry, press <ESC> to return to the

Media Menu.

The Plot Menu

This small menu appears in the upper right corner of the screen after

the plot is completed.  Following are the selections and their

effects.

(A)nalyze and annotate plot

This runs ANALYZE, and the results (gain, beamwidth, front/back or

front/side ratio, etc.) are shown on the plot along with lines

indicating the directions of maximum gain, 3-dB points, and sidelobe.

See (AN)alyze under "The Main Menu", above, for more information.

(P)rint screen

Prints the plot on the printer.  If you've chosen to print an

abbreviated description under the plot (see "The Options Menu" above)

and ANALYZE hasn't been run you'll get an additional prompt:

"(A)nalyze & incl on print".  Typing 'A' at this prompt will cause

ANALYZE to run.  The results will be included in the abbreviated

description below the plot but not on the plot.

57

Trace: C,D,S,R

Typing 'C' (Clear trace) will have an effect only if more than one

trace is on the screen.  A list of recalled traces will appear and you

can choose to clear (remove) one or all of them.

'D' (Delete), 'S' (Save), and 'R' (Recall) operate on traces just like

the equivalent Main Menu selections do on antenna descriptions.  Trace

files are saved in the same subdirectory as the antenna description

(.EN) files but are given the extension ".ENT".  For compatibility

with earlier versions, files with any extension can be recalled or

deleted.  However, files must have the extension ".ENT" to be saved.

This extension will be added by default if you don't specify one.  See

the next section for the use of "wild cards" to list selected files.

SAVING AND RECALLING ANTENNA DESCRIPTIONS

When you recall an antenna description, you are shown an alphabetized

list of the description files in the directory specified by ENSETUP.

If there are accompanying array files (explained below), a distinctive

solid-triangular mark is shown following the description file name.

To choose a description to save, recall, or delete, type its name at

the prompt.  It's not necessary to type the path or directory for the

file names shown on the screen; the correct path will be added by

ELNEC.  Also, the extension ".EN" will be added to antenna description

files and ".ENT" to trace files if you don't specify an extension.

Antenna description files must have an ".EN" extension and trace files

an ".ENT" extension to be saved.  This can be manually entered or

added by ELNEC.  For compatibility to earlier versions, you may recall

or delete files with any extension.

A very convenient feature is the file list "wild card" capability.

"Wild cards" are the characters "*" and "?", which substitute for any

number of following characters and a single character, respectively.

If you specify a file name which includes a "wild card" character,

ELNEC will show all the files with that specification.  For example,

'vert*' will show all files in the .EN file directory beginning with

"VERT" and having an ".EN" extension , e.g., VERT.EN, VERTICAL.EN,

VERT1.EN, etc.  Specifying a "wild card" character when deleting files

will NOT delete all files with the given specification; they will only

be listed as described above.  You can see what .EN files there are in

another directory, say \OLDANTS, by typing '\oldants\*' at the prompt.

When you save an antenna description, ELNEC asks whether you also want

to save "calculated arrays".  When ELNEC runs a far-field analysis, it

calculates several sets, or arrays, of values.  The most important are

the impedance array, containing self and mutual impedances of all

pulses (segment junctions); the current array, containing the current

at each pulse; the voltage array, containing the voltage across each

segment junction; and the dB array, containing the field strength at

each specified azimuth or elevation angle.  If you change, for

example, the azimuth or elevation angle for the plot, only the dB

58

array needs to be recalculated; if you change sources or loads, only

the current, voltage, and dB arrays need recalculation.  ELNEC version

2 allows you to save these arrays if desired so future calculations

with the same antenna description won't unnecessarily have to be

repeated.  To do so, simply type 'y' at the prompt asking whether you

want to.  (If no arrays have been calculated, the prompt won't

appear.)  Before saving the arrays, ELNEC checks available disk space

and will refuse to save them if there's not enough.  The arrays are

saved as files in the same directory as the ".EN" (antenna

description) files, as specified by ENSETUP (see ELNEC FILES, below).

The largest files are the ones containing the impedance arrays.  There

are two of these, and they can be as large as 64k bytes each for a

127-pulse antenna.  (Note:  With the Maximum Pulse Option, there can

be several sets of these arrays, with a total size of greater than

500,000 bytes.)  The other arrays are relatively small.  If you

specify that you don't want to save arrays with an antenna description

and arrays previously have been saved with that description, THE

EXISTING ARRAY FILES WILL BE DELETED.

TRACEVIEW

Starting TraceView

The TraceView mode is a way to look at, compare, or print saved traces

without having to run a far-field analysis.  To use TraceView, start

ELNEC by typing 'ELNEC TV'.  The program will begin by showing you the

saved trace files.  You need to select one of these -- it will become

the "primary" trace.  (The primary trace plays the same role as the

calculated trace in ELNEC's normal mode.  ANALYZE is done on the

primary trace.  The primary trace also determines the value of the

plot outer ring if automatic scaling is chosen, and an abbreviated

description under the printed plot will contain information on the

primary trace.)  After you choose the primary trace, the Main Menu

will appear.  Note, however, that most of the selections aren't

available.  This is because ELNEC doesn't do any calculations or

antenna modifications in TraceView mode -- it's strictly for showing

existing traces.  <RET> will plot the primary trace.  You can print

the plot or recall additional traces just like you do in ELNEC's

normal mode.

ANALYZE and TraceView

If you wish, you also can ANALYZE the primary trace.  Note, however,

that the analysis resolution is fixed at the plot step size.  This is

because TraceView works only with the trace data and doesn't calculate

gain at points between those in the stored trace.

Changing the Primary Trace

You can select a new primary trace by selecting 'RT' (Recall Trace)

from the Main Menu.  This selection appears only in TraceView mode.

59

Ending TraceView

When you exit TraceView by selecting QU (QUit), the LAST file is NOT

altered.

ELNEC FILES

The only ELNEC files which are readable by using DOS commands TYPE or

PRINT, or utility programs such as PCTools or Norton Utilities, are

READ.ME, ANTNOTES.DOC, ELNEC.DOC, and outputs which can be "printed"

to a file (source, load, current, and pattern data).  All the others

will appear as unreadable ASCII characters.

Files on the Distribution Disk(s)

If your ELNEC program came on two 360k disks, one disk is labeled

MANUALS and contains the .DOC files, and the other disk contains the

remainder.  Otherwise, all files are on the single 720k or 1.22 Meg

disk.

ELNEC.EXE and ENSETUP.EXE: These are the main program and the setup

program.

.EN (antenna) files: These files contain antenna descriptions, and

may be saved, recalled, and deleted from the Main Menu.  Whenever you

save an antenna description, it is given the extension ".EN" and

stored in the subdirectory specified by ENSETUP unless you specify

otherwise when you save it.  (If ENSETUP hasn't been run, .EN files

are saved in the current subdirectory.)

LAST.EN: This antenna file contains the description of the last

antenna analyzed.  It's read when ELNEC is started, and the antenna

description present when ELNEC is quit is written into this file.

(Exception: If ELNEC is ended by selecting 'EX' from the Main Menu,

LAST.EN isn't changed.)

MSHERC.COM: This special driver must be loaded (run) before

starting ELNEC if your system has Hercules graphics.  Once loaded, it

doesn't need to be reloaded until the computer is rebooted.

ELNEC.DOC: This file.

ANTNOTES.DOC: Contains notes about the furnished example antenna

description (.EN) files.

READ.ME: Contains brief instructions on how to print this file.

60

Files Created by ELNEC and/or ENSETUP

ELNEC.CFG: This file is created by ENSETUP the first time it is

run, or by ELNEC if you choose MP (Make Permanent) from the Options

Menu.  It contains the printer type, path to the .EN files, date

format, screen colors, and Options Menu choices.  Each time ELNEC is

run, it looks for this file in the current directory, reads it, and

uses the values it finds.  If ELNEC.CFG isn't found, ELNEC uses

default values.  ELNEC.CFG is modified by running ENSETUP or by

choosing MP in the Options Menu.  See RUNNING ENSETUP in the GETTING

STARTED chapter and "The Options Menu" in this chapter for more

information.

.ENT (trace) files: These files contain trace information and are

saved and recalled from the Plot Menu.  They are stored in the same

subdirectory as the .EN files.

.ZR, .ZI, .V, .I, .DB files: These files contain arrays calculated

by ELNEC.  They are created if you choose to save calculated arrays

with an antenna description or with the LAST file.  They are stored in

the same subdirectory as the .EN files.

If the Maximum Pulse Option (MaxP) is being used, additional temporary

files are created.  See the MaxP manual for more information.

PRINTERS

Many of today's 8- or 9-pin dot-matrix printers can emulate an Epson

FX printer, and many 24-pin printers can emulate the Epson LQ series.

For laser printers, the HP LaserJet series has become a standard.

Drivers also are included for the Epson MX, which has a smaller set of

graphics density options, and for the HP DeskJet.  The 8/9 pin IBM

Proprinter is compatible with the Epson MX for ELNEC graphics.  Even

if your printer can't imitate any of these exactly, it may respond

correctly to the few necessary commands and work satisfactorily.  If

you can't make plots print correctly, and have tried the suggestions

under PROBLEMS, your printer isn't a compatible type.

Note that the printer must be connected to the primary parallel

printer port (LPT1:).  HPIB and serial busses aren't supported.

If your printer isn't compatible, I may be able to write a printer

driver for your printer type.  What you will need to do is:

Send me the printer type and model and a copy of the portions of

the manual which describe graphics operation and printer commands

(usually called "escape sequences").  Include any information on

the appearance of the plot (distorted, garbled, negative, etc.).

If it's possible for me to write the driver, I'll send you a new

copy of ELNEC which includes the driver.  There will be a nominal

charge for this service.

I reserve the right to discontinue or change the price of this offer

without prior notice.

61

PLOTTERS

I've received a few requests for plotter drivers.  ELNEC currently

creates a plot for the screen then "dumps" the screen data onto a

printer.  This can't be done with a plotter.  A plotter requires

creation of a separate, special plot just for its use.  Because of the

relative complexity of the task and the limited demand, I don't plan

to provide plotter drivers for ELNEC.

PROBLEMS

Plot grid appears but no plotting apparently takes place:

You may be plotting the pattern in a direction in which the field

intensity is very low (< -50 dBi).  Although nothing seems to be

happening, the program is calculating and plotting the far field but

the plot appears as a dot in the center of the grid.  When calculation

is complete, the plot will expand and become visible (if outer ring

scaling is automatic), and the maximum gain will be shown to be very

low (frequently -99.99 dBi).  The most common situation causing this

result is an azimuth plot at zero degrees elevation of a horizontal

antenna over ground.

ELNEC says it doesn't detect a graphics adapter:

If you have Hercules graphics, you must load (run) MSHERC before

running ELNEC.  See RUNNING ELNEC.  If you have a CGA, MCGA, EGA, VGA,

or compatible system and get this message, please notify me; it

shouldn't happen.

Plot is distorted on screen:

ELNEC is designed to automatically adjust for the resolution of your

graphics system type, and it assumes a standard 3:4 aspect ratio.

ELNEC presently contains no provision for changing the geometry of the

plot on screen.  Distortion caused by the monitor can only be

corrected by adjusting or repairing the monitor.  Distortion on the

screen won't cause distortion of the printed plot, however.

Printed plot is distorted:

The most likely cause is that you've specified the wrong type of

printer (8/9 pin instead of 24 or vice-versa).  See RUNNING ENSETUP.

If changing the printer selection won't correct the problem, your

printer isn't a compatible type; see PRINTERS, above.

Printed plot is negative:

Your printer isn't a compatible type; see PRINTERS, above.

62

The program always starts with the "Default" antenna, or it shows

the "Default" antenna when a different one was recalled:

The program reverts to the Default when it can't find the LAST.EN file

or when a recalled file has been corrupted and ELNEC can't read it.

The first situation could happen if ELNEC.EXE has been moved since it

was last run, and can't find the path to the .EN files.  If the second

situation occurs, the corrupted file is no longer usable and should be

erased.

Printed plot is garbled:

You may have specified a laser printer but are using a dot-matrix

printer, or vice-versa, or you've specified an FX-type printer and

your printer will recognize only MX graphics.  See RUNNING ENSETUP in

the GETTING STARTED chapter.  Another possibility is that you have

your printer set to emulate an IBM Proprinter.  This option, available

on many printer types, is typically selected with a DIP switch

setting.  If your printer is set to this mode you must use the Epson

MX driver.  If none of these is the case, the printer isn't a

compatible type; see PRINTERS, above.

A '%' sign appears in a menu entry:

This happens if the number of digits required to display the entry

exceeds the number of spaces allotted for it in the menu.  It

otherwise has no affect on program operation.  If this occurs

frequently enough to be a nuisance, let me know and we'll see if the

menu format needs changing.

Changing ELNEC.CFG using ENSETUP doesn't have any effect:

Some changes to ELNEC.CFG won't have any effect on certain systems.

For example, since the plot is monochrome on CGA monitors, plot color

choices won't change plot colors on a CGA system.

ELNEC always looks in the current subdirectory for ELNEC.CFG.

Likewise, ENSETUP always modifies the ELNEC.CFG file in the directory

which is current when ENSETUP is running.  You may be modifying one

ELNEC.CFG file and ELNEC is reading another.  To make sure that

ENSETUP is modifying the correct file:

1. Make sure ELNEC.EXE and ENSETUP.EXE are in the same subdirectory

(hard disk systems) or on the same diskette (floppy drive systems).

The following step assumes that the subdirectory is named ELNEC (hard

disk systems) or the program disk is in drive A: (floppy disk

systems).

2. Make sure that the subdirectory containing ELNEC.EXE and

ENSETUP.EXE is the current subdirectory when running either ELNEC or

ENSETUP, by typing 'CD ELNEC' (hard disk systems) or 'A:' (floppy disk

systems) at the DOS prompt before running the program.

63

The screen is blank or the plot or grid is invisible:

A bad choice of colors has been made.  You can change colors by

running ENSETUP or you can erase the file ELNEC.CFG which will force

use of the default colors.

The program crashes when it's started or when a file is recalled:

ELNEC won't permit you to save a defective file.  When a file is

written, a code number is included and this number is checked whenever

the file is read.  If it's incorrect the file won't be accepted and

the "Default" antenna is shown instead.  It's highly unlikely, but

possible, that a file could be corrupted without changing the code

number.  If this were to happen to the LAST.EN file, which is read and

processed each time the program starts, a crash could happen.  The

cure for this would be to erase the LAST.EN file and any accompanying

calculated array files by typing 'del last.*'.  Likewise, a file which

causes crashing when read has been corrupted and should be erased.

Again, calculated array files also should be erased by typing 'del

filename.*' where filename is the name of the file without the

extension.  This is an unusual circumstance but has happened.

The program crashes under any other circumstances:

A large amount of effort has gone into making ELNEC "crash-proof."

There should be no condition or action on the user's part which causes

a program crash.  (Exception: Turning off the printer during printing

will cause a crash.  It wasn't felt that prevention of this occurrence

was worth the reduction in plot printing speed and increase in code

size.)  If you encounter a crash, please record the error message.  If

it's possible to duplicate the crash, record the sequence of events

leading up to it.  If a particular description file is involved, print

the description using 'PD' in the Main Menu.  Send or fax the

information to me and I will find and correct the problem as quickly

as possible and send you a corrected copy without charge.

ERROR MESSAGES

ELNEC error messages are intended to be as self-explanatory as

possible.  However, more information sometimes is useful, so some

messages include an "ELNEC error" number.  This section gives more

information about what causes the error and how to avoid or correct

it.  Following the numbered errors is additional explanation of

several others.

64

ELNEC error 1

ELNEC is unable to start in the TraceView mode because it can't recall

a primary trace file.  ELNEC is looking for a file with extension .ENT

in the .EN files directory, and is unable to locate one, either

because of difficulty accessing the .EN files directory, or because no

.ENT files are present in the directory.  If you haven't saved any

.ENT (trace) files when running ELNEC, you will get this result;

TraceView's purpose is to view these traces.  If the problem is caused

by inability to access the directory, correct the problem described in

the message.  The .EN files directory can be changed with ENSETUP or

from the Options Menu in ELNEC, but at least one trace file must be in

the directory for TraceView to run.

ELNEC error 2

When calculated array files are recalled, they are put into arrays for

ELNEC to use.  These arrays reside in RAM.  If insufficient RAM is

available to load them, it means that more RAM was available when the

calculations were originally done than is at present.  When this

happens, ELNEC will load the antenna description, but not the

calculated arrays.  However, you'll probably find that ELNEC won't

perform calculations on the antenna because it will detect that not

enough RAM is available.  For more information on how to make more RAM

available, see MEMORY CONSIDERATIONS on p. 9.

ELNEC error 3

The antenna contains too many "pulses" for ELNEC to analyze.  The

number of pulses approximately equals the total number of segments.

ELNEC is limited to 127 total unless the Maximum Pulse Option is

present.  The only way to make a significant reduction in the number

of pulses is to reduce the number of wire segments.  For more

information on pulses, and a technique for doubling the number of

available pulses, see pages 68 and 26.

ELNEC error 4

Insufficient free RAM is available to run ELNEC.  You will have to

make more RAM available or reduce the antenna complexity.  Reducing

the total number of segments will have the greatest effect.  For more

information on RAM and how to make more available, see MEMORY

CONSIDERATIONS on p. 9.  If you exit ELNEC by typing 'QU' from the

Main Menu in order to free more RAM, the antenna description will be

saved and reloaded when you restart ELNEC.

ELNEC error 5

ELNEC was unable to access the default .EN directory.  This directory

is specified by running ENSETUP or by choosing it in the Options Menu.

If this error occurs, you must either correct the problem shown in the

error message or change the .EN directory.

65

ELNEC error 6

When ELNEC ends, it saves the current antenna description in file

LAST.EN in the .EN file directory.  This error occurs when it is

unable to do so.  If you can't correct the problem shown in the error

message, you must either change the .EN file directory (selection "EN"

in the Options Menu) or exit the program by typing 'EX'.  If you do

the latter, the antenna description will not be saved.

ELNEC error 7

This message will occur if you have selected "Save all calculated

arrays" for selection "LF" in the Options Menu and insufficient disk

space is available to save them.  When this option is chosen, ELNEC

saves calculated arrays with the LAST file in the .EN directory when

the program ends.  You must either change the .EN path to a drive with

enough space (Options Menu selection "EN") or change Options Menu

selection "LF" to "Save description only".  If you do the latter, the

calculated arrays won't be saved, and calculations will have to be

redone when ELNEC is restarted.

ELNEC error 8

An error was encountered while writing ELNEC.CFG.  This file contains

the information specified by the Options Menu and the ENSETUP program.

ELNEC.CFG is always written in the current directory (the directory

you were in when you started ELNEC).  All that can be done is to

correct the error reported by ELNEC.  Until the problem is corrected,

no changes can be made to ELNEC.CFG.

ELNEC error 9

ELNEC had trouble reading the file LAST.EN.  This file always is

written when ending ELNEC with 'QU'.  It contains the antenna

description present when ELNEC ended, and is automatically read when

ELNEC starts.  LAST.EN resides in the directory specified by ENSETUP

or by Options Menu selection "EN".  If you see this error, ELNEC will

be unable to save or recall antenna descriptions until the .EN file

directory is changed to one which can be accessed or the problem is

corrected.

ELNEC error 10

An illegal extension was specified for an antenna description file or

a trace file.  Extensions .V, .I, .DB, and .Z*, where "*" is any

combination of characters, are reserved for calculated array files.

Files with these extensions can't be saved, recalled, or deleted by

ELNEC.  Files with other extensions can be recalled or deleted, except

that files with the extension .EN can't be recalled as a trace file,

and files with extension .ENT can't be recalled as an antenna

description file.  Antenna description files must have the extension

.EN, furnished either by ELNEC or the user, to be saved.  Trace files

must have the extension .ENT to be saved.

66

The following errors occur only when ELNEC is operating in conjunction

with the Maximum Pulse Option, MaxP.  They are included here because

they are generated by ELNEC.

ELNEC error MP1

ELNEC was unable to find one of the temporary files recorded by MaxP,

so is unable to make use of the data it has calculated.  This file is

recorded in the directory specified by selection "MA" in the Options

Menu.  The only cause for this error I can foresee is if the MaxP

directory is on a floppy disk and the disk is removed during program

execution.  Please notify me if you see this error.

ELNEC error MP2

When you change the directory for the temporary MaxP files, ELNEC

attempts to copy them from the old directory to the new one.  (If

successful, it then erases the files in the old directory.)  This

error occurs if ELNEC was unable to copy the files.  When this

happens, it leaves the MaxP temporary file directory unchanged.

ELNEC error MP3

When MaxP has run and you choose to save the calculated arrays, ELNEC

copies the temporary files containing the impedance data from the MaxP

temporary directory to the .EN file directory.  This error occurs if

ELNEC is unable to find the temporary files in the MaxP directory or

if it is unable to access the MaxP directory.  There is no obvious

fault which would cause this error; please notify me if you see it.

ELNEC error MP4

ELNEC is unable to write the temporary MaxP files to the MaxP

temporary file directory for the reason shown in the error message.

The MaxP temporary file directory can be changed with selection "MA"

in the Options Menu.  The temporary files which ELNEC writes are small

-- two are 512 bytes and the third seldom exceeds 2k or so.  However,

before running MaxP, ELNEC makes sure there's enough disk space to

hold the large temporary arrays that MaxP will generate.  The required

space may exceed 500k bytes.  The only solution is to specify a drive

for the MaxP temporary file path which contains adequate space.

ELNEC error MP5

When calculated array files are recalled from antennas containing more

than 127 pulses (arrays calculated by MaxP), they aren't loaded into

ELNEC but are copied into the MaxP temporary files directory as

temporary files.  This error occurs when ELNEC is unable to copy them

into the MaxP temporary file directory.  You must correct the problem

shown in the error message or change the MaxP temporary file path with

selection "MA" in the Options Menu to a drive/directory which can be

accessed and which has adequate room.

67

Zero-length wire(s)

If zero-length wires are present, ELNEC can't place sources or loads

properly or do other essential calculations.  Therefore you aren't

permitted to leave the Wires Menu while this problem exists.  When you

attempt to leave, however, you're given the option of deleting all

zero-length wires.  Agreeing to the deletion will clear the error and

permit you to return to the Main Menu.

Other errors:

Wire(s) in or on the ground plane

If a real or perfect ground has been specified, wires aren't permitted

to extend into the ground plane (negative z-coordinate) or lie on the

ground plane (both z-coordinates zero).  If this condition exists,

ELNEC can't perform calculations and you're prevented from running the

program or saving the antenna description.  You also can't exit ELNEC

using (QU)it from the Main Menu because this would save the defective

description in the LAST file.  If you don't want to correct the

problem by changing the ground type or wire coordinates, you can exit

the program by typing 'EX' at the Main Menu.

Source(s) or load(s) on a nonexistent wire or open wire end

This condition prevents ELNEC from correctly performing calculations.

One circumstance which can cause this error is to delete all wires

which have sources placed on them.  ELNEC will leave one source (since

one source is required) but place it on a nonexistent pulse.  Like the

above error, it will prevent you from running ELNEC, saving the

antenna description, or exiting using (QU)it.  If you want to exit the

program without fixing the problem, use (EX)it in the Main Menu.

PULSES

Perhaps the nicest thing about ELNEC is that you don't have to

understand "pulses", and how MININEC numbers them, to use it.

However, pulses are fundamental to the computation method so the term

appears here and there in ELNEC.  Their significance in source/load

placement is explained in "The Sources Menu" above.  The other place

where they have particular significance is in determining the size of

an important internal array, the impedance array.  The size of this

array can't exceed 127 pulses unless the Maximum Pulse Option is used.

If your antenna description includes more than 127, ELNEC will tell

you so and refuse to perform the calculations.  The number of pulses

is closely related to the total number of wire segments, so if the

number of pulses exceeds 127, fewer wires, or wires having fewer

segments, must be specified.  More information on pulses can be found

in "MININEC: The Other Edge of the Sword", in February, 1991 QST, and

in the MININEC manual referenced at the end of this chapter.

68

NOTES FOR EXPERIENCED MININEC USERS

ELNEC describes antenna and ground radial wires by diameter instead of

radius.  This was done because wire tables generally give diameter,

and tubing is usually measured by diameter.  ELNEC uses elevation

angle (measured upward from horizontal) rather than zenith angle

(downward from vertical).  Although zenith angle is mathematically

cleaner, I found myself having difficulty thinking in terms of zenith

angle so changed to elevation instead.  Media boundaries are entered

differently; you enter the beginning boundary of the medium being

described rather then the boundary of the next medium.  MININEC

doesn't permit radials if only one medium is specified; ELNEC does,

and assumes infinite radial length in that case.  And as explained

elsewhere, if source/load positions are defined by wire number and

position in wire (as recommended), the source and load positions will

behave differently than in MININEC as geometry is changed.

When you select source or load data, powers are shown.  The powers

shown by ELNEC are exactly twice those shown by MININEC.  MININEC

apparently interprets source voltages as peak values, while ELNEC

interprets source voltages and currents as rms.

If you compare ELNEC and MININEC results, you'll find small

differences (typically 0.01 dB or so).  This is because ELNEC uses

precise values for the speed of light and the permittivity of free

space to calculate derivative units, while MININEC uses several

constants of various precision to define derivative units.  Also,

certain calculations are done in a different order, changing

cumulative errors.

You may find substantial differences in ELNEC and MININEC far-field

results if you have described a multi-media ground with the media at

different heights.  ELNEC contains a correction for the simplified way

MININEC uses to determine reflection point.  This is briefly described

in "MININEC: The Other Edge of the Sword", in February, 1991 QST.

No more fussing with pulses!  You'll love ELNEC's source/load entry

system and smooth handling of wire/source/load changes.

MORE MININEC INFORMATION

If you would like to learn more about the mathematics, basic

functioning, and limitations of MININEC, obtain a copy of

The New MININEC (Version 3): A Mini-Numerical Electromagnetic Code,

by J.C. Logan and J.W. Rockway, Naval Ocean Systems Center, San

Diego, CA (NOSC Technical Document TD 938)

69

It can be ordered as NTIS document number ADA181682 from

National Technical Information Service

U.S. Department of Commerce

5285 Port Royal Road

Springfield, VA 22161

(703) 487-4650

This is a highly technical manual.

A comprehensive article on the limitations of MININEC and its

derivatives such as ELNEC is "MININEC: The Other Edge of the Sword",

in February, 1991 QST.  Although much of its content is included in

this manual, I recommend reading it.  (For one thing, it has the major

advantage of containing pictures!)

HELP!

As a customer, you have the right to expect a program you can use.  If

you encounter a problem with ELNEC, I'll do everything I can to

resolve it.  Before contacting me, however, please consider that I'm

employed full time outside the pursuit of writing and supporting ELNEC

which limits the total time I have available.  I've tried to

anticipate as many questions as possible in the manual; please first

try to find the answer to your question here.  If you can afford the

time to write, I'll try to answer your letter the day after I receive

it, or will call you if you prefer.  If you need a faster response,

and again, if you can't find the answer in the manual, call my home at

(503) 646-2885 and I'll be glad to help you.  I'm on Pacific time.  I

now have fax capability at the same telephone number.  The fax machine

is generally on 24 hours a day and will automatically respond to fax

machines which send a "CNG" (beep) tone.

It's my sincere intent to make ELNEC as intuitive and easy to use as

possible while retaining the impressive power of MININEC's method-of-

moments analysis.  If you have any suggestions for improvements or

would care to comment on anything you like or don't like about the

program, I would very much appreciate hearing from you.  I would

especially like to know if you've observed any malfunctioning,

unpredictability, or program "crashing".  These are not acceptable and

will be corrected as soon as the cause is discovered.  Later versions

will be offered to current users at a substantial discount.

Thanks for choosing ELNEC.  Good luck with your antenna projects!

73,

Roy Lewallen, W7EL

P.O. Box 6658

Beaverton, OR 97007

70

71

I N D E X

.DB files  61

.EN (antenna description) files  12, 13, 58-60

extension  58

path  13, 15, 63

path, default  13, 47

.ENT (trace) files  12, 13, 58, 61

extension  58

path  13

path, default  13, 47

.I files  61

.V files  61

.ZI files  61

.ZR files  61

Aluminum

alloy, resistivity  42

ANALYZE  44, 57

and TraceView  59

resolution  43

Antenna

viewing  45, 50

Antenna description

abbreviated, printing below plot  47

deleting  58

recalling  58

saving  58

ANTNOTES.DOC  12, 60

Azimuth angle  24, 41, 42

Azimuth plot

description  24

Beamwidth  44, 57

Calculated arrays

saving  44, 58, 61

saving with LAST  46, 61

Color

background  13, 15

plo  13, 15

recalled trace  13

Conductivity

scaling  27

Coordinate system  40

Current

abrupt changes  20, 25

and segments  19

importance of  25

interpreting  25

phase of  22

polarity  25

symmetry  25

Current directory

72

defined  31

Date format  13

dBd  29

dBi  29

Default antenna  63

Deleting

antenna descriptions  44, 58

traces  58

Directories  60

Directory

current, defined  31

starting ELNEC from a different  31

Disk space, freeing, floppy drive system  12

Elevation angle  24, 41, 42, 69

Elevation plot

description  24

ELNEC

ending with QUit  46

exiting without saving in LAST  46

files  60

limitations of  70

starting  31

upgrading from v. 1.xx  16, 17

version 2 new features  17

ELNEC.CFG  12, 13, 15, 46, 61, 63

default values  13

ELNEC.DOC  13, 60

ELNEC.EXE  12, 17, 60, 63

ENSETUP  12, 59, 63

running  13

ENSETUP.EXE  12, 17, 60, 63

Error messages  64

maximum pulse option  67

Errors

sources/loads on nonexistent wire  68

sources/loads on open wire end  68

wire coordinate  52

wires in or on ground plane  68

zero-length wires  68

EXit  46, 68

Field strength

units  47

Field(s) to plot  43

Files

ELNEC  60

Frequency  41

Front view  45

Front/back ratio  44, 57

quad antenna  21

Front/side ratio  44, 57

Front/sidelobe ratio  44

Gain  57

antenna over ground  26, 29

ELNEC vs MININEC  69

73

forward  44

reference  29, 44

Graphics adapter  8, 62

Ground  57

conductivity  55

conductivity and dielectric constant, table  55

conductivity, default  46, 55

connecting loads to  54

connecting sources to  54

connecting wires to  24, 50

dielectric constant  55

dielectric constant, default  46, 55

height  56

modeling  23

perfect  23

radials  23, 56

radials, adding or changing  57

real  23

real, limitations of model  23, 55

shielding effects  23

specifying  55

Ground description, real  42

Ground system

efficiency  23, 57

Ground type  42

Group dia.  49

Guarantee  6

Hardware requirements  8

Height, antenna

changing  50

Help

how to get  70

Horizontal antennas, low  23

HPIB  9, 61

Impedance

feedpoint  25, 45

source  25, 45

user-defined, for SWR  44

Installation  12

Inverted vee antenna  35, 49

Isotropic radiator  29

Laplace transform

automatic RLC entry  54

using  54

LAST.EN  46, 60

and TraceView  60

defective  64

not found  63

saving calculated arrays with  46, 61

Letters, lowercase and uppercase  40

License agreement  6

Load data  26, 45

Loading coils  23, 26

Loads  42

74

adding  54

at junctions of more than two wires  27

connecting to ground  54

deleting  54

impedance, specifying or changing  54

Laplace transform  54

loss  26

on nonexistent wire  68

on open wire end  68

position of, specifying  54

position of, specifying pulse number  54

using  23

Loads Menu  54

Log periodic antenna  21

Loop

quad  20, 21

Loss

load  26

Main Menu  41

Maximum Pulse Option  8, 17, 21, 47, 59, 61, 65, 68

error messages  67

Maximum Pulse Option (MaxP)

description  10

MAXP.EXE  31

Media  23

adding  56

boundary  56

boundary, changing type  57

deleting  56

height  56

multiple, at different heights  69

multiple, example  56

Memory

expanded and extended  10

Memory requirements  8, 9

Menus  41

Loads Menu  54

Main Menu  41

Media Menu  55

Options Menu  46

Plot Menu  57

Sources Menu  52

Wires Menu  48

MININEC  5, 8, 20, 23, 27, 55, 57, 68

limitations of  19, 70

manual  69

notes for experienced users of  69

MININEC: The Other Edge of the Sword  19, 68-70

Modeling  18

antenna structure  19

complex structures  28

Monitor

color  15

LCD  13, 15

75

monochrome  13, 15

MSHERC.COM  13, 31, 60

Multiband antennas  21

Options Menu  46

Patterns  24

Permeability

wire, specifying  42

Phased array antenna

feed systems  22

Phased array antennas  22

Plot

distorted  62

distorted, printed plot  62

garbled, printed plot  63

generating  44

grid style  47

outer ring value  43, 47

printing  57

style  47

Plot Menu  57

Plot type  42

Plot/table range  43

Plotters  9, 62

Polarization  43

Power

ELNEC vs MININEC  69

loss in loads  26

Primary trace  59

Print quality  47

Printer port  9, 61

Printers  13, 61

dot matrix  14

driver, special  61

Epson FX  14, 61

Epson LQ  14, 61

Epson MX  14, 61

HP DeskJet  14, 61

HP LaserJet  14, 61

HPIB  61

IBM  14, 61, 63

laser  14, 61

serial bus  61

types  14

Problems  62

% sign in menu entry  63

blank screen  64

default antenna  63

distorted plot  62

distorted printed plot  62

ENSETUP has no effect  63

garbled printed plot  63

graphics adapter not detected  62

grid but no plot  62

negative printed plot  62

76

program crashes  64

Pulses  8, 68

doubling the number of  26

maximum number of  21, 26, 68

Quad antenna  21

front/back ratio  21

Quad loop  20, 21

Quality  6

QUit  46, 68

RAM  8, 9

READ.ME  13, 60

Real ground description  42

Recall Trace  59

Recalling

antenna descriptions  44, 58

traces  58

Resistance

low  25

negative  25, 45

Resistivity

wire, scaling  27

wire, specifying  42

Right side view  45

RLC, series or parallel, entering  54

Saving

antenna descriptions  44, 58

traces  58

Scaling  27, 41

Scientific notation  40

Segments  19

and accuracy  20

and computation time  20

length limits  20, 21

number of, choosing  20, 21, 25, 26

number of, entering  50

number of, total  50

tapering  26, 51

Sidelobe  44, 57

Significant digits, number of  40

Skin effect  42

Source data  45

Sources  41

adding  53

amplitude and phase  53

at junction of two wires  22

at junctions of more than two wires  27

connecting to ground  54

current  22, 53

deleting  53

multiple  22

on nonexistent wire  68

on open wire end  68

polarity  22

position of, specifying  53

77

position of, specifying pulse number  53

using  22

voltage  22, 53

Sources Menu  52

Step size  43

SWR  25, 44, 45

Templates  28

Test drive  32

Time remaining display  40

Tips  26

Title  41

Top view  45

Trace

clearing  58

deleting  58

recalling  58

saving  58

TraceView mode  59

ending  60

starting  31, 59

Traps  23, 26

TSR programs  9

Units  42

changing without changing numbers  51

V/m  47

View Antenna  45, 50

Wild card  58

Wire Coordinate Errors  52

Wire gauge  41

radials  57

wire diameter, specifying as  49

wire diameter, specifying as, example  33

Wire loss  25, 42

Wires  19, 41

adding  50

closely spaced  20, 21

connecting to ground  24, 50

connecting to other wires  19, 48

deleting  50

diameter, changing group  49

diameter, entering or changing  49

diameter, limits  19

diameter, specifying as wire gauge  49

diameter, specifying as wire gauge, example  33

direction  22

end coordinate, errors  52

end coordinates  19

end coordinates, entering  48

end coordinates, specifying  22

in or on ground plane  68

joining at an angle  20, 26

length, changing  48

permeability  42

renumbering  50

78

resistivity  42

rotating  49

rotating, example  37

zero-length  68

Wires Menu  48

Zenith angle  69

Zero-length wires  68

79