��    ��   �����     ����     ����

�۱   �۱ �۱����   �۱���   �۱���

��  �۱� �۱   ۱ �۱�  ۱ �۱�  ۱

���۱�   ��    � �۱    � �۱    �

�۱�     ���    �۱      �۱

����        ��   �۱      �۱

�۱���   �    ��  �۱    � �۱    �

�۱�  ��  ��   �۱  ��  �۱  ��  �۱

�۱   �۱  ����۱�   ���۱�   ���۱�

��    ��   �����     ����     ����

Flexnet XSCC driver

Version 0.14 (30 August 1998)

Zilog Z8530 SCC driver for

PA0HZP-OptoPcScc Card

PE1PET SCC Card

DRSI Card

BayCom Digi SCC Card

BayCom USCC Card

by Henk de Groot, PE1DNN

Based on TFPCX 2.21 SCC driver by Ren� Stange, DG0FT

Linked with FlexNet driver library 1.5a, by Gunter Jost, (D)K7WJ

0 Contents

----------

1       Introduction

1.1     Foreword

1.2     What is it?

2       Operation

2.1     Command line options

2.2     Flexnet Interface

2.3     Examples

3       Odds and ends

3.1     To Do

3.2     Thanks

1 Introduction

--------------

This document describes the XSCC driver for FlexNet.

In section 1 you will find the foreword and short explanation what  it

is.

In  section  2  the  Operation  is  explained  with explanation of the

command-line options, FlexNet interface and examples.

1.1 Foreword

Being involved in  packet radio since  about 1995 I  slowly discovered

how the AX 25 network worked. I  started off like a lot of hams  using

a BayCom serial modem. This is a cheap starter and without it I  would

never would have started. That's why I strongly support to keep  cheap

1k2 packet, even though it is  slow. The cheap 9k6 YAM modem  might be

an alternative, but a suitable 9k6 compatible TRX is needed for that.

I used  the TFX  driver by  NORD><LINK with  the BayCom  modem but was

bothered by  some bugs  in it.  I tried  to get  hold of NORD><LINK by

trying to connect to  the DB0FC node. From  my place it was  very hard

to find a  route to that  node. I've been  at a lot  of places but not

where I wanted to be. Until I connected to PI1DRS. From there I  could

just simply connect to DB0FC!  It turned out PI1DRS used  FlexNet! Now

people say  I can  do the  same without  FlexNet. It  maybe so,  but I

could not manage to do it using the help the nodes provided. My  first

experience with FlexNet was very good.

My second good experience  with FlexNet came when  I tried to use  the

BayCom  modem  from  Windows95.  I  wanted  to  do  this  to check out

TSTHWIN,  which  does  not  run  from  DOS.  FlexNet  worked   without

problems.

Then in  1997 Ren�  Stange, DG0FT,  dropped the  development of  TFPCX

2.21.  Since  TFPCX  still  lacked  proper  DAMA  support  I  took the

opportunity to  build the  latest version  of NORD><LINK's TheFrimware

into TFPCX. Later I added driver  support for the 9k6 YAM modem.  That

also     got     me     into     the     9k6     modem      operation.

After some time of 9k6  operation with TCP/IP (using SV2AGW's  dial-up

networking  product  and  FlexNet)  I  noticed  that  the Dutch TCP/IP

network is not at  its best. I also  found out that many  people would

like to  try FlexNet  but that  FlexNet lacked  PA0HZP Opto SCC driver

support, a commonly used SCC card in The Netherlands.

Since I had a  good working SCC driver  sample with TFPCX I  thought I

would be able  to build a  FlexNet driver without  having an SCC  card

myself. After one  initial contact to  Gunter Jost I  did not get  any

answers (still don't know why) and  it turned out others had the  same

experience.  This  was  very  de-motivating,  and  I dropped the whole

idea.

Hans, PA3EZQ  finally got  me into  it again.  He met  Gunter Jost and

acquired  the  needed  driver-kit.  Since  operators  at my local node

(PI1VRZ) also indicated they wanted  to try FlexNet I decided  to have

a  look  at  the  driver-kit.  After  looking  at  it I concluded that

re-using  the  TFPCX  SCC  driver  was  not  too difficult and started

building.

The result of all this is what  you have here. I hope there will  be a

lot of experiments with  this and I hope  I did not do  it for nothing

(don't think so!). Although I do not  have an SCC card I think I  will

personally benefit from this driver as a user of the HAM network.

Henk de Groot - PE1DNN, Apeldoorn 30 August 1998

1.2 What is it?

XSCC is a FlexNet SCC driver for a variety of SCC cards which use  one

or more Zilog Z8530 SCC chips or look-alike. It is based on the  TFPCX

SCC driver and any SCC card which used to work with TFPCX should  work

with XSCC.

It  is  tested  in  conjunction  with  the FlexNet software (including

FlexNet Node)  both with  and without  DAMA. Since  it adheres  to the

FlexNet  driver  interface   specification  (using  (D)K7WJ's   driver

library) it should also work  with TNL1.EXE for TNN and  XNET (version

1.15 or better).

XSCC is tested on the PA0HZP Opto-SCC card (4 ports), PE1PET SCC  card

(2 ports), DRSI card (2 ports) and BayCom USCC card (4 ports).

It supports full-duplex operation but will interrupt reception if  the

used modem does not support this.

2 Operation

-----------

In  this  chapter  I   will  provide  command-line  options,   FlexNet

interface and examples.

2.1 Command line options

When running XSCC -? you will get the following help screen:

-----------------------------------------------------------------------------

FlexNet XSCC driver V0.14 (C) Aug 30 1998 H. de Groot PE1DNN

Driver for OSCC, PSCC, DRSI, DSCC and USCC cards

Usage: XSCC -t:<type> -p:<baseaddr> -i:<irq> -c:<channels>

-t:<type>    = OSCC, PSCC, DRSI, DSCC or USCC default OSCC (PA0HZP)

-p:<baseaddr>= Address of SCC, default 0x150 (OSCC/PSCC) or 0x300 (others)

-i:<irq>     = IRQ setting of SCC card, default 3 (OSCC/PSCC) or 7 (others)

-c:<channels>= Number of channels you want to use on the card, shall be

equal or less then the number of channels supported by the

card. Default is maximum number of channels of the card.

FlexNet device driver interface V1.5a (C)1995-1997  G.Jost (D)K7WJ

-----------------------------------------------------------------------------

-t: SCC card type

With the '-t' option you can  specify which SCC card you want  to use.

The type  is not  case sensitive  and only  the first character(s) are

evaluated. The following SCC cards are supported:

OSCC ('-t:OSCC' or '-t:o')

Specifies the PA0HZP Opto  SCC card, this is  also the default if  the

option  '-t'  is  not  specified.  When  using this card the following

defaults are used for the other options:

baseaddr = 0x150

irq      = 3

channels = 4

Don't forget to specify 'r' with  the FlexNet MODE command if you  are

using a real PA0HZP card (not  for PE1PET or DRSI) (see later  in this

document).

PSCC ('-t:PSCC' or '-t:p')

Specifies the  PE1PET SCC  card. When  using this  card the  following

defaults are used for the other options:

baseaddr = 0x150

irq      = 3

channels = 2

Note that this  is equivalent to  '-t:OSCC -c:2', but  now the correct

name will appear during startup and in a status overview.

DRSI ('-t:DRSI' or '-t:dr')

Specifies  the  DRSI  SCC  card.  When  using  this card the following

defaults are used for the other options:

baseaddr = 0x300

irq      = 7

channels = 2

Note that this is equivalent to '-t:OSCC -p:0x300 -i:7 -c:2', but  now

the correct name will appear during startup and in a status overview.

DSCC ('-t:DSCC' or '-t:d')

Specifies  the  BayCom  Digi  SCC  card.  When  using  this  card  the

following defaults are used for the other options:

baseaddr = 0x300

irq      = 7

channels = 4

USCC ('-t:USCC' or '-t:u')

Specifies the  2 port  and 4  port BayCom  USCC card.  When using this

card the following defaults are used for the other options:

baseaddr = 0x300

irq      = 7

channels = 4

Don't forget to specify '-c:2' if your USCC card has only 2 ports.

Note: the  USCC-8 port card cannot  be used with XSCC, use the FlexNet

USCC driver for that card.

-p Port base-address

The  '-p'  option  specifies  the  base-address  of  the card. Default

base-address is 0x150  for the PA0HZP  Opto SCC card  and PE1PET card.

For  the  other  cards  the   default  base  address  is  0x300.   The

base-address  is  always  divisible  by  8.  The exact location of the

control-  and  data-registers  of  the  SCC  chips depends on the card

type. After loading XSCC the actual settings are shown on the screen.

-i Interrupt request number

The '-i' option  specifies the IRQ  number used for  the card. Default

IRQ is 3 for the PA0HZP Opto  SCC card and PE1PET card. For the  other

cards the  default IRQ  is 7.  Not every  IRQ can  be used,  the IRQ's

standard used on PC's are disabled.

IRQ can be 2, 3, 4, 5, 7, 9, 10, 11, 12, 14 or 15.

-c Number of channels

The '-c'  parameters specifies  the number  of active  channels on the

card. The minimum value is 1, maximum depends on the card type.

OSCC:  4 channels

PSCC:  2 channels

DRSI:  2 channels

DSCC:  4 channels

USCC:  4 channels

Note  if  you  have  a  USCC-2  card  you  should  specify 2 channels,

although the  program will  accept 4  (but detection  will fail due to

lack of hardware).

The  type  of  modem  connected  to  each channel is determined by the

FlexNet MODE command, see later.

2.2 FlexNet Interface

There are several FlexNet utilities which interface to XSCC, they  are

described in this section.

FSET

XSCC  is  programmed  by  the  FSET  command  (or  via  the digipeater

software).

FSET TXDelay

If FSET TXD is not used on a  port then the txdelay is set to 25  (250

ms).

Internal timer  for TX-Delay  has a  54ms resolution  (that's the PC's

interrupt 0x1C timer, can't help  it, there is nothing better  that is

compatible on any PC).

FSET TXD 0 25 will in fact  give a 216 to 270ms TX-delay on  channel 0

instead of  250ms (depending  on when  txdelay is  set with respect to

the next timer tick).

FSET MODE

If FSET MODE is not  used on a port then  the baudrate is set to  1200

baud. Otherwise the  baudrate is set  with FSET if  it is between  100

and 65500 baud. Note  that your SCC card  and modem should be  capable

of what is specified, XSCC cannot check on this and just programs  the

clock-speed for the set baudrate.

Note  that  your  PC  has  to  keep  up  with the specified speed. The

program STAT can be used to judge if you PC is fast enough.

Besides  the  baudrate  you  can  also  specify other things with FSET

MODE.

'-' Will switch off the port, can be re-enabled later however.

'c' Will select Soft DCD, leaving out 'c' will select hardware DCD.

'd' Will select Full Duplex. This is handled in the FlexNet kernel,

XSCC will report it back to the FlexNet kernel is the driver

status is fetched. If the connected modem does not allow

full-duplex reception will be interrupted on transmission. XSCC

does not check this! Leaving out 'd' will select half-duplex.

'z' Will switch on NRZ coding, needed for the DF9IC modem. Leaving

out 'z' will select NRZI coding.

The 'r' and 't' flags map onto the modems as known in TFPCX:

'-'   Disable      Port disabled (Switched Off)

''    Softclock    The transmit and receive clocks are created

internally for use with (A)FSK-Modems (duplex mode

not possible)

't'   Hardclock    The transmit clock will be generated by the modem,

the receive clock is created internally (e.g. G3RUH)

'rtz' DF9IC-Modem  The transmit and receive clock will be generated by

the modem, NRZ-Mode.

'r'   PA0HZP-Port  The receive clock will be created internally,

divided externally by 32 and passed back to the

SCC-Controller as transmit clock. (for Opto

SCC-card). NOTE: the 'r' parameter does not give

an external RX clock in XSCC! I reused the 'r'

setting so the PA0HZP port setting could be mapped

on the FlexNet flags! When 'r' is used with 't' it

will give an external RX (and TX) clock however!

'r' only functions with the PA0HZP Opto SCC card. The other modes  are

used for the  PSCC,  DRSI,  DSCC and USCC card. Note that modems which

need an external  RX clock and  internal TX clock are not supported by

XSCC.

Other options are not relevant for  XSCC except for '-p' which is  not

supported  on  purpose  (may  violate  your  countries  regulation  by

sending an idle carrier with no data and station-identification).

STAT

Stat  will  show  statistics  for  XSCC  and  others. Here I will only

explain the meaning of the shown statistics for XSCC.

Command line: XSCC -t:USCC -p:0x300 -i:7

Possible output of stat:

FlexNet version 3.3g

uptime: 8s; total 20k, max 0k, used 1k

po device  version    txframes rxframes    terr    rerr   rberr ioerr

0 U:XSCC4  0.11           265     1827       0       2       0     0

1                       24361    24852       1       0       1     0

3                        1927    24361       0       0       0     0

Column  'po'  shows  the  port  number  assigned by FlexNet (port 2 is

switched off by using 'FSET MODE 2 -').

Column   'device'   shows   the   identification   of the  device. The

first letter  indicates the  type of  SCC card  used. This  is 'O' for

PA0HZP Opto SCC, 'P' for PE1PET SCC, 'R' for DRSI SCC, 'D' for  BayCom

Digi SCC and 'U' for BayCom USCC.

The column and XSCC is fixed and identifies the XSCC driver. The  last

number indicates  the number  of ports,  which is  defined by the '-c'

command-line parameter.

The  'txframe'  column  shows  the  number  of successfully send AX.25

frames.  The  'rxframe'  column  shows  the  number  of   successfully

recieved AX.25 frames.

The 'terr' counts the number  of failed transmissions. This is  caused

by so  called 'underrun',  when the  CPU is  not capable of delivering

data  fast  enough  to  the  SCC  chip  during  transmission.  On 19k2

foreinstance a new byte  has to be written  to the SCC chip  every 0.5

ms.  If  the  CPU  is  held  up  and  cannot write the data in time an

'underrun' situation occurs  (The SCC needs  to send the  next byte in

the frame which is not present).

The 'rerr' counts a similar problem  on reception. In this case it  is

caused by so called 'overrun'. The CPU is not capable of reading  data

fast enough from the SCC  chip during reception. On 19k2  foreinstance

a new byte has to be read from  the SCC chip every 0.5 ms. If the  CPU

is held up  and cannot read  the data in  time an 'overrun'  situation

occurs (The SCC  needs to store  the next received  byte in the  frame

but the previous byte is still present in the register).

The 'rberr'  counts when  XSCC itself  runs out  of buffer-space. This

occurs when there  are too many  received frames and  FlexNet does not

read them  fast enough.  The number  of frames  XSCC can  buffer is  a

trade-off between running out of buffers and the driver-size.  Buffers

are shared for all ports, a high speed port can cause an 'rberr' on  a

low-speed port.

Both 'terr' and 'rerr' are a sign that either:

- The speed of the PC is not sufficient

- The PC has a too high load

- One of the applications on the PC blocks interrupts too long

- The there are too many interrupts/second

- XSCC's code can not keep up with the speed

The 'rberr' is a sign that either:

- XSCC does not have enough buffers

- Too many ports are used for high speed

Currently  there  are  16  transmit  buffers  and  32  receive buffers

(receive and transmit buffers are separated to avoid dead-locks)

CALIB

Calib is use to keep a channel transmitting and is supported by XSCC.

2.3 Examples

In this chapter I present some examples how to startup XSCC.

For starting FlexNet  I use a  batch-file called LOADFLEX.BAT.  Ensure

the FlexNet directory is in the PATH environment parameter,

----------------------------------------------------------------------

FLEXNET 20

XSCC -t:USCC -p:0x300 -i:7 -c4

FLEX

TFEMU

FSET TXD 0 25

FSET TXD 1 18

FSET TXD 2 18

FSET TXD 3 15

FSET MODE 0 1200c

FSET MODE 1 1200c

FSET MODE 2 9600

FSET MODE 3 19200rtz

----------------------------------------------------------------------

In this example XSCC is loads for the BayCom USCC card on base address

0x300 and IRQ 7. On the ports the following modems are present:

port 0: an AFSK modem (1k2)

port 1: an AFSK modem (1k2)

port 2: a  FSK  modem (9k6)  (type Cadams)

port 3: a  FSK  modem (19k2) (type DF9IC)

Ports 0 and 1 are set to 1200  baud with soft DCD. TX delay on port  0

is 250 ms and  on port 1 180  ms. Port 2 is  used with 9k6 baud  and a

TXD of 180 ms. Port 3 is 19k2 baud and a TXD of 150 ms.

Other examples for the XSCC line:

If you have 4 ports and 2 Z8530 SCC chips:

Example: XSCC -t:OSCC -p:0x150 -i:3 -c:4

If you have 2 ports PE1PET SCC and 1 Z8530 SCC chip:

Example: XSCC -t:PSCC -p:0x150 -i:3 -c:2

Example MODE commands:

FSET MODE 0 1200rc   PA0HZP port with 1200 baud and soft-DCD

FSET MODE 1 1200c    FSK or AFSK modem, software DCD

FSET MODE 1 9600t    G3RUH modem at 9k6 baud and hardware-DCD

FSET MODE 2 19200rtz DF9IC modem (NRZ coding, extern RX/TX clocks) at

19k2 baud and hardware DCD

3 Odds and ends

---------------

3.1 To Do

The  following  things  are  on  my  todo  list.  If  you  have   more

suggestions then send me a message. If you can help me out with  ideas

I would also like it...

-There is no TX-Delay measurement on reception. Don't know how  to  do

it!

-I whish there was a more  accurate timer than the 54ms PC  timer. One

which would not give  conflicts with other FlexNet-drivers  and could

be used under Windows also.  Why does the FlexNet kernel  not provide

a timer,  everybody has  this problem  I think  (except ethernet-card

drivers e.t.c.).

-I'm  trying to  support USCC-8  but no  success so  far. That's why I

dropped this card for now (USCC-8 users van use the USCC driver).

-Maybe I should add support for the z85230 ESCC chip.

-Found some  other SCC  cards in the Linux and JNOS SCC drivers, Eagle

and Primus. Maybe I should support those too.

-Still looking  for reviewers  to examine  the code. You can also have

the code. The FlexNet driver-kit you have to get from DK7JW!

3.2 Thanks

First  of  all  I  like  to  thank  Hans,  PA3EZQ,  for  providing the

driver-kit, without his contribution this driver would not exist.

The  following  people  helped  me  to  Alpha  test  XSCC  and to gave

suggestions for improvements (random order):

Frits    - PD0RSN

Pete     - PE1MHO

Jean-Luc - F5IBH

Marco    - PE1PQK

Gerard   - PE1BSC

Hans     - PD0DAD

Henk     - PE1DOQ

Freek    - PE1RDD

Wim      - PA3AKK

I would like to thank them very much. Since I do not have an SCC  card

I totally relied on these people. Although the first versions did  not

run at all, lucky enough none of them ruined their PC. Special  thanks

to Marco, PE1PQK, who ran this version with some load on PI8ZWL.  This

triggered me  to completely  redesign the  buffer management  in XSCC.

Wim, PA3AKK, did not test the driver itself but gave valuable  comment

as user of PI8ZWL on the performance of XSCC.

Also others  contributed since  some of these  people shared XSCC with

others, but these are the people I had direct contact with.