# Button 1 - Display

## Cycles through display modes.

## UTC Time

### Shows Universal Coordinated Time in 24-hour format.

### Rotating hand shows 1/8ths of second.

## Local Time

### Shows local zone time in 12-hour format.

### Includes daylight savings time correction.

### Rotating hand shows 1/8ths of second.

## Latitude

### Shows latitude in degrees, minutes, seconds.

### Last character will show the site survey status:

#### ‘ ‘: No survey

#### ‘s’: Survey in progress

#### ‘h’: Survey complete, position hold

## Longitude

### Shows longitude in degrees, minutes, seconds.

### Last character will show the site survey status:

#### ‘ ‘: No survey

#### ‘s’: Survey in progress

#### ‘h’: Survey complete, position hold

## Master status

### If an external device is communicating with Oncore via the RS-232 port, clock will switch to SLAVE status.

### Reverts to MASTER status after 90 seconds of serial inactivity.

# Button 2 - Menu

## Cycles through available menus.

## When Display (button 1) is pressed, returns to previously selected display mode.

## Time zone change

### Use Select (button 3) to select time zone.

### To make selection permanent, commit with 2+3 combo.

### Message will confirm commitment to EEPROM.

## Error log

### Errors accumulated in EEPROM and are not erased on startup.

### On entry, will display the number of errors logged.

### Errors can be cycled through with Select (button 3).

### For each error, 20 decimal digits will be displayed.

#### First 10 digits are the internal clock time the error was logged

##### Format: MMDDhhmmss

#### If this is a time mismatch error, the second 10 digits will be the Oncore time.

##### Format: MMDDhhmmss

##### There must be 5 mismatches in a row before the error is logged, after which the internal clock will be re-synchronized.

##### Timer mismatch errors are suppressed for the first minute after startup.

#### If this is any other error, the second 10 characters will be repeating and indicate the error code:

##### 4444444444

###### Timer 1 (1PPS) timer overflow

##### 5555555555

###### Serial receive framing error

##### 6666666666

###### Serial receive overflow error

##### 7777777777

###### T1 (1PPS) edge out of phase with internal scheduler

### Reset the entire error log with 2+3 combo.

## Satellite status

### Use Select (button 3) to cycle between lines

### First line shows number of satellites tracked and DOP (Dilution of Precision) from 0 to 99.9

#### DOP will be 000 when site position is fixed

### Subsequent lines will show status of receiver channels

#### Channel number (1 through N)

#### GPS satellite ID number (eg: Sat7)

#### Channel tracking mode (eg: m8)

##### 0: code search

##### 1: code acquire

##### 2: AGC set

##### 3: preq acquire

##### 4: bit sync detect

##### 5: message sync detect

##### 6: satellite time available

##### 7: ephemeris acquire

##### 8: available for position

###### Satellite carrier to noise density ratio (s0-255 dB-Hz)

#### Channel status flags (‘d’ followed by two hex digits)

##### Bit 7: using satellite for position fix

##### Bit 6: satellite momentum alert flag

##### Bit 5: satellite anti-spoof flag set

##### Bit 4: satellite reported unhealthy

##### Bit 3: satellite reported inaccurate

##### Bit 1: using satellite for time solution

##### Bit 0: parity error

## CPU clock frequency

### Accuracy +/-64Hz

## Oncore Identity

### Use Select to cycle between lines of Oncore ID message.

### Lines longer than display will scroll back and forth automatically.

### Not available in SLAVE mode.

## Site location survey

### Asks if you want to start site survey.

### Use 2+3 combo to initiate survey.

### Survey takes several hours to complete.

### Not available in SLAVE mode.

## Oncore Self-test

### Use 2+3 combo to start self-test.

### Will usually result in errors being logged.

### Not available in SLAVE mode.

# Button 3 - Select

## Used to select between options in Menu.

## See individual Menu states for details.

# Combo 1+2 - Audible Tick Toggle

## Turn simulated WWV sounds on/off.

# Combo 2+3 - Commit

## Used to commit menu options.

## See individual Menu states for details.

# Combo 1+3 - Dial Sync

## Activate when dial second hand is straight up.

## Dial will freeze until the start of the next minute.