YA YAGI ANALYZER

Copyright 1994 by Brian Beezley, K6STI

All Rights Reserved

YA Yagi Analyzer...............................1

Running YA.....................................1

F/B and F/R....................................3

Frequency Menu.................................3

Height Menu....................................3

Elements Menu..................................4

Taper Menu.....................................4

Bracket Menu...................................4

Boom Compensation..............................5

Element Conductivity...........................6

Two-Element Yagis..............................6

Match Menu.....................................6

Save Menu......................................8

Algorithm Limitations..........................9

Notepad.......................................10

Graphs........................................10

Patterns......................................10

Options Menu..................................11

Changing Screen Colors........................12

Printing The Screen...........................12

Index.........................................13

---- YA YAGI ANALYZER ------------------------------------------

YA.EXE is a simple but accurate program for analyzing

Yagi-Uda antenna designs on your PC.

YA analyzes Yagis hundreds of times faster than NEC- or

MININEC-based antenna-modeling programs.  YA uses an analysis

algorithm that is more complex than the W2PV Yagi model, simpler

than the MININEC model, and more accurate than either.  YA is

calibrated to NEC, the high-accuracy Numerical Electromagnetics

Code from the Lawrence Livermore National Laboratory.  YA and

NEC typically are within 0.05 dB for forward gain, a dB or two

for F/B, and a couple ohms for input impedance.  Radiation

patterns calculated by YA and NEC are virtually identical for

most designs.

---- RUNNING YA ------------------------------------------------

If you have an EGA or VGA display, you can execute YA

directly.  For an HGC or CGA, first do the following:

HGC      Load the HERCULES.EXE graphics driver.  To do

this, type HERCULES at the DOS prompt.  Since

this must be done just once per boot, it's often

convenient to load HERCULES from AUTOEXEC.BAT.

CGA      Load GRAFTABL and GRAPHICS.  You can load

them from AUTOEXEC.BAT.  These DOS utilities

provide additional CGA fonts and let you print

CGA graphics screens.

YA stores information for Yagi designs in individual

files.  Yagi files contain element dimensions, analysis

frequencies, and other information.  You specify the name of a

Yagi file when you start YA.  After YA loads the file, you can

examine or modify the design.  You can save the current design

in a file at any time.  Yagi files use the extension .YAG.

If you know which Yagi file you want, give the filename

on the YA command line, for example, YA PV4 (the extension isn't

needed).  Otherwise, type YA and Yagi files in the current

directory will be listed.  To list Yagi files in a different

directory, specify the directory name on the command line.

Select a file by moving the lightbar with the arrow

keys, PgUp, PgDn, Home, or End.  Press Enter to select the

highlighted file.  Alternatively, you can enter a filename by

typing it.  As you type, the lightbar moves to the first

filename that matches the characters entered.  Press Enter

whenever the desired file is highlighted.

1

Select "Other" to enter a file or directory name not

listed.  If you enter a directory containing Yagi files, YA

lists them and you can select one.

Press Esc to hide the Main Menu to view the whole

screen.  Press Esc to return to the Main Menu from a submenu.

You can also return by pressing the command key a second time

(but not from Save).  This feature is handy when making a quick

check.

You can terminate data entry and return to the Main Menu

with one keystroke by pressing Esc instead of Enter.  If you do

this, whenever you return to the submenu the cursor will be

positioned at the item last entered.  This makes it easy to

change a parameter repeatedly to experiment with its effect.

You can exit most submenus (not Save or Notes) by

pressing a valid command key.  YA will execute the command

immediately, bypassing the Main Menu.  This is a quick way to go

from one submenu to another.  Whenever you press a command key

during data entry, YA accepts the data and then executes the new

command.

Use an arrow key with numerical input to enter data and

simultaneously move the cursor up, down, right, or left to

another item.  Use the spacebar to enter data without advancing

to another item.  This is particularly useful with the Match

Menu and Bracket Menu to change a parameter value repeatedly to

experiment with its effect.

To reduce screen clutter, YA does not label the numbers

within Yagi patterns.  They have the following meaning:

Frequency

Forward Gain

Front-to-Rear Ratio

Input Impedance

Standing-Wave Ratio

Elevation Angle

YA defaults to a generalized definition of front-to-back

ratio described in the next section.  YA displays elevation

angle only for Yagis over ground.

The notation 12.7-j15.4 means a resistance of 12.7 ohms

in series with a reactance of -15.4 ohms.  Z stands for

impedance.  The letter w appended to boomlength means

wavelengths.  The path and name of the current Yagi file appear

in the lower-right screen corner.

Although YA doesn't require a math coprocessor, it will

run up to 20 times faster when one is present.  YA may run up to

50% faster if you disable your memory manager.

2

---- F/B AND F/R -----------------------------------------------

YA uses a generalized notion of front-to-back ratio as a

measure of pattern quality.  Conventional F/B is the ratio of

forward power (at 0 degrees) to power radiated in the opposite

direction (at 180 degrees).  YA's generalized F/B is the ratio

of forward power to the peak power in the region 90 to 180

degrees to the rear of the antenna.  This pattern-quality

measure is called front-to-rear ratio.  The F/R backlobe region

is the rear half-plane for azimuth patterns and free-space

elevation patterns.  For elevation patterns of Yagis over

ground, the backlobe region begins at the rear horizon and

extends to the zenith.  In general, the F/R figure differs in

each analysis plane.

Yagi designs maximizing conventional F/B may have large

backlobes at angles other than 180 degrees.  These backlobes can

cause unwanted signal pickup, but F/R takes them into account.

YA uses F/R by default.  Use the Options Menu to select

conventional F/B.

---- FREQUENCY MENU --------------------------------------------

YA models Yagi designs at a spot frequency or over a

frequency band.  You specify a single analysis frequency for

spot-frequency designs, and low, middle, and high frequencies

for designs covering a band.  YA models Yagis to 2 GHz.

You can add, change, or delete analysis frequencies with

the Frequency Menu.  You must use one or three frequencies--YA

doesn't allow two.  To change to a spot-frequency design, enter

zero for the low or high frequency.

---- HEIGHT MENU -----------------------------------------------

Use the Height Menu to specify antenna height above

ground.  YA orients Yagis for horizontal polarization when

modeling over ground.  Set height to zero to model a Yagi in

free space.  Elevation angle is that at which YA computes

forward gain, F/R, and the azimuth pattern.

YA takes into account mutual impedances between Yagi

elements and their ground images.  This lets you see the effect

of ground proximity on azimuth patterns and input impedance.

However, YA models perfectly conducting ground, not real earth

with finite conductivity and dielectric constant.  Yagis over

real earth may have several dB less gain overhead and up to 1 dB

less gain near the horizon than the figures YA computes.

Elevation-pattern nulls may be less deep.  Nevertheless, you can

use YA to make accurate relative comparisons among Yagi designs

over ground.

3

---- ELEMENTS MENU ---------------------------------------------

Use the Elements Menu to change element positions and

lengths.  YA displays element half-length, the distance from the

center of the boom to element tip.  For tapered elements YA

displays the lengths of electrically equivalent untapered

elements.  These are not physical lengths and should not be used

for antenna construction.  Use the Taper Menu to inspect and

change physical lengths of tapered elements.

YA can model Yagis with up to 17 elements.  Use the Ins

and Del keys to add or delete elements.  Press Del to delete the

element at the cursor.  Press Ins to add an element below the

cursor.  When you add or delete an inner element, YA respaces

elements to avoid bunching or gaps.  When you add or delete an

end element, YA does not respace and the boomlength changes.

---- TAPER MENU ------------------------------------------------

Fast Yagi-modeling algorithms represent each element as

a thin cylinder of constant diameter.  A tapering algorithm is

necessary to convert elements of tapered telescoping tubing to

untapered cylindrical equivalents.  For practical purposes,

tapered and untapered elements are electrically equivalent when

their self-impedances are equal.  Tapered elements are always

physically longer than their untapered equivalents.  Tapering

must be accurately modeled to realistically characterize HF

Yagis.

YA uses a modified version of the tapering algorithm

developed by James Lawson, W2PV, from the ARRL book "Yagi

Antenna Design."  The modified algorithm yields untapered

equivalents with self-impedances much closer to those of the

tapered originals.  It also better predicts the real-world

performance of Yagis with heavily tapered elements.

YA can model Yagis with up to eight taper sections.  Use

the Taper Menu to change the diameter or length of any section.

For any length change except to the tip section itself, YA

readjusts tip length to maintain an electrically equivalent

design.  Use the Ins and Del keys to add or delete taper

sections.  You can enter a taper length of zero for elements

that don't use a particular tubing diameter.

---- BRACKET MENU ----------------------------------------------

A conductive element-to-boom mounting bracket increases

element effective diameter at the bracket.  The amount of

increase depends on the size and shape of the bracket.  YA uses

the equations of D. Jaggard to model flat, rectangular mounting

plates and Hy-Gain element clamps.  For details of the method,

see the ARRL book "Physical Design of Yagi Antennas" by Dave

Leeson, W6QHS.

4

Use the Bracket Menu to define or modify a mounting

bracket.  Select bracket type with the tab key.  After you enter

bracket dimensions, YA calculates the length and diameter of an

equivalent taper section.  The length of the section is the

half-length of the bracket.  Its diameter is electrically

equivalent to the element and bracket combined.

When you return to the Main Menu, YA updates all

elements of the current design with the new taper section.

Whenever the length of the existing first taper section is less

than 5% of the element half-length, YA assumes that the section

already represents a mounting bracket.  YA will modify the

section instead of adding a new one.

If you enter zero for a bracket dimension so that YA

lists "None" for the equivalent taper, when you return to the

Main Menu YA removes the first taper section if it represents a

mounting bracket (length less than 5% of the element half-

length).

If the Yagi uses mounting brackets of more than one

size, calculate equivalent taper sections with the Bracket Menu

and add them by hand to individual elements with the Taper Menu.

(YA uses reflector tapering to calculate the numbers displayed

in the Bracket Menu.  You may need to temporarily modify the

taper schedule of this element to do calculations for other

elements.)  Use a taper length of zero for elements that don't

use a particular taper diameter.

To avoid disrupting a complex taper schedule entered by

hand, YA will not update the taper schedule when you leave the

Bracket Menu if the length of the first taper section is zero

for any element.  It also won't update if some elements have

mounting brackets and others do not.

The length of a mounting bracket is its dimension along

the element.  Enter the total length (both sides of the boom).

The width of a bracket is its dimension perpendicular to the

element.  The thickness of a Hy-Gain clamp is the total

thickness of the flat part.

It's important to take element mounting into account

when modeling Yagis.  Large mounting brackets may significantly

alter antenna characteristics.  In extreme cases they can move a

desired response completely outside a band.  Small brackets can

upset a carefully optimized pattern at a spot frequency.

However, mounting methods that use compact hardware no larger

than the element diameter generally require no correction.

---- BOOM COMPENSATION -----------------------------------------

When an element is close to or passes through a

conductive boom, its electrical length changes.  YA does not

model this boom effect, but you can use the information in this

section to compensate manually.  All correction values apply to

5

element half-lengths.  Apply the corrections by increasing

calculated element lengths when constructing a Yagi.

Conversely, to model boom effects you must shorten measured

element lengths.

For elements mounted on flat plates in contact with a

boom, W2PV found that element half-lengths should be corrected

by 3% of the boom diameter.  He states that this small effect

diminishes rapidly as the element is spaced away from the boom,

even by a small distance.

For noninsulated through-the-boom mounting, measurements

by Guenter Hoch, DL6WU, were curve-fit by Ian White, G3SEK, to

yield the following boom-correction formula:

C = (12.5975 - 114.5 * B) * B * B

C is the element half-length correction and B is boom

diameter, both in wavelengths.  * means multiply.  For example,

a 0.01-wavelength-diameter boom requires element half-lengths to

be corrected by 0.00115 wavelength.  The experimental data

underlying this formula came from booms with diameters smaller

than 0.055 wavelength; the formula isn't valid for larger booms.

G3SEK says that the correction required for insulated through-

the-boom mounting is close to 50% of C.

---- ELEMENT CONDUCTIVITY --------------------------------------

YA accounts for ohmic losses in Yagis due to imperfect

element conductivity and skin effect.  These losses usually are

small for tubing elements at HF, but they can be significant for

rod elements at VHF/UHF.  YA assumes 6061-T6 aluminum-alloy

conductors for all models.  6061-T6 material has a volume

resistivity 23% higher than that of 6063-T832 alloy and 51%

higher than that of pure aluminum.

---- TWO-ELEMENT YAGIS -----------------------------------------

YA expects every Yagi to have a reflector element.  You

can model two-element Yagis with a driven element and director

by placing a dummy reflector a great distance behind the two

active elements.  This arrangement will satisfy YA while having

negligible effect on the response of the two-element array.

---- MATCH MENU ------------------------------------------------

The Match Menu lets you design a variety of matching

networks.  Use the tab key to select one of the following

network types:

6

1.  Perfect Match

The perfect match is an idealized matching network.

Because it's always perfectly matched at one frequency, it's

very handy.  You can set this frequency in the Match Menu.  SWR

is always 1 at the match frequency.  YA uses the complex

conjugate of antenna input impedance at the match frequency as

the SWR reference impedance at all frequencies.  This

effectively models a perfectly broadband matching network.

Therefore, SWR variation using the perfect matching network

reveals the inherent impedance-bandwidth properties of the

antenna.

2.  Hairpin Match

The hairpin match is an L-network using distributed

reactances.  The driven element is shortened from its resonant

length to raise the equivalent parallel input resistance to that

of the feedline.  The hairpin acts as a shunt inductance to

cancel the capacitive reactance of the shortened driven element.

Hairpin matches are commonly fed with coax through a balun.  YA

models the hairpin as a transmission line of parallel rods.

You specify rod diameter, rod length, and center-to-

center rod spacing.  You can also specify balun-lead length.

Enter the length of one lead from the axis of the balun to the

element-attachment point.  YA assumes the leads use #12 wire.

At HF, YA models the leads as fanning out from a spacing of 1.5"

at the balun to the hairpin-rod spacing at the element.  For

frequencies above 30 MHz, YA assumes the leads fan out from

0.15".  This lead spacing is typical of 3/8"-diameter coax

(presumably loaded with a few ferrite beads).

You can specify the shunt capacitance between driven-

element halves.  This is particularly useful with Hy-Gain

element clamps.  Use 26.5 pF for 20-meter clamps and 9.5 pF for

15- and 10-meter clamps.

Hy-Gain calls their version of the hairpin match a Beta

match.  It uses two rods which straddle the boom and a shorting

strap that connects all three together.  The presence of the

boom has a negligible effect on hairpin inductance, so a Beta

match is electrically equivalent to a conventional hairpin

match.

NOTE:  When using a hairpin match, measure driven-

element half-length from the point where the feedline and

hairpin attach to the element, not from the center of the boom.

3.  Gamma Match

YA uses equations developed by Harold Tolles, W7ITB, to

model gamma matches.  You specify gamma-rod diameter, rod

length, center-to-center spacing from the driven element, and

7

series capacitance.  To model lead inductance, specify the lead

length from the gamma rod to the coax connector.  YA assumes the

lead uses #12 wire.  When the lead runs parallel to the driven

element, include its length as part of the gamma rod rather than

specifying it separately.

4.  T Match

A T match is a balanced gamma match.  Rod length and

capacitance values apply for each side of the driven element.

Some T matches are fed with 50-ohm coax through a 4:1 balun.

Enter a feed impedance of 200 ohms for these systems (which

often have negligible lead length).  You can also feed a T match

with wire leads from a 1:1 balun.  YA makes the same assumptions

about these balun leads as it does for a hairpin match.  YA

assumes the distance between T-match rod ends is half their

distance to the driven element.  To model a T match without

capacitors, enter a high capacitance value.  To model a folded

dipole, extend the T-match rod to the end of the element.

Gamma- and T-match rods increase the effective diameter

of the inner part of a driven element.  Because YA does not

model this effect, you may need to compensate by increasing

driven-element length when constructing a Yagi.

YA uses a velocity factor of 0.975 for the transmission

lines formed by gamma- or T-match rod and driven element,

hairpin rods, and balun leads.  For driven-element diameter, YA

uses a weighted logarithmic average of the diameters of the

taper sections spanned by gamma- or T-match rods (diameters are

weighted by taper-section length).  YA draws rod length to scale

on the Yagi sketch, but not rod spacing.

Matching-network performance depends critically on a

number of physical parameters that are difficult to measure and

to model accurately.  Calculated and actual matches may differ

due to input-impedance modeling error, series- or shunt-

capacitance estimation error, lead-spacing variation, proximity

of other antennas or guy wires, etc.  When constructing a Yagi,

always make the matching network adjustable!

---- SAVE MENU -------------------------------------------------

Use the Save command to save the current Yagi design.

You can specify path and filename.  If you enter the name of a

Yagi file that already exists, YA asks permission before

overwriting it.  For fast periodic saves, press Enter to

overwrite the last file saved (YA doesn't ask permission).  When

no file has been saved, YA saves the design in SAVE.YAG.  YA

does not save matching networks or mounting-bracket dimensions.

In case you forget to save a design, YA automatically

saves the current design in the file OUT.YAG whenever you exit

8

the program.  You can recover the design by specifying OUT.YAG

as an input file.

---- ALGORITHM LIMITATIONS -------------------------------------

YA closely predicts actual antenna performance as long

as you accurately characterize element tapering and mounting and

you follow the guidelines in this section.

YA is calibrated to NEC, the Numerical Electromagnetics

Code.  MININEC-based antenna-analysis programs may give

different results.  The MININEC algorithm has an inherent

frequency-offset error.  MININEC gives results similar to those

of YA and NEC but at a frequency up to 2% higher, depending on

element diameter.

YA closely tracks NEC for element diameters up to 0.01

wavelength (0.27" at 432 MHz).  Yagis with thicker elements may

exhibit some frequency offset from calculated designs, although

YA gives reasonable agreement with NEC for diameters up to about

0.04 wavelength.

YA uses a narrowband modeling algorithm.  Accuracy may

degrade at frequencies more than about 5% away from the central

design frequency.

YA models with input impedances of a few ohms are not

likely to be accurate.  At these impedance levels element

currents are very large and fields nearly cancel.  This

condition greatly magnifies small model inaccuracies.  Low-

impedance Yagis should be avoided for practical reasons as well.

Dimensions become critical, skin effect can cause considerable

loss, impedance matching becomes difficult, and bandwidth is

severely restricted.

Input-impedance accuracy decreases for maximum-gain or

long-boom designs.  YA is typically a couple ohms high for

maximum-gain designs with impedances under 10 ohms.  YA is

typically a few ohms low for designs with a dozen or more

elements and impedances in the 20- to 30-ohm range.

Modeling accuracy may degrade for elements spaced closer

than about 0.05 wavelength.  In practice, HF elements spaced

this close may move enough in the wind to affect the response of

critical designs.  YA may exhibit a frequency offset of several

percent for designs with element half-lengths shorter than about

0.18 wavelength.  Element lengths this short are unusual and

normally aren't used in designs of 17 or fewer elements.

To prevent gross modeling errors, YA won't let you enter

element half-lengths shorter than 0.15 wavelength or longer than

0.3 wavelength.  It won't let you position elements closer than

0.03 wavelength.  Finally, it won't let you enter taper

diameters greater than 0.05 wavelength.

9

---- NOTEPAD ---------------------------------------------------

You can enter, edit, and display design notes with the

Notes command.  YA displays the Yagi title at the top of the

notepad and you can edit it, too.  The title and notes are saved

in all output files.

The notepad editor automatically wraps words within a

paragraph.  A blank line or a line with leading space begins a

new paragraph.  Use the arrow keys, Home, End, PgUp, and PgDn to

move the cursor.  Use Del or Backspace to delete characters and

Alt-D to delete a line.  Use PrtSc to print the notepad screen.

---- GRAPHS ----------------------------------------------------

YA graphs forward gain, F/R, SWR, and impedance curves

versus frequency.  Regular command keys are active while viewing

graphs.  For example, you can press M and G to alternate between

the Match Menu and graphs when adjusting a matching network.  F5

changes screen colors, while PrtSc prints the screen.

YA scales graphs automatically.  The F/R or F/B curve

reflects the backlobe region selected in the Options Menu.  The

impedance curve graphs the real part of input impedance.

---- PATTERNS --------------------------------------------------

YA displays patterns using the standard ARRL log-dB

polar scale.  Half-scale is about 12 dB down and quarter-scale

is about 24 dB down.  You can directly compare YA patterns with

those in ARRL publications since both use the same scale.  To

reduce screen clutter, YA draws pattern sketches without grids

or scale markings.  The sketches have 5-degree resolution.

Use the Plot command to generate high-resolution

patterns with grids, scale markings, and annotation.  YA

calculates these patterns at the middle analysis frequency with

1-degree resolution.  The Plot command generates a plot file and

then displays it.  YA saves patterns in files so that you can

review and compare them later.  A plot file uses the Yagi

filename and the extension .PLT.  If you plot, change antenna

geometry, and press P again, YA overwrites the plot file with

new patterns.  But if you press Alt-P, YA generates a new plot

file.  Each new file has an incrementing digit appended to its

name.  Use Alt-P to avoid overwriting plot files generated

earlier in a session.  (For both P and Alt-P, YA writes a plot

file only when the pattern has changed.)

You can review a plot from DOS by typing YA PLOT.  This

bypasses the analysis part of YA and lists plot files in the

current directory.  Specify a directory name to list plot files

in a different directory.  You can specify the name of a plot

file, for example, YA PLOT PV4, to display a pattern

immediately.

10

Use the O key to overlay two patterns.  YA displays

filenames instead of plot titles for overlays.  YA coordinates

overlay annotation with patterns by color for EGA/VGA systems.

YA identifies free-space patterns as E-Plane or H-Plane.

It uses the annotations Azimuth or Elevation for over-ground

patterns.

Dots in the sparse radial lines are spaced 2 dB apart.

Dots in the outer two circles are spaced 1 and 2 degrees apart.

These calibrations allow you to read directivity values from

plots with good accuracy.

YA plots are perfectly circular on monitors with

standard 4:3 aspect ratio.  Adjust your monitor's vertical-

height control to correct elliptical plots.

---- OPTIONS MENU ----------------------------------------------

The Options Menu lets you control some additional

aspects of YA.

The E-plane is the plane containing Yagi elements.

Select this plane to analyze the azimuth pattern of a Yagi over

ground.  The H-plane is the plane perpendicular to the elements.

Select this plane to analyze the elevation pattern of a Yagi

over ground.

YA displays gain figures in dBi by default.  Gain in dBi

is antenna response compared with that of an isotropic radiator

in free space.  An isotropic antenna radiates uniformly in all

directions.  Select dBd to compare antenna response with that

broadside to a free-space, half-wave dipole.  Gain figures are

2.15 dB lower when expressed in dBd.

YA uses a backlobe region of 90 to 180 degrees for the

pattern-quality figure by default (F/R).  For conventional F/B,

select a backlobe region of 180 degrees.

YA defaults to a 24-pin printer.  Select Laser for

DeskJet printers.  YA does not print color images.

YA uses units of inches by default.  Select mm to use

millimeters.  YA reads Yagi files defined either way and

converts file dimensions to the units currently selected.

You can save all settings in the Options Menu (and all

screen colors for EGA/VGA systems) with the Save Options

command.  This creates or modifies the YA.INI initialization

file.  If this file is present when YA begins execution, it sets

all options and colors accordingly.  Use this feature to

customize YA.  Delete YA.INI to use default options and colors.

11

---- CHANGING SCREEN COLORS ------------------------------------

Press F5 to change screen colors for EGA and VGA

systems.  Use the left-arrow and right-arrow keys to select a

screen item (an item blinks once when selected).  Then use the

up-arrow and down-arrow keys to cycle forward and backward among

the 64 available colors.  Press S to save color codes in the

YA.INI file.  Use F5 to change colors for any screen.  YA saves

colors for all screens whenever it saves those for one.

---- PRINTING THE SCREEN ---------------------------------------

YA prints screen images on most printers (but not

PostScript printers).  For EGA, VGA, and HGC systems, use the

Options Menu to specify and save printer type.  For CGA systems,

load the DOS GRAPHICS program before starting YA.  See your DOS

manual to specify printer type when using GRAPHICS.  Press PrtSc

to print any screen.

12

INDEX

#12 wire  7

5% of the element half-length  5

6061-T6  6

6063-T832  6

Accuracy  1, 9

Algorithm Limitations  9

Alt-D  10

Alt-P  10

Aluminum  6

Always make the matching network adjustable!  8

Arrow keys  1, 10

AUTOEXEC.BAT  1

Azimuth  11

Backlobe region  3

Backspace  10

Balun  8

Balun-lead length  7

Blink  12

Boom Compensation  5

Boom-correction formula  6

Bracket Menu  5

CGA  1, 12

Color  10, 11

Command line  1

Conductivity  3, 6

Conventional F/B  3

Current directory  1

DBd  11

DBi  11

Del  4, 10

Dielectric constant  3

Directory  1, 10

DL6WU  6

Driven-element diameter  8

Dummy reflector  6

E-Plane  11

Effective diameter  8

EGA  1, 11, 12

Elements Menu  4

Elevation  11

Elevation angle  3

End  1, 10

Enter  1, 2, 8

Esc  2

F/B  3

F/R  3

F5  10, 12

13

Folded dipole  8

Forward gain  1

Free space  3

Frequencies above 30 MHz  7

Frequency band  3

Frequency-offset error  9

G3SEK  6

Gamma match  7

GRAFTABL  1

GRAPHICS  1, 12

Graphs  10

Ground  3

H-Plane  11

Hairpin match  7

Height Menu  3

HERCULES.EXE graphics driver  1

HGC  1, 12

Hide the Main Menu  2

High-resolution patterns  10

Highlighted file  1

Hoch  6

Home  1, 10

Hy-Gain element clamps  7

Idealized matching network  7

Impedance  2, 3, 4, 7, 8, 9, 10

Initialization file  11

Ins  4

Insulated through-the-boom mounting  6

Isotropic antenna  11

Jaggard  4

Lawrence Livermore National Laboratory  1

Lawson  4

Lead length  8

Leeson  4

Lightbar  1

Logarithmic average  8

Main Menu  2

Match frequency  7

Match Menu  6

Math coprocessor  2

Memory manager  2

MININEC  1, 9

Mounting bracket  4

Mutual impedance  3

NEC  1, 9

Notes  10

Numerical Electromagnetics Code  1, 9

14

Options Menu  11

Other  2

OUT.YAG  8

Overlays  11

Peak power  3

Perfect match  7

PgDn  1, 10

PgUp  1, 10

Plot file  10

PLT  10

Polar scale  10

Polarization  3

Printer  11, 12

PrtSc  10, 12

Quick check  2

Respaces elements  4

Return to the Main Menu  2

Rod diameter  7

Rod length  7

Rod spacing  7

Save Menu  8

SAVE.YAG  8

Self-impedance  4

Shunt capacitance  7

Skin effect  9

Spacebar  2

Spot frequency  3

Submenu  2

SWR  7

SWR reference impedance  7

T match  8

Tab key  5, 6

Taper Menu  4

Tapering algorithm  4

Terminate data entry  2

Through-the-boom mounting  6

Tolles  7

Two-element Yagis  6

Velocity factor  8

VGA  1, 11, 12

W2PV  1, 4, 6

W6QHS  4

W7ITB  7

White  6

YA PLOT  10

YA.INI  11, 12

Yagi files  1

Yagi title  10

Z  2

15