Transmission Line Impedance Matching Program

INTRODUCTION

Performs the mathematics for transforming complex impedance sets

that represent actual or theoretic loads.

Typically one obtains up to 21 measurements at equally spaced

frequencies across an amateur band.  The data may be entered via this

program, be prepared by an ascii editor, or sent to a disk file after

entering.  Once the data is in disk file, it may be used in future

sessions to explore various matching approaches.

The operator makes all decisions and the PC does the calculation and

plotting.

Transmission lines RG 8, RG 11, RG 58, RG 59 are accurately represented

including attenuation when used either as series elements or as stubs.

Series and parallel LC sections may be placed either in series with the

center conductor, or in parallel with the coaxial cable.  Single L or C

elements may be used also.

Transformers  and Inductors are represented as ideal devices (Lossless

and no stray components)

Design results may be dumped to a file for later printing(after exiting).

Final or intermediate impedance values may be deposited into a disk

file which may be the starting place for a subsequent session.

Intermediate impedance values may be shown on the screen as tabular

values(complex numbers).

OPERATING PROCEDURE

Start from DOS by entering MATCH.

Shortly, a prompt will appear which allows you to select the basic system

impedance.  Pressing the "enter" key will enter the default value of 50 ohms.

Otherwise, the value you enter will be the system impedance for the duration

of the session.

VSWR Prompt.  This prompt has a default value of 2.0.  It is used to

plot a constant radius circle on the SMITH chart and to place a target

line on the VSWR plot.  Other values can be entered.

LASERJET PROMPT.  The next prompt asks whether the print files are

intended for either LASERJET III or IV.  Either one causes the printout

to include a SMITH chart impedance plot along with the VSWR.  Further,

there is a distinction between the III and IV versions.  For some

reason the III does not support as much printout on the SMITH chart as

does the IV.  If neither LASERJET is expected,  select No.  The

printout still contains the impedance and VSWR values and a crude VSWR

plot.

FINALLY, the operating screen will appear with another prompt, the

DISK FILE option.  This option allows you to work with data that have

been saved to disk prior to starting the session.  If you answer "yes",

it will show you all the files in your directory that have a .CIR

extension.  Then it waits for you to enter one of the names of available

files. (no need to enter the extension).  If you answer "no", it will

prompt you for the frequency and impedance values.

At this point, whether you selected a disk file of data or entered new

data, the top portion of the screen will be blank  and there will be a

skimpy SMITH chart along with a VSWR plotting area.  The impedance set

you entered(or brought from disk) will be plotted on the SMITH chart

and the corresponding VSWR will be plotted on the left hand portion of

the bottom section.

OPERATING THE PROGRAM (RUNNING THE SESSION)

The program operates interactively with the operator.  There are

various matching devices that one may want to apply in each situation;

These are selected by use of the function keys usually additional

options are required from the keyboard to complete the selection.

Functions F1,F2,F3,F4 and F6 automatically initialize the components at

a value which provides the amount of "ohms" impedance equal to the

system impedance. For instance, if we are operating with a 50 ohm

system and select F1 S, a capacitor with 50 ohms reactance(at mid band)

will be placed in series with the load.  In the top panel, the device

#1 will be a series capacitor of ?? pf.  The correct reactance will be

used at all other frequencies.

F1      Capacitor   S)eries  P)arallel

F2      Inductor    S)eries  P)arallel

F3      LC Series   S)eries  P)arallel

F4      LC Parallel S)eries  P)arallel

F5      Stub Line   S)horted O)pen    Length in Wavelengths

F6      Resistor    S)eries  P)arallel

F7      Transformer R)atio

F8      Series Line D)irect  R)everse Length in Wavelengths

Chances are, that 50 ohms is not the actual value we would want in the

device.  So we have a way of adjusting the size of the component.

ADJUSTMENT OF COMPONENT VALUES

At the bottom left of the screen are displayed a pair of functions.

Device and Total devices.  These are to keep you informed of which one

of your selected devices, you have current control of, and how many of

them there are in the system.

At the bottom right of the screen are normally four other selections

available:

D)isp      causes the input impedance array to the current device

to be tabulated in the upper panel.  It goes away as

soon as a key is pressed.

N)ewdata   causes the program to go back to allow you to select

a new data set.

O)utdata   causes the output data from the current device to be

sent to a disk file (You will be promped for a

filename)

P)rintfile Causes the entire design to be written to a diskfile

(you will be prompted for an extension-- it will use

the basic filename of your original impedance set)

Pageup/pagedn keys select the device over which controls apply

Up/Dn arrows increase/decrease the component value of the device.  For

instance, if a  305 pf. Capacitor is the current device, and one presses

" Up Arrow " the capacitor would be increased to 335.5 pf (+10%).  In

case the section has two components(CL), the Right/Left arrows increase

and decrease the second component.

Transmission line sections are increased and decreased linearly by .005

wavelengths per step.

Whenever the size of the incremental step needs to be changed, F9 and F10

affect the delta size for first component.  Home/End raise and lower the

size of delta for the second component.

Insert/Del keys cause the implied action. whenever an insertion is done,

a Zerolength series line is inserted.  The operator may change the device

by merely going through the device setting procedure(using the function

Keys)

A COUPLE OF  REAL EXAMPLES:

1.  80 Meter full wavelength triangular loop antenna.  The data were

obtained by using a noise bridge and lots of painful algebra.  Anyway,

the following impedance readings were obtained:

Freq       real     imag     VSWR

3.500      84       -j170    9.05

3.550      86       -j138    6.58

3.600      88       -j110    4.87

3.650      90       -j96     4.16

3.700      98       -j39     2.36

3.750      101       j0      2.02

3.800      114       j34     2.53

3.850      122       j63     3.19

3.900      143       j130    5.39

3.950      145       j172    7.19

4.000      190       j216    8.00

These values are contained in a diskfile called loop80.cir on the disk.

Match the antenna for VSWR under about 2.0 across the whole band.

o From a DOS prompt start the program >"match"

o Answer the first prompt with an "ENTER" (if we wanted to use some

other impedance cable than 50 ohms, we would enter it like, 75).

o Likewise, on the VSWR question.

o If you have either a HP LASERJET III or IV "Enter" else "n enter".

o Likewise here. it only affects how the output printout looks. It does

not affect the way the program calculates or displays on the screen.

o On the disk file question, just "ENTER", then it will show you all the

files on the directory that have "*.cir".

o Respond with "fs80 ENTER", shortly there will be semi-circular

impedance plot on the smith chart on the display. Also, nootice the

VSWR plot on the left.

o Start the solution by tapping "F8" to request a series section of

cable.  You are prompted for whether the line is D)irect or R)everse

direction.(normally use direct unless you want to go toward the

antenna instead of from it)  Tap the "D" key.

o Next prompt is for type of cable.  "1" for RG8/U

o Next prompt is for the approximate length in Wavelengths. ".6ENTER"

o Now you should see a second plot ( the result of having put the 0.6

wavelength of RG8/U in between the antenna and the place of

observation) also, notice the information at the top left corner.

Tap the "uparrow" key once or twice and notice that the line segment

is lengthened by .005 wavelengths per click.  Try the "downarrow" key

and see that the line is adjusted accordingly.

o Adjust it to .665 wavelengths. (this will lead to a good match)

o Tap the "pageup" key to advance us to the next device.

o Choose "F5" to select a stub.  It will prompt for S)horted or O)pen.

Tap "S" It will ask for cable type "1" for RG8/U again. it will ask

for length in wavelengths. ".1ENTER"

o This completes the match.  Notice that the impedance plot just about

lies within the VSWR = 2.0 circle.

o You may see the resulting impedance set by tapping "D" the data are

shown in the upper window.  pressing any key will remove the data from

the screen.

o If you tap the "P" key you will be asked for the 3 letter suffix (Wed

for instance). this will cause a report to be written to the disk

under the name "FS80.wed".  The report can be printed as ascii to a

printer.