FlexNet-Treiberdokumentation    PCISCC.EXE         (C)2001 St.Koehler DH1DM

Stand: 01/2002

Der Treiber wurde speziell f�r PCI-Bus SCC-Karten entwickelt, die mit

dem DSCC4-Controller PEB-20534H der Firma Infineon ausger�stet sind.

Einschr�nkungen:

- Der Treiber kann Zugriffe auf den SCC-Controller nur im Protected Mode

ausf�hren (PCI Memory Mapped IO). Aus Performance-Gr�nden wird momentan

die Umschaltung aus dem virtuellen Modus (VM86) nicht unterst�tzt, was

die absolute Unvertr�glichkeit des Treibers mit EMM386 und Windows zur

Folge hat.

- Die TX-Delay Messung beim Empfang wird momentan nicht durchgef�hrt.

Treiberoptionen:

/c=<num> w�hlt eine PCISCC-Karte aus, falls mehrere Karten installiert sind

ohne Angabe wird die PCISCC-Karte 0 ausgew�hlt

/t=<ms>  setzt die TX-Delay Skalierung, standartm��ig werden 10ms angenommen

----------------------------------------------------------------------------

FlexNet driver documentation    SCCDRV.EXE         (C)2001 St.Koehler DH1DM

Date: 01/2002

The driver was developed specifically for PCI bus based SCC cards, that

are based on the PEB-20534H DSCC4 controller by Infineon.

Limitations:

- The driver is only capable to access the SCC controller in protected mode

(PCI memory mapped IO). Because of performance reasons the mode switch

from the virtual mode (VM86) is currently not supported, which makes the

driver unusable in a EMM386 or Windows environment.

- TX delay is currently not measured.

Command line options:

/c=<num> select a PCISCC card if more than one is present

default card number is 0

/t=<ms>  set TX delay scaling, default value is 10 ms