The Yagi Optimizer

YO.EXE is a Yagi-Uda antenna analysis and optimization program for

IBM-PC and compatible computers.  YO will analyze a Yagi hundreds of times

faster than general-purpose antenna modeling programs, such as MININEC or MN.

A fast Yagi model is implemented with high accuracy for forward gain and

overall pattern, and good accuracy for F/B and input impedance.  The Yagi model

used in YO is significantly more complex than the well-known W2PV Yagi model.

YO includes an algorithm for automatically optimizing the forward gain,

radiation pattern, and input impedance of a Yagi.  This algorithm

simultaneously adjusts all element lengths and spacings to iteratively converge

an antenna design to an optimal set of dimensions, using performance criteria

and objectives which you specify.

The Yagi analysis algorithm used in YO employs a method-of-moments

solution for element currents.  It includes correction factors to prevent

convergence to supergain geometries, and to more closely match the YO-computed

gain and pattern to those obtained with MN.  Compared with MN, the accurate

arbitrary-geometry antenna modeling program based on MININEC, YO is typically

within 0.1 dB for forward gain, within a few dB for F/B, and within a couple of

ohms for input impedance.  Patterns produced by YO and MN for the same Yagi

usually appear nearly identical.  Occasionally, F/B differences of 10 dB or

more will occur for unusual Yagis which YO computes as having all backlobes

more than 30 dB down.

The speed of the YO Yagi analysis algorithm permits several Yagi

iterations per second to be performed for small Yagis (8 MHz XT with 8087).

Using YO, a small beam can be completely optimized for gain, pattern, and SWR

over a frequency band in just a few minutes.  For antennas having many

elements, such as long-boom VHF Yagis, the optimizer may be run unattended with

constraints to automatically find the best antenna -- while you sleep, if

necessary.  Yagis having up to 50 elements can be modeled.

YO will analyze a Yagi at a spot-frequency or at the low, middle, and

high frequencies of a band of interest.  It will update plots on your screen of

the far-field radiation patterns at each frequency while it iterates the Yagi

dimensions, and will continuously display the forward gain, F/B, input

impedance, and SWR.  This running graphics/alphanumeric display lets you

interactively direct the optimization.  You may interrupt the optimization at

any time to save the current design or to change optimization strategy.

System Requirements

YO.EXE requires an IBM-PC or compatible computer with 384K bytes of

memory.  You'll need 512K bytes if you invoke the high-resolution PLOT.EXE

plotting program directly from YO.  A Color Graphics Adapter, Enhanced Graphics

Adapter, or Hercules Graphics Card is necessary.  YO will run much faster if a

math coprocessor chip is installed, but one is not required.  A hard disk is

not necessary.  An editor or wordprocessor program is needed to create or

modify Yagi files.  DOS 3.00 or later is recommended in order to avoid certain

bugs in earlier DOS versions which may impact YO.

Input Files

YO reads the geometry of a Yagi from an input text file.  The file may

be in one of three formats.

The YO file format uses an easy-to-read table for Yagi dimensions,

specifies one or three analysis frequencies, and allows a separate tapering

schedule to be used for each element.  YO files always have the extension .YAG.

YO will also read files in MN format.  These files always have the

extension .ANT.  You may analyze and optimize a Yagi which you have previously

examined using MN.  MN-format files specify only one frequency.  The elements

must not be tapered to be read correctly by YO, and they must be listed in

order, starting with the reflector.  The Yagi must be aimed in the +X MN

direction, but it may have any polarization.

The last file format supported is that used by the N6BV Yagi analysis

programs.  Two frequencies, the low and high band limits, are specified in the

file, and independent tapering is allowed for each element.  Many antenna

designs, including commercial antennas and high performance contest arrays, are

available in N6BV format.

Yagi files are always ordinary DOS text files composed of ASCII

characters.  You may create your own Yagi files by using any text editor or

wordprocessor.  Refer to the YO file format description given later.  If you

use a wordprocessor to create Yagi files, be sure to have it generate a

standard DOS text file instead of a file in wordprocessing format.  Files in

wordprocessing format contain embedded control codes which will confuse YO.

Output Files

When you exit YO, the current design will be output using one of the

three file formats.  The output file format can be selected with the Other

Parameters command.  The name of the output file is always OUT.YAG, OUT.ANT, or

OUT.BV, depending on the format selected.  You may also save an intermediate

design without quitting YO by using the Save This Design option.  The name of

the first file saved is OUT1.* and the digit is incremented each time another

design is saved during a YO run.  Don't forget to rename the output files

before your next YO run if you want to save them, or they may be overwritten.

When making an MN-format file YO will specify 24 analysis segments per

element if possible.  The self-impedance computed by MN for dipole elements is

most accurate when 24 segments are used.  If the Yagi has six or more elements,

so that the MN limit of 126 pulses per antenna would be exceeded, YO will

partition the available segments evenly among the elements.  Then it will

allocate any leftover segments beginning with the reflector and proceeding

forward along the boom.  If you want to reduce the MN analysis time you may

lower the number of segments in the MN-format file by editing the file.  For

MN-format files the center of the Yagi boom is put at X=0.  This facilitates

studies of stacked dissimilar Yagis using MN.

Before Running YO ...

Before running YO do the following, according to your display type:

HGC:	The HERCULES.COM graphics driver must be loaded.  To do

this, simply enter HERCULES at a DOS prompt.  This

needs to be done only once per boot, so HERCULES may be

put in your AUTOEXEC.BAT file for convenience.

CGA:	Load GRAFTABL and GRAPHICS once per boot (you may put

them in your AUTOEXEC.BAT file).  These are standard DOS

programs.  They will improve the CGA graphics characters

and allow you to transfer the CGA plots to your printer.

EGA:	The EGA requires no preliminaries.

When executed, YO automatically determines which type of graphics

display is present, and then configures itself accordingly.  The graphics

features of YO vary according to the display type.  For example, screen writing

is visible on the CGA during plot updates because the CGA has only one graphics

buffer.  For the EGA and HGC, YO composes a new screen in one graphics buffer,

while the current screen is being displayed from another, and then switches the

screens instantly.  This gives the illusion of motion.  The patterns appear to

be objects which change shape, rather than sequences of static drawings.  Color

is not used on the YO screens, but the high-resolution PLOT program will

produce color plots on the EGA.

Starting Up YO

You may specify a Yagi filename on the command line when you invoke YO,

for example, YO 204SSB.  If you know the filename you want this is the quickest

way to get going.  Otherwise, YO will list the *.YAG files available, and then

prompt you for a filename.  If you enter <Esc> YO will terminate and return you

to DOS.

The following special command line sequence will not be interpreted as

a Yagi filename specification:

YO PRINT Filename

For this special sequence only, YO will format and print the text file

specified and then terminate.  You may use upper or lower case for the PRINT

keyword.  The documentation files supplied with YO are formatted for direct

viewing on your screen, allowing a quick paperless reference.  YO PRINT will

add margins and pages while printing these files.

Normally you do not need to specify a file extension or a directory for

Yagi files.  If neither is specified YO will first look for the file in the

current directory in YO format using the extension .YAG.  If the file is not

found it then looks for the file in MN format using the extension .ANT.  If

still not found it looks for the file in N6BV format using no extension.  If

the file cannot be located in the current directory the whole process is

repeated using the Yagi library directory (see below).

If you specify a file extension YO looks for that exact file, in both

the current and library directories.  If you specify a drive or path, YO looks

for the file only in the directory specified.  You may specify an extension if

necessary to resolve ambiguities, or when analyzing an N6BV file where the

extension usually denotes the Yagi boom length in feet.

After making a menu selection, <Esc> is used to return to the main

menu.  It is used both for normal returns and to abort an operation.  If you

have not entered anything, pressing <Esc> during a data entry operation will

cancel the operation and return you to the main menu.  If the data has been

typed but not entered, pressing <Esc> will enter the data, terminate the

operation normally, and return you to the main menu.

<Esc> is also used to get rid of the main menu in order to reveal the

whole screen.  The menu can be brought back by pressing any key other than

<Esc>, and if this key corresponds to a valid command, the command will be

executed.

For menu selections involving numerical input you may use an arrow key

to enter the number and simultaneously advance to a new item.  <Enter> is used

to enter a number and remain at the item selected.  This is useful for changing

the value of one item repeatedly to experiment with its effect.

When YO asks a question you may press "y" for "yes" and any other key

for "no", except the <Esc> key.  <Esc> will cancel the operation and return you

to the main menu.

The numbers listed inside each of the Yagi patterns are not labeled in

order to reduce screen clutter.  The numbers have the following meanings:

Frequency

Forward Gain

F/B or F/Worst Backlobe

Input Impedance

SWR

The Yagi count in the lower left corner is incremented each time a new

Yagi evaluation is performed.  The evaluations which do not result in a pattern

update are either part of the gradient computation described later, or are

iteration trials which do not result in improved antenna performance.  The

complete filename, including path and extension, appears in the lower right

corner.

Frequencies

YO handles either spot-frequency Yagi designs or designs intended to

operate over a frequency band.  A single frequency is specified for

spot-frequency designs, while low, middle, and high frequencies are specified

for designs covering a band.  The middle frequency should be between the low

and high frequencies, but otherwise there are no restrictions.

In YO-format files you may specify one or three frequencies.  In N6BV

files only the low and high frequencies are specified, and YO uses the

geometric mean of the two as the middle frequency.  This is the way the N6BV

Yagi programs treat the two frequencies.  MN-format files have only one

frequency, and this is used to specify a spot-frequency by YO.

You may add, change, or delete frequencies with the Frequency command.

This command changes only the analysis frequencies; it does not rescale the

beam dimensions.  You must use one or three frequencies, two are not allowed.

To convert to a spot-frequency analysis enter 0 for either the low or the high

frequency; YO will then set both frequencies to 0.  It takes longer for YO to

iterate a design over a frequency band than at a spot-frequency.  You should

always use just one frequency for spot-frequency designs to get the fastest

iteration cycles.

A Yagi may be modeled at any frequency with YO.  Frequencies of 1 GHz

or more will cause frequency digits to be displayed with a leading % character,

indicating screen field overflow, but otherwise YO will operate normally.

Frequency fields in output files will not overflow until 10 GHz.

Pattern Optimization Criteria

YO can use the standard front-to-back power ratio (F/B) as a quality

measure for Yagi patterns during automatic optimization.  A second pattern

quality criterion is also available.  This is the ratio of the forward power to

the total power in the rear half-plane (F/R).  Designs maximizing F/B often

have large backlobes -- they are just not at 180 degrees.  When used as a

pattern quality measure during automatic optimization, the F/R criterion yields

antennas with better overall patterns.  This criterion prevents large lobes

from appearing anywhere to the rear of the antenna, including at 180 degrees.

YO measures the rear power by computing the radiated power every 5

degrees and then summing the terms from 95 to 180 degrees.  You may select

either the F/B or F/R pattern optimization criterion with the Other Parameters

command.  You may choose to have either F/B or the ratio of the forward power

to that of the worst backlobe displayed numerically for each frequency.  The

latter display seems more appropriate when using the F/R design criterion, and

it can be related directly to the patterns you see on the screen.  Try

optimizing the patterns of several antennas using both the F/B and F/R methods

to get a feel for the differences involved.

Simple Optimization

You can direct YO to maximize forward gain, optimize the radiation

pattern, or increase the input impedance.  The latter also reduces the SWR

across a frequency band, so this operation is referred to as "minimizing the

SWR" when three frequencies are specified.  For each of these objectives YO

will employ the Method of Steepest Descent, an iterative optimization technique

described later.  You may select one of these performance objectives, run for a

while, and then select another and iterate with it.  This simple optimization

mode is useful for exploring the characteristics and performance tradeoffs of a

particular Yagi configuration, and in finding limiting values.  However, in

general, optimizing an antenna solely using one of these criteria will not

result in a satisfactory design for the following reasons:

In general, maximum-gain antennas are characterized by very low input

impedance, small SWR bandwidth, a narrow front lobe, and a high-amplitude rear

lobe.  Some maximum-gain designs that YO will find will have an input impedance

of only an ohm or two.  These are wholly impractical antennas, but they do

serve as useful reference points when determining the amount of gain you must

give up to get a realizable design.  Other maximum-gain designs having input

impedances in the 5-10 ohm region, while still high-Q antennas, are quite

practical for narrowband applications, such as use on the 12 meter HF band or

for weak-signal work at 144.200 MHz.

Antennas optimized for best pattern are generally good all-around

antennas for general-purpose work.  They are often selected for receiving

applications.  (However, in situations involving omnidirectional or random

direction noise or QRM, an antenna designed for maximum gain will deliver a

higher average receive S/N than any other antenna design).  Most commercial

Yagis are optimized for best F/B.  Antennas designed for best pattern tend to

have 1-2 dB less gain than maximum-gain designs, reasonable input impedances,

broad front lobes, and low-amplitude rear lobes.  However, it is frequently the

case that a small sacrifice in pattern shape will result in a worthwhile

increase in forward gain, or a welcome broadening of the SWR bandwidth.  As

with maximum-gain designs, you pay some price in the other performance

parameters when you optimize strictly for best pattern.

Yagis are not optimized with impedance itself as an objective.  Rather,

the impedance value, or the SWR bandwidth in the case of antennas operated over

a frequency band, acts as a constraining characteristic.  Some combination of

gain and pattern is optimized, as long as the impedance is high enough or the

SWR bandwidth is great enough.

Constrained Optimization

Because of the shortcomings of Yagis designed using simple

optimization, YO includes an optimization mode where you may pick your own

gain/pattern/SWR tradeoff and then optimize an antenna within the chosen

constraints.  For example, you might maximize the antenna forward gain subject

to the constraints that the F/B not drop below 25 dB and that the SWR across

the band not rise higher than 2.  Or, you might optimize the pattern subject to

the condition that the gain not fall below 6 dBd and the SWR remain less than 2

across the band.  Automatic constrained optimization is a very powerful tool

for arriving at a final design.  Even when using antenna modeling programs,

optimizing an antenna by trial-and-error experimentation while trying to

satisfy constraints can consume a great deal of time and effort.

To get the most out of a particular Yagi you need to explore the

antenna first using the unconstrained optimization mode.  This will enable you

to get a feel for the potential of the antenna configuration and for the way

that the various performance parameters trade off among themselves.  You may

even discover a region of antenna geometry where the desired objectives tend to

maximize simultaneously.  After exploring the Yagi design in this way you will

be able to choose the best set of starting dimensions for constrained

optimization, and a set of realistic constraint values.

If two constraints are in effect YO will attempt to satisfy both.  If

that is not possible then it will satisfy one or the other.  If that is not

possible then you should set more realistic constraint values.

When F/B is used for the pattern quality measure the pattern constraint

is a minimum value of F/B.  When F/R is used the constraint is a minimum value

of the ratio of the forward power to the power of the largest lobe in the rear

half-plane.  The latter constraint is indicated by "Min F/L" on the constraint

menu.

The default values for the constraining variables are such that no

constraints are in effect.  Use the Constraints command to change the

constraint values.  Note that if you are maximizing gain the constraining value

for gain is inoperative.  Similarly, while optimizing the pattern the pattern

constraint has no meaning and is not in effect.

Manual Optimization

YO provides a way for you to change the element lengths and positions

individually by hand.  This is a great way to get a feel for how each element

affects the overall antenna performance.  It is also useful for touching up the

results of the automatic optimization, especially the pattern at the low and

high frequencies.  In order to run quickly, the YO optimizer only operates at

the middle frequency.  As a result the patterns at the outer frequencies are

never as good as that at midband when you optimize for best pattern.  If you

want an antenna with a more uniform pattern across the band you can modify the

elements by hand to achieve the desired response.  In general, to broaden the

response lengthen the reflector and shorten one or more directors.  For long

Yagis you may manually modify the first 19 elements.

Optimization Method

The automatic optimization procedure used by YO employs a modified

Method of Steepest Descent technique.  This technique is also known as Newton's

Method.  (Yagis had not yet been invented in Newton's time, so he had to make

do with dipoles).

First, each element length (and position, if enabled) is changed

individually by a very small amount, with all other antenna dimensions

unchanged, to calculate the sensitivity of the desired objective (gain,

pattern, or impedance) to each variable.  The collection of these sensitivities

is called a gradient.  The gradient amounts to an n-dimensional vector, where n

is the number of elements plus element spacings, which points in the direction

of maximizing the objective.  After the gradient is calculated, an iteration

step is taken wherein all of the element lengths and positions are updated

simultaneously along the gradient vector, each in proportion to its respective

sensitivity, and all in proportion to a step size.  If the step size is not too

large, after the update the new antenna will have incrementally higher

performance, according to the objective selected.

Because it takes a long time to calculate the gradient, the modified

algorithm will keep iterating the lengths and positions along the same gradient

vector until no further improvement is found.  A new gradient is then

calculated and the process repeated.  The size of the iteration steps is

controlled automatically by various internal criteria with the objective of

finding the maximum value of the objective function in the smallest number of

iterations.  (The step size is too small when a larger step would have improved

performance more, but too large when unstable convergence occurs or the

performance maximum is overshot.)  The whole process is terminated either by

operator interruption or when the step size is automatically reduced many times

in a row, that is, when essentially no further improvement in the objective is

possible with small perturbations of the antenna dimensions.

This optimization technique is quite efficient, locating optimal

dimensions for an antenna without wild goose chases.  However, the method does

not guarantee that the global optimum is found (nor does any other optimization

technique, except for exhaustive search).  The global optimum is the very best

set of antenna dimensions out of all possible dimensions, according to your

chosen criteria of "best".  The reason that the global optimum may be missed is

that the optimization algorithm may converge to a local optimum, and then

terminate because immediate movement in any direction only produces worse

results.

Optimization using perturbation techniques is inherently nearsighted.

If the algorithm is trying to maximize forward gain, for example, and there is

a small bump in the gain curve for a certain set of element lengths and

positions, it is possible that it may wind up on top of the bump rather than at

the highest overall point of the whole curve.  If you imagine trying to climb

to the very top of a mountain range having multiple peaks, in dense fog, you

will be able to visualize the problem.

If you believe that YO is converging to a local optimum you may vary

the starting dimensions to begin the search in a different region of the

performance space.  In general, converging to a local optimum is not much of a

problem with Yagi antennas when using simple optimization.  However, when

optimizing with constraints you are more likely to encounter this situation.

The constrained optimizer tends to stop at the first set of Yagi dimensions it

encounters which are just within the constraint limits.  If you have explored

the Yagi design first using simple optimization you will have some idea whether

a better region of the performance space exists, and therefore whether to try

another set of initial dimensions.

Automatic optimization is done at the middle frequency only.  If the

optimizer were run at the low and high frequencies as well, the program would

execute quite a bit more slowly.  Once you get the hang of working with YO it

is pretty easy to control the response over the whole band while optimizing

only at the middle frequency.  For example, you may equalize the response at

the low and high frequencies by changing the location of the middle frequency,

as it need not be at the center of the band.  In addition, manual changes may

be made to the individual element dimensions to touch up the response at the

outer frequencies.  Future versions of YO may offer the option of automatic

optimization over a band.  (By its nature, the SWR constraint takes the SWR at

the low and high frequencies into account.)

To avoid terminating the optimization prematurely, YO will continue to

iterate even when only very small performance improvements are being made.

Occasionally this will slowly inch the design into a new region of the

performance space where significant improvements become possible.  However,

this situation seldom occurs, so when you are satisfied that no further

improvements are possible you may press any key to interrupt the optimization

and pop up the main menu.  The optimization may be restarted if you change your

mind.  YO will terminate optimization itself eventually, but only after the

step size has been automatically reduced many times in succession, indicating a

performance maximum has been found.

If the key used to interrupt optimization is a valid YO command key,

the command will be executed immediately.  For example, you may press <Esc> to

stop the optimization and also get rid of the main menu, in order to view the

whole screen.

During optimization, checks are made on the reasonableness of the Yagi

dimensions.  Elements are not allowed to become shorter than 0.4 wavelengths,

nor longer than 0.55 wavelengths.  No pair of elements are allowed to migrate

closer than .05 wavelengths.

SWR

During optimization, YO computes the SWR at the low, middle, and high

frequencies using a simple broadband matching network (see below).  The

matching network is adjusted each iteration cycle for unity SWR at a fourth

frequency, called the matching frequency.  In general this frequency is not the

same as the middle frequency, although it is usually somewhere near the middle

of the band.  The matching frequency itself is adjusted each iteration to make

the SWR at the low and high frequencies approximately equal.  This condition

minimizes the maximum SWR across the band.  The matching frequency can also be

updated in a separate iterative loop by using the B command to balance the SWR

at the low and high frequencies.  When you eventually construct the Yagi it

should be matched for unity SWR at the matching frequency, not at the center of

the band or at the middle analysis frequency, in order to achieve the maximum

possible SWR bandwidth.  Note that the SWR displayed for the middle frequency

will not be 1.00 unless the matching frequency is very close to the middle

frequency.

For spot-frequency designs SWR is not an important design parameter

because you can always, in principle, match the antenna perfectly to any

feedline.  In this case, instead of minimizing SWR, you can tell YO to raise

the antenna input impedance.  This is useful for getting out of the low ohmic

range where conductivity losses in the elements may significantly lower the

realizable gain, or where small construction errors in a high-Q design may

result in performance degradation.  YO tries to raise the input impedance in 1

ohm steps.  This permits you to watch the operation proceed and then terminate

it when your desired impedance is reached.  The impedance-raising loop differs

from the gain and pattern maximization loops in that a new gradient is

calculated every iteration cycle.  While slower, this strategy causes the

design to take the straightest path to a desired impedance condition, thereby

sacrificing the least gain or pattern when starting from an optimized design.

Matching Networks

YO contains models for three fundamental matching networks, plus some

variations.

1.  Broadband Network

The simple broadband matching network consists of a broadband

transformer and a series reactance.  The transformer matches the resistive part

of the Yagi input impedance to the feedline impedance, and the series reactance

tunes out any residual Yagi reactance.  While you might actually construct such

a network, it is mainly useful for finding the inherent bandwidth properties of

the Yagi, independent of any particular reactive network which might modify the

SWR bandwidth.  The transformer is modeled as being perfectly broadband in

response, and the small series reactance has an insignificant effect on SWR

bandwidth.  This simple matching network is used during all optimization

iterations, even after another matching network has been designed, because it

allows well-behaved SWR convergence during constrained optimization.

2.  Gamma Match

YO also contains a model for gamma matches.  A gamma matching network

uses a small-diameter rod parallel to one side of the driven element.  The rod

extends from the center of the element and is strapped to the element some

distance down the rod.  The coax center conductor is attached to the gamma rod

and the shield is connected to the center of the driven element.  A capacitor

is placed in series with the gamma rod, usually at the feedpoint, to compensate

for the inductance introduced by the rod.  The driven element is not broken at

the center and may be attached electrically to the boom.

The YO gamma match model uses the W7ITB equations.  You specify the

gamma rod diameter and center-to-center spacing from the driven element.  YO

will then calculate the required rod length and series capacitance, and will

adjust the matching frequency to balance the SWR at the band edges.  The

element diameter used in the gamma match equations is that of the first taper

section which is longer than 1/20th of the element half-length.  This prevents

a short inner taper section representing a mounting plate from being

erroneously employed in the gamma match.  No correction is made if the gamma

rod extends into the next taper section, and no check is made for rods which

extend beyond the end of the driven element.

Although it is not well-known by amateurs, the gamma match is capable

of compensating for the input impedance variation of a Yagi across a frequency

band.  You may take a narrowband high-performance Yagi design and flatten out

the SWR over a wide band by using a carefully designed gamma match.  Because

the operational bandwidth of Yagis is typically limited by their SWR bandwidth,

rather than by their gain or pattern bandwidth, this is a very useful property

of the gamma match, not shared by other matching networks in common use.  The

drawback of a compensating gamma match is that it usually requires a long rod.

This makes it more sensitive to geometry changes caused by mechanical flexing

of the element and matching assembly.

3.  T Match

A T match is a balanced double-sided gamma match which is fed by

connecting a balanced transmission line (or coax through a balun) between the

two gamma rods ends near the boom.  Two series capacitors are used to tune out

the rod reactance.  You can design a T match with YO by using the gamma match

model and specifying half the actual feedline impedance.  The rod length and

capacitance calculated should be applied each side of the driven element.

4.  Hairpin Match

YO also contains a model for hairpin matches.  The hairpin match uses a

shunt inductance across the split driven element feedpoint to raise the

resistive part of the Yagi input impedance to that of the feedline.  The driven

element is then shortened from its resonant length to compensate for the added

inductive reactance, and the system is fed with coax through a balun.  The

inductance can be provided by a coil, by a loop composed of stiff wire or rod,

or by a shorted transmission line consisting of two parallel rods.  YO will

compute the inductance required, and for the transmission line inductor, will

calculate the distance to the shorting bar.  You specify the hairpin rod

diameter and spacing between rods.  YO then iteratively modifies the driven

element length and the matching frequency to obtain unity SWR at the matching

frequency and equal SWR at the low and high frequencies.

Hairpin matches on commercial Yagis use wide spacing between the

hairpin rods.  For the small values of inductance normally required the

distance to the shorting bar is on the order of the rod spacing.  This

geometry, which amounts to a rectangular loop, is not well-modeled by a shorted

transmission line.  For this kind of hairpin you should expect to require a

longer distance to the shorting bar than indicated by the transmission line

model.  However, the values calculated by YO for inductance and driven element

length are accurate for all inductor geometries.  The inductance value is

displayed for cases where it is more convenient to use a lumped inductor

directly across the feedpoint rather than a loop or transmission line.  Future

versions of YO may contain a separate model for stubby hairpins.

The hairpin match will not compensate for Yagi input impedance

variations as will the gamma match, but it will not significantly narrow the

inherent bandwidth of the Yagi either.  Hairpin matches are favored by those

troubled by the unbalanced nature of a gamma match, and by the extra mechanical

components involved in a balanced T match.  However, unlike these systems, the

hairpin match requires a split driven element which must be insulated from the

boom.

5.  Beta Match

The beta match is Hy-Gain's version of the hairpin match.  It uses two

rods which straddle the boom, and a shorting bar across the rods which also

electrically connects the rods to the boom.  The presence of the boom has a

negligible effect on the inductance of the hairpin, so the beta match is

electrically identical to a conventional hairpin match.

Keep in mind that the broadband matching network is used during all

optimization iterations.  When doing optimization with an SWR constraint, you

should set the SWR constraint value according to the matching network you

eventually intend to use.  For example, if you intend to use an

SWR-compensating gamma match, you might choose an SWR constraint of 4, knowing

that it is easy to design a noncritical gamma match which will reduce this SWR

to well below 2.  Conversely, the hairpin match will not compensate for SWR

variations, so you should set the constraint to your ultimate SWR limit.

The matching network menu operates like a spreadsheet.  You may reenter

a network parameter and YO will immediately recalculate the network dimensions

and the SWR at all frequencies.  You may use this feature to interactively

design a matching network, or to explore the impedance-compensating properties

of gamma matches.  You should try various driven element lengths when designing

gamma matches.  The different Yagi input reactances which result will enable

additional gamma match geometries to become realizable designs.

Occasionally a Yagi may exhibit an unusual input impedance variation

with frequency which inhibits convergence of the SWR balancing operation during

gamma and hairpin match design.  If this occurs simply press any key to

terminate the balancing operation.

Driven Element Length

The gain and pattern of a Yagi are not significantly affected by the

length of the driven element, so this length is not changed by the YO

optimization algorithm.  YO modifies the driven element length only during

adjustment of the hairpin match.  You may change the driven element length

manually when designing a gamma match to vary the input impedance seen by the

matching network, permitting different gamma dimensions to be used.  You may

also change the driven element length to accomodate a matching network not

modeled by YO.

One interesting matching scheme involves lengthening the driven element

until the real part of the Yagi input impedance is 50 ohms.  Then a series

capacitor is added to tune out the inductive reactance caused by the long

driven element.  In addition to simplicity, this matching scheme has the

advantage that the higher input impedance results in a somewhat broader SWR

bandwidth, without resorting to an impedance-compensating gamma match.

Element Tapering

Yagi modeling algorithms internally work with cylindrical elements

having a uniform diameter.  A tapering algorithm is necessary to convert from

real antennas using tapered telescoping tubing to the cylindrical (monotaper)

equivalent.  Generally the monotaper length is chosen such that the monotaper

dipole exhibits the same reactance as that of the tapered element at the

central design frequency.  Tapering can greatly affect the reactance of a Yagi

element, and therefore can have a substantial effect on antenna performance.

Tapered elements must be physically longer than their monotaper equivalents to

exhibit the same reactance (for some tapers, much longer).  Tapering must be

accurately modeled to realistically characterize HF Yagis.

YO contains two tapering algorithms.  One is the W2PV tapering

algorithm, from the ARRL Yagi Antenna Design book and the most recent edition

of the ARRL Antenna Book.  This algorithm has been widely used for Yagis and

generally gives good results.  However, a careful study using MN of many

different tapered dipoles and their W2PV-derived monotaper equivalents showed

that often the computed reactance values of the dipoles really were not very

close.  In addition, the reactance values diverged rapidly with frequency

excursion from the central design frequency.

YO contains a second tapering algorithm designed to correct these

shortcomings.  This algorithm is a modified version of the W2PV.  The modified

algorithm yields monotaper equivalent dipoles whose computed reactances are

much closer to those computed for the original tapered elements.  For some

taper schedules the resulting difference in predicted Yagi performance is

dramatic.

In addition, the new tapering algorithm computes the diameter of the

monotaper equivalent in such a way that its reactance and that of the original

tapered element match over a wide frequency band, not just at a single

frequency.  Effectively, the Q of the monotaper equivalent dipole is made equal

to that of the original tapered element.  Using the original W2PV algorithm,

the monotaper equivalent diameter ("standard diameter") is chosen variously as

7/8", as the middle taper diameter, or in some implementations as the average

diameter of the taper sections.  Such equivalent diameters are approximately

correct for many taper schedules in common use.  In addition, for

spot-frequency designs the equivalent diameter can be chosen arbitrarily

without introducing error.  However, to accurately model Yagi performance over

a frequency band, either the equivalent diameters must be accurately

represented, or else the taper conversion must be completely redone at each

frequency analyzed.  Reactance errors in the element models away from the

central design frequency can give a misleading indication of antenna

performance at the band edges.

The new tapering algorithm is the default for YO.  To experiment with

the original W2PV tapering algorithm use the Other Parameters command.  The

standard diameter used with the YO implementation of the original W2PV tapering

algorithm is the average diameter of the taper sections, where the diameter

contribution of each section is weighted by the length of the section.  This

method avoids problems introduced by very short taper sections which represent

element mounting plates.  The maximum number of taper sections is seven.

YO will output the current design using the current taper schedule, for

YO and N6BV output file formats.  For output files in MN format the equivalent

monotaper lengths are always output directly, without tapering.  When the

output file is written, the length of the outermost taper section (the element

tip) is adjusted to reflect the current antenna design, while the inner

sections remain as specified in the taper schedule.  It is possible for an

element to acquire a negative tip length if it gets shortened enough during

optimization.  To allow you to exit YO quickly, this condition is not brought

to your attention as the output file is written.  An error will be generated

later if YO tries to read such a file.  The error will also be caught if you

try to construct the antenna.  Not many hardware stores stock tubing in

negative lengths these days.

Element Mounting Brackets

A conductive element-to-boom mounting bracket lowers the inductance of

an element in the vicinity of the bracket, raising the element resonant

frequency.  This effect may be modeled by increasing the diameter of the

element where it passes over the bracket.  The amount of the increase depends

in a complicated way on the geometry of the bracket and its proximity to the

element.  The Mushiake-Uda equations are used to model this effect for one

common type of mounting bracket, the flat rectangular mounting plate.

The mounting bracket model is selected with the [ or ] keys.  After you

specify the plate geometry, YO will calculate the length and diameter of a new

inner taper section which will be added to the taper schedule.  The new section

has a diameter which is equivalent electrically to the combination of the

original element and the mounting plate.  The length of the section is the same

as the half-length of the mounting plate.

When you return to the main menu all elements of the current Yagi

design will be updated with the new taper section, and it will appear in the YO

output files.  The original input file is not modified.  To permanently recover

the mounting plate correction simply copy the output file onto your original

input file.  If your Yagi uses more than one size of mounting plate you should

use YO to calculate the dimensions of the equivalent taper sections and then

add them by hand to your Yagi file.  Don't forget to shorten the original inner

taper section by the length of the new section added, as the mounting plate

does not alter the total length of tubing in the Yagi.

If YO finds that the existing inner taper section is very short (less

than 1/20th of the element half-length), it assumes that this section already

represents a mounting plate correction.  Additional mounting plate computations

will modify this section, and not cause a new section to be added.  If you are

using a very short inner taper section of tubing for mechanical reinforcement

of the element, rather than to represent a mounting plate correction, you

should use YO to calculate any mounting plate corrections and then add them by

hand to the Yagi file.  Otherwise, YO will substitute mounting plate

corrections for your reinforcement section.

If you enter zero for the plate width, length, or thickness, so that YO

lists "None" for the equivalent taper, when you return to the main menu any

existing very short inner taper section will be removed from the taper

schedule.

The width of the mounting plate is its dimension along the boom, and

the length is its dimension along the element.  Enter the dimensions of the

whole plate, even though YO uses half-lengths for element dimensions.  The

element-plate distance is the closest distance between the element and mounting

plate.  If U-bolt saddles are not used then this distance is 0.

The element dimensions used during on-screen mounting plate

calculations are those of the driven element.  However, when YO adds the new

taper section to the current design, the correction is recalculated for each

individual element to ensure that the correct element diameter is used.

The element mounting method should always be taken into account when

modeling Yagis.  Large mounting plates can significantly affect the frequency

response of a Yagi, moving the desired response completely out of a band in

extreme cases.   Small mounting plates can upset a carefully optimized F/B at a

spot frequency.  However, some mounting methods which use very small mounting

hardware require no correction factor.  One such method is that used by

Cushcraft for their smaller HF beams.  This method employs a single U-bolt and

a small saddle which fits between the boom and element; no mounting plate is

used.  This mounting method should have a negligible effect on element

resonance.  Other element mounting arrangements are not well-modeled by a

simple flat plate.  These include the Hy-Gain clamping mount which surrounds

the element and effectively passes it through the boom, and the Wilson

right-angle bracket mount.  More information on element mounting correction can

be found in the ARRL Yagi Antenna Design book.

Frequency and Taper Scaling

YO contains a scaling routine permitting any Yagi design to be

translated to a new frequency while maintaining essentially the same

performance characteristics.  This provides an easy way to replicate a favorite

Yagi design on a new band without having to design an entirely new antenna.

The taper schedule may be completely different at the new frequency.  YO will

take everything into account, generating a set of dimensions for a Yagi having

electrical performance nearly identical to that of the original antenna.

Taper modifications may also be made in the absence of any frequency

scaling.  This is useful when experimenting with different tubing diameters to

determine the effect on bandwidth characteristics.  It is also useful in

generating an equivalent electrical design for new mechanical parameters.  For

example, you might use the taper scaling feature if you come across a bargain

hamfest beam having good aluminum but unusual tapering and unknown electrical

design.  You could replicate your favorite Yagi design onto this beam by taper

scaling, as long as the boom is long enough and the element positions are

adjustable (if they aren't you can always reoptimize).

Although the YO scaling feature can virtually duplicate the performance

of an existing Yagi at a new spot-frequency, the duplication will not be exact

across a new frequency band unless the diameters of the monotaper equivalent

elements happen to scale in exact proportion with frequency.  This is seldom

the case because practical mechanical considerations or tubing material on hand

normally dictate element diameters.  In addition, the two bands may not have

the same percentage bandwidth.  Therefore, after frequency scaling you should

check the performance of the Yagi over the new band of interest.  Usually the

unmodified performance of scaled antennas is entirely satisfactory.  If not,

the new design can be used as the starting point for a quick reoptimization.

The frequency and taper scaling algorithm used by YO does not rely on a

simple polynomial approximation for element reactance as does the W2PV scaling

technique.  Instead, the method-of-moments algorithm used to compute

self-impedance during normal YO Yagi analysis is invoked iteratively for each

element while the tip lengths are adjusted.  The adjustment loop is terminated

when the reactance of the new element matches that of the original element to

within 0.1 ohm.

The YO and N6BV file formats permit each element to have a completely

different taper schedule.  This is very handy when building a Yagi using

elements obtained from a variety of old antennas.  However, to keep things

simple, during taper scaling YO will apply only one new taper schedule to scale

all elements.

The new taper schedule is entered by selecting Tapering/Scaling from

the main menu.  Simply enter the new taper diameters, separated by spaces, on

the Diameters line.  Then enter the new taper lengths on the Lengths line.  The

length of the last taper section, which varies among the elements and is

indicated by [var] for the current schedule, need not be entered.  YO will

compute this length for each element according to the new schedule.  The set of

dimensions listed for reference as the current taper schedule are those of the

driven element.  If you wish to use the listed diameters or lengths for the new

schedule (for all elements) just press <Enter> on the Diameters or Lengths

line.  This is handy when frequency-scaling a Yagi within a band.

If you press <Esc> when prompted for the new diameters you will exit

Tapering/Scaling.  Use this to examine the current taper schedule without

making any changes.  Once you have specified the diameters you are committed to

a taper scaling operation, and entering <Esc> on the Lengths line will complete

the calculations rather than aborting.  If YO finds that the taper lengths

specified result in an element tip having a negative length, you are prompted

to reenter the lengths.

The tapering algorithm selected (W2PV or modified W2PV) will affect the

results during taper or frequency scaling.  Select the algorithm you prefer

before beginning any scaling operations.

Two-Element Yagis

YO expects every Yagi to have a reflector element.  Two-element Yagis

having a driven element and a director pose a special problem.  These antennas

can be modeled by placing a dummy reflector a great distance away from the two

active elements.  This arrangement will satisfy YO while leaving the response

of the two-element array unaffected.

Pattern Plots

The plots displayed on the screen by YO during optimization use the

standard ARRL log dB antenna polar plotting scale.  Half-scale represents about

12 dB down and quarter-scale is about 24 dB down.  Yagis plotted with this

scale produce familiar beam shapes which may be compared with those in ARRL

antenna publications and QST articles.  Scale markings are not shown in order

to reduce screen clutter.  Yagi response is computed every 5 degrees and these

points are connected with straight lines in the plots.  The slight asymmetry in

the patterns near the polar centers is due to inherent inaccuracies in the

Microsoft plotting routines.

You may also plot the mid-frequency pattern in high resolution with the

H command.  This invokes PLOT.EXE, a general-purpose antenna pattern plotting

program.  PLOT produces annotated polar and rectangular plots with a variety of

display options.  The patterns are generated every 1 degree for Yagis having 15

elements or more, and every 2 degrees for smaller antennas.  You may do a

switched-overlay comparison of two Yagis using the instant plot comparison

feature in PLOT (unless you have a CGA screen).  This feature can reveal subtle

differences between Yagis which would otherwise go unnoticed.

When PLOT is invoked from YO, a plot file is generated with the same

root name as that of the Yagi file, but with the extension .PLT.  Later, after

quitting YO, you may execute PLOT by itself to view and compare the plot files

you have generated.  PLOT uses its own highly accurate drawing routines to

avoid the plotting errors inherent in the Microsoft routines.  The PLOT drawing

routines are not used by YO because they are slower.  See PLOT.DOC for more

information on PLOT.

Other Parameters

The Other Parameters menu selection allows you to control some

additional aspects of YO.

If you change the tapering algorithm, and if the Yagi is not a

monotaper design, when you exit Other Parameters you will asked whether the

Yagi file should be reread using the new tapering algorithm.  The current

design will be overwritten if the file is reread.  To save the current design

first, press <Esc> to cancel the question, and then select Save This Design.

When you select fixed element positions only the element lengths are

varied during automatic optimization.  This is useful for optimizing antennas

whose element positions cannot be changed due to mechanical constraints, such

as through-the-boom mounting.  It is also useful when you know that the element

positions are approximately correct, and you want the optimization to run

faster.  The gradient computations execute twice as fast when the element

positions are fixed.

If you choose not to allow a longer boom, YO will monitor the position

of the last director during optimization.  It will allow this element to

migrate closer to the reflector than the initial design distance, but not

farther away.  The position of the reflector is not varied during optimization,

as it is the reference point for the whole array.  The boom length limit is

updated if you lengthen the boom manually by moving the reflector or the last

director.

The numerical display selection controls the number displayed on the

third line inside each plot.  This number represents pattern quality.

If you select vertical polarization the H-plane pattern is used instead

of the E-plane pattern.  A dot is placed at the plot origin for reference.  The

angular range for the F/R calculation remains 95 to 180 degrees for vertically

polarized antennas, even though you do not get a free null at 90 degrees in the

H-plane as you do in the E-plane.

If you select Save These Settings a file named YO.PAR is written to the

current directory when you return to the main menu.  Codes for the Other

Parameters settings are placed in YO.PAR.  When YO begins execution, it checks

for YO.PAR, and if it exists, the codes are read and the Other Parameters are

set accordingly.  You can use this feature to customize the default setting for

each of the Other Parameters.

Making a Hardcopy of the Screen

You may print the screen if you have a dot matrix printer with

Epson-compatible graphics.  On 84-key keyboards press <Shift><PrtSc>.  For

other keyboards press <Shift><F10> instead.  For the EGA and HGC, if you start

a screen printout at the top of the page, a second printout may be put on the

same page without any manual realignment of the printhead.  YO does not respond

to <PrtSc> or <F10> for the CGA, or perform any printing function.  Instead,

DOS will intercept <Shift><PrtSc> and print your plot if the DOS program

GRAPHICS is loaded before starting YO.  Polar plots will come out somewhat

elliptical on paper, especially for the EGA.  The EGA ellipticity will be

reduced in future versions of YO (and PLOT), and support for 24-pin printers

will be added as well.

DOS Environment Space

DOS provides a convenient way for you to specify configuration

information to YO and to PLOT.  The DOS SET command places information into the

DOS Environment Space in memory, where it can be retrieved later by a program.

SET commands can be put into your AUTOEXEC.BAT file and they will be

automatically executed every time you boot the computer.

You may see what is currently in the DOS Environment Space by typing

SET.  You may eliminate an individual parameter by typing SET [parameter]=.

There are several SET parameters used by YO and PLOT:

1. Library Directories

When you accumulate many Yagi and plot files it is nice to have

separate directories for them, so that the current directory doesn't get so

cluttered.  You might use the current directory for Yagi experiments, saving

optimized Yagi files and their plots elsewhere.  You can tell YO and PLOT to

automatically reference other directories with SET commands.  One directory can

be set up for Yagi files and another for plot files.  These libraries are used

only for loading files; when YO creates a plot file or an output file it always

writes it to the current directory.

If your Yagi library is in the subdirectory YAGIS and your plot library

is in PLOTS then do the following:

SET YAGLIB=YAGIS

SET PLTLIB=PLOTS

You may specify a drive and path for the directories if necessary.

When this library facility is used YO and PLOT will list available files both

in the current directory and in the appropriate library directory.  To specify

a file from the library directory you only need to enter the filename.

YO and PLOT always search the current directory first.  If you need to

force the program to use the library (when the same filename appears in the

current directory) just specify the complete path and filename.

If you are using a dual floppy disk system you can put your Yagi

library and plot library on a second disk drive.  This will give you plenty of

working space on your main drive and still allow access to the library files.

2. Reference dB

To cause YO and PLOT to display gain figures in dBi rather than dBd do

the following:

SET DB=dBi

You may use any combination of upper or lower case for the three dBi

letters.  Gains in dBd are referenced to the peak gain of a half-wave dipole in

free space.  This is common for amateur antennas and is the YO default.  Gains

in dBi are referenced to an isotropic antenna in free space, which is an

antenna that radiates equally in all directions.  A dipole has 2.15 dB gain

over an isotropic antenna, so YO converts from one gain reference to the other

by adding or subtracting 2.15 dB.

3.  Location of COMMAND.COM

YO needs to have access to the DOS command interpreter COMMAND.COM in

order to list Yagi files and to call PLOT.  PLOT needs COMMAND.COM to list plot

files.  Normally the DOS Environment Space will contain the location of

COMMAND.COM.  If it doesn't then you should do the following:

SET COMSPEC=Drive:\COMMAND.COM

where Drive indicates the disk drive whose root directory contains COMMAND.COM

while YO is executing.  If you are using a single floppy disk system you must

copy COMMAND.COM from your DOS system disk onto the YO disk, or use a RAMDISK.

YO File Format

Here is an example of a beam using the YO file format.  You may include

descriptive text at the end of any Yagi file, after all of the essential

information.  The elements are always specified from the rear of the beam first

(R, DE, D1, D2, etc.), and element half-lengths must be used, i.e., from the

center of the boom to the element tip, not tip to tip.

K7HYR's Max Gain Yagi

24.890  24.940  24.990  MHz

4 elements, inches

1.617     1.250     1.125     0.875

0.000     2.938    15.062    66.000    33.305

124.000     2.938    15.062    66.000    28.248

248.000     2.938    15.062    66.000    26.815

372.000     2.938    15.062    66.000    28.313

The first line is a title for the antenna which will be displayed at

the bottom of the YO screen.  The title may be up to 80 characters long, but

very long titles will be truncated when displayed.

The second line consists of 1 or 3 analysis frequencies.  You may also

use Hz, KHz, or GHz.  If no frequency units are specified then MHz is assumed.

The third line gives the number of elements and the dimension units.

You may use feet, meters, centimeters, or millimeters as well as inches.  The

spelling of these keywords must be exact.

The next line lists the taper diameters, starting with those closest to

the boom.  If you use tabs to separate the numbers, including an initial tab as

illustrated, the taper diameters will line up with the taper lengths which

follow.  The maximum number of taper sections is seven.  The first taper

section of this antenna represents the mounting plates.

The remaining lines list the element position followed by the taper

lengths, beginning with the taper closest to the boom.  The elements positions

do not need to begin at 0, but positions, not spacings, must be used.

You may use any combination of spaces, commas, and tabs as separators,

and any combination of upper and lower case letters for the keywords.

The example above uses the same taper schedule for all elements.  Here

is a Yagi which uses a monotaper (single diameter) for the driven element.

Three taper diameter lines are used, the last specified remaining in effect

until a new one is encountered:

N2FB 6 el 20m, NCJ Jan. 86

14.000   14.174   14.350

6 Elements, Inches

1.250     1.125     1.000

0.000      18.000    54.000   138.970

1.000

86.380    196.578

1.250     1.125     1.000

164.090    18.000    54.000   126.640

321.590    18.000    54.000   121.950

464.450    18.000    54.000   121.900

643.990    18.000    54.000   113.520

You may include taper lengths of zero for convenience.  The following

file is equivalent to the file above, but is easier to read:

N2FB 6 el 20m, NCJ Jan. 68

14.000   14.174   14.350

6 Elements, Inches

1.250     1.125     1.000

0.000    18.000    54.000   138.970

86.380     0         0       196.578

164.090    18.000    54.000   126.640

321.590    18.000    54.000   121.950

464.450    18.000    54.000   121.900

643.990    18.000    54.000   113.520

If the last taper section has zero length for an element (because the

tubing size is used only with other elements) you may omit the trailing zero

altogether.  The following Yagi has a 0.5 inch diameter tip for the reflector

only:

Cushcraft 12-4CD

24.94 MHz

4 elements, inches

.875	.75	.625	.5

0	25	45	45	6

67.625	25	45	45

115	25	45	42.5

210	25	29	44

Any of the dimensions may have one of the following symbols appended to

it to use a different length unit than that specified on the number-of-elements

line.  Don't leave a space between the number and the unit abbreviation.  This

feature allows you to use feet for the taper lengths and inches for the taper

diameters, for example.  Here are the abbreviations:

ft   '   in   "   m   cm   mm