From info-serv@arrl.org Thu May 26 21:20:11 1994

Return-Path: <info-serv@arrl.org>

Received: from uu7.psi.com by orion.url.es (4.1/SMI-4.1)

id AA09771; Thu, 26 May 94 21:18:25 +0200

Received: from mgate.arrl.org by uu7.psi.com (5.65b/4.0.071791-PSI/PSINet) via SMTP;

id AA07496 for elsfredi@url.es; Thu, 26 May 94 15:24:20 -0400

Received: from jrb by mgate.arrl.org with smtp

(Smail3.1.28.1 #6) id m0q6l1R-000B9fC; Thu, 26 May 94 15:23 EDT

Received: from mt by jrb with SMTP

id AA39500 ; Thu, 26 May 94 15:22:18 GMT

Date: Thu, 26 May 94 15:20:43 GMT

Message-Id: <1373@mt>

From: info-serv@arrl.org (ARRL HQ AUTOMATED ELECTRONIC MAIL SERVER)

Reply-To: info-serv@arrl.org

To: elsfredi@url.es

Subject: INFO response: AX25-2.DOC

Status: RO

2.3.4.3.3  Frame Reject (FRMR) Response

2.3.4.3.3.1

The FRMR response frame is sent to report that the  receiver

of  a frame cannot successfully process that frame and  that  the

error condition is not correctable by sending the offending frame

again.  Typically this condition will appear when a frame without

an  FCS  error  has  been received  with  one  of  the  following

conditions:

1.  The  reception of an invalid or not implemented  command  or

response frame.

2.  The reception of an I frame whose information field  exceeds

the agreed-upon length.  (See 2.4.7.3, below.)

3.  The  reception  of an improper N(R).  This  usually  happens

when the N(R) frame has already been sent and  acknowledged,

or when N(R) is out of sequence with what was expected.

is  not  allowed, or the reception of a U or S  frame  whose

length is incorrect.  Bits W and Y described in  2.3.4.3.3.2

should both be set to one to indicate this condition.

5.  The reception of a supervisory frame with the F bit set

to  one,  except  during a  timer  recovery  condition  (see

2.4.4.9), or except as a reply to a command frame sent  with

the  P  bit  set to one. Bit W  (described  in  2.3.4.3.3.2)

should be set to one.

6.  The reception of an unexpected UA or DM response frame.  Bit

W should be set to one.

7.  The reception of a frame with an invalid N(S).  Bit W should be

set to one.

An  invalid N(R) is defined as one which points to  an  I

frame  that previously has been transmitted and acknowledged,  or

an  I  frame which has not been transmitted and is not  the  next

sequential I frame pending transmission.

An  invalid N(S) is defined as an N(S) that is  equal  to

the  last transmitted N(R)+k and is equal to the  received  state

variable  V(R),  where  k is the maximum  number  of  outstanding

information frames as defined in 2.4.7.4 below.

An  invalid  or not implemented command  or  response  is

defined  as a frame with a control field that is unknown  to  the

receiver of this frame.

2.3.4.3.3.2

When a FRMR frame is sent, an information field is added  to

the  frame that contains additional information indicating  where

the  problem  occurred.  This information field is  three  octets

long and is shown in Fig. 9.

Information Field Bits

23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

0  0  0  0  Z  Y  X  W   V(R)    C   V(S)  0  Rejected Frame

R            Control Field

Fig. 9 -- FRMR frame information field

Where:

1.  The rejected frame control field carries the control  field

of  the frame that caused the reject condition.  It  is  in

bits 0-7 of the information field.

2.  V(S)  is  the  current send state variable  of  the  device

3.  The  CR bit is set to zero to indicate the  rejected  frame

was a command, or one if it was a response.

4.  V(R)  is the current receive state variable of  the  device

reporting rejection (bit 13 is the low bit).

5.  If W is set to 1, the control field received was invalid or

not implemented.

6.  If  X  is  set  to 1, the  frame  that  caused  the  reject

condition  was considered invalid because it was a U  or  S

frame  that had an information field that is  not  allowed.

Bit W must be set to 1 in addition to the X bit.

7.  If  Y is set to 1, the control field received and  returned

in   bits   exceeded  the  maximum   allowed   under   this

recommendation in 2.4.7.3, below.

8.  If  A is set to 1, the control field received and  returned

in bits 1 to 8 contained an invalid N(R).

9.  Bits 8, and 20 to 23 are set to 0.

2.3.4.3.4  Unnumbered Acknowledge (UA) Response

The  UA  response  frame  is  sent  to  acknowledge   the

reception  and  acceptance of a SABM or DISC  command  frame.   A

received command is not actually processed until the UA  response

frame  is  sent.  Information fields are not permitted  in  a  UA

frame.

2.3.4.3.5  Disconnected Mode (DM) Response

The  disconnected  mode response is sent whenever  a  DXE

receives  a  frame  other  than a SABM or UI  frame  while  in  a

disconnected  mode.   It  is  also sent to  request  a  set  mode

command,  or  to indicate it cannot accept a  connection  at  the

moment.  The DM response does not have an information field.

Whenever a SABM frame is a received, and it is determined

that  a  connection is not possible, a DM frame  shall  be  sent.

This  will  indicate  that the called  station  cannot  accept  a

connection at that time.

While a DXE is in the disconnected mode, it will  respond

to  any command other than a SABM or UI frame with a DM  response

with the P/F bit set to 1.

2.3.4.3.6  Unnumbered Information (UI) Frame

The   Unnumbered  Information  frame  contains  PID   and

information fields and is used to pass information along the link

fields  to go back and forth on the link bypassing flow  control.

Since   these  frames  are  not  acknowledgeable,  if  one   gets

obliterated, there is no way to recover it.  A received UI  frame

with  the  P bit set shall cause a response  to  be  transmitted.

This response shall be a DM frame when in the disconnected  state

or  a  RR  (or  RNR, if appropriate)  frame  in  the  information

transfer state.

2.3.5  Link Error Reporting and Recovery

There are several link-layer errors that are  recoverable

without  terminating the connection.  These error situations  may

occur  as  a  result  of  malfunctions  within  the  DXE,  or  if

transmission errors occur.

2.3.5.1  DXE Busy Condition

When  a  DXE  becomes temporarily  unable  to  receive  I

frames,  such  as when receive buffers are full, it will  send  a

Receive Not Ready (RNR) frame.  This informs the other DXE   that

this  DXE  cannot handle any more I frames at the  moment.   This

condition is usually cleared by the sending of a UA, RR, REJ,  or

SABM command frame.

2.3.5.2  Send Sequence Number Error

If the send sequence number, N(S), of an otherwise error-

free  received frame does not match the receive  state  variable,

V(R),  a  send sequence error has occurred, and  the  information

field will be discarded.  The receiver will not acknowledge  this

frame, or any other I frames, until N(S) matches V(R).

The  control  field of the erroneous I frame(s)  will  be

accepted so that link supervisory functions such as checking  the

P/F  bit can still be performed.  Because of this  updating,  the

retransmitted I frame may have an updated P bit and N(R).

2.3.5.3  Reject (REJ) Recovery

REJ  is  used  to request a retransmission  of  I  frames

following  the  detection  of a N(S) sequence  error.   Only  one

outstanding  "sent  REJ" condition is allowed at  a  time.   This

condition  is  cleared  when  the  requested  I  frame  has  been

received.

A DXE receiving the REJ command will clear the  condition

by  resending all outstanding I frames (up to the  window  size),

starting with the one indicated in N(R) of the REJ frame.

2.3.5.4  Time-out Error Recovery

2.3.5.4.1  T1 Timer Recovery

If  a DXE, due to a transmission error, does not  receive

in  a sequence of I frames, it will not detect  a  send-sequence-

number  error,  and therefore will not transmit a REJ.   The  DXE

which transmitted the unacknowledged I frame(s) shall,  following

the  completion of time-out period T1, take appropriate  recovery

action  to determine when I frame retransmission should begin  as

described  in 2.4.4.9, below.  This condition is cleared  by  the

reception of an acknowledgement for the sent frame(s), or by  the

link being reset.  See 2.4.6.

2.3.5.4.2  Timer T3 Recovery

Timer  T3 is used to assure the link is still  functional

during  periods of low information transfer.  Whenever T1 is  not

running  (no  outstanding I frames), T3 is used  to  periodically

poll  the  other DXE of a link.  When T3 times out, a RR  or  RNR

frame  is transmitted as a command and with the P bit  set.   The

waiting  acknowledgement  procedure  (2.4.4.9,  below)  is   then

executed.

2.3.5.5  Invalid Frame or FCS Error

If  an invalid frame is received, or a frame is  received

with  an FCS error, that frame will be discarded with  no  action

taken.

2.3.5.6  Frame Rejection Condition

A  frame  rejection condition occurs  when  an  otherwise

error-free  frame  has been received with one of  the  conditions

listed in 2.3.4.3.3 above.

Once  a  rejection  error occurs, no more  I  frames  are

accepted  (except for the examination of the P/F bit)  until  the

error is resolved.  The error condition is reported to the  other

DXE by sending a FRMR response frame.  See 2.4.5.

2.4  Description of AX.25 Procedures

The  following  describes the procedures used  to  setup,

use, and disconnect a balanced link between two DXE stations.

2.4.1  Address Field Operation

2.4.1.1  Address Information

All   transmitted  frames  shall  have   address   fields

conforming  to  2.2.13, above.  All frames shall  have  both  the

destination device and the source device addresses in the address

field,  with the destination address coming first.   This  allows

many links to share the same RF channel.  The destination address

is  always  the address of the station(s) to receive  the  frame,

while the source address contains the address of the device  that

sent the frame.

sign if the point-to-multipoint operation is allowed.   Operation

with  destination addresses other than actual amateur call  signs

is a subject for further study.

2.4.1.2  Command/Response Procedure

AX.25  Version 2.0 has implemented  the  command/response

information   in  the  address  field.   In  order  to   maintain

compatibility    with   previous   versions   of    AX.25,    the

command/response information is conveyed using two bits.

An  upward-compatible AX.25 DXE can determine whether  it

is  communicating  with  a DXE using an  older  version  of  this

protocol by testing the command/response bit information  located

in  bit 7 of the SSID octets of both the destination  and  source

address subfields.  If both C bits are set to zero, the device is

using  the  older protocol.  The newer version  of  the  protocol

always has one of these two bits set to one and the other set  to

zero, depending on whether the frame is a command or a response.

The  command/response  information is  encoded  into  the

address field as shown in Fig. 10.

Frame Type          Dest. SSID C-Bit    Source SSID C-Bit

Previous versions             0                    0

Command (V.2.0)               1                    0

Response (V.2.0)              0                    1

Previous versions             1                    1

Fig. 10 -- Command/Response encoding

Since  all  frames  are  considered  either  commands  or

responses, a device shall always have one of the bits set to one,

and the other bit set to zero.

The  use  of the command/response  information  in  AX.25

allows  S frames to be either commands or responses.   This  aids

maintenance   of  proper  control  over  the  link   during   the

information transfer state.

2.4.2  P/F Bit Procedures

The next response frame returned by the DXE to a SABM  or

DISC command with the P bit set to 1 will be a UA or DM  response

with the F bit set to 1.

The next response frame returned to an I frame with the P

bit  set  to 1, received during the information  transfer  state,

will be a RR, RNR, or REJ response with the F bit set to 1.

The next response frame returned to a supervisory command

frame  with the P bit set to 1, received during  the  information

transfer state, will be a RR, RNR, or REJ response frame with the

F bit set to 1.

The  next  response frame returned to a S  or  I  command

frame  with  the  P bit set to 1, received  in  the  disconnected

state, will be a DM response frame with the F bit set to 1.

The  P  bit  is used in  conjunction  with  the  time-out

recovery condition discussed in 2.3.5.4, above.

When not used, the P/F bit is set to zero.

2.4.3  Procedures For Link Set-Up and Disconnection

2.4.3.1  LAPB Link Connection Establishment

When  one DXE wishes to connect to another DXE,  it  will

send  a SABM command frame to that device and start  timer  (T1).

If  the other DXE is there and able to connect, it  will  respond

with  a UA response frame, and reset both of its  internal  state

variables  (V(S)  and V(R)).  The reception of  the  UA  response

frame  at  the  other  end will  cause  the  DXE  requesting  the

connection  to  cancel the T1 timer and set  its  internal  state

variables to 0.

If the other DXE doesn't respond before T1 times out, the

device requesting the connection will re-send the SABM frame, and

start  T1  running  again.  The DXE should  continue  to  try  to

establish  a  connection  until it has  tried  unsuccessfully  N2

times.  N2 is defined in 2.4.7.2, below.

If,  upon  reception of a SABM command, the  DXE  decides

that  it  cannot enter the indicated state, it should send  a  DM

frame.

When  receiving a DM response, the DXE sending  the  SABM

should  cancel  its  T1 timer, and  not  enter  the  information-

transfer state.

The  DXE sending a SABM command will ignore  and  discard

any  frames except SABM, DISC, UA, and DM frames from  the  other

DXE.

Frames  other  than UA and DM in response to  a  received

SABM  will  be  sent  only after the link is set  up  and  if  no

outstanding SABM exists.

2.4.3.2  Information-Transfer Phase

After establishing a link connection, the DXE will  enter

the  information-transfer  state.  In this state,  the  DXE  will

accept  and  transmit I and S frames according to  the  procedure

When  receiving a SABM command while in the  information-

transfer  state,  the  DXE will follow  the  resetting  procedure

outlined in 2.4.6 below.

2.4.3.3  Link Disconnection

2.4.3.3.1

While  in  the information-transfer state,  either  DXE  may

indicate a request to disconnect the link by transmitting a  DISC

command frame and starting timer T1 (see 2.4.7).

2.4.3.3.2

A DXE, upon receiving a valid DISC command, shall send a  UA

response  frame  and enter the disconnected state.  A  DXE,  upon

receiving  a  UA  or DM response to a sent  DISC  command,  shall

cancel timer T1, and enter the disconnected state.

2.4.3.3.3

If  a UA or DM response is not correctly received before  T1

times out, the DISC frame should be sent again and T1  restarted.

If  this happens N2 times, the DXE should enter the  disconnected

state.

2.4.3.4  Disconnected State

2.4.3.4.1

A  DXE  in  the disconnected state  shall  monitor  received

commands  and react upon the reception of a SABM as described  in

2.4.3.1 above and will transmit a DM frame in response to a  DISC

command.

2.4.3.4.2

In the disconnected state, a DXE may initiate a link  set-up

as outlined in connection establishment above (2.4.3.1).  It  may

also  respond  to  the  reception  of  a  SABM  and  establish  a

connection, or it may ignore the SABM and send a DM instead.

2.4.3.4.3

Any  DXE receiving a command frame other than a SABM  or  UI

frame  with the P bit set to one should respond with a  DM  frame

with  the  F  bit  set to one.  The  offending  frame  should  be

ignored.

2.4.3.4.4

When  the DXE enters the disconnected state after  an  error

condition  or if an internal error has resulted in the DXE  being

in  the  disconnected  state, the DXE  should  indicate  this  by

sending  a  DM response rather than a DISC frame and  follow  the

link  disconnection procedure outlined in 2.4.3.3.3, above.   The

DXE  may then try to re-establish the link using the link  set-up

procedure outlined in 2.4.3.1, above.

2.4.3.5  Collision Recovery

2.4.3.5.1  Collisions in a Half-Duplex Environment

Collisions  of  frames in a half-duplex  environment  are

taken  care  of  by  the  retry  nature  of  the  T1  timer   and

retransmission count variable.  No other special action needs  to

be taken.

2.4.3.5.2  Collisions of Unnumbered Commands

If sent and received SABM or DISC command frames are  the

same,  both  DXEs  should  send a UA  response  at  the  earliest

opportunity, and both devices should enter the indicated state.

If sent and received SABM or DISC commands are different,

both  DXEs should enter the disconnected state and transmit a  DM

frame at the earliest opportunity.

2.4.3.5.3  Collision of a DM with a SABM or DISC

When  an  unsolicited  DM  response  frame  is  sent,   a

collision  between it and a SABM or DISC may occur.  In order  to

prevent  this  DM from being misinterpreted, all  unsolicited  DM

frames  should  be transmitted with the F bit set to  zero.   All

SABM  and DISC frames should be sent with the P bit set  to  one.

This will prevent any confusion when a DM frame is received.

2.4.3.6  Connectionless Operation

In  Amateur  Radio,  there  is  an  additional  type   of

operation  that is not feasible using level 2 connections.   This

operation  is  the  round table, where several  amateurs  may  be

engaged  in one conversation.  This type of operation  cannot  be

accommodated by AX.25 link-layer connections.

The   way   round-table  activity   is   implemented   is

technically  outside  the AX.25 connection, but still  using  the

AX.25 frame structure.

AX.25 uses a special frame for this operation, called the

Unnumbered  Information (UI) frame.  When this type of  operation

is  used,  the  destination  address  should  have  a  code  word

installed  in  it to prevent the users of that  particular  round

table from seeing all frames going through the shared RF  medium.

An example of this is if a group of amateurs are in a round-table

discussion  about  packet  radio, they could put  PACKET  in  the

destination  address,  so  they would receive  frames  only  from

others in the same discussion.  An added advantage of the use  of

AX.25  in this manner is that the source of each frame is in  the

source  address  subfield,  so  software  could  be  written   to

automatically display who is making what comments.

Since  this  mode  is connectionless, there  will  be  no

requests for retransmissions of bad frames.  Collisions will also

occur, with the potential of losing the frames that collided.

2.4.4  Procedures for Information Transfer

Once  a  connection  has been  established,  as  outlined

above, both devices are able to accept I, S, and U frames.

2.4.4.1  Sending I Frames

Whenever  a DXE has an I frame to transmit, it will  send

the  I frame with N(S) of the control field equal to its  current

send  state  variable V(S).  Once the I frame is sent,  the  send

state  variable  is  incremented  by one.  If  timer  T1  is  not

running, it should be started.  If timer T1 is running, it should

be restarted.

The DXE should not transmit any more I frames if its send

state variable equals the last received N(R) from the other  side

of  the link plus seven.  If it were to send more I  frames,  the

flow control window would be exceed, and errors could result.

If  a  DXE is in a busy condition, it may  still  send  I

frames as long as the other device is not also busy.

If  a  DXE is in the frame-rejection mode, it  will  stop

sending I frames.

2.4.4.2  Receiving I Frames

2.4.4.2.1

If  a DXE receives a valid I frame (one with a  correct  FCS

and  whose  send sequence number equals  the  receiver's  receive

state variable) and is not in the busy condition, it will  accept

the  received I frame, increment its receive state variable,  and

act in one of the following manners:

1.  If it has an I frame to send, that I frame may be sent with the

transmitted N(R) equal to its receive state variable V(R) (thus

acknowledging the received frame).  Alternately, the device may

send a RR frame with N(R) equal to V(R), and then send the I

frame.

2.  If there are no outstanding I frames, the receiving device will

send a RR frame with N(R) equal to V(R).  The receiving DXE may

wait a small period of time before sending the RR frame to be sure

additional I frames are not being transmitted.

2.4.4.2.2

If  the  DXE  is  in a busy condition,  it  may  ignore  any

received  I  frames without reporting this condition  other  than

repeating the indication of the busy condition.

If  a busy condition exists, the DXE receiving  the  busy

condition   indication  should  poll  the  sender  of  the   busy

indication periodically until the busy condition disappears.

frames with the P bit set to one.

The  reception  of  I  frames  that  contain  zero-length

information  fields  shall be reported to the next level  but  no

information field will be transferred.

2.4.4.3  Reception of Out of Sequence Frames

When  an I frame is received with a correct FCS, but  its

send sequence number, N(S), does not match the current receiver's

receive  state  variable, the frame should be discarded.   A  REJ

frame  shall be sent with a receive sequence number equal to  one

higher (modulo 8) than the last correctly received I frame if  an

uncleared  N(S) sequence error condition has not been  previously

established.   The  received state variable and poll bit  of  the

discarded  frame should be checked and acted upon, if  necessary,

before discarding the frame.

2.4.4.4  Reception of Incorrect Frames

When  a  DXE receives a frame with an incorrect  FCS,  an

invalid  frame, or a frame with an improper address,  that  frame

shall be discarded.

2.4.4.5  Receiving Acknowledgement

Whenever an I or S frame is correctly received, even in a

busy condition, the N(R) of the received frame should be  checked

to  see if it includes an acknowledgement of outstanding  sent  I

frames.   The T1 timer should be cancelled if the received  frame

actually  acknowledges previously unacknowledged frames.  If  the

T1  timer is cancelled and there are still some frames that  have

been sent that are not acknowledged, T1 should be started  again.

If  the T1 timer runs out before an acknowledgement is  received,

the  device  should proceed to the  retransmission  procedure  in

2.4.4.9.

2.4.4.6  Receiving Reject

Upon receiving a REJ frame, the transmitting DXE will set

its  send  state variable to the same value as  the  REJ  frame's

received sequence number in the control field.  The DXE will then

retransmit  any  I  frame(s) outstanding at  the  next  available

opportunity conforming to the following:

1.  If the DXE is not transmitting at the time, and the  channel

is  open,  the  device  may commence  to  retransmit  the  I

frame(s) immediately.

2.  If   the   DXE  is  operating  on  a   full-duplex   channel

transmitting  a UI or S frame when it receives a REJ  frame,

it may finish sending the UI or S frame and then  retransmit

the I frame(s).

transmitting  another I frame when it receives a REJ  frame,

it  may  abort  the  I  frame  it  was  sending  and   start

retransmission of the requested I frames immediately.

4.  The DXE may send just the one I frame outstanding, or it may

send  more than the one indicated if more I frames  followed

the  first  one not acknowledged, provided the total  to  be

sent does not exceed the flow-control window (7 frames).

If the DXE receives a REJ frame with the poll bit set, it

should  respond with either a RR or RNR frame with the final  bit

set before retransmitting the outstanding I frame(s).

2.4.4.7  Receiving a RNR Frame

Whenever  a  DXE  receives a RNR  frame,  it  shall  stop

transmission  of  I  frames until the  busy  condition  has  been

cleared.   If timer T1 runs out after the RNR was  received,  the

waiting  acknowledgement  procedure  listed  in  2.4.4.9,  below,

should  be  performed.  The poll bit may be used  in  conjunction

with  S  frames  to test for a change in  the  condition  of  the

busied-out DXE.

2.4.4.8  Sending a Busy Indication

Whenever a DXE enters a busy condition, it will  indicate

this  by sending a RNR response at the next  opportunity.   While

the  DXE is in the busy condition, it may receive and  process  S

frames,  and if a received S frame has the P bit set to one,  the

DXE should send a RNR frame with the F bit set to one at the next

possible  opportunity.   To  clear the busy  condition,  the  DXE

should  send either a RR or REJ frame with the received  sequence

number equal to the current receive state variable, depending  on

whether the last received I frame was properly received or not.

2.4.4.9  Waiting Acknowledgement

If timer T1 runs out waiting the acknowledgement from the

other DXE for an I frame transmitted, the DXE will restart  timer

T1  and transmit an appropriate supervisory command frame (RR  or

RNR)  with  the  P  bit set.  If the  DXE  receives  correctly  a

supervisory  response frame with the F bit set and with  an  N(R)

within  the  range from the last N(R) received to the  last  N(S)

sent  plus  one, the DXE will restart timer T1 and will  set  its

send  state  variable  V(S) to the received N(R).   It  may  then

resume   with   I  frame  transmission  or   retransmission,   as

appropriate.  If, on the other hand, the DXE receives correctly a

supervisory response frame with the F bit not set, or an I  frame

or  supervisory command frame, and with an N(R) within the  range

from  the last N(R) received to the last N(S) sent plus one,  the

DXE will not restart timer T1, but will use the received N(R)  as

an  indication of acknowledgement of transmitted I frames  up  to

and including I frame numbered N(R)-1.

with  the  F  bit set is received, the  DXE  will  retransmit  an

appropriate supervisory command frame (RR or RNR) with the P  bit

set.  After N2 attempts to get a supervisory response frame  with

the  F bit set from the other DXE, the DXE will initiate  a  link

resetting procedure as described in 2.4.6, below.

2.4.5  Frame Rejection Conditions

A  DXE  shall initiate the frame-reset procedure  when  a

frame  is received with the correct FCS and address field  during

the information-transfer state with one or more of the conditions

in 2.3.4.3.3, above.

Under these conditions, the DXE will ask the other DXE to

reset  the  link by transmitting a FRMR response as  outlined  in

2.4.6.3, below.

After sending the FRMR frame, the sending DXE will  enter

the  frame reject condition.  This condition is cleared when  the

DXE that sent the FRMR frame receives a SABM or DISC command,  or

a DM response frame.  Any other command received while the DXE is

in the frame reject state will cause another FRMR to be sent  out

with the same information field as originally sent.

In  the  frame rejection condition, additional  I  frames

will not be transmitted, and received I frames and S frames  will

be discarded by the DXE.

The DXE that sent the FRMR frame shall start the T1 timer

when  the  FRMR is sent.  If no SABM or DISC  frame  is  received

before the timer runs out, the FRMR frame shall be retransmitted,

and   the  T1  timer  restarted  as  described  in  the   waiting

acknowledgement section (2.4.4.9) above.  If the FRMR is sent  N2

times without success, the link shall be reset.

2.4.6  Resetting Procedure

2.4.6.1

The   resetting  procedure  is  used  to   initialize   both

directions  of  data  flow  after  a  nonrecoverable  error   has

occurred.   This resetting procedure is used in the  information-

transfer state of an AX.25 link only.

2.4.6.2

A DXE shall initiate a reset procedure whenever it  receives

an unexpected UA response frame or an unsolicited response  frame

with  the  F bit set to one.  A DXE may also initiate  the  reset

procedure  upon receipt of a FRMR frame.  Alternatively, the  DXE

may  respond to a FRMR by terminating the connection with a  DISC

frame.

2.4.6.3

A  DXE  shall  reset the link by sending a  SABM  frame  and

starting  timer  T1.  Upon receiving a SABM frame  from  the  DXE

a  UA  frame back at the earliest opportunity, set its  send  and

receive  state  variables,  V(S) and V(R), to zero  and  stop  T1

unless it has sent a SABM or DISC itself.  If the UA is correctly

received by the initial DXE, it resets its send and receive state

variables, V(S) and V(R), and stops timer T1.  Any busy condition

that previously existed will also be cleared.

If  a  DM response is received, the DXE  will  enter  the

disconnected  state  and  stop timer T1.  If timer  T1  runs  out

before  a UA or DM response frame is received, the SABM  will  be

retransmitted  and timer T1 restarted.  If timer T1 runs  out  N2

times,  the  DXE  will  enter the  disconnected  state,  and  any

previously existing link conditions will be cleared.

Other  commands  or  responses received by  the  DXE  before

completion of the reset procedure will be discarded.

2.4.6.4

One  DXE  may request that the other DXE reset the  link  by

sending  a  DM response frame.  After the DM frame is  sent,  the

sending DXE will then enter the disconnected state.

2.4.7  List of System Defined Parameters

2.4.7.1  Timers

To   maintain  the  integrity  of  the  AX.25   level   2

connection, use of these timers is recommended.

2.4.7.1.1  Acknowledgement Timer T1

The  first timer, T1, is used to make sure a DXE  doesn't

wait  forever  for a response to a frame it  sends.   This  timer

cannot be expressed in absolute time, since the time required  to

send frames varies greatly with the signaling rate used at  level

1.   T1  should take at least twice the amount of time  it  would

take  to send maximum length frame to the other DXE, and get  the

proper response frame back from the other DXE.  This would  allow

time for the other DXE to do some processing before responding.

If  level  2 repeaters are to be used, the  value  of  T1

should be adjusted according to the number of repeaters the frame

is being transferred through.

2.4.7.1.2  Response Delay Timer T2

The  second timer, T2, may be implemented by the  DXE  to

specify  a maximum amount of delay to be introduced  between  the

time an I frame is received, and the time the resulting  response

frame is sent.  This delay may be introduced to allow a receiving

DXE to wait a short period of time to determine if there is  more

than  one frame being sent to it.  If more frames  are  received,

the  DXE can acknowledge them at once (up to seven), rather  than

acknowledge  each individual frame.  The use of timer T2  is  not

Note  that, on full-duplex channels, acknowledgements should  not

be delayed beyond k/2 frames to achieve maximum throughput.   The

k parameter is defined in 2.4.7.4, below.

2.4.7.1.3  Inactive Link Timer T3

The third timer, T3, is used whenever T1 isn't running to

maintain  link integrity.  It is recommended that whenever  there

are  no  outstanding  unacknowledged I  frames  or  P-bit  frames

(during  the information-transfer state), a RR or RNR frame  with

the  P  bit set to one be sent every T3 time units to  query  the

status  of the other DXE.  The period of T3 is  locally  defined,

and  depends greatly on level 1 operation.  T3 should be  greater

than T1, and may be very large on channels of high integrity.

2.4.7.2  Maximum Number of Retries (N2)

The maximum number of retries is used in conjunction with

the T1 timer.

2.4.7.3  Maximum Number of Octets in an I Field (N1)

The maximum number of octets allowed in the I field  will

be 256.  There shall also be an integral number of octets.

2.4.7.4  Maximum Number of I Frames Outstanding (k)

The  maximum number of outstanding I frames at a time  is

seven.

END.

--------------------------------------------------------------------------