** Recopilado por:

������������������������������������������������Ŀ

�  EB3AMT � op.Albert � Barcelona � JN11CJ  � o o�

�         EB3AMT@EB3EHJ.EAB.ESP.EU          �  ! �

���������������������������������������������  - �

����������������������������������������������

From info-serv@arrl.org Thu May 26 21:19:33 1994

Return-Path: <info-serv@arrl.org>

Received: from uu7.psi.com by orion.url.es (4.1/SMI-4.1)

id AA09760; Thu, 26 May 94 21:17:57 +0200

Received: from mgate.arrl.org by uu7.psi.com (5.65b/4.0.071791-PSI/PSINet) via SMTP;

id AA07492 for elsfredi@url.es; Thu, 26 May 94 15:23:53 -0400

Received: from jrb by mgate.arrl.org with smtp

(Smail3.1.28.1 #6) id m0q6l1Q-000B9dC; Thu, 26 May 94 15:23 EDT

Received: from mt by jrb with SMTP

id AA39498 ; Thu, 26 May 94 15:22:16 GMT

Date: Thu, 26 May 94 15:20:37 GMT

Message-Id: <1371@mt>

From: info-serv@arrl.org (ARRL HQ AUTOMATED ELECTRONIC MAIL SERVER)

Reply-To: info-serv@arrl.org

To: elsfredi@url.es

Subject: INFO response: AX25-1.DOC

Status: RO

AX25.DOC

AX.25 Amateur Packet-Radio Link-Layer Protocol

Version 2.0  October 1984

Copyright (c) 1984 by

The American Radio Relay Legue, Inc.

Blanket permission to copy this publication by end users for

noncommercial purposes is hereby granted.  No part of this

work may be reproduced in any form where such copy is offered

in exchange for any payment unless written permission has first

been secured from the publisher.

2.  AX.25 Link-Layer Protocol Specification

2.1  Scope and Field of Operation

In  order  to  provide  a  mechanism  for  the   reliable

transport  of  data  between  two  signaling  terminals,  it   is

necessary  to define a protocol that can accept and deliver  data

over a variety of types of communications links.  The AX.25 Link-

Layer  Protocol is designed to provide this service,  independent

of any other level that may or may not exist.

This protocol conforms to ISO Recommendations 3309,  4335

(including DAD 1&2) and 6256 high-level data link control  (HDLC)

and  uses  some terminology found in these  documents.   It  also

conforms with ANSI X3.66, which describes ADCCP, balanced mode.

This  protocol  follows,  in principle,  the  CCITT  X.25

Recommendation,  with the exception of an extended address  field

and  the addition of the Unnumbered Information (UI)  frame.   It

also follows the principles of CCITT Recommendation Q.921  (LAPD)

in the use of multiple links, distinguished by the address field,

on a single shared channel.

As  defined,  this  protocol will work  equally  well  in

either half- or full-duplex Amateur Radio environments.

This protocol has been designed to work equally well  for

direct  connections between two individual  amateur  packet-radio

stations or an individual station and a multiport controller.

This  protocol allows for the establishment of more  than

one  link-layer  connection  per  device, if  the  device  is  so

capable.

This  protocol  does not  prohibit  self-connections.   A

self-connection  is considered to be when a device establishes  a

link  to  itself using its own address for both  the  source  and

destination of the frame.

Most  link-layer  protocols assume that one  primary  (or

master)  device  (generally  called  a  DCE,  or  data   circuit-

terminating equipment) is connected to one or more secondary  (or

slave)  device(s)  (usually  called a DTE,  or  data  terminating

equipment).   This type of unbalanced operation is not  practical

in a shared-RF Amateur Radio environment.  Instead, AX.25 assumes

that  both  ends  of  the link are of  the  same  class,  thereby

eliminating  the two different classes of devices.  The term  DXE

is  used in this protocol specification to describe the  balanced

2.2  Frame Structure

Link  layer packet radio transmissions are sent in  small

blocks of data, called frames.  Each frame is made up of  several

smaller groups, called fields.  Fig.1 shows the three basic types

of  frames.  Note that the first bit to be transmitted is on  the

left side.

First

Bit Sent

Flag      Address     Control     FCS      Flag

01111110  112/560 Bits  8 Bits    16 Bits  01111110

Fig. 1A -- U and S frame construction

First

Bit Sent

Flag      Address      Control    PID    Info.       FCS      Flag

01111110  112/560 Bits   8 Bits   8 Bits  N*8 Bits   16 Bits  01111110

Fig. 1B -- Information frame construction

Each field is made up of an integral number of octets (or

bytes), and serves a specific function as outlined below.

2.2.1  Flag Field

The flag field is one octet long.  Since the flag is used

to  delimit  frames, it occurs at both the beginning and  end  of

each  frame.  Two frames may share one flag, which  would  denote

the  end of the first frame, and the start of the next frame.   A

flag consists of a zero followed by six ones followed by  another

zero,  or  01111110 (7E hex).  As a result of bit  stuffing  (see

2.2.6,  below),  this sequence is not allowed to  occur  anywhere

else inside a complete frame.

2.2.2  Address Field

The address field is used to identify both the source  of

the  frame and its destination.  In addition, the  address  field

contains  the  command/response information  and  facilities  for

level 2 repeater operation.

The encoding of the address field is described in 2.2.13.

The  control field is used to identify the type of  frame

being  passed  and  control several attributes  of  the  level  2

connection.   It  is  one octet in length, and  its  encoding  is

discussed in 2.3.2.1, below.

2.2.4  PID Field

The  Protocol  Identifier  (PID) field  shall  appear  in

information  frames (I and UI) only.  It identifies what kind  of

layer 3 protocol, if any, is in use.

The PID itself is not included as part of the octet count

of the information field.  The encoding of the PID is as follows:

M      L

S      S

B      B

yy01yyyy AX.25 layer 3 implemented.

yy10yyyy AX.25 layer 3 implemented.

11001100 Internet Protocol datagram layer 3 implemented.

11001101 Address resolution protocol layer 3 implemented.

11110000 No layer 3 implemented.

11111111 Escape character.  Next octet contains more Level 3

protocol information.

Where:

A y indicates all combinations used.

Note:

All  forms  of  yy11yyyy and yy00yyyy  other  than  those

listed  above  are  reserved  at this time  for  future  level  3

protocols.   The  assignment of these formats is  up  to  amateur

agreement.   It  is  recommended that the  creators  of  level  3

protocols   contact  the  ARRL  Ad  Hoc  Committee   on   Digital

Communications for suggested encodings.

2.2.5  Information Field

The  information field is used to convey user  data  from

one  end of the link to the other.  I fields are allowed in  only

three  types of frames:  the I frame, the UI frame, and the  FRMR

frame.   The  I  field can be up to 256 octets  long,  and  shall

contain  an integral number of octets.  These  constraints  apply

prior to the insertion of zero bits as specified in 2.2.6, below.

Any  information  in the I field shall be passed along  the  link

transparently,  except  for the zero-bit  insertion  (see  2.2.6)

necessary  to prevent flags from accidentally appearing in the  I

2.2.6  Bit Stuffing

In  order to assure that the flag bit sequence  mentioned

above  doesn't appear accidentally anywhere else in a frame,  the

sending  station  shall monitor the bit sequence for a  group  of

five  or more contiguous one bits.  Any time five contiguous  one

bits  are sent the sending station shall insert a zero bit  after

the  first  one  bit.   During frame  reception,  any  time  five

contiguous  one  bits  are  received,  a  zero  bit   immediately

following five one bits shall be discarded.

2.2.7  Frame-Check Sequence

The  frame-check sequence (FCS) is a  sixteen-bit  number

calculated  by  both the sender and receiver of a frame.   It  is

used  to  insure that the frame was not corrupted by  the  medium

used to get the frame from the sender to the receiver.  It  shall

be calculated in accordance with ISO 3309 (HDLC) Recommendations.

2.2.8  Order of Bit Transmission

With  the  exception of the FCS field, all fields  of  an

AX.25 frame shall be sent with each octet's least-significant bit

first.  The FCS shall be sent most-significant bit first.

2.2.9  Invalid Frames

Any frame consisting of less than 136 bits (including the

opening  and closing flags), not bounded by opening  and  closing

flags, or not octet aligned (an integral number of octets), shall

be  considered  an  invalid frame by the link  layer.   See  also

2.4.4.4, below.

2.2.10  Frame Abort

If a frame must be prematurely aborted, at least  fifteen

contiguous ones shall be sent with no bit stuffing added.

2.2.11  Interframe Time Fill

Whenever   it  is  necessary  for  a  DXE  to  keep   its

transmitter  on  while  not actually  sending  frames,  the  time

between frames should be filled with contiguous flags.

2.2.12  Link Channel States

Not applicable.

2.2.13  Address-Field Encoding

The  address  field of all frames shall be  encoded  with

both the destination and source amateur call signs for the frame.

Except  for the Secondary Station Identifier (SSID), the  address

characters only.  If level 2 amateur "repeaters" are to be  used,

their call signs shall also be in the address field.

The  HDLC address field is extended beyond one  octet  by

assigning  the  least-significant  bit of each  octet  to  be  an

"extension bit".  The extension bit of each octet is set to zero,

to indicate the next octet contains more address information,  or

one,  to  indicate  this is the last octet of  the  HDLC  address

field.   To make room for this extension bit, the  amateur  Radio

call sign information is shifted one bit left.

2.2.13.1  Nonrepeater Address-Field Encoding

If  level  2 repeaters are not being  used,  the  address

field is encoded as shown in Fig. 2.  The destination address  is

the call sign and SSID of the amateur radio station to which  the

frame is addressed, while the source address contains the amateur

call  sign  and SSID of the station that sent the  frame.   These

call signs are the call signs of the two ends of a level 2  AX.25

link only.

First

Octet Sent

Address Field of Frame

Destination Address         Source Address

A1 A2 A3 A4 A5 A6 A7    A8 A9 A10 A11 A12 A13 A14

Fig. 2 -- Nonrepeater Address-Field Encoding

A1  through A14 are the fourteen octets that make up  the

two  address  subfields of the address  field.   The  destination

subaddress is seven octets long (A1 thru A7), and is sent  first.

This  address sequence provides the receivers of frames  time  to

check  the  destination address subfield to see if the  frame  is

addressed to them while the rest of the frame is being  received.

The  source  address subfield is then sent in octets  A8  through

A14.   Both  of these subfields are encoded in the  same  manner,

except  that  the last octet of the address field  has  the  HDLC

address extension bit set.

There  is  an octet at the end of each  address  subfield

that contains the Secondary Station Identifier (SSID).  The  SSID

subfield  allows an Amateur Radio operator to have more than  one

packet-radio station operating under the same call sign.  This is

useful when an amateur wants to put up a repeater in addition  to

a regular station, for example.  The C bits (see 2.4.1.2,  below)

and H bit (see 2.2.13.2, below) are also contained in this octet,

along with two bits which are reserved for future use.

mode of operation.

Octet   ASCII   Bin.Data  Hex Data

Flag            01111110     7E

A1       K     10010110     96

A2       8     01110000     70

A3       M     10011010     9A

A4       M     10011010     9A

A5       O     10011110     9E

A6     space   01000000     40

A7     SSID    11100000     E0

A8       W     10101110     AE

A9       B     10000100     84

A10      4     01100100     68

A11      J     10010100     94

A12      F     10001100     8C

A13      I     10010010     92

A14    SSID    01100001     61

Control    I     00111110     3E

PID    none    11110000     F0

FCS    part 1  XXXXXXXX     HH

FCS    part 2  XXXXXXXX     HH

Flag            01111110     7E

Bit position    76543210

Fig. 3A -- Nonrepeater AX.25 frame

The frame shown is an I frame, not going through a  level

2 repeater, from WB4JFI (SSID=0) to K8MMO (SSID=0), with no level

3  protocol.   The P/F bit is set; the  receive  sequence  number

(N(R)) is 1; the send sequence number (N(S)) is 7.

2.2.13.1.1  Destination Subfield Encoding

Fig.  3 shows how an amateur call sign is placed  in  the

destination address subfield, occupying octets A1 thru A7.

Octet   ASCII   Bin.Data   Hex Data

A1       W     10101110      AE

A2       B     10000100      84

A3       4     01101000      68

A4       J     10010100      94

A5       F     10001100      8C

A6       I     10010010      92

A7     SSID    CRRSSID0

Bit Position-->   76543210

Fig. 3 -- Destination Field Encoding

Where:

1.  The top octet (A1) is the first octet sent, with  bit

0  of each octet being the first bit sent, and bit  7

being the last bit sent.

2.  The  first (low-order or bit 0) bit of each octet  is

the HDLC address extension bit, which is set to  zero

on all but the last octet in the address field, where

it is set to one.

3.   The  bits marked "r" are reserved bits.  They may  be

used in an agreed-upon manner in individual networks.

When not implemented, they should be set to one.

4.  The  bit marked "C" is used as  the  command/response

bit of an AX.25 frame, as outlined in 2.4.1.2 below.

5.  The  characters of the call sign should  be  standard

seven-bit  ASCII  (upper  case only)  placed  in  the

leftmost seven bits of the octet to make room for the

address  extension  bit.  If the call  sign  contains

fewer  than six characters, it should be padded  with

ASCII spaces between the last call sign character and

the SSID octet.

6.  The  0000  SSID is reserved for  the  first  personal

AX.25 station. This establishes one standard SSID for

"normal" stations to use for the first station.

2.2.13.2  Level 2 Repeater-Address Encoding

If  a  frame  is to go through  level  2  amateur  packet

repeater(s), there is an additional address subfield appended  to

the end of the address field.  This additional subfield  contains

the call sign(s) of the repeater(s) to be used.  This allows more

than one repeater to share the same RF channel.  If this subfield

exists,  the  last octet of the source subfield has  its  address

extension  bit  set to zero, indicating that  more  address-field

data  follows.  The repeater-address subfield is encoded  in  the

same  manner  as the destination and  source  address  subfields,

except for the most-significant bit in the last octet, called the

"H" bit.  The H bit is used to indicate whether a frame has  been

repeated or not.

In  order  to provide some method of  indicating  when  a

frame has been repeated, the H bit is set to zero on frames going

to  a repeater.  The repeater will set the H bit to one when  the

discard  any frames going to the repeater (uplink frames),  while

operating  through  a repeater.  Fig. 4 shows how  the  repeater-

address subfield is encoded.  Fig. 4A is an example of a complete

frame after being repeated.

Octet   ASCII   Bin.Data  Hex Data

A15      W     10101110     AE

A16      B     10000100     84

A17      4     01101000     68

A18      J     10010100     94

A19      F     10001100     8C

A20      I     10010010     92

A21    SSID    HRRSSID1

Bit Order  -->    76543210

Fig. 4 -- Repeater-address encoding

Where:

1.  The  top octet is the first octet sent, with bit  0  being

sent first and bit 7 sent last of each octet.

2.  As  with  the  source and  destination  address  subfields

discussed  above, bit 0 of each octet is the HDLC  address

extension  bit, which is set to zero on all but  the  last

address octet, where it is set to one.

3.  The  "R"  bits are reserved in the same manner as  in  the

source and destination subfields.

4.  The  "H" bit is the has-been-repeated bit.  It is  set  to

zero  whenever a frame has not been repeated, and  set  to

one by the repeater when the frame has been repeated.

Octet    ASCII   Bin.Data  Hex Data

Flag             01111110     7E

A1        K     10010110     96

A2        8     01110000     70

A3        M     10011010     9A

A4        M     10011010     9A

A5        O     10011110     9E

A6      space   01000000     40

A7      SSID    11100000     E0

A8        W     10101110     AE

A9        B     10000100     84

A11       J     10010100     94

A12       F     10001100     8C

A13       I     10010010     92

A14     SSID    01100000     60

A15       W     10101110     AE

A16       B     10000100     84

A17       4     01101000     68

A18       J     10010100     94

A19       F     10001100     8C

A20       I     10010010     92

A21     SSID    11100011     E3

Control     I     00111110     3F

PID     none    11110000     F0

FCS     part 1  XXXXXXXX     HH

FCS     part 2  XXXXXXXX     HH

Flag             01111110     7E

Bit position     76543210

Fig. 4A -- AX.25 frame in repeater mode

The  above frame is the same as Fig. 3A, except  for  the

addition of a repeater-address subfield (WB4JFI, SSID=1).  The  H

bit is set, indicating this is from the output of the repeater.

2.2.13.3 Multiple Repeater Operation

The  link-layer AX.25 protocol allows  operation  through

more  than  one  repeater, creating  a  primitive  frame  routing

mechanism.   Up to eight repeaters may be used by  extending  the

repeater-address subfield.  When there is more than one  repeater

address,  the repeater address immediately following  the  source

address  subfield  will be considered the address  of  the  first

repeater  of  a multiple-repeater chain.  As a  frame  progresses

through  a chain of repeaters, each successive repeater will  set

the  H bit (has-been-repeated bit) in its SSID octet,  indicating

that  the  frame has been successfully repeated through  it.   No

other  changes  to the frame are made (except for  the  necessary

recalculation of the FCS).  The destination station can determine

the  route  the frame took to each it by  examining  the  address

field.

The  number of repeater addresses is variable.   All  but

the last repeater address will have the address extension bits of

all  octets  set to zero, as will all but the  last  octet  (SSID

octet) of the last repeater address.  The last octet of the  last

repeater address will have the address extension bit set to  one,

indicating the end of the address field.

It should be noted that various timers (see 2.4.7, below)

may  have  to be adjusted to accommodate  the  additional  delays

encountered  when a frame must pass through  a  multiple-repeater

same path before reaching the source device.

It  is anticipated that multiple-repeater operation is  a

temporary method of interconnecting stations over large distances

until  such  time that a layer 3 protocol is in use.   Once  this

layer 3 protocol becomes operational, repeater chaining should be

phased out.

2.3  Elements of Procedure

2.3.1

The  elements of procedure are defined in terms  of  actions

that occur on receipt of frames.

2.3.2  Control-Field Formats and State Variables

2.3.2.1  Control-Field Formats

The control field is responsible for identifying the type

of  frame  being sent, and is also used to  convey  commands  and

responses  from  one  end of the link to the other  in  order  to

maintain proper link control.

The  control  fields  used in AX.25 use  the  CCITT  X.25

control fields for balanced operation (LAPB), with an  additional

control field taken from ADCCP to allow connectionless and round-

table operation.

There are three general types of AX.25 frames.  They  are

the Information frame (I frame), the Supervisory frame (S frame),

and  the  Unnumbered  frame (U frame).  Fig. 5  shows  the  basic

format  of  the  control field associated  with  these  types  of

frames.

Control-Field      Control-Field Bits

Type         7  6  5  4  3  2  1  0

I Frame          N(R)   P    N(S)   0

S Frame          N(R)  P/F S  S  0  1

U Frame        M  M  M P/F M  M  1  1

Fig. 5 -- Control-field formats

Where:

1.  Bit 0 is the first bit sent and bit 7 is the last bit

sent of the control field.

2.  N(S) is the send sequence number (bit 1 is the LSB).

LSB).

4.  The  "S" bits are the supervisory function bits,  and

their encoding is discussed in 2.3.4.2.

5.  The  "M" bits are the unnumbered frame modifier  bits

and their encoding is discussed in 2.3.4.3.

6.  The  P/F bit is the Poll/Final bit.  Its function  is

described in 2.3.3.  The distinction between  command

and response, and therefore the distinction between P

bit and F bit, is made by addressing rules  discussed

in 2.4.1.2.

2.3.2.1.1  Information-Transfer Format

All I frames have bit 0 of the control field set to zero.

N(S)  is  the sender's send sequence number  (the  send  sequence

number  of  this frame).  N(R) is the sender's  receive  sequence

number (the sequence number of the next expected received frame).

These numbers are described in 2.3.2.4.  In addition, the P/F bit

is to be used as described in 2.4.2.

2.3.2.1.2  Supervisory Format

Supervisory  frames  are denoted by having bit 0  of  the

control  field set to one, and bit 1 of the control field set  to

zero.   S  frames  provide  supervisory  link  control  such   as

acknowledging or requesting retransmission of I frames, and link-

level window control.  Since S frames do not have an  information

field,  the  sender's send variable and  the  receiver's  receive

variable are not incremented for S frames.  In addition, the  P/F

bit is used as described in 2.4.2.

2.3.2.1.3  Unnumbered Format

Unnumbered frames are distinguished by having both bits 0

and 1 of the control field set to one.  U frames are  responsible

for  maintaining additional control over the link beyond what  is

accomplished  with  S  frames.  They  are  also  responsible  for

establishing  and  terminating link connections.  U  frames  also

allow  for the transmission and reception of information  outside

of   the  normal  flow  control.   Some  U  frames  may   contain

information and PID fields.  The P/F bit is used as described  in

2.4.2.

2.3.2.2  Control-Field Parameters

2.3.2.3  Sequence Numbers

Every  AX.25  I  frame shall be  assigned,  modulo  8,  a

sequential  number  from  0 to 7.  This will allow  up  to  seven

outstanding I frames per level 2 connection at a time.

2.3.2.4  Frame Variables and Sequence Numbers

2.3.2.4.1  Send State Variable V(S)

The send state variable is a variable that is internal to

the  DXE  and  is never sent.  It contains  the  next  sequential

number  to  be assigned to the next transmitted  I  frame.   This

variable is updated upon the transmission of each I frame.

2.3.2.4.2  Send Sequence Number N(S)

The send sequence number is found in the control field of

all  I  frames.  It contains the sequence number of the  I  frame

being sent.  Just prior to the transmission of the I frame,  N(S)

is updated to equal the send state variable.

2.3.2.4.3 Receive State Variable V(R)

The receive state variable is a variable that is internal

to the DXE.  It contains the sequence number of the next expected

received I frame.  This variable is updated upon the reception of

an  error-free  I  frame whose send sequence  number  equals  the

present received state variable value.

2.3.2.4.4  Received Sequence Number N(R)

The  received sequence number is in both I and S  frames.

Prior  to  sending an I or S frame, this variable is  updated  to

equal  that  of  the received  state  variable,  thus  implicitly

acknowledging  the  proper  reception of all  frames  up  to  and

including N(R)-1.

2.3.3  Functions of Poll/Final (P/F) Bit

The  P/F bit is used in all types of frames.  It is  used

in  a  command  (poll) mode to request an immediate  reply  to  a

frame.   The  reply  to this poll is  indicated  by  setting  the

response  (final)  bit  in  the  appropriate  frame.   Only   one

outstanding  poll condition per direction is allowed at  a  time.

The procedure for P/F bit operation is described in 2.4.2.

2.3.4  Control Field Coding for Commands and Responses

The following commands and responses, indicated by  their

control field encoding, are to be use by the DXE:

2.3.4.1  Information Command Frame Control Field

The  function  of  the  information  (I)  command  is  to

transfer   across  a  data  link  sequentially  numbered   frames

containing an information field.

The  information-frame control field is encoded as  shown

subfield to maintain control of their passage over the link-layer

connection.

Control Field Bits

7 6 5 4 3 2 1 0

N(R) P  N(S) 0

Fig. 6 -- I frame control field

2.3.4.2  Supervisory Frame Control Field

The supervisory frame control fields are encoded as shown

in Fig. 7.

Control Field Bits       7   6   5   4   3   2   1   0

Receive Ready     RR        N(R)    P/F  0   0   0   1

Receive Not Ready RNR       N(R)    P/F  0   1   0   1

Reject            REJ       N(R)    P/F  1   0   0   1

Fig. 7 -- S frame control fields

The Frame identifiers:

C or SABM          Layer 2 Connect Request

D or DISC          Layer 2 Disconnect Request

I                  Information Frame

RR                 Receive Ready. System Ready To Receive

RNR or NR          Receive Not Ready. TNC Buffer Full

RJ or REJ          Reject Frame. Out of Sequence or Duplicate

FRMR               Frame Reject. Fatal Error

UI                 Unnumbered Information Frame. "Unproto"

DM                 Disconnect Mode. System Busy or Disconnected.

2.3.4.2.1  Receive Ready (RR) Command and Response

Receive Ready is used to do the following:

1.  to  indicate  that the sender of the RR is  now  able  to

receive more I frames.

2.  to  acknowledge  properly  received I frames  up  to,  and

including N(R)-1, and

3.  to clear a previously set busy condition created by an RNR

command having been sent.

requested  by  sending a RR command frame with the P-bit  set  to

one.

2.3.4.2.2  Receive Not Ready (RNR) Command and Response

Receive Not Ready is used to indicate to the sender of  I

frames  that  the receiving DXE is temporarily  busy  and  cannot

accept any more I frames.  Frames up to N(R)-1 are  acknowledged.

Any I frames numbered N(R) and higher that might have been caught

between states and not acknowledged when the RNR command was sent

are not acknowledged.

The RNR condition can be cleared by the sending of a  UA,

RR, REJ, or SABM frame.

The status of the DXE at the other end of the link can be

requested  by sending a RNR command frame with the P bit  set  to

one.

2.3.4.2.3  Reject (REJ) Command and Response

The  reject frame is used to request retransmission of  I

frames  starting  with N(R).  Any frames that were  sent  with  a

sequence number of N(R)-1 or less are acknowledged.  Additional I

frames may be appended to the retransmission of the N(R) frame if

there are any.

Only  one  reject  frame condition  is  allowed  in  each

direction  at  a time.  The reject condition is  cleared  by  the

proper  reception of I frames up to the I frame that  caused  the

reject condition to be initiated.

The status of the DXE at the other end of the link can be

requested  by sending a REJ command frame with the P bit  set  to

one.

2.3.4.3  Unnumbered Frame Control Fields

Unnumbered  frame control fields are either  commands  or

responses.

Fig.  8 shows the layout of U frames  implemented  within

this protocol.

Control Field                Type     Control Field Bits

7  6  5  4  3  2  1  0

Set Asynchronous Balanced Mode-SABM  Cmd    0  0  1  P  1  1  1  1

Disconnect-DISC                      Cmd    0  1  0  P  0  0  1  1

Disconnected Mode-DM                 Res    0  0  0  F  1  1  1  1

Unnumbered Acknowledge-UA            Res    0  1  1  F  0  0  1  1

Frame Reject-FRMR                    Res    1  0  0  F  0  1  1  1

Fig. 8 -- U frame control fields

2.3.4.3.1  Set Asynchronous Balanced Mode (SABM) Command

The  SABM  command  is  used  to  place  2  DXEs  in  the

asynchronous balanced mode.  This is a balanced mode of operation

known as LAPB where both devices are treated as equals.

Information fields are not allowed in SABM commands.  Any

outstanding  I frames left when the SABM command is  issued  will

remain unacknowledged.

The  DXE  confirms  reception and acceptance  of  a  SABM

command   by  sending  a  UA  response  frame  at  the   earliest

opportunity.   If  the  DXE is not capable of  accepting  a  SABM

command, it should respond with a DM frame if possible.

2.3.4.3.2  Disconnect (DISC) Command

The  DISC  command is used to terminate  a  link  session

between  two  stations.  No information field is permitted  in  a

DISC command frame.

Prior  to  acting on the DISC frame,  the  receiving  DXE

confirms acceptance of the DISC by issuing a UA response frame at

its  earliest opportunity.  The DXE sending the DISC  enters  the

disconnected state when it receives the UA response.

Any  unacknowledged  I frames left when this  command  is

acted upon will remain unacknowledged.