ARRL : 8th Computer Networking Conference - October 7, 1989 - page 203-209

DAMA - A NEW METHOD OF HANDLING PACKETS ?

==========================================

by Detlef J. SCHMIDT, DK4EG   Steinbrecherstr. 22    D-38106 BRAUNSCHWEIG

NORD><LINK e.V.

c/o Peter Glzow, DB2OS

Allensteiner Str. 5

D-30880 LAATZEN

Germany

Translation : Mark Bitterlich, WA3JPY

Reprint     : Pierre Cornelis, ON7PC

Lately it seems we are hearing more and more stories about hams who are

having trouble using their local node or digipeater. It seems that the user

has no trouble hearing the digi, but the digi doesn't seem to hear the user

at all. The symptoms almost match those where the receiver at the digi site

is either dead or close to it. While that kind of failure is always a

possibility, it is not the subject of this article.

The condition that this paper will talk about is one where the above

symptoms do actually occur, but not from any lack of receiver sensivity.

Instead it is due to the digi's receiver hearing too many signals all at

once and the remote user pretty much gets lost in the "noise".

The reason for this becomes obvious when we consider that while all the

users may hear the digi/node just fine, they in many cases don't hear each

other. Thus in some cases, more than one station will transmit at the same

time causing packet collisions. This situation is referred to as "a hidden

station" problem, and for remotely located users access to his or her

favorite digipeater become difficult to impossible during rush hour

periods.

This is not a new problem, and in fact there are other services

experiencing the same difficulties. A real world example is ships on the

open sea trying to gain access to a communication satellite.

Several different experiments have been made to overcome this dilemma on

amateur packet radio. One possible solution that is being pursued is

through the use of full duplex digipeaters (BTMA), however there are

several disadvantages to this approach. In a full duplex the hardware

expense will normally be much higher and the system will occupy two

frequencies but will only realize the maximum throughput of one. A better

approach might be to increase the throughput by reducing the collisions on

a single channel system rather than spreading the load onto two channels.

It would be ideal if we could incorporate a system that did this with

something so minor as software change (such as replacing the EPROM in a

TNC) or by changing some operational parameters.

One of the methods used that attempts to solve the hidden station problem

while still using a single frequency is called DAMA (Demand Assigned

Multiple Access). A description of this method follows.

In a connection oriented protocol environment, an end user will try to

connect to the master (satellite) by means of a slotted ALOHA method

(channel access without any coordination). Collisions might occur during

this phase but they are tolerable since they are relatively rare. Once a

connect request is recognized by the master, the connecting stations

identification is added to the polling list and from this point on the

master controls all connected stations. Permission to send data is granted

by means of polls which might be included in ACK packets or even in

transferred data frames.  so in this case a user will only be allowed to

transmit after receiving "permission" in the form of a poll sent from the

master station. Once permission is granted several frames might be

transmitted in a block. However, if the user does not respond within a

given time frame (say around 1/2 second) then the master assumes that the

poll got clobbered or the user never received it for some reason. The

master then passes permission to to transmit to all other active stations

and when completed comes back to the first user and gives him another

chance.

On the other hand, if the user (slave) actually receives the poll and

replies with sent I frames, the master will not acknowledge them until the

next time around after serving all the other active stations. If when

polled by the master, the user responds with an empty frame (Receive

Ready/Final), then the master will reduce the user in polling priority and

will skip him on the next time around.

As the activity on the frequency increases, the polling priority of

inactive users might be further decreased, but when these stations respond

with an I-frame they will again regain their original priority.

If you understand the description just given, you might think that you are

reading about AX.25 level 2 protocol and this is why DAMA has a chance of

working over amateur packet radio. AX.25 L2 provides all the protocol

elements that are needed to implement DAMA and no new syntax is required.

Most of the new functions required could be obtained simply by patching

existing operational parameters while the rest could be achieved by making

some minor changes to the TNC's firmware.

So how do we actually go about incorporating DAMA using AX.25 protocol ?

Due to the fact that there are no new syntax elements required, the

following description will only use standard AX.25 terms. Since CSMA

(Carrier Sense Multiple Access) as well as DAMA is used, please interpret

all future references DAMA as CSMA-DAMA. The term "poll" used throughout

this text in no way refers to the poll bit  in the control field of packet

frames and this bit remains unchanged to ensure compatibility. The

different phases of the protocol will be described separately below.

Connect Establish :

-------------------

When a node attempts to connect to a user, the node adds the users ID to

it's polling list and begins to send SABMs to that station. If after a

certain amount of tries no UA is received, the user is assumed to be

inoperable and is removed from the polling list.

When a new user starts a connect sequence to the node, he begins by sending

SABMs to the master in a simple CSMA manner duplicating the existing method

used today. Collisions are possible during this phase, so it might be

necessary to repeat the SABMs several times until the node replies with a

UA. Once the node recognizes the users connection attempt, the users ID is

added to the polling list in a fashion very similar to the one used by

TheNet nodes (TheNet userlist) and the node (master) is now in control of

the uplink users station. After the user sends SABMs and the node replies

with a UA, the user replies with an RR0 to signal to the node that it had a

successful reception of UA.

Idle state :

------------

As long as no information transfer occurs between user and node, (idles)

then the node sends its polls as an RR with the corresponding count. If the

response by the user is just an RR#, then the time until the next poll to

the user will be lengthened to avoid unnecessary channel load. The exact

amount of time added is determined by the total channel activity.

If information transfer by other users on the node is high (as determined

by the number of I-frames being sent) then the amount of time  added before

the next poll occurs to an inactive station is longer than in cases where

there is only very little  channel activity. Thus when the frequency is

basically clear, the waiting times are reduced to a minimum so that no

decrease in channel throughput takes place. This is the principle of the

self-alignment mechanism of DAMA, where a channel is always regulated to

insure its maximum possible throughput.

If the node ever fails to receive an RR from the user (due to a collision

of the nodes poll or the users RR response) then the node will proceed on

to the other stations on its polling list. The node will come back and try

this station again after all the other users on its list have been

serviced. If after a certain number of transmitted polls this station still

has not answered, then it is considered to be unavailable by the node and

is dropped completely from the list. This is analogous to those "keep-alive

polls" that we have today.

Data transfer : Node --> User :

-------------------------------

There is no difference between regular CSMA and DAMA in this case. Because

it is always up to the master (node) to act first, it could send one or

more I-frames or a poll to the user. The user will acknowledge I-frames

immediately with an RR#, but could also send its own I-frames with the

corresponding count (having to correct the count on the sent I-frame serves

the same purpose as an ACK with AX.25). The meaning of the Poll/Final bit

remains unchanged.

Data transfer : User --> Node :

-------------------------------

As mentioned before, the node will send polls to all users that are

uplinked to it and the user will not respond until it receives this poll or

an I-frame from the node. It may be wise to point out that when a user is

polled he must always come back with some kind of response, even if it is

an RNR#. If the node fails to hear any kind of response from the user then

it assumes something went wrong (such as a collision) and moves on to the

next user on its polling list.

This method of always waiting for a poll before transmitting is the central

aspect used to avoid collisions in a situation where hidden stations exist.

This is in contrast to the usual CSMA method where several stations can

actually transmit at the same time. Additionally the problem of deadtime

collisions is resolved. Deadtime refers to the period from when the TNC

realizes the channel is free and starts transmitting, to when he has been

on the air long enough for other TNCs to recognize his carrier. This is

really not a rare case, as exemplified by the case where two or more TNCs

are waiting for a digipeaters carrier to vanish so that they can leap on

the frequency. Using DAMA the node will not acknowledge received frames the

instant it hears them. Instead it will first service all other uplinked

stations and then come back with an RR# to the sending stations I-frames

along with a poll to that station. This poll basically says "Have you got

any else for me ?".

Disconnecting :

---------------

If the master intends to cut the connection, it will send the usual

DISC-frame to the user. The user will then promptly respond with the

UA-frame (final bit set). If the node fails to receive the UA and again

sends a DISC-frame, the user will respond with a DM-frame. This is

identical to the actual CSMA version.

When the user wants to disconnect from the node, he will wait to send his

DISC-frame until polled by the master. AT this point it makes no major

difference whether the node responds to the user right away with a UA or

goes through another polling cycle to do so, however an immediate UA is

preferred.

UI-frames :

-----------

In CSMA as well as in a DAMA environment, the UI-frames are treated in a

special way. I.E. These frames are used to carry some information besides

the regular protocol traffic. Normally UI-frames are never sent from a user

to a node, and it is not good headwork to make a habit of making UI-frame

direct QSOs on the input frequency of a node. However, in contrast to a

duplex system, it is possible to actually do this. So although the rare

UI-frames will reduce the throughput to the CSMA value, it will not drop

the much lower ALOHA value that would occur with a duplex digi having a QSO

on its input frequency. UI-frames originated by the node are no problem

since all stations receive these frames.

Other protocol elements :

-------------------------

So we have gone from the beginning to the end in describing a complete DAMA

session. We have not translated each and every AX.25 element into one that

has a special significance to DAMA. This is not required since many of them

will keep their initial meaning. DM, RNR, REJ, etc will all be used as they

were before. The only deviation from the pure CSMA version is in the fact

that the users will only be allowed to transmit these frames after

receiving permission from the master (node) in the form of a poll. The node

will only transmit these frames after all other users on its list are

served by completion of one polling cycle.

Compatibility of DAMA and CSMA :

--------------------------------

One advantage of the DAMA method is that it does not require everybody to

change everything all at once. However as additional users convert their

TNCs to work with DAMA the more pronounced will become the increase of

throughput. Even stations that are waiting to switch over could help to

increase the areas throughput by changing a few operational parameters. For

example the delay between the reception of a frame and the TNCs response

(sometimes called T2 or DWAIT) should be reduced to a value under 1 second.

In addition the time interval from when an I-frame is sent to when the TNC

sends an RR# to ask for a pending ACK, should be set to a value that is

clearly higher than the time between two polls of the master (usually more

than 30 seconds at 1200 Bd).

To fully benefit from DAMA both the node and the user must work together in

the master/slave relationship. Assuming that the users TNC is capable of

both the normal and the DAMA mode, there still remains the problem of how

to tell the user to "turn DAMA mode on". There are several ways that this

could be done:

1. Automatic detection of the protocol version by means of the protocol

identifier byte or reserved SSID-octet-bits of the node (preferred

version).

2. Implementation of a channel specific parameter which controls the

protocol version.

3. Implementation of a new UPLINK command besides the current CONNECT

command.

4. Implement a further protocol element such as a SABM-frame (similar to

X.25) so that at connect time the node could alert the user to the

increased features.

In case #1 of the above it would be sufficient to tell the user to switch

DAMA mode only once, at connect time. This state would then remain in

effect until disconnect. However since there is no PID  field in

SABM-frames this information has to be carried in some other way, such as

utilizing the dormant bit 5 of the master's SSID address field. It is

proposed that DAMA test versions set this bit to 0 to convey the necessary

information to the users TNC.

Conclusion :

------------

The existing AX.25 version was established in 1982 when packet radio was

not widespread as it is today. Most stations in the beginning were pretty

much equal and there was no distinction made between DTE and DCE functions.

However with the implementation of wide area networks not all stations are

performing the same function. In fact today the network nodes are acting in

DCE function considering their control and information exchanging aspects.

These functions will be better served with the implementation of DAMA.

The methods discussed in this article could increase the throughput on an

AX.25 channel tremendously. One advantage is the avoidance of system

breakdown which occurs with channel overload. Using DAMA, the throughput

will increase continuously up to its maximum. There is no foldback effect

like that which occurs using CSMA where at a special limit (above ca 60%)

the throughput is actually reduced.

There is also a strong "social" aspect of DAMA wherein even the weak

stations can work through the node reliably without being overpowered by

stations close to the node.

It is possible to make direct connections with other HAMs on the uplink

frequency unlike that of a duplex system. In addition the users TNCs still

retain the digipeater capability inherent in our present simplex system.

All protocol elements keep their original meaning which allows both

versions to be utilized on the same frequency, yet throughput increases as

more and more users switch over to the new method.

Literatur

Fox,T.        AX.25 Level 2 protocol specifications    AMRAD

Kauffels,F.J. Lokale Netze                             R.Mller Verlag

Mahle,C.      Satellite Scenarios and Technology       IEEE J. on selected

Hyde,G        for the 1990's                           Areas in communication

Inukai,Th.                                             May '87

Schmidt,D.J.  DAMA, ein neues Verfahren fr            cq-DL, 4/89

Packed-Radio?

Schmidt,D.J.  Synchrone DF-Protokolle mit 6809-Micro- TU-Script BS '81

Computern in heterogenen Sternnetzen

Tanenbaum,A.  Computer Networks                        Prentice Hall Verlag

-------------------------------------------------------- July 24, 1992 ---

Note by DB2OS: The full DAMA protocoll is implemented in the latest

version of TheNetNode (TheNet for PC) as a DAMA MASTER

and TheFirmware (WA8DED Hostmode for TNC2) as a DAMA

SLAVE by NORD><LINK. DAMA is also supported by TFPCX, a

resident AX.25 software TNC for the PC (only external

Modem required, no TNC) and by TFKISS (former TFPCR).

TFKISS emulates a TNC with TheFirmware and any TNC

can be used if it can be switched into KISS-Mode!!!

DIGICOM and BAYCOM Software also now supports DAMA.

DAMA is now used on several german TNN digipeaters and

the results are of great promise.