A Receiver Distribution Amplifier

David Johnson, G4AON, dave.g4aon@gmail.com

This amplifier is designed to be used with an active antenna to provide signals for up to four receivers in the ham shack, giving the opportunity to easily monitor several frequencies simultaneously. Such amplifiers are often used in commercial installations, yet circuits are rarely seen, nor do such units often appear on the surplus market.

Signal handling is more than sufficient when used with a PA0RDT type of active antenna and is low noise. Commonly available parts are used and a print circuit layout is available for home etching. The board is 122mm x 75mm and single sided.

The design is based on a generic receiver preamp with each stage being adjustable over roughly a 10 dB range, with a maximum gain of approximately +7 dB. The measured 3rd order input intercept of the assembled unit was +15 dBm with an input level of -20 dBm for each of two tones, giving unwanted products of -90 dBm. Isolation between channels is in the order of 20 to 25 dB.

* R7 and VR1 set the gain per stage, the values shown were fine with BSX20 transistors, but may need different values if other types of transistors are used.

While I used BSX20 transistors, 2N5109 or similar transistors with a reasonably high fT of around 500 MHz can be used, in which case a single stage could be built "rats nest" style to check the performance. The Club free “2N3866-like” devices should also be ok. The print board layout uses a TO5 spacing to more readily accommodate alternative transistors.

The amplifier comprises 4 common emitter preamps in parallel, each section is designed for an input impedance of 200 Ohms giving a combined input impedance of 50 Ohms, with an output impedance of 50 Ohms. Each stage draws just under 20 mA at a supply Voltage of 13.8 Volts. Protection from reversed supply is via a series diode with a 250 mA polyfuse to limit the current in case of a short circuit.

If testing a single stage, use a 68 Ohm resistor across the input BNC socket to simulate the combined input impedance of the stage under test and the 3 “missing” amplifiers (3 X 200 ohms in parallel = 66.7 ohms).

The bandwidth is approximately 30 KHz to 100 MHz at the -3 dB points. At the expense of a reduced lower frequency response, the more common FT50-43 cores could be substituted and will give a bandwidth from around 200 KHz to more than 100 MHz. There is no filtering to limit the upper frequency response, or to reduce broadcast stations on lower frequencies. If needed, an external filter could be added, although unless you have a nearby broadcast station, it is unlikely to be necessary.

There is no automatic switching to disable the amplifier when transmitting, at QRP power levels (and reasonable spacing of your active antenna from your main antenna), disabling the active antenna may not be needed during transmissions. If needed, there is a suitable switching circuit in the active antenna link below.

Construction

The design could be built “Manhattan style”, or using a home etched and drilled board using the layout in the link below. Each transformer is wound using a pair of 0.25mm enamelled copper wires tightly twisted together using 2 or 3 turns per centimetre. It is important to twist the wires, parallel wires have less bandwidth. Take care to not damage the enamelled coating.

Single sided PCB was used with unused areas flooded with copper to provide sufficient ground areas. Lead spacing for the 680nF and 100nF capacitors is 7.5mm, smaller components will need their leads adjusted to fit.

Each stage should be checked for an emitter Voltage of around 3 Volts, and the overall current checked (approx 75 mA). Finally mask off the mounting holes, then spray the copper side of the board with print circuit lacquer and secure the transformer cores to the board with "704" silicon, or similar, non acidic fixative. The board is mounted using M3 (3mm thread) hardware. There are no ground connections to the board except via the mounting spacers, check the continuity of each mounting spacer to the negative pin of the board prior to fitting the board in the case. Note in order to avoid ground loops and possible noise pickup, the power input socket negative connection should only go to the pin on the board, not the case.

The gain preset control, VR1, when set to the mid point, should give around unity gain. If additional gain is needed, turning VR1 3/4 clockwise will give approximately 3 dB of gain and close to 10 dB when turned fully clockwise. Avoid the temptation of running all the channels at full gain, most modern receivers have a lot of sensitivity on HF and don't need additional gain from a preamp. Unused channels should be terminated in 50 Ohms using BNC 50 Ohm terminators, these are cheaply available from eBay suppliers.

GM4EVS developed the initial design of the 200R in, 50R out amplifiers.  His LTspice modelling accurately predicted the performance as measured with a spectrum analyser and tracking generator.

Useful links

Board layout and parts list:
https://www.qsl.net/g4aon/resources/dist_amp.zip

A suitable active antenna with transmit switching, as used with this project: https://www.qsl.net/g4aon/pa0rdt_aa/

Enigma Components, a UK supplier of FT50-75 cores: https://enigma-shop.com/