Model:         IC-04AT

Division:      Amateur

Note:          This service bulletin is meant for technical per-

sonnel with experience working on solid state com-

munications equipment.  Damage caused by im-

properly installing this modification may cause

ICOM to charge for subsequent repairs to the

product.  ICOM does not warrant this modification.

Bulletin #:    02988-002

Date:          January 1, 1988

Subject:       Transmitter delay may be excessive at low tempera-

tures (can be up to 2 or 3 seconds).

Procedure

1.  Remove the front and rear covers.

2.  Remove the volume and squelch knobs by pulling up on them.

Push down the high/low power and lamp switch.  Remove the

four screws in the metal chassis corners.  Unfold the chas-

sis.

3.  Replace R262 (currently 560K) with a 2.2 Meg resistor (ICOM

P/N 915-01277).

4.  Reassemble the radio.