Model:         IC-12AT

Division:      Amateur

Note:          This service bulletin is meant for technical per-

sonnel with experience working on solid state com-

munications equipment.  Damage caused by im-

properly installing this modification may cause

ICOM to charge for subsequent repairs to the

product.  ICOM does not warrant this modification.

Bulletin #:    28Oct86-019

Date:          October 28, 1986

Subject:       Improve RIT and VXO operation

Procedure

1.  Replace C116 (1 mf 50V currently) on the Main board with a

.22 mf 50 V capacitor.

Bulletin #:    22987-008

Affected S/Ns: Less than S/N 4101

Subject:       Improve the stability of the CTCSS tone in trans-

mit

Procedure

1.  Remove and replace the following components:

Component      Old       Component      New

Identifier     Value     Location       Value

C383           470 pf    PLL Board      Remove this part

R369           39K       PLL Board      22K

R370           2.2K      PLL Board      2.7K

C102           .0027 mf  Main Board     .0018 mf 50 V

Bulletin #:    22987-009

Subject:       No memory retention and CPU locks up

Information:   Lithium battery is dead because D112 is shorting

to the flex board

Procedure

1.  Trim excess lead from D112 and carefully re-solder.