From: WB2ZTK@WB2ZTK

To: ALL@ALLBBS

Hello Out there....

I've just completed my work on the PK 232.  I mention earlier in the week

of a problem with the PK 232 regarding RFI and how the 13 volt supply line is

unprotected from accidentally placing voltages above 13.6 into that box.

This 13.6 v line is connected to a couple of IC and branch to many transistors

and is use as a pull up voltage for many circuits within the box.

Well now that the highly stable voltage regulator in in place along with

a RF choke of 20 uH.  I can easly hear some of our checkins during

the my Faxnet on 28.450.  The RFI from that box was of the low level type and

was knocking out the weak station when the unit was on.

I am very happy with the addition for two reasons.

1.  It protect the PK 232 from Murphy and I.

2.  It gives my rig isolation from the RF that was leaking down the power

supply leads to a common power supply shared with the rigs and other

devices.

For those that plan to add this protection, you'll find plenty of room inside

the PK 232 to place this additonal Voltage reg.  You might want to consider

adding a bridge rectifier and 120 to 18 v @ 1 amp transformer, and connect

the voltage regulator to that.  I feel that the voltage regulator should

be internally placed in the PK 232 along with the choke because of the

leakage that was coming from the leads to a external supply.  Some will

find that placing ferride beads to help from this leakage, and keeping their

external supply as is.  Well some of us use power supplies that are ajustable

and might apply voltages above 18v to the PK 232.  The internal voltage

regulator protects against this over voltage and choke blocks the RFI,

this placement makes the best sense to me and thats how I went.

Having fun and feeling safe, Dan...   WB2ZTK...