I made the mod for expanded coverage on the 2sat and here is the

documentation I came up with. D9 was supposed to be removed. It already

was in my unit but was installed in a second one I modded. Cut it out.

It is located on small standup board after top and bottom are separated.

D6 is changed from a 3 terminal diode with anode on right to one with diode on left. I found that the unit that had D9 did not have D6 and viseversa.

Install D6 with diode on left.

o

----------

|  /     |

| /      |

__________

o      o

anode ^

Range and Mode documentation

FM receive only down to 75 Mhz NBFM sensitivity @ 75 Mhz = 500 uv

90 Mhz = 100 uv

108 Mhz unit starts receiving equally well on am or fm. 2 uv

110 Mhz = 2 uv FM  and 6 uv AM

120 Mhz = .5 uv FM and 2 uv AM

130 Mhz = .75 uv FM and 2 uv AM

131 Mhz the unlock "u" comes on but still receives same as 130 Mhz

135 Mhz = .75 uv FM and 2 uv AM  "u" showing

138 Mhz unit switches off AM receiv and sens on FM = .3 uv

Transmitter enabled and Hi power with internal bat = 1.9 w

All indications good up to 162 Mhz where transmitter unlocks

165 Mhz sensitivity = .6 uv FM rec only

170 Mhz = 1.5 uv FM rec only

175 Mhz = 30 uv FM rec only

180 Mhz = 40 uv

193 Mhz = 50 uv "u" comes on

200 Mhz = No rec at all

204 Mhz unlock goes off and sens = 300 uv

204 - 220 sens dropps to barely receiving 20,000 uv

225 Mhz = 5000 uv

230 Mhz = 3000 uv

240 - 270 = 200 uv

280 Mhz 5 uv

290 Mhz = 1 uv up to 320 Mhz

320 - 367 Mhz = .6 uv

367 and above nothing

Dial runs from 0 to 1310 Mhz

73, N5WX