Model:         IC-3210

Division:      Amateur

Note:          This service bulletin is meant for technical per-

sonnel with experience working on solid state com-

munications equipment.  Damage caused by im-

properly installing this modification may cause

ICOM to charge for subsequent repairs to the

product.  ICOM does not warrant this modification.

Bulletin #:    28Oct86-027

Date:          October 26, 1986

Subject:       To prevent spurious signals on TX when the PA

module was changed form a Toshiba part to a Mit-

subishi part

Procedure

1.  Replace the following parts on the main board:

Current        Component      New

Part           Identifier     Part

470K           R40            680K

560K           R41            1Meg

4.2K           R42            4.7K RH0521CS3J04A

47K            R44            100K RH0651C15J1UA

Bulletin #:    13889-021

Date:          May 1, 1989

Subject:       Some IC-3210s exhibit poor sensitivity in the UHF

band

Procedure

Write to ICOM's Customer Service department for a copy of

this service bulletin.