========================================================================

Date:         3 Dec 87 16:26:31 GMT

From:         Marc Kaufman <kaufman@shasta.stanford.edu>

Subject:      ICOM alignment (was Re: mystery signal)

In article <2131@dasys1.UUCP> schuster@dasys1.UUCP (Michael Schuster) writes:

.In article <2455@ihuxz.ATT.COM> parnass@ihuxz.ATT.COM (Bob Parnass, AJ9S)

writes:

.>    Your ICOM R7000 may be slightly off frequency. Mine is.

.So was mine. Being a perfectionist I put up with it and gritted my teeth dail

.The signal center meter was off too; and inconsistent in its inaccurary from

.band to band. finally as the warranty was near expiration I sent it back to

.Bellevue. They did a re-alignment and included a work order sheet that was mo

.impressive...

Well, now that you are out of warranty, you can put the thing on frequency!

Open the box, remove the metal covers on the right hand circuit board, that

cover the master oscillator, and tweak the coil (I think it's L39 on the

PLL reference oscillator board).  I took advantage of the fact (or probability

at least) that commercial services are very nearly on frequency.. so I

set the radio on a known 480 MHz station, and tweaked for zero deviation on

the meter.  If the IF is alligned properly, center on the meter should

correspond to having the xtal on frequency.

All crystals age, and it is worth knowing how to correct for it.

This is cheaper and more reliable than sending the unit to Washington.

Marc Kaufman (kaufman@Shasta.standford.edu)

G4SPV BBS>