ALR72T

EXTENDED FREQUENCY RX AND TX OF THE ALR72T & ALR72HT

ALINCO 70 CENTIMETER RADIOS.

BEWARE OF ANY STATIC WHEN WORKING WITH THIS RADIO !!!

1 Remove the top and bottom covers of the radio.

2 Remove the 4 screws that hold the front panel assembly to the main

chassis.

3 Pull front panel away from the main chassis very gently.

4 Locate the Volume\Squelch pot on the left side of the radio.

5 Locate the VFO control on the left side of the radio.

6 Between the two above units is an IC that runs vertically between

the two units. (ONLY THE 12 PINS OF THE IC ARE SEEN FROM THE BACK)

7 After locating the IC locate 3 sets of unsoldered jumper pads between

the IC pins.

8 Make a solder bridge between the top 2 pads,then the middle 2 pads,then

between the bottom 2 ONLY of the bottom set of 3 pads.

REAR VIEW OF FRONT PANEL

/ ------\  12 PIN   O  *  O                        /---------\

/ VOLUME  \  IC ->      |<-SOLDER BRIDGE           /           \

/ CONTROL   \         O  *  O                      /             \

/             \                                    /     VFO       \

\             /        O  *  O                      \  CONTROL     /

\         /             |<-SOLDER BRIDGE           \           /

\-----/            O  *  O                        \--------/

O  *  O

*  DO NOT SOLDER THE TOP PAD

O  |< O   OF THESE 3 PADS

* \SOLDER BRIDGE HERE

9 RE-ASSEMBLE RADIO .

10 RESET MICROPROCESSOR.

GOOD LUCK AND HAPPY MODING 73 CARL VE3ZCO@VE3GYQ

CARL@HAMSTER.UWO.BUSINESS.CA.NA [129.100.22.100]

CARL VE3ZCO [44.135.80.8]