amt1.1

6),  7)  THESE MODS ARE MERELY MAKING USE OF THE POINTS THAT WERE

CONVENIENTLY LEFT ON THE BOARD TO DO DATA INVERT SWITCHING.  I USED

DPDT SWITCHES FOR THIS BECAUSE I WANTED A FOOLPROOF WAY OF KNOWING

WHEN THE DATA WAS INVERTED.  I TURN ON A RED LED WITH THE SECOND

SWITCH CONTACT TO TELL ME THAT ONE OF THE SWITCHES IS NOT

NORMALIZED.  THE RED LED IS MOUNTED IN THE SLOT JUST TO THE RIGHT OF

THE SEND LIGHT ON THE FRONT PANEL.  YOU CAN FIGURE OUT THE SERIES

RESISTOR FOR YOUR PARTICULAR LED.  I USED THE SAME STYLE LED THAT

THE REST OF THE UNIT HAS.

THE POINTS TO BE SWITCHED WITH THE TWO SWITCHES ARE ON THE

SCHEMATIC.  THE RCVE DATA INVERT IS LABELED L11 INPUT NOR/REV AND IS

LOCATED IN THE MIDDLE OF THE UPPER LEFT QUARTER OF SHEET 1 OF THE

SCHEMATIC.  THE TX DATA INVERT POINT IS LABELED L10 OUTPUT NOR/REV

AND IS LOCATED IN ABOUT THE SAME AREA OF SHEET 3. I WILL LEAVE YOU

TO FIND THEM ON THE BOARD.  ANY EXISTING JUMPERS WILL HAVE TO BE

REMOVED, OBVIOUSLY.

continued in file amt1.b