AMT2

NOW ON TO THE TWO JACK VERSION.

TWO JACK VERSION...

BACK TO SHEET 1, IC21/22.  CUT THE TRACE FROM IC21, PIN 3 TO IC22,

PIN 4 BEING CAREFUL OF WHERE YOU CUT IT.  THIS IS ONE PLACE THAT YOU

CAN CUT WRONG.  HOOK THE ARMATURE OF A SWITCH SECTION TO IC22, PIN

4.  THEN HOOK THE N.O. CONTACT TO IC21, PIN 3.  MAKE SURE THAT THE

PREVIOUS SWITCH SECTION N.C. CONTACT STILL GOES TO PIN 3 ALSO.  THE

N.C. CONTACT WILL THEN GO TO AN ADDED JACK FOR TTL DATA INPUT TO THE

AMT-1.

LOCATE SHEET 3 AGAIN AND ICS 16 AND 3.  CUT THE TRACE FROM IC16 PIN

3 TO IC3 PINS 12/13.  THE ARMATURE OF A SWITCH SECTION GOES TO IC16,

PIN 3.  THE N.O. CONTACT GOES TO IC3, PINS 12/13 WHICH IS THE SAME

POINT AS LK1 HOLE A AND THE PREVIOUSLY INSTALLED N.C. SWITCH

CONTACT.  HOOKING THE N.C. CONTACT TO ANOTHER SEPARATE TTL OUTPUT

JACK WILL FINISH THIS PART OF THE MOD.

FOR THE ADDED TTL JACKS, I USED A COUPLE OF RCA (PHONO) JACKS.  THEY

ARE EASY TO MOUNT AND USE.  BE SURE TO LABEL THE INPUT AND OUPUT SO

YOU DONT GET THEM MIXED UP.

CONTINUED IN FILE AMT1.8