AMT1.d

12)  THIS ONE IS EASY.  MOUNT ANOTHER RCA JACK ON THE BACK PANEL AND

RUN A WIRE TO POINT B OF THE MATRIX SHOWN IN THE MIDDLE OF THE UPPER

LEFT QUARTER OF SHEET 3.  THATS ALL THERE IS TO IT.  IT WILL PROVIDE

TTL OUTPUT LEVEL TO THE XCVR FOR FSK.  IF OPPOSITE POLARITY IS

NEEDED YOU CAN USE POINT A ON THE MATRIX.  BUFFERING MIGHT BE WISE

BUT NO PROBLEM HAS OCCURRED ON THE ONE I MODIFIED.13)  THIS MOD IS NEEDED ONLY IF YOU HAVE A STRONG SQUARE WAVE SIGNAL

APPEARING ON TP6, THE OUTPUT OF THE ACTIVE FILTER (SHEET 1, TOP,

RIGHT OF CENTER).  THE UNWANTED SIGNALS SECOND HARMONIC WILL FALL

JUST BELOW THE MARK TONE OF 2125 AND DESENSITIZE THE DEMODULATOR

CIRCUIT.  IT WILL ALSO BE APPARENT ON THE FREQUENCY DISPLAY WITH

VERY LOW OR NO AUDIO FEEDING THE AMT-1.

TO MAKE THE MOD, IS SIMPLE.  I HAVE A PHOTOCOPY FROM AEA OF THE

AMT-1 BOARD THAT SHOWS WHERE TO CUT ONE TRACE AND ADD ONE WIRE. I

DONT THINK IT WILL REPRODUCE SO IF IT IS NEEDED, IT WILL BE LOANED

OUT.  IT IS AN EASY FIX FOR A VERY SERIOUS PROBLEM.  ON THE ONE I

DID, IT WIPED THE UNWANTED SIGNAL OUT COMPLETELY.

CONTINUED IN FILE - AMT1.3