AMT1.e

IF YOU REMOVE THE YELLOW TRAFFIC AND IDLE LIGHTS AND THE FOURTH

GREEN LIGHT FROM BOTH ENDS OF THE FREQUENCY DISPLAY AND SWAP THEM

AND THEN DO THE ALIGNMENT MODIFICATION BELOW, YOU WILL HAVE A

PRECISION DISPLAY. BE CAREFUL TO OBSERVE THE POLARITY REVERSAL ON

THE LEDS.  THE YELLOW ONES ARE MOUNTED UPSIDE DOWN ON THE BOARD FROM

THE WAY THE GREEN ONES ARE MOUNTED. IT WILL BE OBVIOUS WHEN YOU HAVE

THEM RIGHT. THEY SHOULD MATCH THEIR NEIGHBORS.

THERE IS SOME MINOR ADJUSTMENT OF THE DISPLAY ALIGNMENT NEEDED TO

GET THEM CORRECT BUT WHEN THEY ARE, YOU WILL BE ABLE TO GET WITHIN

10 HZ OF ANYONE WITH ALL THE LIGHTS OUT AND AT THE SAME TIME YOU

WILL HAVE CONVERTED THE STATUS DISPLAY TO SHOW A GREEN (GO)

INDICATION WHEN YOU ARE SYNCED PROPERLY TO A STATION.

DO A DISPLAY ALIGNMENT PER BOOK INSTRUCTIONS (PAGE 8, PARAGRAPH

10.3) AND THEN FINE TUNE IT.  TO FINE TUNE, FEED THE AMT-1 A TONE OF

2125 AND ADJUST RV3 IN THE DIRECTION TOWARD THE RIGHT HAND YELLOW

LED OF THE FREQUENCY DISPLAY.  THEN FEED 2295 TO THE UNIT AND ADJUST

RV4 THE SAME WAY FOR THE LEFT SIDE.  GO BACK AND FORTH ON THE TWO

TONES AND POTS UNTIL YOU HAVE THEM PRECISELY TUNED TO MARK AND SPACE

ON THE YELLOW LEDS.  TUNE IN A SIGNAL OF KNOWN ACCURACY (SEE MODS

NUMBER 6, 8, AND 9) AND MAKE SURE THAT WHEN YOU MOVE SLIGHTLY OFF

THE SIGNAL FAR ENOUGH THAT THE ADJACENT LED STARTS TO LIGHT THAT THE

SAME THING HAPPENS ON THE OTHER END OF THE DISPLAY.

continued in file amt1.f