DJ160.3

The fifth digit is REWRITTEN whenever the radio receives a proper group or

personal page code, and will display personal code number that was received.   It

is transmitted only when the radio is in GP DSQ mode.  It is NOT transmitted when

the radio is in DSQ or G DSQ modes.  See below.

To set the DSQ number, push [f] [DSQ set], enter five numbers, then push

[V/M].

NOTE:  In the examples that follow, I will assume you have programed the

code 123.4.5 into your DSQ using the DSQ set function.

(a) DSQ Squelch Operation:

Receive Mode:   DSQ squelch is used to cause the radio to be silent

(squelched) while in receive until it receives a three digit code.  IF it

receives the correct three digits during the first second or two of a

transmission, it will release the squelch and the receive the rest of that

transmission.

Transmit Mode:  When used to transmit, the radio will automatically transmit

the three digit DSQ group code right after you press the [PTT] button, so

remember to pause a moment before talking if using the radio in DSQ mode.

To set DSQ Squelch, push [f] [GP DSQ] until DSQ is displayed.   Be sure that

ONLY DSQ is displayed, not G DSQ or GP DSQ.  Normally, if you start with no DSQ

display, hitting [f] [GP DSQ] once will cause DSQ only to be displayed.

Summary of DTMF Squelch function:

Receive:   Radio remains squelched unless it receives the tones 1,2,3

Transmit:  Radio transmits the tones 1,2,3  whenever [PTT] is pressed.

(you must hold down the PTT button throughout your transmission.  If you let go

of the PTT button, you must re-transmit the three digit code if you want the

receiver to hear you!  This is not a problem when using one DJ160 with another,

but can be if you are using the DJ160 with a different radio where manual entry

of the DTMF tones is required)

NOTE:  If using another radio to manually send DTMF squelch codes, the three

digits must be punched in within a space of 1.5 seconds total.

(b) DTMF page function:

This mode is set using the [f] [GP DSQ] buttons.  Hold down the [f] button

and press the [GP DSQ] button until G DSQ or GP DSQ is displayed.  G DSQ stands

for group page mode, and GP DSQ stands for personal page mode.  Normally if no

DSQ function is displayed, you hold down the [f] key and press the [GP DSQ] key

once to display DSQ, twice to display G DSQ, three times to display GP DSQ, and a

fourth time to clear the DSQ function from the display.

CONTINUED IN DJ16OT.4

DJ160T.5

Personal Paging:

This is a somewhat complicated variant of group paging, used to page a

particular person in a group.  Not only is it complicated, but the implementation

on the Alinco DJ 160T differs from that used on some other radios, for the Alinco

uses a single digit personal code where other radios sometimes use a three digit

personal code.  Generally I recommend users not bother with this overly

complicated feature, but I will attempt to at least partially document it here.

Its primary use where you have a party of several individuals all with the same

group but different personal paging codes.  In this situation one can either page

all members of the group using the group page function OR page a given individual

member of the group using the personal page function.

Receive:  The radio may be set to display either "G DSQ" or "GP DSQ".  This

is done using the [f] [GP DSQ] buttons.  If the three digit group code followed

by a single digit personal code followed by a star is received (1,2,3,4,*), the

unit will make the "personal page type" tone (beep boop), the "GP" in the display

will flash, and "Err" will be displayed on the main display.  The "Err" is

displayed because a personal identifier digit was not sent.  If in addition to

the above codes a personal identifier code is received (1,2,3,4,*,7) the radio

will make the personal page type beeping and "GP" will flash.  In addition, the

last digit of the DSQ code will be re-written to 7, to reflect the fact that a

page FROM the person with code number 7 was sent.  Pushing [V/M] button will stop

the flashing of the "GP" in the display.

Transmit:  (Pay CLOSE attention here!  The radio uses the numbers in its DSQ

SET significantly differently when transmitting in GP DSQ mode than it does when

in G DSQ mode!)   Radio MUST be in GP DSQ mode (NOT G DSQ mode) to properly

automatically transmit a personal DTMF page.  Let us assume that you have set

your DSQ code to 123.4.5.  When the [PTT] button is pressed, the radio will send

the following tones:  1,2,3,5,*,4.

CONTINUED IN DJ16OT.6

DJ16OT.6

The first three tones sent are the group code.  This is followed by personal

code of the person you want to PAGE.  Then a star is sent, then YOUR personal

code is sent as an identifier.  Note carefully that it is the FIFTH (last) digit

of the five digit DSQ number that is sent as the FOURTH digit of the DTMF

transmission when you are in GP DSQ mode!  Note too that this last digit is

rewritten every time you RECEIVE a page, so if you want to send a personal page

to someone OTHER THAN the person who paged you last, you must use DSQ SET to

rewrite the last digit of your five digit code to specify the personal code digit

of the person you want to page.  Alternatively, note that when you enter GP DSQ

mode by pressing [f] [GP DSQ] three times, the five digit code is displayed.  At

this point, entering a SINGLE digit will usually cause the fifth digit to be

changed to whatever you want it to be.  Pressing [f] [GP DSQ] a fourth time will

put the radio in GP DSQ mode, removing the display of the five DSQ code digits.

If no code is entered when the GP DSQ display of the code is called up, after

about 5 seconds the display will automatically revert to the normal radio display

with GP DSQ set.  Alternatively, you can reset the entire DSQ display using the

[f] [DSQ set] button combination.  In any case, when using the GP DSQ (personal

page) function, you must each time check the code to make sure you have set the

personal and group code correctly.  Be especially careful that the outgoing

personal code (last digit of the display) is correct for the person you want to

page.

Summary of DSQ Code digits:

123.4.5    in display

a  b c

a  Three Digit Group Code.

DTMF Squelch:

(receive mode) These three digits must be received to break squelch when DTMF

squelch (DSQ only displayed) is used.

(transmit mode) These three digits only are transmitted when in DSQ mode upon

pressing the [PTT] button.  example:  if you have set 123.4.5 as your DSQ code,

when you press [PTT] in DSQ mode 1,2,3 will be transmitted.

Group and Personal Page functions:  These three digits are used as the

group code in both group and personal paging functions and are the first three

digits transmitted as part of a larger group or personal page code transmission.

They are the first three digits that MUST be received when trying to activate a

group or personal page.  See below for more details.

b  This is YOUR personal page code.

DTMF Squelch:  This digit IS NOT USED in DTMF squelch functions.

CONTINUED IN DJ16OT.7

DJ16OT.4

Receive Mode:  In this mode, the radio is silent until it receives the

proper page tone sequence.  At that point, it will beep in a fashion that is

characteristic of either receipt of a group page or of a personal page.  The tone

for a group page consists of several dual beeps, with all beeps being of the SAME

tone.  eg:  beep beep      beep beep     beep beep    etc.

The tone for a personal page consists of several dual beeps, but the two beeps

within each dual beep are of different frequencies.  eg:  beep boop     beep boop

beep boop   etc.

NOTE:  The radio behaves EXACTLY the SAME when in either G DSQ or GP DSQ

modes when it is RECEIVING.  The two modes (G DSQ and GP DSQ) differ ONLY in

terms of what tones get transmitted when you push the [PTT] button!!  I recommend

you use only the G DSQ mode, for it is simpler to understand.  But I will attempt

to shed some light on the GP DSQ mode too.

Group Paging:

Receive:   (Radio may be set to either G DSQ or GP DSQ)  Radio must receive

group code (1,2,3) followed by a star (*) at a minimum in order to produce the

pager tone.  If the radio receives ONLY the three digit group code followed by a

star (1,2,3,*) it will give the group page beep, the "G" in the G DSQ of the

display will flash, but "Err" will be shown in the main display because the

personal code was not properly sent.  If the radio receives a group code followed

by a star followed by a personal code OR a group code followed by a star followed

by a personal code followed by two stars (1,2,3,*,4) or (1,2,3,*,4,*,*)  it will

give the beep for receipt of a group page, the "G" on the display in the G DSQ

will flash, AND, if you press [f] [DSQ set] the fifth digit displayed will be the

personal code that was just received (4 in this case).   If the unit was set for

123.4.5 in the DSQ SET mode, that display will be altered to 123.4.4 to reflect

the fact it received a "4" as a personal code digit.  After a page is received

and the "G" in the display is flashing, pushing [V/M] button once will cause the

G to stop flashing.

Transmit:  The DJ 160T must be in G DSQ mode to properly transmit a group

page (not in GP DSQ mode).   Pressing the [PTT] key will automatically send out

the group code, followed by a star, followed by your personal code (fourth digit

in the DSQ set display), followed by two stars  (1,2,3,*,4,*,*).

CONTINUED IN DJ16OT.5

DJ16OT.8

Group paging:

(receive mode)  This digit is re-written any time a proper group page is

received to reflect the number of the other party who sent you the page.

example:  you have 123.4.5 in your DSQ setting and you now receive a group page

consisting of 123*8**  (or one simply consisting of 123*8).  Your pager will make

the group page type beep, "G" will flash in the display, AND, if you go to DSQ

set mode, you will see that the last digit of the code displayed is now NOT a 5,

but is instead an 8, informing you that "person number 8" was the originator of

that group page.  Thus, if your page code WAS set to 123.4.5 and you receive the

group page 123*8**, when you now press [f] [DSQ set] you will no longer see

123.4.5 but instead see 123.4.8.

(transmit mode)  This digit is NOT USED in group page transmit mode.

Personal paging:

(receive mode) This digit is re-written any time you receive a valid personal

page, and will display the personal code of the person who sent that page.

example:  you have 123.4.5 as your DSQ code, and you receive the following

sequence:  1234*7   Your pager will now make the personal page type beep, "GP"

will flash on the display, and if you press [f] [DSQ set] you will no longer see

123.4.5 displayed, but instead will see 123.4.7.   The last digit reflects the

fact you received a personal page from a person whose own personal code is 7.

(transmit mode) This is the digit that is sent out FOURTH when you push the

[PTT] button.  It specifies which person in the group will receive the personal

page.  You must be careful to set this digit correctly for the person you in the

group who you wish to page, keeping in mind that this digit gets re-written every

time you receive a page of any kind!  If you have 123.4.5 set as your DSQ code,

pressing [PTT] in GP DSQ mode will cause 1,2,3,5,*,4 to be transmitted.  Note

very carefully that this, the fifth digit in your DSQ setting display, is sent

out FOURTH when you are in GP DSQ mode and press the [PTT] button!

CONTINUED IN DJ16OT.9

DJ16OT.7

Group Paging:

(receive mode)  this digit IS NOT USED in group paging receive mode.   ONLY the

first three digits and the last digit are used when receiving a group page.

(transmit mode) When you press [PTT] in G DSQ mode, first the three digit group

code is sent, then a star, then this digit (your personal page code) is sent

followed by two stars (1,2,3,*,4,**).  Thus, this digit is the digit that is SENT

OUT TO OTHERS to identify to them who has sent the page.  It will be displayed in

the fifth digit of the DSQ code display on all radios that received your group

page, to identify you as the one who sent the page.

Personal Paging:

(receive mode) this digit specifies what "other party code" must be sent by

someone trying to page you personally.  In order to receive a PERSONAL page, your

radio must receive the group code followed by this digit followed by a star

(1,2,3,4,*).   Preferably you should receive all that PLUS an additional single

digit that identifies who was sending you the page (1,2,3,4,*,N  where N is a

single digit that identifies the person who sent the page).   If this last

identifier digit is not sent, you will hear a personal page type beep, and "GP"

will flash, but "Err" will be on the main display indicating that a personal

identifier code was not received.

(transmit mode) This is the digit sent out LAST when you press the [PTT] button.

It identifies YOU as the person sending the page.  example:  with DSQ set to

123.4.5  if you press [PTT]  1,2,3,5,*,4  will be sent.   The 4 is received and

displayed as the last digit of the DSQ code on all radios that received your

page.

c This is the single digit code for "the other party".

DTMF Squelch:  This digit is NOT USED in DTMF squelch functions.

CONTINUED IN DJ16OT.8

DJ16OT.9

DTMF Wild Card function:

If you forget a squelch or paging code, you can pick up and decode codes

sent by others.  Use [f] [DSQ set] to set DSQ first to 000.0.0 and then set it to

###.0.0.  (upside down A's will be displayed instead of #'s).

Now set your radio to G DSQ mode.  If you receive ANY valid page, the radio will

beep.  Press [f] [DSQ set] and the ###.0.0 display will be replaced by the page

code you just received.

DTMF Decode function:

At any time, any codes received will be decoded and kept in the M3 dialing

number.  Thus, any time you hear a bunch of DTMF codes on the air, you can press

the [Dial Ch] button, then rotate the top selector know so that M3 is displayed,

then use the up and down arrow buttons (* and #) to scroll the display right (up

arrow) or left (down arrow).  Up to 16 digits will be retained.  You can also set

the radio to DSQ mode while lying in wait to receive DTMF tones.  That way, the

radio will break squelch when it has received three or more DTMF tones, alerting

you to the fact it has captured a code.   Note: according to the manual, you must

have phone numbers stored in one or both dial memories (M1 and M2) for the decode

to work.  But in my experience that does not seem to be required by my unit.

END OF SERIES.