DR110.1

STEP 10- REPLACE LITE UNDER POWER SWITCH, REMEMBER TO

USE THE BLUE COVER OVER THE NEW LITE AS WELL

AS THE YELLOWS SLEEVES ON WIRES. THIS LITE

YOU SOLDER FROM THE FRONT.

STEP 11- REPLACE 3 LITES THAT ARE ABOVE THE MICRO SWITCHES

ON THE FRONT OF THE PCBOARD. THIS WILL HAVE

TO BE UN-SOLDER FROM THE BACK. ALSO REPLACE THE BLUE

COVERS OVER THE 3 LITES.

STEP 12- REPLACE 2 LITES THAT STICK THROUGH HOLES THAT

GO WHERE THE LCD DISPLAY IS, ALSO REPLACE THE

YELLOW SLEEVES. THESE LITES SOLDER FROM THE BACK.

STEP 13- RE-INSTALL THE LCD DISPLAY AND BE SHURE THERE IS

NO CONTAMINATION OF THE CONTACTS ON THE BOARD AND

LCD DISPLAY. BE SHURE TO RE TWIST THE TABS ON THE

DISPLAY THAT GOES THROUGH THE PC BOARD. THIS DISPLAY

MUST BE TIGHT TO THE PC BOARD OR THE DISPLAY WILL

NOT WORK EITHER WITH MISSING LCD SEGMENTS ON OR

NO SEGMENTS ON. THIS LCD DISPLAY SHOWS THE FREQ.

SIGNAL STRENGTH, CHANNEL NUMBER, AND OTHER FUNCTIONS.

STEP 14- TEST THE DISPLAY BY POWERING UP THE RADIO. IF YOU

ARE GETTING MISSING NUMBERS, PULL THE LCD DISPLAY

AND CLEAN THE CONTACTS ON THE PC-BOARD AND THE

DISPLAY. I DID BY SCRAPPING THE PC BOARD WITH A

SCREW DRIVER AS WELL AS THE LCD DISPLAY CONTACT

RIBBON.

STEP 15- ASSEMBLE THE RADIO AND CHECK THE DISPLAY AT EACH

MAJOR ASSEMBLY POINT.

STEP 16- RESET THE CPU AS SHOWN IN YOUR OWNERS MANUAL. THIS

IS BECAUSE THAT THE CPU IS LOCATED ON THE BOARD

YOU WERE WORKING ON AND MAY HAVE SOME BAD DATA AFTER

WORKING ON THE RIG. YOU MAY HAVE TO RESET THE CPU

WHEN YOU TEST THE RIG IN ORDER TO SHOW THE

FULL LCD DISPLAY

ANY QUESTIONS CAN BE SENT TO ME AT MY HOME BBS AND I WILL TRY

TO ANSWER THEM TO THE BEST OF MY ABILITY.

THANKS AND 73'S

FRANK N2JYG @ WA0PTV.#WNY.NY.USA.NOAM

SYSOP OF THE "GLIDA NODES" IN SHERMAN NY

SYSOP OF HIGHLAND BBS IN SHERMAN NY 300-9600 BAUD USR-HST, V.32

V.42 AND NMP5/ AS WELL AS MY FAX NO. VIA A FAX SWITCH (716) 761-6460

A SPECIAL NOTE: I HAVE HAD AN ALINCO DR-110 FOR THE LAST SEVERAL

YEARS ON THE SHERMN <N2JYG-3> NODE F LOACATED IN SHERMAN NY

AND IT KEEPS ON TICKING EVEN WHEN THE STORMS ROLL THROUGH. AS WELL AS

A DR-410 AND NO PROBLEM, EXCEPT FOR THE LED LITES....OH WELL !!!