DR570T.1

RX 850-890Mhz on the DR570T.  10/26/91

If you have already opened up the expanded TX/RX ability of

the DR570T by cutting the jumper and resetting the CPU, you

can also RX 850-890Mhz (and 335-375Mhz) by doing the

following:

1. Display both bands.

2. Put the 2 meter band into VFO mode.

3. Have the ABX function disabled.

3. Press [FUNCTION].

4. Press [BAND/SUB] until you get the desired band.

Note:  While monitoring the 800Mhz band, the 440 band will

be desensed.

73's ---Jim,   N3KHM @ KA3NVP