Frank Gentges K0BRA

## June 11, 2001

Wideband  50 ohm to 200 Transformer

For

Alberto di Bene I2PHD

Input	50 ohms		Red and Red

Output	200 ohms	Green and Orange

20 turns trifilar wire wound on Amidon FT50-J core with two windings in series for the 200 ohm winding.

With a 200 resistor across the secondary the primary impedance is measured as follows.

# Frequency		Impedance

5 kHz			26 ohms	55 degrees phase angle

10 			38		36

20			45		20

50			48		7

100			49		3

200			48.5		0

500			48.5		2

5 MHz			52		8

10			54		15

20			61		28

30			65		34

Input impedance with secondary short circuited (leakage reactance)

30 MHz		50 ohms  	85 degrees phase angle.