FlexNet driver documentation    PAR96.EXE         (C)1995 G. Jost DK7WJ

Translation: Tom Sailer HB9JNX

Date: 10.06.97

Ident: PAR96

Driver for Baycom's FSK modem connected to an LPT interface

LPT1-x may be specified directly, other port adresses or IRQ numbers are

optional. Description with "PAR96 /?".

If different transmit and receive baudrates are to be used, the

transmit baudrate must be specified. In any case, the modem must

be modified accordingly if baudrates other than 9600 baud shall be

used. Warning: the TxDelay is measured incorrectly if the baud rate

differs from 9600 baud.

Parameters:

Baudrate must match TX baudrate

Mode command:

'c' activates the software DCD

It is more reliable to use a hardware squelch and disable the 'c' option.

See the documentation that came with the modem.

The calibrate command pauses every 14.5s for 500ms to reset the PTT watchdog.

Warning: The statistics does not recognize any TX or RX errors because the

hardware does not allow that. Some experimentation is required to know if

the computer or the configuration in conjunction with the other channels

allows an error free operation.