FlexNet driver documentation    SER12.EXE         (C)1994 G. Jost DK7WJ

Translation: Tom Sailer HB9JNX

Date: 05.07.97

Ident: SER12

Driver for the 1200 Baud Baycom modem connected to the COM interface

COM1-4 may be specified directly, different port addresses or IRQ numbers

are optional. Description with "SER12 /?".

Parameters:

Mode command:

Full duplex ('d')

'c' activates the software DCD, only signals with correct baudrate will

be recognized

The calibrate command pauses every 14.5s for 500ms to reset the PTT watchdog.

Warning: The statistics does not recognize any TX or RX problems because the

hardware does not allow that. Some experimentation is required to know if

the computer or the configuration in conjunction with the other channels

allows an error free operation.