From DB2OS  to TNC2   @WW

TS: B$ - Date/time: 0218/1526 - BID: 182308DB0FAU

Title       : TheFirmware TF24c Update Information

Path: !LX0PAC!DB0GE!DK0MTV!DB0GV!DB0SIF!DB0EAM!DB0FAU!

de DB2OS @ DB0FAU

NORD><LINK's THE FIRMWARE

AX.25 Version 2

Multi Channel TNC Firmware

Version 2.4c

1.  Introduction

The Firmware Version 2.4c DAMA/SMACK (xx Channel)

Copyright by NORD><LINK, xx-xx-xx

Free for non-commercial usage

Checksum (xxxx) = xxxx

This is the message that the new version of The Firmware should  display

in Terminal Mode when booting.

The Firmware can be installed in any TNC-2 (or compatible TNC).

This document is all about the latest official version of  The  Firmware

from  NORD><LINK.  The  software no longer mentions all the contributing

authors when booting since there are many that have worked  together  on

developing the software over the years.

The original author of  The  Firmware  was  Michael  (DC4OX).  The  DAMA

implementation was developed by Frank, DL8ZAW. Various improvements have

been contributed by DF2AU, DB2OS,  DK6PX,  DL9HCJ,  DL1MEN,  DF7ZE,  and

others (See Below).

This version (2.4c) is hopefully free of bugs. However,  it  has  always

been  the intention to continually improve the software and to fix known

problems as soon as possible.

2.  What Does "The Firmware" Support ?

This firmware supports fully the AX.25 Link Layer Protocol, Version 2 as

specified  in  October 1984 by the ARRL. The old version of the protocol

(1.x) is virtually extinct and as such, The Firmware no longer  supports

it.

The software  can  support  multiple,  simultaneous  QSO's,  each  on  a

separate  logical  channel  (or stream, as it is sometimes called).  The

number of channels supported by each EPROM is determined at  compilation

time.  The  usual  configuration has 10 channels available, but versions

with upto a maximum of 27 channels can be built  for  use  by  mailboxes

etc.

This version also supports user-access of TheNetNode nodes  running  the

DAMA  protocol.  DAMA is an extension of the AX.25 protocol and provides

users with many advantages and improvements over normal AX.25.

The Firmware will automatically determine whether to use DAMA or  normal

AX.25  when  connecting to a station. Frames received from a DAMA Master

are always displayed with "[DAMA]" appearing after  the  received  text.

The  TNC will then only transmit when instructed to do so by the Master.

Channels connected to non-DAMA equipped stations will be unaffected  and

transmit in the normal manner.

If the TNC does not receive a "Poll" frame from the  "Master"  within  a

user definable interval, it will switch out of DAMA mode.

When the TNC is operating as a DAMA Slave  (that  is,  a  user)  and  it

receives a frame to be digipeated, it does so immediately.

The DAMA Protocol is described in  CQDL,  the  magazine  of  the  German

national  society,  in the April 1989 issue. It is also described in the

paper, "DAMA - A new method of handling packets  ?"  by  Detlef  Schmidt

(DK4EG)   in  the  Proceedings  of  the  8th  ARRL  Computer  Networking

Conference, October 1989 (ISBN 0-87259-251-0).

2.1.  KISS and SMACK

The new KISS compatible protocol "SMACK", developed by DL5UE and  DK5SG,

is  also  supported  by  this  release  of The Firmware. See the release

documentation for details about SMACK.

The user defined Hostmode parameters are remembered when  either  normal

or  SMACK  KISS  modes  are  in  use. They are also retained following a

reset. Switching out of KISS/SMACK mode back to Terminal  mode  restores

the values stored previously.

The SMACK/KISS $FF frame carries the usual meaning and is used to switch

out of that mode and back into normal The Firmware "Terminal" mode.

It should be noted that the internal software clock does not continue to

keep real time whilst either KISS or SMACK is in use.

2.2.  Startup and the EPROM Checksum

Following each reset, the EPROM  checksum  is  recalculated.  This  must

agree  with  the  checksum  shown  in  the round brackets of the startup

message. If the two do not agree, then it is likely that the  EPROM  has

been programmed incorrectly or that the code programmed was corrupted in

some way.

After first installing the EPROM, it is useful to  perform  a  warmstart

using  the  "<ESC>  QRES" command since some TNCs have problems with old

code held in the battery-backed RAM. This will also load  the  RAM  with

the default set of parameters from EPROM.

After the QRES command, the TNC callsign is empty and can be  set  using

the  "<ESC> I" command. Other parameters can be set in a similar manner.

The user defined values are stored in the battery-backed RAM.

3.  Basic Operation in Terminal Mode

All commands and information are sent to the TNC as lines of text.  Each

line  can  be  upto  a maximum of 256 characters in length including the

delimiting Carriage Return (<CR>). If the  256th  character  is  not  an

<CR>, it is ignored and a "Bell" (<BEL>) is sent to the Terminal.

Single characters can be erased using either Backspace (<BS> =  Hex  08)

or Delete (<DEL> = Hex 7F).

Control-U and Control-X will delete a whole line.

Control-R will move the cursor to the first  character  of  the  current

line.  A  second  Control-R moves the cursor back to the end of the line

ready for more input. Between these two  Control-R's,  all  other  input

except for <XON> and <XOFF>, is ignored.

Bell (<BEL>) characters are sent direct to the terminal.

Lines beginning with the Escape (<ESC>) character cause a  asterisk  "*"

to be displayed thus denoting that this is the start of a command line.

Commands sent without parameters will display the current value of  that

parameter.

Lines beginning without <ESC> are taken as  being  Information  and  are

sent out on air.

4.  General Terminal Mode Information

The Logical Channels

The Firmware in its standard form provides 11 logical  TNC  channels  to

the  user.  The  terminal  is always logically connected to one of these

channels which is selected by the "<ESC> S" command.

Channel 0 is reserved for sending only UNPROTO information eg  CQs,  IDs

and  BEACONs.  The path to be used for this channel is, like the others,

set using the "<ESC> C" command.

The other 1-10 channels can also send in UNPROTO mode, as long  as  they

are  not  connected  to another station. Connect requests can be mode on

any free channel. Incoming connect  requests  from  other  stations  are

given  the  next  free  channel  in  sequence providing that the maximum

number of simultaneous connects (set using the "<ESC>  Y"  command)  has

not been exceeded.

4.1.  Selecting Channels

Information arriving on a channel that is  not  currently  selected,  is

stored  in RAM until that channel is chosen for display using the "<ESC>

S" command.

The "<ESC> L" command, without parameters, can be used to ascertain  the

connect  status  of each channel and whether there is data waiting to be

read. If any one of the unselected channels has data waiting to be read,

this is also shown by the STA LED of the TNC lighting up.

Even if a station disconnects, data  is  kept  in  RAM.  This  makes  it

possible  to store messages when the TNC is unattended. In short, a very

simple mailbox!

4.2.  Changing the Connect Path

There is no need to disconnect a station before trying over a  different

path.  Just  use  the  "<ESC>  C"  command to enter the new path for the

desired channel and a reconnect attempt will automatically be made using

the path entered. No data is lost during this process.

It is not possible to connect to the  same  callsign  on  two  different

channels. This is a restriction of the AX.25 protocol.

4.3.  The AX.25 Protocol

The Firmware always uses  version  2  of  the  AX.25  protocol.  Connect

requests  made  using  version  1  of  the  protocol will be immediately

disconnected. Version 2 is better on busy networks.

The Firmware also "polls" in version 2 of AX.25. As soon as there is  no

more  information  to  send,  a timer is started. If this timer runs for

more than 3 minutes, a poll frame is sent to the other TNC thus ensuring

that the link is still in use.

The number of "retries" attempted for any frame is set by the "<ESC>  N"

command.

4.4.  Commands for Individual Channels

The commands, <ESC> F, I, N, O, and V can be set independently  on  each

selected  channel.  Channel  0  holds the default value of each of those

parameters.

A channel is selected, any of the parameters above  set  for  just  that

channel and a connect is made. The QSO then proceeds using those values.

As soon as the QSO ends, the values are  reset  to  those  held  in  the

Channel 0 defaults.

5.  The Firmware's Monitor Functions

The "<ESC> M" command is used to activate the channel  traffic  monitor.

The  parameters given to this command determine the types of frames that

are seen in the monitor output. More than one parameter can be given  in

the same command line.

Parameter     Packet-Type

---------     -----------

N         Monitor OFF

I         Show Information Frames

U         Show UNPROTO Frames

S         Show Supervisory (control) Frames

C         Monitor whilst connected on selected channel

+         Only Packets from/to given Stations (max 8)

-         Show no packets from/to given Stations (max 8)

For example;

<ESC> M N         -    Switch the monitor off

<ESC> M IUS       -    Show Info, Unproto and Supervisory packets

<ESC> M + G6DHU   -    Show only packets to/from G6DHU

<ESC> M - G6DHU   -    Don't show G6DHU's packets

The "+" and "-" parameters may not be used in the same command line  and

they  must appear as the last parameter before the callsign. A parameter

of just "+"  or  "-"  will  erase  the  current  list  of  calls  to  be

monitored/ignored.

5.1.  Monitor Output Format

The station digipeating a packet is shown with an asterisk "*".

AX.25 Frame types as they appear in monitor output are as follows;

Name           Meaning

----           -------

RRa            Ready to Receive (frame acknowledge)

RNRa           Receiver Not Ready (temporarily busy)

REJa           Reject or duplicate packet

UI             UNPROTO information (broadcast)

DM             Busy

SABM           Connect Request

DISC           Disconnect Request

UA             Acknowledge Unnumbered frame

FRMR           Frame Reject (Protocol Failure)

Iab            Information

?ccH           Undefined or Unknown frame

Where;

a  = Sequence Number of the next expected frame

b  = Sequence number of this packet

cc = Hexadecimal number

A  final  character  denotes  the   status   of   the   Poll/Final   and

Command/Response bits of the AX.25 protocol;

<none> = Protocol version 1 without P/F Bit

! = Poll/Final Bit in Protocol version 1

^ = Command Packet in Protocol version 2 (no Poll Bit)

+ = Command Packet in Protocol version 2 (with Poll Bit)

- = Response Packet in Protocol version 2 (with Final Bit)

v = Response Packet in Protocol version 2 (no Final Bit)

The Protocol Identifier (PID) byte is shown in Hexadecimal.

Note: The monitor will only function when there are more than  256  free

buffers available in the TNC.

6.  More General Terminal Mode Commands

6.1.  Connect Text

The "<ESC> U" command allows a connect text to be set  and  switched  on

and  off.  This  allows  you  to  set  a small welcoming message perhaps

inviting a caller to leave a message. The connect text message  is  sent

automatically  to any connecting station and in no way affects operation

by the user.

When Connect Text is enabled, any incoming text and status messages  for

other  channels are saved in RAM until that channel is selected.  Status

messages and data received can thus be seen in the correct chronological

order.

In the event of a channel containing much saved data, remember that  the

"<ESC>  Z"  command  can  be used to select XON/XOFF flow control.  When

enabled, this allows Control-S to stop  the  output  from  the  TNC  and

Control-Q  to  resume  it.  Messages  can  then  be read without loss of

important data.

6.2.  The Heard List

The internal "Heard" List is activiated by the "<ESC> H"  command.   The

heard  list  implemented  by  The Firmware is somewhat different to that

found in other TNC software in a number of ways.

Firstly, it is not a list of the "last 20  stations  heard"  kind.  That

sort  of  list  doesn't  have  much  use  in  today's busy channels. The

Firmware implements a dynamic heardlist. The number of calls  stored  is

determined  by  the free buffer space available in the TNC RAM. With the

standard 32k TNC RAM, it is possible to store upto 600 calls. The growth

of the heard list can be limited by a user definable parameter.

For each station heard, the list saves;

- Callsign and SSID

- Date and Time first heard

- Date and Time last heard

- Number of I-Frames heard

- Number of RR-Frames heard

- Number of REJ-Frames heard

- Number of RNR-Frames heard

The digipeater path is no longer saved since with today's node  network,

this not often of use.

The Heard List can provide useful information about the quality  of  the

local channel and of the links between its users. For example, a station

with a high I-frame count and low RR and REJ counts denotes a good  link

quality.

There is a small performance overhead  in  maintaining  the  heard  list

which  obviously  increases  with  the size of the list. The list update

mechanism can be switched off with the command "<ESC>  H  0".  The  list

remains in memory whilst switched off but incoming packets are no longer

searched for inclusion in the list.

6.3.  The Heard List in Hostmode

The H command also works well in Hostmode.  When  requesting  the  heard

list it is important to remember the following points.

The first line (H, Number stations heard, maximum  number  in  list)  is

sent  with  Hostmode  Code 1 (Success, Message follows null terminated).

The actual heard list is then sent G/G0 to Channel 0 in Hostmode Code  5

"Monitor Header" format (Monitor Header, null terminated, Info follows).

The last line of the list is sent with Hostmode Code 4 (Monitor  header,

null terminated).

The heard list is memory resident, that is,  it  is  saved  following  a

reset or switch off and is held in the battery backed RAM.

6.4.  The Real Time Clock

Version 2.4c of The Firmware also has  a  built-in  24  hour  clock  and

calendar.  It  is  possible to timestamp all status messages (eg CONNECT

REQUEST from, CONNECTED to etc) aswell as the header of  each  monitored

packet.

The "<ESC> K 0" command switches  off  any  timestamping.  "<ESC>  K  1"

switches on timestamping of TNC status messages and "<ESC> K 2" switches

on timestamping of both TNC status messages and monitor headers.

The "<ESC> K" command displays the current Date and Time  and  depending

on how the time and date information was entered, it can be displayed in

either American or European format. Date and time are reset to 00/00/00,

00:00:00  after a cold or "<ESC> QRES" restart.  After a warm start, the

clock continues to run from the previous value stored when the  TNC  was

switched off.

7.  Software Improvements

This section summarises  the  major  changes  in  this  version  of  the

firmware.

7.1.  AX.25 T1 (Acknowledgement Timer) or FRACK

There is no longer a fixed value for the ACK  timer.  The  firmware  now

measures  in  real  time,  the  interval  between  sending  a  frame and

receiving its acknowledgement. This value is then smoothed and  used  to

continuously vary the T1 timer value according to the prevailing channel

conditions.

The time measured, or RTT  (Round  Trip  Time)  is  smoothed  from  wide

variations through the formulae;

- with rising RTT:    SRTT' = ( a1 x SRTT + RTT ) / ( a1 + 1 )

- with falling RTT:   SRTT' = ( a1 x SRTT + RTT ) / ( a2 + 1 )

Therefore, the SRTT (Smoothed Round Trip Time) contains a time  averaged

history  of  previous RTTs. The parameters a1 and a2 are settable by the

user and default to 7 and 15 respectively.

The relationship between the AX.25 T1 timer and the SRTT is;

T1 = a3 x SRTT

where the a3 parameter is user settable (Default = 2)

The SRTT must be initialised to a value before the QSO commences, as  at

that  time,  the SRTT cannot be known. This initial value of SRTT can be

set using the "<ESC> F" command which accepts a parameter,  the  initial

SRTT  in 10ms increments (Default = 700). The RTT measurement process is

started after the connection is acknowledged ie after the  UA  frame  is

sent  by  the station to be connected to. When the connect path involves

digipeaters, the RTT is increased due to the expected extra delay by;

T1 = (2 x "No. Digis" + 1) x IRTT

A1, a2 and a3  can be set with the "<ESC> @A1", "<ESC> @A2"  and  "<ESC>

@A3"  commands.  These  parameters are especially important when using a

FlexNet node which holds back the UA (acknowledgment) frame following  a

connect  request,  until  all nodes along the path have responded to the

request. As soon  as  the  connect  request  is  acknowledged,  the  RTT

measurement is started.

7.2.  DWAIT from DL4YBG

Before each information frame is sent out, the TNC  will  wait  for  one

Slottime  interval  before  the P-Persistence algorithm is started. This

holds irrespective of whether the TNC is operating in SMACK, normal KISS

or non-DAMA The Firmware modes.

When  operating  as  a  DAMA  Slave,  the  persistence  parameters   are

automatically set to Slottime = 0 and Persistence = 255 allowing the TNC

to respond immediately after being asked to send data by the Master.

The  chance  of  packet  collisions  is  reduced  by  implementing  this

strategy.

7.3.  I-Frame Polling from DK6PX

When sending small frames, that are subsequently missed by the receiving

TNC,  it  is sensible to resend the unacknowledged I-frame with the Poll

bit set instead of sending an RR frame.

The "<ESC> @I" command can be used to set the threshold packet length at

which  this strategy reverts to normal polling action. "<ESC> @I 0" will

force normal polling to be used at all times, which is also the  default

when the TNC is operating in DAMA mode.

7.4.  Dynamic MAXFRAME from DK6PX

According to packet length, MAXFRAME is  automatically  increased  when,

for example, other pending packets are of small size.

With MAXFRAME set to 1, a maximum of one frame with  256  characters  is

sent.  If  there are two frames pending with length 128, MAXFRAME is set

to 2. The default initial value of MAXFRAME is 4.

7.5.  DCD/PTT Lock-up

A previous bug in one of the software interrupt  routines,  could  under

certain  circumstances  cause  TNC  lock-up.  This  bug  existed  in all

previous versions of The Firmware  and  was  solved  by  Y51GE.  Another

related bug which caused DCD lockup was also fixed.

7.6.  DAMA Parameters

When DAMA mode is deactivated, the  parameters  P,  W,  B  and  @T2  are

automatically reset to their user defined values.

7.7.  Disconnect from DL1MEN

If a pending connect request is aborted with the "<ESC>  D"  command,  a

DISC (Disconnect) frame is automatically sent. This prevents the station

that was to have been  connected  with,  from  sending  any  unnecessary

frames following the connect abort.

7.8.  Heard List

The heard list now shows the last heard station rather  than  the  first

heard station. If the heard list is full, the oldest call is removed.

8.  The Firmware Command List

8.1.  ESC A [0|1]

Send Linefeed (<LF>) character to terminal after  each  Carriage  Return

(<CR>). (1 = Yes, 0 = No, Default = Yes)

8.2.  ESC B [<n>]

Time interval in seconds after which DAMA mode is  deactivated  when  no

Poll frame from the DAMA Master is received.

"<ESC> B 0" = Switch off DAMA mode completely

"<ESC> B" displays "Interval (Actual Value)" eg "120 (93)"

Default = 120 seconds.

8.3.  ESC C [Call] [Digi List]

Make a connect to the given call. Note that the keyword "v" or "via"  is

*not* required between the Call and the Digi List.

"<ESC> C [Call]" on Channel 0 sets the callsign (and optional  path)  to

be used for UI-frames. For example set;

ESC C ID

or

ESC C CQ

8.4.  ESC D

Disconnect the currently selected channel.

If there is still data to send or  to  be  acknowledged,  the  QSO  will

continue until so done. If this is not what is required, a second "<ESC>

D" command can be given to abort the connection.

If the currently selected channel is already in the disconnected  state,

all channel settable parameters are reloaded with their defaults held in

Channel 0.

8.5.  ESC E [0|1]

Echo all characters sent to the TNC.

1 = Yes, 0 = No, Default= Yes.

8.6.  ESC F [<n>]

Set initial the FRACK value which determines how long  to  wait  for  an

acknowledgement  after an I-frame is sent. This interval can be input in

seconds. If the parameter input is less than 16, the value is multiplied

by  100  and  divided by the A3 parameter (input via "<ESC> @A3"). Input

parameters greater than 15 are taken as milliseconds  (ie  the  Level  2

Round Trip Time).

Default = 250

8.7.  ESC G [0|1]

This is a hostmode only command and is used to inquire as to the  status

of the current channel.

If entered in Terminal mode, an error message will be given.

8.8.  ESC H [<n>]

Display the heard list and set the heard list parameters.

Examples;

H        -  Display Heardlist

H 0      -  Heardlist Update Off

H 1      -  Heardlist Update On

H 2      -  Clear Heardlist

H n      -  Maximum Number of calls in list (n > 2)

The list is shown in order of the last callsign  heard  and  the  oldest

heard callsign will be overwritten when the list becomes full.

The heard list is resident. It remains saved in the battery backed RAM.

Default = 0 (Heardlist off)

8.9.  ESC I [Call]

Set MYCALL for the TNC. After a cold-start, the call is unset.

The callsign can be set independently on  each  channel  and  when  that

channel  is disconnected, the call set in Channel 0 is again assumed for

that channel.

Note that the TNC will only transmit *after* the call has been set using

this command.

8.10.  ESC JHOST[0|1]

This command is used to toggle between Hostmode and Terminal Mode.  Host

mode  is  not  a "human" interface and is designed for computer control.

Packet terminal programs such as GIPSY, THP, AHP, SP (Super Packet)  and

Packet Master use this Host mode interface.

8.11.  ESC K [<n>]

Activate timestamping of status messages and monitor headers. The  built

in real-time clock/calendar is used.

Examples:

K            -  Display current Date and Time

K 0          -  Timestamping OFF

K 1          -  Status Message Timestamping ON

K 2          -  Status Message and Monitor Timestamping ON

K 20.02.88   -  Set Date - European format

K 02/20/88   -  Set Date - American format

K 17:36:00   -  Set Clock time

Default = 0 (All timestamping OFF)

8.12.  ESC L [0..10]

This command is used to display the status of logical TNC channels.   If

given without parameters, the status of all channels will be displayed.

Information such as connect path, frames received, frames  to  be  sent,

frames to be acknowledged and retry counter value are displayed.

The currently selected (ie default) channel  is  displayed  with  a  "+"

mark.

8.13.  ESC M [IUSCN+-]

This command is used to select the monitor mode. The parameters  to  the

command  determine the types of frames displayed by the monitor and also

which stations are selected or filtered from the display.

Examples:

N        None (Monitor OFF)

I        Show only Information Frames

U        Show only UI frames (CQ, BEACON etc)

S        Show Supervisory frames

C        Monitor whilst connected

+        Followed by list of upto 8 calls

which will be the only ones shown.

-        Followed by list of upto 8 calls

which will be ignored.

The "I", "U", "S" and  "C"  parameters  may  be  combined  in  a  single

command.  For  example,  "<ESC>  M IUSC" will display all possible frame

types and the monitor will be active when a channel is in the  connected

state.

The "+" and "-" parameters may *not* be combined in  a  single  command.

The  command  with  just a "+" or "-" parameter will clear the lists and

return to normal operation.

Default = UI  (Monitor displays UI and Information frames)

8.14.  ESC N [0..127]

Set the maximum number of retries for each  Information  frame  sent.  A

value of 0 instructs the TNC to retry forever.

The retry counter can be set independently on each  channel.  When  that

channel  is  disconnected  or  a RESET command is given, the retry value

stored in Channel 0 becomes the default.

Default = 10 (Tries)

8.15.  ESC O [1..7]

Set the maximum number of information frames that may be outstanding and

unacknowledged at any one time (MAXFRAME).

The MAXFRAME can be set independently on each channel. When that channel

is disconnected or a RESET command is given, the value stored in Channel

0 becomes the default.

Default = 2 (Frames unacknowledged)

8.16.  ESC P [0..255]

Set the TNC's Persistence value for determining channel access priority.

DAMA Slaves always set P = 255.

Default = 32 (32/255 chance of access)

8.17.  ESC QRES

Cold start The Firmware and reload the EPROM defaults.

8.18.  ESC R [0|1]

Enable the TNC digipeat function. 1 = Yes, 2 = No.

Default = 1 (Allow TNC to digipeat)

8.19.  ESC S [0..10]

Select the given logical channel. 0 is the Monitor channel.

Default = 0 (Default channel is the monitor channel).

8.20.  ESC T [0..127]

Set the  delay  between  keying  the  transmitter  and  sending  a  data

(TXDELAY).  The  value given is in 10ms increments. For example, a delay

of 300ms would be "<ESC> T 30". The value should be experimented with to

suit your TX, but set it as low as possible.

Default = 25 (250ms)

8.21.  ESC U [0|1]

Set (or display) the Connect Text for the TNC. The text is saved even if

the function is switched off.

Examples:

U 1 "Hello!"      Enter Connect Text

U 1               Enable Connect Text

U 0               Disable Connect Text

U                 Display Connect Text

Default = 0 (No Connect Text, Disable Connect Text).

8.22.  ESC V

Display the current AX.25 Protocol Version in use. Protocol version 1 is

no longer supported by The Firmware from v2.4 onwards.

8.23.  ESC W [0..127]

Set (or display) the TNC Slot Time to be  used  in  determining  channel

access priority. The value given is in milliseconds.

Default = 10 (ms)

8.24.  ESC X [0|1]

Disable the transmitter PTT line.

Default = 1 (PTT enabled)

8.25.  ESC Y [0..10]

Set (or display) the maximum number of simultaneous connects allowed  to

or  from  the TNC. If given without parameters, the command will display

"Max No. Connects (Occupied Channels)". This is only the case  when  all

channels have the same SSID set as that on Channel 0.

Example: "4 (1)"

Default = 4 (A maximum of 4 channels in use at any one time).

8.26.  ESC Z [0..3]

Determine TNC flow control and XON/XOFF handshaking.

If flow control is enabled, the TNC will not send data to  the  terminal

if  a  command or data to be sent is being entered. When flow control is

disabled, the TNC  will  immediately  send  any  data  to  the  terminal

regardless of whether a command or data is being entered.

If XON/XOFF handshaking is enabled, the user may stop  TNC  output  with

the Control-S key and restart it with Control-Q.

Examples:

0      Flow  On     , XON/XOFF  Off

1      Flow  Off    , XON/XOFF  Off

2      Flow  On     , XON/XOFF  On

3      Flow  Off    , XON/XOFF  On

Default = 3 (Flow control off, XON/XOFF on)

9.  Extended TNC Commands

There are a number of extended TNC commands that are prefixed  with  the

"@" character eg "<ESC> @A1".

9.1.  ESC @A1 [<n>]

Set or display the a1 SRTT calculation parameter. Default = 7.

9.2.  ESC @A2 [<n>]

Set or display the a2 SRTT calculation parameter.  Default = 15.

9.3.  ESC @A3 [<n>]

Set or display the a3 SRTT calculation parameter.  Default = 3.

9.4.  ESC @B

Display the free buffer count in the software.

9.5.  ESC @D [0|1]

Enable Full-duplex mode. Default = 0 (simplex)

9.6.  ESC @I [<n>]

Set (or display) the I-poll threshold. The value given is the number  of

bytes  at  which  if packets longer than that are waiting to be sent, I-

poll will not be used.

Default = 60 (bytes)

9.7.  ESC @K

Start KISS/SMACK mode.

9.8.  ESC @M [0|1]

Set 7 or 8 bit (binary) terminal mode.

When in 7 bit mode, control characters are automatically displayed  with

a  leading  "^"  prefix. In 8 bit or binary mode, control characters are

passed through without translation.

Note that XON/XOFF should not be used at the Terminal end as it may hang

when seeing the XOFF ("^S") character.

The default is to operate in binary mode (@M  =  1)  and  the  character

translation  does not occur. The command is however important when using

Hostmode software eg THP, SP etc.

0 = 7-Bit; 1 = 8-Bit (only in Terminal-Mode)

Default = 1 (binary, 8 bit mode)

9.9.  ESC @T2 [<n>]

Time interval to delay acknowledgement of received frames.

Default = 150 (1.5 seconds)

9.10.  ESC @T3 [<n>]

Set or display the link  "keep  alive"  timer.  If  the  link  has  been

inactive for @T3 seconds, the link partner is polled for activity.

Default = 18000 (ms)

9.11.  ESC @V [0|1]

Enable  callsign  checking,  eg  rejection  of  connect  requests   from

"NOCALL". (1 = Enabled, 0 = Disabled).

Default = 0 (disabled)

10.  Acknowledgements

Original documentation in German by DB2OS, DF2AU und DC4OX.

Updated by DJ1OR September 1992.

English translation by Mike Chace (G6DHU) February 1993.