VIA BATTERY VOLTAGE MONITOR

A Battery Voltage monitor has been added since there are several power schemes being used (Alkaline, NiCD, NiMH, LiPoly, etc) and it's useful to know the battery voltage/status. During the initial "Splash Screen" the voltage is displayed. In addition, a button has been added to the Setup Screen.  The "Batt" button toggles a voltage measurement function.

When toggled on, the voltage is displayed with a 10Hz update rate. Tapping any button on the Setup Screen will turn off the interrupt that triggers the voltage samples.

There is a hardware modification required to implement this feature. Add a 10K resistor

from the STM32F407 board pin labeled "PC1" to the adjacent Ground pin. Add a 68K resistor from your battery to pin "PC1". Connect the wire to the battery at the INPUT of the 5V Switcher/Linear Regulator. Verify the voltage reading after measuring the battery with an accurate voltmeter.

The 2 resistors will be included with future AQRP VIA kits.