# Installing Open-embedded

http://www.gumstix.org/software-development/open-embedded/61-using-the-open-embedded-build-system.html

1.

$ mkdir -p ~/dev/overo-oe

2.

$ sudo apt-get install git subversion gcc patch help2man diffstat texi2html texinfo libncurses5-dev cvs gawk python-dev python-pysqlite2 python-psyco unzip chrpath ccache

3.

$ sudo dpkg-reconfigure dash

Choose "No"

4.

$ cd ~/dev/overo-oe

$ git clone git://gitorious.org/gumstix-oe/mainline.git org.openembedded.dev

5.

$ cd org.openembedded.dev

$ git checkout --track -b overo origin/overo

$ git checkout --track -b overo-2011.03 origin/overo-2011.03

6.

$ cd ~/dev/overo-oe

$ git clone git://git.openembedded.net/bitbake bitbake

$ git clone http://cgit.openembedded.org/cgit.cgi/bitbake/ bitbake

$ cd bitbake

$ git checkout 1.12.0 1.10.2

7.

$ cd ~/dev/overo-oe

$ cp -r org.openembedded.dev/contrib/gumstix/build .

8.

$ cd ~/dev/overo-oe/build

$ vim profile

change OVEROTOP to "${HOME}/dev/overo-oe"

$ cp ~/.bashrc ~/.bashrc.bak

$ cat profile >> ~/.bashrc

$ source ~/.bashrc

# Build image (omap3-console-image) with Bitbake

1.

Add

PARALLEL_MAKE = "-j2"

OE_ALLOW_INSECURE_DOWNLOADS = "1"

to ~/dev/overo-oe/build/conf/local.conf

2.

$ cd ~/dev/overo-oe

$ bitbake omap3-console-image

Issues that may arise:

Issue A: perl-native_5.10.1.bb failed with undefined reference to `pthread_getspecific`

Sol A:

$ git clone git://gist.github.com/925006.git

open 925006/0001-perl-native-multilib-patch-for-ubuntu-11.04.patch and replace `uname -m` with 'i386' (without quotes)

$ git apply --verbose 925006/0001-perl-native-multilib-patch-for-ubuntu-11.04.patch

$ bitbake -c clean perl-native

$ bitbake perl-native

Issue B:

Log data follows:

| sed: can't read docbook.cat: No such file or directory

| ERROR: Function do_compile failed

NOTE: package docbook-sgml-dtd-3.1-native-1.0-r2.0: task do_compile: Failed

Sol B:

$ cd ~/dev/overo-oe/tmp/work/i686-linux/docbook-sgml-dtd-3.1-native-1.0-r2.0/docbook-sgml-dtd-3.1-1.0

$ cp ../* .

3.

$ bitbake x-load

# Installing DSPLink (Bitbake ti-dsplink)

1.

$ bitbake ti-dsplink

Issues that may arise:

Issue A. Bitbake can't fetch ti_cgt_c6000_6.1.17

Sol A:

User has to manually download ti_cgt_c6000_6.1.17 from ti's website https://www-a.ti.com/downloads/sds_support/CodeGenerationTools.htm

Once downloaded:

$ cd ~/dev/overo-oe/sources

$ rm ti_cgt_c6000_6.1.17_setup_linux_x86.bin.lock

$ cp ~/Downloads/ti_cgt_c6000_6.1.17_setup_linux_x86.bin

$ md5sum ti_cgt_c6000_6.1.17_setup_linux_x86.bin > ti_cgt_c6000_6.1.17_setup_linux_x86.bin.md5

# Copy Angstrom Filesystem to MicroSD

1.

Follow instructions at http://www.gumstix.org/create-a-bootable-microsd-card.html to make a bootable MicroSD. In our case we will be using the Angstrom file system built with bitbake omap3-console-image so only the section on partioning the card needs to be followed.

2.

$ mkdir /media/card

$ sudo mount /dev/sdd1 /media/card

$ cd ~/dev/overo-oe/tmp/deploy/glibc/images/overo

$ sudo cp MLO-overo /media/card/MLO

$ sudo cp u-boot-overo.bin /media/card/u-boot.bin

$ sudo cp uImage-overo.bin /media/card/uImage

3.

Create a file name 'script.boot' and add the following lines to the file

echo ===== DSPLink Settings=====

setenv mmcargs ${mmcargs} mem=126M

if run loadimage; then

run mmcboot

else

run nandboot

fi

Then run the following commands:

$ apt-get install uboot-mkimage

$ mkimage -A arm -O linux -T script -C none -a 0 -e 0 -n 'P25 OMAP3 DSP: 126M Linux' -d script.boot boot.scr

4.

$ cp boot.scr /media/card/boot.scr

5.

$ sudo umount /dev/sdd1

6.

$ sudo mount /dev/sdd2 /media/card

$ cd /media/card

$ sudo tar xvaf ~/dev/overo-oe/tmp/deploy/glibc/images/overo/omap3-console-image-overo.tar.bz2

7.

$ cd  ~/dev/overo-oe/tmp/deploy/glibc/ipk/overo

$ mkdir /media/card/home/root/ipk

$ cp ti-* /media/card/home/root/ipk/

$ sudo umount /dev/sdd2

# Booting OveroTide with MicroSD

1.

$ sudo apt-get install minicom

2.

$ minicom -s

Under "Serial Port Setup"

set Serial Device: /dev/ttyUSB0 (or whatever it is on your system)

Bps/Par/Bits: 115200 8N1

Hardware Flow Control: No

Software Flow Control: No

Hit [ESC] until you return to the main configuration menu

Select "Save Setup as dfl" to save the default setup

Select "Exit from Minicom" to exit the program

3.

Connect the usb cable to the console usb port on the Overo Tobi

then run minicom:

$ minicom -o

4.

Plug the power adapter into the power jack of the Overo Tobi.  When connected and powered, you should see a message from U-boot followed by the normal Gumstix boot sequence in the minicom window.

Log in for the first time with

username: root

(no password)

5.

When finished, you can exit Minicom by typing [CTRL-A] then pressing Q.

# Setup Environment for OMAP Arm and DSP Code build

1.

$ mkdir ~/dev/omap_tools

Copy the required tools from the open-embedded working directory to your dev folder. Note that the version numbers may not be the same as yours but the directory structure should be the same (hopefully)

2. copy ti-ctg6x

$ cp -a ~/dev/overo-oe/tmp/work/overo-angstrom-linux-gnueabi/ti-cgt6x-1_6_1_17-r4/cgt6x_6_1_17 ~/dev/omap_tools/ti-cgt6x-tree

3. copy ti-xdctools

$ cp -a ~/dev/overo-oe/tmp/work/overo-angstrom-linux-gnueabi/ti-xdctools-3_20_05_76-r2/xdctools_3_20_05_76 ~/dev/omap_tools/ti-xdctools-tree

4. copy ti-dspbios

$ cp -a ~/dev/overo-oe/tmp/work/overo-angstrom-linux-gnueabi/ti-dspbios-5_41_07_24-r1/bios_5_41_07_24 ~/dev/omap_tools/ti-dspbios-tree

5. copy ti-dsplink

$ cp -a ~/dev/overo-oe/tmp/work/overo-angstrom-linux-gnueabi/ti-dsplink-1_1_65_00_03-r100i/dsplink_linux_1_65_00_03/dsplink ~/dev/omap_tools/dsplink

6. copy Linux kernel

$ cp -a ~/dev/overo-oe/tmp/sysroots/overo-angstrom-linux-gnueabi/kernel ~/dev/omap_tools/kernel

7. copy armv7a tools

$ cp -a ~/dev/overo-oe/tmp/sysroots/i686-linux/usr/armv7a ~/dev/omap/armv7a

# Build and Run OMAP Arm and DSP code

Directory Structure of ~/dev/p25/omap

omap

├── arm_ambe

│   └── Linux

├── dsp_ambe

│   └── DspBios

│       └── 5.XX

│           └── OMAP3530

├── gpp_host_intf

├── include

│   ├── dprintf

│   │   ├── dsp

│   │   └── gpp

│   ├── dvsi

│   └── utils

├── lib

│   ├── dprintf

│   │   ├── dsp

│   │   └── gpp

│   ├── dvsi

│   └── utils

└── scripts

└── omap_env_scripts

arm_ambe and dsp_ambe directories contain the source files specific to the arm and dsp processors respectively.

include and lib directories contain shared libraries and headers used by both Arm and DSP code. The lib/dvsi and include/dvsi directory should contain the apcop2_c64x.lib and apcop2[fec].h files respectively.

script directory contains script to help setup build environment and build DSP and ARM code. You may want to add this script directory to your $PATH.

1.

Open script/setup_dev_env.sh and modify the paths to the DVSI libraries and your OMAP development folder

2. run $ ./setup_dev_env.sh

This will copy the library and include files from the DVSI folder into their respective folders in the OMAP dev folder

3.

Open script/omap_env_scripts/omap_setup_env.sh and make sure the path variables correspond to your setup.

4. run $ ./omap_setup_env.sh

This will configure ti-dsplink and build the necessary libraries for ARM and DSP source compilation.

5. Build Source Code (You'll need to add scripts/omap_env_scripts/omap_build.sh to your $PATH environment variable for this part)

6.

$ cd ~/dev/omap

$ omap_build.sh arm_ambe gpp release

$ omap_build.sh dsp_ambe dsp release

7.

The binaries will be located in /opt/bin/gpp/BUILD/ARM_AMBE/BIN/RELEASE/arm_ambe and /opt/biin/dsp/BUILD/OMAP3530_0/DSP_AMBE/BIN/RELEASE/dsp_ambe.out

Copy these files to the Overo Tide (OMAP Arm Processor) and also copy scripts/setup_dsp_env.sh

8. Connect to the Gumstix with Minicom and run the following

$ ./setup_dsp_env.sh

$ ./arm_ambe dsp_ambe.out 51718 51719