- P25 OSSIE Overo Waveform Installation instructions:

- 1.

Install OSSIE 0.8.2, it's dependencies, and parts of GNU Radio

- For instructions on how to download and install OSSIE, follow the instructions on Virginia Tech's website:  http://ossie.wireless.vt.edu/download/user_guides/OSSIE_0.8.2_User_Guide.pdfhttp://ossie.wireless.vt.edu/download/user_guides/OSSIE_0.8.2_User_Guide.pdfhttp://ossie.wireless.vt.edu/download/user_guides/OSSIE_0.8.2_User_Guide.pdfhttp://ossie.wireless.vt.edu/download/user_guides/OSSIE_0.8.2_User_Guide.pdfhttp://ossie.wireless.vt.edu/download/user_guides/OSSIE_0.8.2_User_Guide.pdf.

It will guide you through installing OSSIE, UHD drivers, and GNU Radio as well.  The rest of this guide will assume you have installed OSSIE into the root directory of /sdr/.

As a test to make sure the UHD detects a USRP present, simply run

- $ uhd_usrp_probe

while a USRP is plugged in.

Before proceeding with the following instructions, make sure the necessary steps in the “Installation and Setup for OMAP” pdf have been completed.

- 2.

- $ mkdir ~/dev/

- $ cd dev/

- 3.

Download or unzip the latest usrp_br subversion into current directory.  The rest of this guide will assume you have unzipped or downloaded the usrp_br folder into ~/dev/.

- 4.

- Set up Gumstix Overo Tide COM to use USBnet via the instructions here: http://wiki.gumstix.org/index.php?title=Category:How_to_-_usbhttp://wiki.gumstix.org/index.php?title=Category:How_to_-_usbhttp://wiki.gumstix.org/index.php?title=Category:How_to_-_usbhttp://wiki.gumstix.org/index.php?title=Category:How_to_-_usbhttp://wiki.gumstix.org/index.php?title=Category:How_to_-_usb.

The code supplied and the rest of this guide assume the Gumstix uses USB, the IP address of the device is 10.10.10.15, and the two ports it uses to communicate with the host side are 51718 and 51719.  If another IP address or ports are to be supplied the OveroVocoderDevice.cpp code located in ~/dev/usrp_br/gpp_ossie/OveroVocoderDevice/ needs to be changed from

- hg = new HostGPPInterface (“10.10.10.15”, 51718);

- hg_enc = new HostGPPInterface(“10.10.10.15”, 51719);

to

- hg = new HostGPPInterface (“x”, y);

- hg_enc = new HostGPPInterface (“x”, z);

- Where x is the IP address of the Gumstix board, and y and z are the two chosen open ports.

- 5.

Once an IP has been assigned to the Gumstix run the script and specify the IP address of the board to load on the executables for the ARM and DSP

- $ cd ~/dev/usrp_br/omap/

- $ ./build_gpp_dsp.sh 10.10.10.15

- 6.

Build Modem and Baseband code

- $ cd ~/dev/usrp_br/gpp/

- $ make

- 7.

Configure, then build OSSIE components

- $ cd ~/dev/usrp_br/gpp_ossie/

- $ make configure

- $ sudo make

This will build all of the p25_ossie_overo waveform components as well as the p25_ossie_overo waveform itself and install the waveform into the /sdr/dom/waveforms/ folder.  The components should only be configured once for each platform they are run on.

- 8.

Configure, then build the custom Sound_out and Sound_in devices provided

- NOTE 1:  We found that by using an external USB sound card peripheral, the audio was both more reliable and robust.  If you would like to use the built-in sound hardware on the host machine, you must edit the line

- char *pcm_name = “plughw:1,0”;

to

- char *pcm_name = “plughw:x,y”;

- where x is the number of the sound card, and y is the number of the audio device (usually 0,0 for built-in audio hardware) in both soundCardCapture in ~/dev/usrp_br/gpp_ossie/Sound_in/ and soundCardPlayback in ~/dev/usrp_br/gpp_ossie/Sound_out/.

Build and install both custom sound components

- $ cd ~/dev/usrp_br/gpp_ossie/Sound_in

- $ ./configure; automake –install-missing; make; sudo make install

- $ cd ~/dev/usrp_br/gpp_ossie/Sound_out

- $ ./configure; automake –install-missing; make; sudo make install

- NOTE 2:  The binary file contained in Sound_out (soundCardPlayback) may install into the wrong directory: /sdr/dev/dev/bin/ instead of /sdr/dev/bin/.  This is believed to be a bug in the OSSIE code after configuration.  To fix this, edit the Makefile.am inside the Sound_out folder as follows

- $ vim ~/dev/usrp_br/gpp_ossie/Sound_out/Makefile.am

Change the following lines from

- xmldir = $(prefix)/dev/xml/$(ossieName)

- bindir = $(prefix)/dev/bin

to

- xmldir = $(prefix)/xml/$(ossieName)

- bindir = $(prefix)/bin

Sound_out will then need to be reinstalled

- $ cd ~/dev/usrp_br/gpp_ossie/Sound_out/

- $ sudo make install

An alternative is to simply copy the binary into the correct directory

- $ sudo cp /sdr/dev/dev/bin/soundCardPlayback /sdr/dev/bin/

- 9.

Edit the following file in vim or another text editing program

- $ vim ~/dev/usrp_br/gpp_ossie/ModemDevice/ModemDevice.cpp

Change

- chdir(“/home/sdr-overo/dev/usrp_br/run”);

to

- chdir(“/home/USERNAME/dev/usrp_br/run”);

Where USERNAME is the username of the current account on the PC.  In later versions this will be a parameter than can be edited from within the OSSIE perspective, but for now is hard-coded.

- 10.

Copy the run_nodebooter script from the usrp_br to /sdr/ and run.

- $ sudo cp ~/dev/usrp_br/gpp_ossie/run_nodebooter.sh /sdr/

- $ cd /sdr/

- $ sudo ./run_nodebooter.sh

This will start the OSSIE Nodebooter

- P25 OSSIE Overo Running Waveform

- 11.

Run the following on the Overo Tide.  You can either run it via sshing to the device, or using minicom -o

- #./setup-env.sh; ./arm_ambe dsp_ambe.out 51718 51719

If you chose different ports for setup in step 4, those will need to be supplied instead of the default 51718 and 51719 ports.

- 12.

Open the Java GUI, Modem and Baseband via the script p25_run.sh in another window

- $ cd ~/dev/usrp_br

- $ sudo ./p25_run.sh -u

- 13.

Start the Wave Loader in another terminal.

- $ sudo c_wavloader

Select the number that corresponds to the p25_ossie_overo waveform, and start it

- 14.

At this point, the waveform should be loaded, and have already established a connection to the Overo board.  Click on the “connect” button located on the GUI.  The radio is now ready to use.