Digital Storage Oscilloscope and Logic Analyzer

Operating Instructions

The operation of the device is fairly straight forward.  In simplest terms, the PC sends out a series of commands which set up the hardware and then tells the system to start sampling the data.  It monitors three status signals in order to tell when the sampling is complete.  The PC then comands that the device send it the data from each input card.  The PC does some minimal processing of the data and then displays it.

An INI file is used to set many of the operating characteristics of the device.  It is a text file which you can customize to meet your particular requirements.  I suggest that you have one (or more) INI files for each of your projects.  I have supplied an INI file with this package which you should use the first time you run the program.  It should be placed in the same folder as DSOLA.exe.  You can use the various menus to customize the system configuration and then save your own INI file.

The program makes a single entry in the Windows Registry which points to the last used/saved INI file.  You can find the entry by searching the Registry for “DSO.”

Following is a brief explanation of each of the menu bar items.  My intention is to make all of the menu items available in the INI file.  One of the reasons for this is that when you store the data collected by the DSO/LA an INI file is created with all the operating parameters.  This allows you to retrieve the data and be sure that you can view it the same way as when it was stored.

Menues:

File

INI Read - read the INI file

INI Save - save the INI file using the same name

INI SaveAs - save the INI file using a specified name

Data Read - read a previously stored BIN file

Data Save - save the captured data

save in either BINary or CSV format

save as byte or bits

save entire data set, samples on screen, or custom range

Boards - allows you to define each board type

LA boards:

channel name

whether or not it is to be displayed

A/D boards:

name

voltage range

trigger voltage

whether or not the signal is inverted

trace and background color

trace thickness

Trigger

Pre-trigger count

enable/disable forcing the main cursor to the trigger position

trigger state for each of the eight trigger inputs

Display

Time Base select

Enable/Disable Voltage grid lines for A/D

Zoom and Slide factors

LA and A/D size as a percentage of the total screen

Cursors and Grids

Cursors Color and Thickness

Enable up to four cursor markers

Enable/Disable Time Grids

Time Grid Color and Thickness

Time per DIvision

Function Keys

Shows the Function Key mapping for various controls

Debug

Printer Port - control the printer port outputs

DSO Functions - perform DSO functions

Data - load data into the memory buffers for each board.  This allows you to “play” with the system without having any hardware.

DSO Controls:

Arm - start a data capture cycle, draw the data upon completion

Draw - draw the data - useful when you have changed some of the operating parameters

Start Sample - select the starting sample number

Cursor - place the cursor

T - force the cursor to the trigger sample

Zoom - zoom in (higher resolution) or out (lower resolution) on the cursor

Slide - slide the display keeping the same number of samples

T - slide to the cursor

End Sample - select the ending sample number

When entering Start Sample, End Sample, or Cursor position the background will change color.  The typed-in value will not take affect until “Enter” is pressed at which time the background color will return to white.

Displayed Information:

The text box under the Arm and Draw buttons has multiple purposes:

- status while capturing data

- analog voltage at the cursor position of the selected A/D board

The text box under the Zoom control indicates the logic state of the LA channels under the cursor