To give you a better idea of the ease of use of PLDs (Programmable Logic Devices), I will expand on the 17-bit counter example mentioned in the text.

The address generator requires a 17-bit (131072 count) synchronous counter.  A synchronous counter is required so that all the address bits change at the same time - synchronously.  The discreet circuit I initially implemented is shown in the figure.  Note that it uses 5 ICs.  Even though I use only one stage of the fifth device, it is required to maintain synchronous operation.

The PLD implementation is quite a bit simpler.  All I needed to do was select the synchronous counter from a list of device models, select and define the needed parameters, and place it in my schematic.  Below is the complete list of possible parameters.  The only inputs I needed were

aclr and clk, the only output was q[] and the parameters I specified were direction and width.

Once the part was placed, I then had to "wire" the I/O ports to the appropriate circuits or I/O pins.  See the AddrClock circuit of the Time Base and Address Generator schematic.

When the schematic entry has been completed, the next step is to "compile" it.  This basically assigns the schematic elements to the internal circuit blocks of the PLD and determines whether or not the circuit fits.  If the circuit does not fit, there are several options available which

can be used to reduce the amount of resources used.  Some of these are:

1) disabling JTAG (Joint Test Action Group) capability.

2) disabling globally assigned signals such as clocks and clears.

Once this stage of compilation is complete, the pin assignments must be made.  This can be done automatically or manually.  I always chose manual, because I wanted the PLD I/O to be such that it allowed the easiest printed circuit board layout.  The manual method was really quite easy.  I had previously determined the signal to pin assignments based on the PC board layout.  The PLD software presents a list of signals as well as a "picture" of the PLD.  All I had to do was "drag and drop" each signal onto the desired pin.

Now the final compilation phase can be executed.  This phase attempts to "fit" the circuit into the PLD with the pin assignments.  The software I used does a number of iterations in order to complete the "fit" process.  I have seen up to 20.  If it is successful, a file is created which is used to program the PLD.  If not successful, then some pin reassignment will be necessary.

It is now time to program the PLD.  A programming device or equivalent circuit is required.  I was able to obtain a programmer from the PLD manufacturer; but, since the circuit consists of a common IC and a few resistors, it would not be difficult to build one.  The PLD I selected is capable of In-System Programming.  All I needed to do was implement the appropriate connector on the circuit board with four connections to specific PLD pins.  These four pins can be used for I/O, but I chose to not do that.  The programmer connects to the printer port of the PC and the connector on my board.  The last step is to use the software to program the PLD and then remove the programming cable from the board.

The circuit is now ready for the "smoke" test!  Since it is impossible to probe the internal circuit points, I strongly suggest that test points be designed into the PLD.  At one point of my design cycle, I had three of them.  These allowed me to 'scope some of the critical timing points.  Because I had previously implemented identical circuits using discrete logic, my debugging was minimal, so I did not need many test points.  Untested, or first try, circuits will probably require more.  I recommend that outputs be buffered so that external loading of the I/O pins does not adversely affect system operation.

This was my first attempt at using PLDs and I found it to be much easier than expected.  The free software, downloaded from the Web, is fairly intuitive and comes with a tutorial.  I ran the tutorial through the first dozen or so steps; and then, like a typical engineer and ham, went off on my own.  I was able to go from starting with no previous experience to a finished product within a few weeks working an hour or so several evenings a week.

Following is a complete list of the possible I/O signals and parameters which may be used to define the operation of the counter.

INPUTS

Port Name	Required	Description / Comments

data[]		No		Parallel data input to the counter.  Input port LPM_WIDTH

wide. Uses load or sload.

clock		Yes		Positive-edge-triggered Clock.

clk_en		No		Clock Enable input. Enables all synchronous activities.  Default = 					1 (enabled).

cnt_en		No		Count Enable input. Disables the count when low (0) without

affecting sload, set, or sclr.  Default = 1 (enabled).

updown		No		Controls the direction of the count. High (1) = count up. Low (0)

= count down.  	Default = up (1). If the LPM_DIRECTION

parameter is used, the updown port cannot be connected. If

LPM_DIRECTION is not used, the updown port is optional.

cin		No		Carry-in to the low-order bit. 	If omitted, the default is 0.

aclr		No		Asynchronous Clear input.  Default = 0 (disabled). If both aset

and aclr are used and asserted, aclr overrides aset.

aset		No		Asynchronous set input.	Default = 0 (disabled). Sets q[] outputs

to all 1's, or to the value specified by LPM_AVALUE. If both aset

and aclr are used and asserted, aclr overrides aset.

aload		No		Asynchronous load input. Asynchronously loads the counter with

the value on the data input. Default = 0 (disabled). If aload is

used, data[] must be connected.

sclr		No		Synchronous Clear input. Clears the counter on the next active

Clock edge. Default = 0 (disabled). If both sset and sclr are used

and asserted, sclr overrides sset.

sset		No		Synchronous set input. Sets the counter on the next active Clock

edge. Default	= 0 (disabled). Sets q outputs to all 1's, or to

the LPM_SVALUE value . If both sset and sclr are used and

asserted, sclr overrides sset.

sload		No		Synchronous load input. Loads the counter with data[] on the

next active Clock edge.	Default = 0 (disabled). If sload is used,

data[] must be connected.

OUTPUTS

Port Name	Required	Description / Comments

q[]		No		Data output from the counter.	Output port LPM_WIDTH wide.

Either q[] or at least one of the eq[15..0] ports must be

connected.

eq[15..0]	No		Counter decode output Active high when the counter reaches the

specified count value.	(AHDL only) Either the q[] port or eq[] 					port must be connected. Up to c eq ports can be used (0 <= c

<= 15).  Only the 16 lowest count values are decoded. When

the count value is c, the, the eqc output is set high (1). For

example, when the count is 0, eq0 = 1, when the count is 1, eq1

= 1. Decoded output for count values of 16 or greater require

external decoding. The eq[15..0] outputs are asynchronous to 					the q[] output.

cout	 	No		Carry-out (borrow-in) of the MSB.

Parameters:

Parameter		Type	Required	Description

LPM_WIDTH		Integer	Yes	The number of bits in the count, or the width of

the q[] and data[] ports, if they are used.

LPM_DIRECTION	String	No	Values are "UP", "DOWN", and "UNUSED". If the

LPM_DIRECTION parameter is used, the updown port

cannot be connected. When the updown port is not

connected, the default for the LPM_DIRECTION

parameter is "UP".

LPM_MODULUS		Integer	No	The maximum count, plus one. Number of unique states

in the counter's cycle. If the load value is larger than the

LPM_MODULUS parameter, the behavior of the counter

is not specified.

This information is from the MAX+plus II help file.  The software is available from the Atmel website: www.Atmel.com