SUPER MORSE (TM)

QUICK.DOC

Ver. 4.16

Copyright 1986-1995

M. Lee Murrah

3-25-95

I. Introduction....................   1

A. This Document..............................................   1

B. Suggested Shareware Contribution...........................   1

C. Super Morse License........................................   1

II. Getting Started...............   2

A. Equipment and System Requirements and Limitations..........   2

1. General Requirements....................................   2

2. System Setup............................................   2

B. Distribution Files.........................................   2

C. Installation...............................................   4

1. If you have a hard disk.................................   4

2. If you have a floppy drive..............................   4

D. To run under Windows as a DOS application..................   5

E. Upgrading from Earlier Versions............................   5

F. Using a Sound Blaster card.................................   5

III. Super Morse Basics...............   6

A. Starting and Quitting Super Morse..........................   6

B. The Super Morse Screen.....................................   6

1. The Message Line (Top)..................................   6

2. The Status Lines (Bottom)...............................   7

C. Program Operation and Control..............................   7

1. Making Selections and Entering Information..............   7

2. The Function Keys.......................................   7

3. The <ALT> and Other Control Keys........................   8

4. Repeat a transmission...................................   8

5. Obtaining Help..........................................   8

6. Saving Program Parameters...............................   9

7. Exiting a Program Function..............................   9

D. The Morse Character Set....................................   9

1. Standard Characters.....................................   9

2. Nordic Characters.......................................   9

E. Code Speed Definition and Calibration......................  10

1. Speed Standards.........................................  10

2. Speed Control...........................................  10

a. Character, Send and Word Speeds......................  10

b. Farnsworth Method....................................  10

3. Calibration.............................................  10

IV. Using Super Morse..............  12

A. Selecting a Major Program Function.........................  12

1. Major Selections........................................  12

2. Pull-Down Menus.........................................  12

B. The Learn Selections.......................................  12

1. Lesson 1 - DITs and DAHs................................  13

2. Lesson 2 - DIT and DAH Combinations.....................  13

3. Lesson 3 - Character Steps..............................  13

4. Lesson 4 - Character Runs...............................  13

5. Lesson 5 - Character Groups.............................  13

6. Lesson 6 - Character Levels.............................  13

7. All Characters..........................................  13

8. User Selected Characters................................  14

9. Flash Characters........................................  14

10. Oddball Characters.....................................  14

11. View Group Definitions.................................  14

C. The Build Selections.......................................  14

1. Copy Modes -- Regular, Zoom and Match...................  14

2. Groups..................................................  15

a. Type -- Random, Variable, and Structured.............  15

b. Mode -- Normal, Number, and Time.....................  15

3. Words...................................................  15

4. Message.................................................  15

5. QSO.....................................................  15

6. Disk....................................................  16

a. Text Files...........................................  16

b. Scripted Files.......................................  16

D. The Enhance Selections.....................................  16

1. Boost...................................................  16

2. Head....................................................  17

3. Lag.....................................................  17

4. Q & A...................................................  17

5. Typewriter..............................................  17

6. Word....................................................  17

7. Keyboard................................................  17

8. Manual..................................................  17

E. The Measure Selections.....................................  18

1. Exam Formats............................................  18

a. Exam Sources.........................................  18

b. QSO Templates........................................  18

2. Simulated VEC Exam......................................  18

3. Auto....................................................  19

4. Generate, File, and Run.................................  19

5. Check...................................................  19

6. Display and Print.......................................  19

7. Keyboard................................................  19

8. Solid...................................................  20

F. Operation Selections.......................................  20

1. Modem...................................................  20

2. TeleKey.................................................  22

3. Radio...................................................  22

G. Review Selections..........................................  22

1. Session Selections......................................  22

2. DOC.....................................................  22

3. Stats...................................................  23

a. Stats Groups.........................................  23

b. "Mastery" Defined....................................  23

c. Clearing Stats.......................................  23

H. Options Selections.........................................  23

1. Code....................................................  24

a. Timing Method........................................  24

b. Timing Factor........................................  24

c. Calibration..........................................  25

d. Tone Split...........................................  26

e. Favorite Speed.......................................  26

f. Farnsworth Lo and Farnworth Hi.......................  26

g. Learn Speed..........................................  27

h. Speed Lock...........................................  27

i. Standard Lock........................................  27

2. Chars...................................................  27

3. Ports...................................................  28

b. The Telephone Port...................................  28

i. Selection.........................................  28

ii. Setting Communications Parameters................  28

c. The Keying Port......................................  29

4. Colors..................................................  29

5. Miscellaneous...........................................  30

a. Prompt Sound.........................................  30

b. Video Type...........................................  30

c. Show DIT/DAH.........................................  30

d. Learn Review.........................................  30

e. Learn Choices........................................  31

f. Jump Spaces..........................................  31

g. Boost Factor.........................................  31

h. Tie Speeds...........................................  31

i. Clear Screen.........................................  31

j. Sound Device.........................................  31

k. File Reader..........................................  31

6. Initial.................................................  32

7. Save....................................................  32

V. Business Details....................  33

A. Feedback...................................................  33

B. Distribution and Support...................................  33

1. Distribution Policy.....................................  33

2. Suggested Contributions.................................  33

a. General..............................................  33

b. International Users..................................  34

c. Credit Cards.........................................  34

d. Payments via Compuserve..............................  34

3. Support, Inquiries and Correspondence...................  34

C. Help Others Learn About Super Morse........................  35

D. Send Me Your Good News!....................................  35

VI. Troubleshooting....................  36

Page 1

I. Introduction

---------------

A. This Document

-----------------

This document contains the basic information needed to run

Super Morse.  Information of interest to advanced users is

contained in the file ADVANCE.DOC.  If you need information that

is not contained in one of these two files, please contact the

author.

B. Suggested Shareware Contribution

-----------------------------------

$20 is the suggested contribution, although any amount will

be appreciated. Payments may be charged to your Compuserve

account in the Shareware Registration Database (GO SWREF, Program

No. 649).

C. Super Morse License

----------------------

The author retains ownership of the copyright in Super

Morse. However, You are granted the right to use the program for

private, non-commercial purposes without further permission from

the author. That is, you can copy it, use it, and share it

(including distribution at hamfests and in ham radio and computer

equipment stores) so long as you do not charge any money for it

other than the cost of the disk or other transfer medium, not to

exceed $3.  You may not make a profit or recover labor costs for

copying or distributing Super Morse. Super Morse must be

distributed in its original form and content. Please note that

you MUST return a completed registration form as contained in the

file REGISTER.FRM as a condition of using Super Morse.

Shareware distributors may distribute Super Morse so long

as they include on or in close associate with the distribution

diskette a PROMINENT NOTICE stating that the user must also make

a Shareware contribution to the author.  The copy fee may not

exceed $5.

Page 2

II. Getting Started

-------------------

A. Equipment and System Requirements and Limitations

----------------------------------------------------

1. General Requirements

-----------------------

Memory:        384K

Drive :        Floppy disk or hard drive

Mouse :        Microsoft

Serial Ports:  3 for full use of Super Morse features.

Video :        Monochrome or color

2. System Setup

---------------

Your CONFIG.SYS file should authorize at least 20 files.

This can be done with the following statement:

FILES = 20

B. Distribution Files

---------------------

Super Morse is normally shipped as a self-extracting archive

file with a name such as SM401.EXE, containing all the files

necessary for operation.  The number in the file name is the

program version number.

If you would like to give a copy of Super Morse to a friend

or want to upload it to a computer bulletin board system, please

upload or share the archive file in its original condition.  Do

not add or remove files, since that can create confusion to other

users. In addition, do not distribute or share the files in

de-archived form since you may omit an important file!

Page 3

The Super Morse distribution file contains the following

files:

Needed

Name           Description                               to Run?

-----------    -----------                               -------

ANTENNAS.SM  - Names of antennas for QSO feature           Yes

CITIES.SM    - Names of cities for QSO feature             Yes

CLASSES.SM   - Definition of license classes               Yes

GROUPS.SM    - Code groups for Learn phase                 Yes

GROUPS.1     - Alternate code group for Learn phase         No

GROUPS.2     - Alternate code group for Learn phase         No

HAMWORDS.SM  - List of "ham" words for receive feature     Yes

JOBS.SM      - Name of occupations for QSO feature         Yes

NAMES.SM     - Names of operators for QSO feature          Yes

Q&A.SM       - Sample question and answer file             Yes

QSO1.SM      - QSO template for Measure phase              Yes

QUICK.DOC    - This DOC file                                No

README.1ST   - Important information about setup            No

REGISTER.FRM - For user registration and suggestions        No

REGWORDS.SM  - List of "regular" words for Receive feature Yes

RIGS.SM      - Names of radio equipment for QSO feature    Yes

RUNS.SM      - Character runs for Learn feature            Yes

SM.EXE       - the main executable file                    Yes

SM.ICO       - Icon file for use in Windows                 No

SM.PIF       - Program information file for Windows        Yes*

SMBLD.HLP    - Help file for Build phase                   Yes

SMBYE.HLP    - Sign off message                            Yes

SMHOME.HLP   - Help file for control keys                  Yes

SMENH.HLP    - Help file for Enhance phase                 Yes

SMLRN.HLP    - Help file for Learn phase                   Yes

SMMAIN.HLP   - Main help file                              Yes

SMMEAS.HLP   - Help file for Measure phase                 Yes

SMMORSE.HLP  - Help file for Morse characters              Yes

SMONLINE.HLP - Help file for the Modem feature             Yes

SMOP.HLP     - Help file for the Operate phase             Yes

SMRADIO.HLP  - Help file for radio terminal feature        Yes

SMSTATS.HLP  - Help file for Review|Statistics             Yes

SMTKEY.HLP   - Help file for telekey feature               Yes

STATES.SM    - States                                      Yes

STEPS.SM     - Character pairings for Learn feature        Yes

WHATSNEW.nnn - New features in version nnn                  No

* Needed for Windows use only

The data files ANTENNAS.SM, CITIES.SM, MAMWORDS.SM, JOBS.SM,

NAMES.SM, Q&A.SM, QSOn.SM, REGWORDS.SM, RIGS.SM, and STATES.SM

can be edited with a line editor or word processor in the ASCII

output mode.  For more information on editing, see the

ADVANCE.DOC file.

Page 4

C. Installation

---------------

In the following examples "<ENTER>" means to press the key

labelled "ENTER" or "RETURN".  Also version 4.01 is used as the

example, but for later versions, you should substitute the

correct version number. This can be found in the name of the

distribution file, which is in the form SMnnn.EXE, where nnn is

the version number.  For example, SM401.exe contains version

4.01.

1. If you have a hard disk

--------------------------

1. Make a directory, preferably named "\SM", as follows:

C> MD \SM <ENTER>

2. Change the current directory to the newly created

directory as follows:

C> CD \SM <ENTER>

3. Place the distribution diskette in a floppy drive.  We

will assume drive A: in this example.

4. Copy the program files to the newly created directory as

follows:

C> COPY A:*.* C:\SM <ENTER>

5. Extract the files from the archive file as follows:

C> SM401 <ENTER>

6. Run the program as follows:

C> SM <ENTER>

2. If you have a floppy drive

-----------------------------

1. Extract the files from the archive file follows:

following

A> SM401 <ENTER>

2. Run the program as follows:

A> SM <ENTER>

Page 5

3. Do not use a write protect tab on the floppy.

NOTE: If you have a 360K floppy drive, you will not be able

to extract the files onto the distribution diskette.  You will

have to find a friend with a high density drive or hard drive,

copy the files onto that machine, and do the extraction there.

You can then copy the extracted files (less the ZIP distribution

file) back onto a 360K diskette to run on your machine.

D. To run under Windows as a DOS application

--------------------------------------------

1. Follow the steps for hard disk above.

2. Make sure SM.PIF and SM.ICO are in the \SM directory.

3. Enter Windows and select the program group in which you

want Super Morse to appear.

4  Choose "File".

5. Select "Program Item".

6. Define Program "Properties":

Description: "Super Morse"

Command Line: "\SM\SM"

Working Directory: "\SM"

7. Use Browse to select the SM.ICO from the \SM directory

(or whatever directory you install Super Morse in).

8. Use the loop timing method and speaker sound output.

E. Upgrading from Earlier Versions

----------------------------------

If you are upgrading from version 4.07 or earlier, delete

SM.CFG and all SMn.USR files (e.g., SM1.USR).

F. Using a Sound Blaster card

-----------------------------

To use a Sound Blaster or compatible card, insert an

environment string in your AUTOEXEC.BAT file as described in your

Sound Blaster manual, using a text editor or word  processor in

ASCII output mode. If you are using Windows and have a Sound

Blaster installed as you Windows sound device, you will not be

able to use it with Super Morse.  You must use the speaker as the

sound output device or exit Windows and run Super Morse under

DOS.

Page 6

III. Super Morse Basics

-----------------------

A. Starting and Quitting Super Morse

------------------------------------

To start Super Morse:

1. At the prompt type SM <ENTER>.

2. At the bottom of the title screen, enter a user number

selected from 0-9 and press <ENTER>. User 1 is the default.

To quit Super Morse:

1. Press <ESC> or select Quit from the main menu.

2. Answer whether you want to save the new program

parameters, if prompted.

B. The Super Morse Screen

-------------------------

1. The Message Line (Top)

-------------------------

At all times while Super Morse is running, the top line of

the screen is used to identify the current function and the key

necessary to abort that function.

Page 7

2. The Status Lines (Bottom)

----------------------------

Likewise, the bottom three lines display important program

parameters as follows:

Wt        Code weighting (ratio of DAH to DIT length)

Std       Code speed standard, either text or random

characters.

Zoom      Zoom mode status.

Mat       Match mode status. Match mode checks keyboard

input against characters previously sent.

Int       Interference status

Jit       Code jitter status

Sou       Sound output status

Ch        Currently active characters

Txt       Hide or show text status

ETim      Elapsed time

SSp       Send speed

Csp       Character speed

WSp       Word speed

Class     License class represented by speed

Tone      Speaker tone frequency or note

User      Current user number

C. Program Operation and Control

--------------------------------

1. Making Selections and Entering Information

---------------------------------------------

Selections are made from pull-down and line menus using

either the arrow keys, the mouse, or entering a highlighted

letter in the menu selection via the keyboard.  The up and down

arrow keys move a pointer bar up and down in pull-down menus, and

the right and left arrow keys move the pointer laterally in line

menus.  A selection can be highlighted by pointing and clicking

the left mouse button.  The highlighted selection can be invoked

by pressing <ENTER>, double clicking the left mouse button, or by

pressing the highlighted character in the selection name.

2. The Function Keys

--------------------

The function keys are active at any input prompt, and while

code is being sent.  They are defined as follows:

F1  - Decrease SEND Speed   F2  - Increase SEND Speed

F3  - Decrease CHAR Speed   F4  - Increase CHAR Speed

F5  - Decrease WORD Speed   F6  - Increase WORD Speed

F7  - Lower Speaker Tone    F8  - Raise Speaker Tone

F9  - Decrease All Speeds   F10 - Increase All Speeds

Page 8

The operations performed by these keys are discussed in more

detail in several of the following sections.

3. The <ALT> and Other Control Keys

-----------------------------------

Super Morse parameters may also be set using <ALT> key

combinations and other keys.  Some are active while code is being

sent and others only when an input is required.

The following <ALT> combinations and other keys are

available:

<ALT-C>  - Clear the screen (not active during menu

presentation)

<ALT-E>  - Set to Extra class speed

<ALT-F>  - Set to User's favorite speed

<ALT-G>  - Set to General class speed

<ALT-H>  - Context sensitive help

<ALT-I>  - Set Interference type

<ALT-J>  - Toggle code speed jitter

<ALT-K>  - Toggle speaker

<ALT-N>  - Set to Novice class speed

<ALT-P>  - Pause

<ALT-S>  - Toggle between code speed standards (PARIS and

CODEX)

<ALT-T>  - Reset elapsed time clock

<ALT-V>  - Save new settings

<ALT-W>  - Set code weight (2.0-6.0)

<ALT-X>  - Set text display mode

<ALT-Z>  - Toggle "Zoom" mode

<ALT-1>  - Set class 1 speed

<ALT-2>  - Set class 2 speed

<ALT-3>  - Set class 3 speed

<ALT-=>  - Set cs,ss and ws to the same value

<HOME>   - Quick key help

<ESC>    - Quit a function

4. Repeat a transmission

------------------------

In most exercises characters sent by Super Morse can be

repeated by pressing the <TAB> key at any prompt.

5. Obtaining Help

-----------------

Most menu's have a HELP selection which provides a brief

help screen.  Help for the control keys is available by pressing

<HOME>.

Page 9

6. Saving Program Parameters

----------------------------

Program parameters can be saved using the <ALT-V> command,

the Options|Save command, or when prompted upon exiting the

program.

7. Exiting a Program Function

-----------------------------

Most program functions can be aborted or exited using the

<ESC> key.

D. The Morse Character Set

--------------------------

1. Standard Characters

----------------------

Super Morse implements the standard Morse code character set

used in amateur radio.

Procedure signs ("prosigns") which are not printable as

ASCII characters are printed on the screen as a combination of

characters in pointed brackets.  Super Morse permits prosigns to

be entered either as a combination of the two printable

characters or a special ASCII character.  The prosigns and the

ASCII representations are as follows:

Prosign      ASCII Representation    Meaning

-------      --------------------    -------

<AR>                 +              End of message

<SK>                 $              End of contact

<AS>                 !              Wait, stand-by

<KN>                 #              Over-specified station

only

<BT>                 =              Double dash

<AA>                 @              New line

<KA>                 &              Attention

<SN>                 %              Understood

Double characters may be entered on the keyboard by pressing the

<SHIFT> key.

2. Nordic Characters

--------------------

In addition, Super Morse supports Nordic characters and

non-amateur punctuation(colon, semicolon, left parenthesis, and

right parenthesis).

Page 10

E. Code Speed Definition and Calibration

----------------------------------------

1. Speed Standards

------------------

The PARIS standard for random characters is used for all but

the Build/Groups functions where the CODEX method for plain text

is used.  As a consequence, that function will seem to be faster

than the other functions, but do not be alarmed.  That is the way

it should be!  You can toggle the code speed standard at any time

by pressing <ALT-S>.

2. Speed Control

----------------

a. Character, Send and Word Speeds

----------------------------------

Super Morse provides control over three aspect of code

speed. The "character" speed is the speed at which individual

elements comprising the Morse characters (DITS, DAHS, and

inter-element spacing) are sent.  Normally, the spacing between

characters is sent at the same speed as the character elements.

However, when the Farnsworth method is used, the spacing between

characters is sent at a slower speed than the characters

themselves.

Super Morse provides a further refinement in permitting you

to also control the "word speed", which controls the spacing

between words or character groups.  This is, of course, available

only when words or character groups are being sent.  Only

advanced users are likely to find any need for this setting, and

all others can ignore it.

b. Farnsworth Method

---------------------

The Farnsworth method is implemented in Super Morse by

permitting you to control the "send" speed, which takes into

consideration the present character speed and increases the

inter-character spacing to provide the indicated overall send

speed.

3. Calibration

--------------

In order for Super Morse to send properly-timed code,

calibration to each computer is required.  Super Morse uses two

different calibration methods, Clock and Loop, in order to handle

various types of PC designs and operating systems. The method is

selected from the Options|Code menu.

If the clock calibration method is used, Super Morse

automatically calibrates each time it loads, and the speed should

Page 11

be independent of your computer clock speed.  You may need to

touch up the calibration the first time you run the program.

If you use the loop method, you must manually calibrate

Super Morse. You must also recalibrate Super Morse each time you

add or remove a memory resident program if you use the loop

method.

The loop method must be used if your computer does not

implement the system clock chip in the same way as the original

PC design.  Super Morse is initially set for the clock method;

but if the code is erratic or choppy on your computer, you should

change to the loop method. You must also use the loop method if

you are running Microsoft Windows or a PC emulator on a McIntosh!

Page 12

IV. Using Super Morse

---------------------

A. Selecting a Major Program Function

-------------------------------------

1. Major Selections

-------------------

The following major program functions are provided:

Learn     Learn Morse code characters

Build     Build receiving speed

Enhance   Enhance sending and receiving skills

Measure   Measure progress

Operate   Operate via telephone or radio

Review    Review pertinent information

Options   Set program parameters

Help      Obtain help

Quit      Quit Super Morse

2. Pull-Down Menus

------------------

When one of the major selections is chosen, a pull-down

menu with additional selections is then presented.  One of the

selections is always a Help screen, and the last selections is

always Quit. Help and Quit are not shown in any of the subsequent

discussions of menus, but you should understand that they are

there.

B. The Learn Selections

-----------------------

The Learn menu includes the following selections:

DIT/DAH   Learn the basic DIT and DAH sounds

Combos    Learn the basic DIT and DAH combinations

Steps     Learn characters in steps of similar sounding

characters.

Runs      Drill using sequential groups of similar sounding

characters from the Steps function.

Groups    Drill using character groups defined by the ARRL.

Levels    Drill using combinations of Groups.

All       Drill using all Morse characters

User      Drill with user-selected characters shown on the

status line.

Oddball   Select the one character that is different

Flash     Character flashcards

Groups    View the Steps, Runs and Group definitions

Page 13

Many of the features are scored and the result is shown in the

status area showing number right, number wrong, consecutive

number correct, and percentage correct.

In addition each exercise has a mastery level associated

with it (see Review|Stats), and the user is notifed with message

when it is attained.

1. Lesson 1 - DITs and DAHs

---------------------------

Hear the DIT and DAH sounds and learn to distinguish them in

an interactive drill.

2. Lesson 2 - DIT and DAH Combinations

--------------------------------------

Hear the four basic DIT and DAH combinations and learn to

distinguish them in an interactive drill.

3. Lesson 3 - Character Steps

-----------------------------

Interactively learn characters in 69 steps of closely

related character pairs.  Enter the correct character when

prompted.

4. Lesson 4 - Character Runs

----------------------------

Interactively learn runs of characters, each successive one

of which differs from by only a DIT or a DAH from the previous

character. Enter the correct character when prompted.

5. Lesson 5 - Character Groups

------------------------------

Interactively learn from standard character groups as used

by the ARRL and others. Enter the correct character when

prompted. Groups are customizable.

6. Lesson 6 - Character Levels

------------------------------

Interactively learn from character levels formed from the

groups. Enter the correct character when prompted.

7. All Characters

-----------------

Drill using all Morse characters.  Enter the correct

character when prompted.

Page 14

8. User Selected Characters

---------------------------

Take interactive quiz involving a customized group of

characters from the currently active character set.  Enter the

correct character when prompted.

9. Flash Characters

-------------------

Listen to the sounds of randomly selected characters as

Super Morse flashes them on the screen in large size. No user

keyboard response is required.

10. Oddball Characters

---------------------

Listen to a group of five characters and then enter the one

character that is different from the rest.

11. View Group Definitions

--------------------------

The character steps and groups used in the preceding

selections may be displayed using the View selection.

C. The Build Selections

------------------------

The Build selections include the following:

Groups         Receive groups of random characters

Words          Receive whole words

Message        Receive a message entered by the user

QSO            Receive a simulated radio conversation

Disk           Receive contents of a disk file

1. Copy Modes -- Regular, Zoom and Match

----------------------------------------

All functions are sent at the speed shown on the status

line. However, the Groups, Words and Disk functions can be sent

in the "zoom" mode, which is enabled and disabled by pressing the

<ALT-Z> key, and its current status is shown on the status line.

The zoom mode sends a specified number of words at one speed and

then bumps the speed to a higher speed in a specified increment.

The Zoom start and stop speeds, speed increment and number of

words at each speed is set in Options|Zoom.

Code can be copied directly on the keyboard by setting the

Match mode using <ALT-M>. Correct characters ares shown in normal

video and incorrect characters are shown in reverse video.

Page 15

2. Groups

--------

a. Type -- Random, Variable, and Structured

-------------------------------------------

Choose either Random, Variable or Structured groups from the

line menu. Then choose Normal, Timed or Number modes.

Random sends characters in randomly selected 5 character

groups. Structured groups are similar to the Random groups

except that the first and second randomly selected characters of

a group are repeated in reverse order as the fourth and fifth

characters. The third character is always a randomly selected

number.  This permits you to receive random code but have the

ability to check his answer without looking at the screen.

Variable groups are also similar to Random groups except

that the length of the groups is variable from 1 to 10.  This

helps you become accustomed to copying normal words, which are of

differing lengths.

b. Mode -- Normal, Number, and Time

-----------------------------------

Normal mode sends continuously without limit. Number mode

sends the number of groups specified by the user, and Time mode

sends for a limited time specified by the user.

3. Words

--------

This selection permits you to work with real words rather

than groups. You can select either Regular or Ham words,

Call signs, or words from a user disk file.

4. Message

-----------

Type in a message and press <ENTER> to send it.  Press <TAB>

to repeat it. The maximum length of the message is 255

characters.

5. QSO

------

Use the QSO function to copy a simulated radio contact

between two stations (know to hams as a "QSO").  Random text is

sent containing randomly chosen call signs, names, etc.,

providing a variety of QSO combinations.

Page 16

6. Disk

-------

a. Text Files

-------------

To send a text stored in a disk file, enter a filename,

including disk drive letter and subdirectory at the prompt.  The

last file used is shown as a suggestion, and it may be selected

by pressing <ENTER>. If you do not know the name of the file you

wish to receive, you may press <SPACE> to obtain a disk

directory.  At the prompt enter the file specification in the

form D:\SUBDIR\FILENAME.EXT, where "D:" is the drive designator,

"SUBDIR" is the subdirectory name if applicable.  Wildcards are

may be used.

Disk files can be sent either in Normal or Reverse modes as

selected by the user when prompted.

b. Scripted Files

-----------------

Scripted disk files can also be used.  See SCRIPT.SM for an

example, or the ADVANCE.DOC file for complete instructions.

D. The Enhance Selections

-------------------------

The following enhance selections are provided:

Boost          Speed is automatically boosted as long as you

maintain a specified accuracy level.

Head           Copy code in your head by memorizing a string

of characters as they are sent.

Lag            Learn to copy behind - max 50 characters

behind. Both random lag and specified number

behind

Q & A          Answer questions sent in code.

Typewriter     Send using keyboard as a typewriter.  There

is an 80 character type ahead buffer.

Word           Copy whole words.

Keyboard       Copy on the keyboard

Manual         Send code using designated key on keyboard,

mouse key or external telegraph key with

automatic decoding to screen. Default is LSHIFT

key.

1. Boost

--------

Code is sent beginning at 3 WPM and gradually increases in

speed as long as the accuracy level set in Options|Misc is

maintained.  Copying is on the keyboard.  The maximum speed

Page 17

reached is recorded, and Boost begins at slightly below that

speed when this feature is next used.

2. Head

-------

A string starting with length 1 is sent again and again,

adding one character each time, until the user misses.  This

forces you to memorize a string of increasing length.

3. Lag

------

Learn to copy behind the current character being sent in two

modes.  In the first mode, the user enters a lag number and

sending begins.  The program asks the user to enter not the

current character but the one sent the lag number times ago.  In

the Random lag mode, several characters are sent and the user is

asked to enter a randomly selected one of them.

4. Q & A

--------

Super Morse sends a question taken from the Q&A.SM disk file

in code, and the use is asked to enter the answer.  The answer

must match the word shown in the line immediately following the

question in the disk file.

5. Typewriter

-------------

Copy randomly sent characters on the keyboard.  Correct

characters are shown in normal video, and errors in reverse

video.

6. Word

-------

Whole words are sent, and the user is asked to type the word

on the keyboard.

7. Keyboard

-----------

Characters typed on the keyboard are sounded by Super Morse.

8. Manual

---------

The user sends Morse code manually using either the ESC,

Right SHIFT, Left SHIFT, or left mouse button.  The key is chosen

in Options|Misc, and the default is LSHIFT. Super Morse decodes

Page 18

the characters sent and prints them on the screen.  The program

adapts to send speed.

E. The Measure Selections

-------------------------

Measure provides the following selections:

Auto      Take an internally generated test

File      Take a test stored in a disk file

Check     Check character content of a test

Display   Display the current test

Print     Print the current test

Generate  Generate a test

Keyboard  Test receive speed on the keyboard

Solid     Test ability to copy characters without a mistake

1. Exam Formats

---------------

a. Exam Sources

---------------

Several of the Measure functions use examinations based upon

simulated radio contacts, or QSO's.  The format and content of

these QSO's are determined either by an internal QSO generation

routine or an external template.  The internal routine is used

exclusively in the Auto exam function and is the default

selection in the Generate exam function.

b. QSO Templates

----------------

The external template may be selected in the Generate exam

function.  The external template is contained in a disk file

named QSOn.SM where "n" is a number from 1 to 99.  The file

QSO1.SM is provided as an example, but you may create other

ASCII files yourself.  Instructions for creating templates is

provided in the ADVANCE.DOC file.

2. Simulated VEC Exam

---------------------

All but the last two selections are associated with a

simulated code test much like the one you would take when sitting

for the license exam given by the FCC-approved Volunteer

Examination Coordinator (VEC).  All VEC exam-related functions

are based upon the simulated contact used for the QSO command

discussed above. In the exam functions the student is asked to

set the exam speed either at novice, general class, extra class

speed, or other user-specified speed.  If one of the three

license classes is chosen, you are further given the opportunity

to select either regular or Farnsworth spacing.

Page 19

The user is cautioned that the actual exam may not be

exactly like the one used in Super Morse!

3. Auto

-------

This is a fully integrated self-test for the student.  All

exam questions are displayed on the screen, and the exam is

automatically graded.

4. Generate, File, and Run

--------------------------

Prepare an exam in advance and give it using the Generate

exam command and Run exam file commands. These commands are

intended primarily for use by an instructor to create and give a

simulated test to a student or students. The Generate command

creates an exam QSO either internally or from an external

template as selected and stores it in a special file on disk. The

Generate command also permits a new set of questions to be

created for the existing exam.

The Run command loads the exam previously created using the

Generate command and sounds it at the Novice, General, or Extra

exam speed as selected. You can also select None, and the speed

will default to that shown on the status line.

5. Check

--------

Check tallies and reports the characters used and not used

in the current exam file.  This may be used to determine whether

an exam meets the requirements for an official code test.

6. Display and Print

--------------------

Review the text of the test QSO by displaying it on the

screen or by listing it on a line printer with the Display

and Print commands. Either command permits you to select either

the text of the exam, the related exam questions, or the exam

answers.

In the Print mode you are prompted to turn on the line

printer and press a key to begin.  If the printer is not on, an

error will occur, and the program will terminate.

7. Keyboard

-----------

This command lets you test your approximate receive speed.

You are prompted to choose either random characters or the

contents of a disk file. The computer sends a character and waits

Page 20

for you to type in the correct key.  The accuracy of the copy and

the approximate copy speed are displayed on the status line.

If the incorrect key is typed, the character is printed in

reverse video (color monitors only) and sent again.  The computer

also remembers wrong characters and sends them more often than

the other characters.

The character set used in the test can be configured using

the Options|Chars command.

8. Solid

--------

This function tests your ability to copy random characters

without a mistake.  A sequence of characters of increasing length

is sent, and you are asked to type in the sequence exactly as

sent. When a mistake is made, the test ends and you are given a

report of the number of consecutive characters copied correctly

and the number of minutes and seconds during which "solid copy"

occurred.

F. Operation Selections

------------------------

The Operations function contains the following selections

Modem          Send and receive via a telephone modem

Radio          Send and receive using a radio

1. Modem

--------

The Modem command provides a simple terminal program which

permits you to use a telephone modem connected to your computer

to call another similarly equipped user and carry on a real-time

QSO in Morse code.  A modem using the Hayes AT command set is

assumed.

When Modem is selected, the computer's selected

communications port is initialized to the parameters stored in

the configuration file, the default values of which are COM1,

1200 baud, 8 bits, no parity, and 1 stop bit.  The modem is

initialized to automatically answer calls from another computer

on the first ring.

The Modem commands may be displayed using the usual <ALT-H>

specific help key.  All Modem commands are invoked using the

<CTRL> key in combination with another key.  The following

commands are provided:

<CTRL-C>       Activate CW (Morse) mode

<CTRL-D>       Dial a number

<CTRL-E>       Echo/No echo of characters sent to modem

Page 21

<CTRL-F>       Send a disk file

<CTRL-H>       Terminate telephone connection (hang up)

<CTRL-L>       Add/remove linefeeds from received text

<CTRL-M>       Hang up modem

<CTRL-P>       Set baud rate and other communications

parameters

<CTRL-R>       Enable/disable Morse mode for received

characters

<CTRL-S>       Enable/disable Morse mode for transmitted

characters

<CTRL-T>       Activate Terminal (non-Morse) mode

Initially, the Modem function is in the "Terminal" mode,

which means that it will act just like a normal modem program and

will not sound Morse code.  The terminal mode should be used to

make a connection with the other computer, and then you should

switch to the "CW" mode by pressing <CTRL-C>.  You can switch

back to the terminal mode by pressing <CTRL-T>.

Initially Super Morse will sound both the received and sent

characters.  This can be selectively changed using the <CTRL-R

and CTRL-S> commands.  Super Morse also adds line feeds to all

carriage return characters.  If received lines are printed

double-spaced, this can be changed with the <CTRL-L> command.  If

you would like to see characters sent to the modem during the

time that Super Morse is not connected to another computer,  use

the <CTRL-E> command.

To connect with another computer and use the Modem command,

do the following:

a. Prearrange the call with the other person.  Both copies

of Super Morse should be in the Modem function, and both should

be set to the terminal mode with <CTRL-T>. Both must set their

communications parameters, other than the communications port, to

the same values.

b. Dial the other computer by entering the number in the

dial box, which pops up automatically when Modem is entered or

when the <CTRL-D> command is entered.

c. Enter the telephone number and press <ENTER>.

d. Wait for the other party's telephone to ring and the his

modem answer the phone, followed by a brief carrier tone. Both

screens should indicate "Connected to Remote SUPER MORSE."

e. Both users then enter the CW mode by pressing <CTRL-C>

and proceed with the conversation from the keyboard or from a

file entered using the <CTRL-F> command.

f. To disconnect the phone, use the <CTRL-H> command.

Page 22

2. TeleKey

----------

TeleKey permits use of a manual telegraph key to send Morse

code to another coputer via modem.  Whereas Modem send characters

in ASCII format and translates them into code. TeleKey sends the

actual code as entered using a manual key.  Either an external

manual key or one of the keyboard keys can be used as the manual

key. The CTRL key should not be chosen since it will interfere

with recognition of the CTRL key commands.  Only the <CTRL-D>,

<CTRL-E> and <CTRL-H> commands are used in TeleKey.  The

connection is made in the same way as in Modem.

3. Radio

--------

The Radio command is for advanced users and is described in

the ADVANCE.DOC file.

G. Review Selections

--------------------

The Review menu provides the following selections:

Session        Examine and replay the most recent Super

Morse session

Keys           View function and ALT key assignments

Morse          Display the Morse code definitions

DOC            Display this file.

QSO            Display the most recent QSO

Stats          Examine user statistics

Only those selections that require any further explanation

or user input will be covered below.

1. Session Selections

---------------------

A secondary pull-down menu presents the following Session

selections:

View           Display the most recent session

Play           Replay the most recent session in Morse code

Erase          Erase it

Print          Print it to a line printer

File           Save it to a disk file

2. DOC

-----

This QUICK.DOC file may be viewed using this command.

Page 23

3. Stats

--------

a. Stats Groups

---------------

This selection permits you to review statistical information

about your progress recorded in several Super Morse functions.  A

tertiary pull-down menu presents the following selections for the

current user:

General        Number sessions, total time and current step,

run, group, and level in learn function.

Learn          Mastery status of DITs/DAHs, combinations,

all characters, steps, runs, groups and levels.

Progress       Progress measured by boost copy speed, longest

head copy, solid copy time at each speed, best

auto exam score, and best type test speed.

Erase          Erase statistice for current user.

b. "Mastery" Defined

--------------------

Mastery is defined as a certain number of consecutive

correct answers for all characters in the current exercise.  The

numbers for each exercise are as follows:

DIT/DAH             10

Combinations         5

Steps                5

Runs                 3

Groups               3

Levels               3

All characters       1

A mastery level of 1 is also set for user characters, but the

results are not shown since it is not a fixed grouping.

c. Clearing Stats

-----------------

The user statistics can be cleared with the <E>rase User

command from the Stats display.  Use this command with caution!

H. Options Selections

---------------------

The Options menu provides the choices of the following

groups of options setting:

Code      Set parameters relating to code generation

Chars     Define character set

Ports     Set up ports

Page 24

Colors    Set screen colors

Misc      Miscellaneous settings

Initial   Reinitialize parameter settings to default

Save      Save current parameter settings

All parameters can be reset to their initial values using

the "Initial" command, and the current values can be saved using

the "Save" command.

1. Code

-------

Following are the selections in the code menu with example

settings shown.

Timing Method       : Timer

Timing Factor       : 1083

Calibration         : XXXX

Tone Split          : On

Set Fav. Speed      : 5-14-5

Learn Speed         : On

Farnsworth Lo       : 18

Farnsworth Hi       : 25

Speed Lock          : Off

Standard Lock       : Diff

a. Timing Method

----------------

Super Morse uses two different calibration methods that may

be used depending upon your equipment requirements.  The first is

the Timer method, which directly uses the PC's clock chip.  This

method provides automatic calibration each time Super Morse is

run and produces more accurate times and thus better sounding

code.

The Loop method is provided as an alternative since not all

PC's support the clock chip in exactly the same way (and

therefore that method will not work on all PC's), the Timer

method seems to be incompatible with Microsoft Windows, and the

Timer method will not work with PC emulators for other machines

such as the Apple McIntosh. The disadvantage of the Loop method

is that it requires manual calibration and must be recalibrated

each time a memory resident program (including Windows) is added

or removed.

b. Timing Factor

----------------

The timing factor helps match the code speed to the computer

speed.  When switching between the Timer and Loop timing methods,

you will find that the timing factor is radically different.  In

some cases the calibration test may be so slow that you will have

Page 25

to abort it and guess at a an appropriate timing factor.  As a

rough guide, the timing factor can vary from 100-12000 or greater

depending upon clock speed and computer design.

c. Calibration

--------------

While the Timer timing method automatically calibrates each

time Super Morse is run, you should adjust the timing factor the

Page 26

first time you run Super Morse running manual Calibration. Once

Super Morse is adjusted to your computer when using the Timer

method, you should not need to change it.

If you use the Loop method, calibration must be performed

manually each time you add or delete memory resident programs.

Super Morse tests calibration by sending the standard word

PARIS the same number of times as the send speed shown on the

status line.  The send, character and word speeds can be set to

any desired speed. If Super Morse is correctly calibrated, the

test should be completed in exactly one minute.  Super Morse

times the test and displays the result along with a

recommendation for a revised Timing Factor. The calibration

should be repeated until the recommended and the current Timing

Factor are very close. You should seek an elapsed time of 59-60

seconds so that any errors will be on the too-fast side of the

indicated speed.

d. Tone Split

-------------

The tones for sending DITs and DAHs are normally the same,

but as an additional aid in distinguishing between the two, Super

Morse permits you to split the two tones such that the DAH is 10

Hz. lower than the DIT.  This is accomplished using the Tone

Split selection in the Options|Code menu. The Tone Split feature

is not available when the Sound Blaster is used for sound output.

e. Favorite Speed

-----------------

You can directly set the current speed to a speed that you

commonly use by pressing the <ALT-F> key combination.  The speed

used by the <ALT-F> command is set to the current speed shown on

the status line using the Set Fav. Speed selection in the

Options|Code menu.  Thus, to set a favorite speed, set the speeds

on the status line using the function keys and then press <ALT-

F>. The speeds shown in the menu are in the same order as on the

status line.

f. Farnsworth Lo and Farnworth Hi

---------------------------------

The character speed for the Farnsworth method can be set for

both low and a high speed ranges.  The Farnsworth speed for send

speeds below 20 WPM is set in the range of 13-30 WPM with the

"Farnsworth Lo" selection; and for speeds above 20 WPM in the

range 20-30 WPM with the "Farnsworth Hi" selection. The

Farnsworth speed is also used in the quick speed settings, using

ALT keys.

Page 27

g. Learn Speed

--------------

Using the Learn Speed selection, you can determine whether

Super Morse switches to special learn speeds when the Learn phase

is entered or whether the speeds remain the same as for other

phases.

h. Speed Lock

-------------

When using Super Morse for classroom teaching, it may be

helpful to lock the speed settings to prevent the student from

changing them to an easier speed.  This setting permits the speed

change keys to be locked and unlocked selectively.

i. Standard Lock

----------------

Normally Super Morse toggles between the PARIS (text) and

CODEX (random) speed standards as the context dictates.  However,

those users are distracted by the change a lock may be set to

maintain the PARIS standard regardless of the type of characters

being sent.  In addition the standard lock also disables the

<ALT-S> standard toggle.  Use of the lock is not recommended

since it will help you on the test to have studied random

characters at the seemingly "higher" random character speed.

2. Chars

--------

The set of characters that you desire to receive may be set

by choosing this command. The Chars screen displays the entire

character set in a line at the top and a series of x's and -'s

just below them.  The x's indicate that character just above it

is currently active while the -'s indicate the opposite.  The

status can be altered in groups or individually using the

following commands:

Letters        The letters A-Z

Numbers        The numbers 0-9

Punctuation

Prosigns       The Procedure signs

National       The international characters

Group 1        The character groups used in Learn

Group 2             "

Group 3             "

Group 4             "

Group 5             "

Group 6             "

Group 7             "

Default        The default set

Type           Set individual characters by typing

All            All characters

Page 28

Note that the prosigns are represented by their single

character ASCII equivalent character. The default character set

has the letters and numbers enabled and the punctuation and

prosigns disabled. The National command is active only when the

special Nordic character set has been enabled. When using Groups,

note that the alternate character groups file (GROUPS.2) has only

six groups, and the "Group 7" selection will thus not be active

if that file is used.

To change the status of individual characters, select "Type"

and then press the desired keys.  When finished entering

individual characters, press <ESC> to exit back to the character

menu.

3. Ports

--------

Super Morse uses two different serial ports, a "keying port"

to interface with a keyer or other device and a "telephone port"

to interface with a telephone modem. The setting for these ports

are determined using the following menu:

Phone Port          : COM1

Baud Rate           : 1200

No. Bits            : 8

Parity              : N

Dial type           : Tone

Keying Port         : COM2

Input Keying        : ALT

RS-232 Input        : DSR

Input Sense         : +

Output Keying       : Off

RS-232 Output       : DTR

Output Sense        : +

b. The Telephone Port

---------------------

i. Selection

------------

Super Morse makes use of the PC serial communications ports

for the Operate|Modem feature. You can set either COM1 or COM2

using the Phone Port selection.

ii. Setting Communications Parameters

-------------------------------------

The baud rate for the telephone port can be set at 300-9600

baud with the Baud Rate selection, the number of bits can be set

Page 29

either at 7 or 8 using the No. Bits selection, and the parity can

be set at either Even, Odd, or None using the Parity selection.

The dialing type used by your telephone is set to either Tone or

Pulse using the Dial type selection.

c. The Keying Port

------------------

Setting up and using the keying port for external input and

output is an advanced feature that is covered in detail in the

ADVANCE.DOC file. Only the onboard functions will be covered

here.

To use the keyboard as a manual key, set input keying to

either of the select either CTRL, Left SHIFT, Right SHIFT, or

Mouse.  This designates the selected key for use as a manual key

in the Enhance|Manual function.  If the mouse is selected, the

left button is used.

4. Colors

---------

Screen colors can be set using the following commands:

A - Main Bg         Main screen

B - Main Fg

C - Main Hi

D - Menu Bg         Menus

E - Menu Fg

F - Menu Hi

G - MBar Bg         Menu selector bar

H - MBar Fg

I - MBar Hi

J - Status Bg       Status line

K - Status Fg

L - Status Hi

M - SBar Bg         Status line selector bar

N - SBar Fg

O - SBar Hi

P - Line Bg         Line menu

R - Line Fg

S - Line Hi

T - Info Bg         Information box

U - Info Fg

V - Info Hi

W - IBar Bg         Information box selector bar

X - IBar Fg

Y - IBar Hi

1 - Blue Set        Predefined blue color set

2 - Red Set         Predefined red color set

3 - Green Set       Predefined green color set

4 - Mono Set        Predefined Monochrome colors

Page 30

where:

Bg = background

Fg = foreground

Hi = highlight

As A-Y are selected, the indicated areas are cycled through

the CGA adapter colors. 1-4 select entire predefined color sets.

5. Miscellaneous

----------------

A miscellaneous group of parameters is et using the

following menu selections:

National Chars      : English

Video Adapter       : CGA

Show DIT/DAH        : On

Learn Review        : On

Learn Choices       : On

Boost Factor        : 90%

Prompt Sound        : On

Jump Spaces         : Yes

Tie Speeds          : On

Clear Screen        : On

Sound Device        : Speaker

File Reader         : Int.

a. Prompt Sound

---------------

The prompt sounds ("beep", "diddle", "click")can be toggled

On and Off using this selection.

b. Video Type

-------------

At program start Super Morse automatically determines the

video type used in your computer. However, this can be changed

between Mono and Color if necessary with the Video Adapter

selection.

c. Show DIT/DAH

---------------

The Learn phase uses a screen display of DIT and DAH to

reinforce the audio.  This feature can be turned on and off using

the Show DIT/DAH selection.

d. Learn Review

---------------

Normally when a mistake in the character quizzes in Learn,

you are given the opportunity to review the characters missed and

Page 31

the character actually chosen.  This can be turned off and on

with this selection.

e. Learn Choices

----------------

In the character quizzes in the Learn, the prompt normally displays

the character list from which the character was chosen.  This can be

toggled off and on using this selection.

f. Jump Spaces

--------------

The Jump Spaces selection permits multiple spaces in a disk

file to be skipped.

g. Boost Factor

---------------

The percentage of correct answers necessary to increase the

speed in the Boost feature can be adjusted using the Boost Factor

selection.

h. Tie Speeds

-------------

Super Morse permits you to tie the send, character and word

speeds above 20 words per minute for easy one-key settings.

However, this can be disabled using the Tie Speeds selection.

i. Clear Screen

---------------

Setting this choice to "On" causes the screen to be cleared

when exiting a function.

j. Sound Device

---------------

Super Morse permits selection of either the PC speaker or

the Sound Blaster card for audio output, or a flashing light for

a silent code output, using the Sound Device selection. The Sound

Blaster card provides a much higher output level than the PC

speaker and can easily be fed to an external amplifier or

recorder if desired.

The flashing light option permits Super Morse to be used

where the audio output is inappropriate.

k. File Reader

--------------

Super Morse permits use of either an internal file reader to

display this QUICK.DOC file or the very popular LIST program by

Page 32

Vernon Buerg. The latter is recommended since it is a very

flexible program which permits scrolling forward and backward and

text searches. LIST must be in the same directory as the Super

Morse files.  LIST can be downloaded from many computer bulletin

boards or purchased from Shareware copy services.

The File Reader selection in the Options|Misc menu toggles

between the two readers, which are displayed as "Int." for the

internal reader and "LIST" for the LIST program.

6. Initial

----------

Super Morse parameters can be reset to their initial

settings with the Options|Initial selection.

7. Save

-------

This selection saves the current parameters.

Page 33

V. Business Details

-------------------

A. Feedback

-----------

It is my goal that Super Morse be absolutely the best Morse

code training program available anywhere, commercial or

otherwise. If it is not the best, I will endeavor to make it so.

To achieve this I NEED YOUR INPUT!  Please let me know what you

like about Super Morse, what you don't like about it, any bugs or

cosmetic defects you have discovered, what you would like to see

changed or added, etc.  If you have used another Morse code

program that you like better, tell me the name of the program and

why you like it better. Please take the time to answer and return

the questions in the feedback form contained in the file

FEEDBACK.FRM to me. If that form is inadequate, write me a

letter.  If a letter won't do the job, give me a call!

B. Distribution and Support

---------------------------

1. Distribution Policy

----------------------

Super Morse is distributed primarily through Compuserve

"Hamnet" and computer bulletin board systems.  If possible, users

should obtain the program in that manner.  If the user does not

have a modem and a Compuserve subscription, the author will make

a courtesy copy if the user will send a formatted diskette

(preferably high density--either 5 1/4 or 3 1/2) and a stamped,

addressed diskette mailer.  A contribution to support further

development and distribution of the program as described below is

requested but not required. The contribution is not a purchase

price, and the author therefore will not accept "orders" for

Super Morse--a diskette and mailer must be provided by the user

in order to receive a copy.

The author does not notify the users of updates.  The users

should check Compuserve "Hamnet" and other bulletin boards

periodically for new versions.  If you wish, you can send the

author a diskette and mailer as described above to be held until

a new version is released.

2. Suggested Contributions

--------------------------

a. General

----------

In order to cover expenses associated with the development

and distribution of Super Morse, such as equipment, software,

telecommunications charges, postage, supplies, and long distance

Page 34

calls to users,  I suggest that you make a voluntary contribution

of $20 (or whatever is affordable to you--or more if you think it

is worth it) for each copy of the program. For use by ham clubs

in organized classes, I suggest a contribution of $5 for each

student, unless the students make their own contributions

individually. If you have already made a contribution for a an

earlier release, you do not need to make an additional

contribution for a new release.

If you cannot afford a monetary contribution, at least

write the author a note and give me an "attaboy" or two.  The

author is a always a sucker for an ego massage.

b. International Users

-----------------------

Please send contributions denominated in U. S. Dollars, if

possible. Changing foreign currency in Cedar Rapids, IA requires

a trip to a downtown bank, and I am extremely lazy. If you cannot

obtain U.S. currency, you can send an International Money Order

through your national post office. The order will be forwarded to

the United States Postal Service which will process and mail a U.

S. money order to the author.

c. Credit Cards

---------------

Sorry, I cannot accept credit cards.

d. Payments via Compuserve

--------------------------

Super Morse contributions may also be made via Compuserve's

Shareware Registration Database.  Enter GO SWREG and choose

program no. 649.

3. Support, Inquiries and Correspondence

----------------------------------------

The author is happy to help you with problems in using Super

Morse.  However, please keep in mind that there is only one of

the author and thousands of you.  Therefore, before contacting

the author, please read this document and if possible consult

with a friend who is also a Super Morse user. If you are have a

question about DOS or WINDOWS, please check with your computer

vendor or a knowledgeable friend first.

The author can be contacted on Compuserve (ID No.:

71016,1355), GEnie (Address: L.MURRAH), on packet @WA0RJT or by

U.S. mail at the following address:

10 Cottage Grove Woods, S.E.

Cedar Rapids, IA 52403

Page 35

If you write the author, you must enclose a self-addressed,

stamped envelope.  The author does not normally respond without

one.

The author can also be reached by telephone at 319-365-6530,

evenings and weekends. Please do not call the author at his work

place!

C. Help Others Learn About Super Morse

--------------------------------------

If Super Morse is helpful to you, you can help others learn

about it in the following ways:

1.  Give a copy to your friends.

2.  Upload Super Morse to your local computer bulletin

board system.

3.  Leave a message on your computer bulletin board

recommending Super Morse.

4.  Tell your ham club about it, especially the person in

charge of license classes.

5.  Write a letter to a ham radio magazine recommending Super

Morse.

6.  Write a review of Super Morse for a ham radio or

computer magazine.

D. Send Me Your Good News!

--------------------------

If Super Morse helps you obtain your license or upgrade to a

higher class of license, I would love to hear about it!  Please

take the time to drop me a note and let me hear your good news!

Page 36

VI. Troubleshooting

------------------

If you have difficulty in using Super Morse, you should

begin by reading the instructions!  Most problems are due to a

misunderstanding of how Super Morse operates.  However, if you

continue to have a problem, the following points may be helpful.

Problem or message       Solution

------------------       --------

Code seems too fast      Not a problem. See discussion of

for speed                code speed.

'File Not Found'         -All files with SM and HLP extension

should be in same directory as SM.EXE.

-Check file list to see if all are

there.

-Program disk must be in the current.

drive.

-Make sure SM.CFG is present and

not shown with 0 bytes. If so, erase it.

-If files dearchived onto a floppy, copy

distribution file to a hard disk and

try again. The floppy may have

overflowed.

'I/O Error 02'           Ditto.

'I/O Error 100'          Erase SM.CFG file. If that does not

work, erase the SMn.USR files. The

current version of one of these files is

probably longer than the previous

version.

'I/O Error 101'          Disk is full.  Usually happens when

trying to run from a floppy.  Run on a

hard disk, or remove all unnecessary

files, such as QUICK.DOC, from the

floppy.

'I/O Error 150'          Remove write protect tab from diskette!

Super Morse must be able to write to

diskette to create configuration and

user files.

Strange Values on        Erase SM.CFG and/or SMn.USR files.

Status Line and Other    and start over.

Weird Happenings

'Disk Read Error'        Erase SM.CFG and/or SMn.USR and start

over.

Page 37

Erratic/Choppy Code      Switch to Loop timing method.  If in

Windows, go to DOS and run it (Fully

exit Windows--do not use MSDOS prompt!

Some new, faster machines have problems

running Super Morse for unknown reasons

but probably due to unusual designs to

increase speed.  We are monitoring the

situation and will fix it if possible.

Code Speed Changes       If loop timing method is used, speed

will be affected each time you add or

remove a memory resident program. You

must recalibrate each time.

Mouse Does Not Work      Reboot computer

Low Speaker Volume       Inherent PC problem.  Install a Sound

Blaster or other sound card.

Low Sound Card Volume    Use the utility program that comes with

your sound card to increase volume.  SM

does not control sound card volume.

If all else fails        Reboot or reset computer.  If that

fails, turn it off and then back on

(after the hard drive stops spinning!).