| To: | Do the following |
| --- | --- |
| Turn the radio on | Press the POWER key |
| Adjust the volume | Repeated press the UP or DOWN keys |
| Adjust the squelch | While holding the FUNCTION button, repeatedly press either the UP or DOWN keys. |
| Set the frequency | - If M or C is displayed, press the A key. - Using the keypad, enter all 6 digits of the desired frequency |
| Change to Simplex | If a + or – is displayed, hold the FUNCTION button and press the 2 key until they disappear. |
| Set offset for repeater (duplex) operation | - Hold the FUNCTION button and press the 2 key until either a + (for positive offset) or a – (for negative offset appears. Release the FUNCTION button. - The offset frequency 0.60 should be displayed. If it is not, or a different frequency is desired, rotate the dial until the desired offset frequency is displayed. - Briefly press PUSH TO TALK. |
| Set tone encode (PL) | - Press and hold the FUNCTION button, then press the 4 key until a T is displayed. Release the FUNCTION key. - Rotate the dial until the desired tone frequency is displayed. - Briefly press PUSH TO TALK. |
| Change power | Press and hold the FUNCTION button, then press the LAMP H/L key. An L is displayed when the radio is set to low power. |

| To: | Do the following |
| --- | --- |
| Recall a pre-programmed frequency | - If an M is not displayed, press the A key. - Rotate the dial until the desired memory is displayed. |
| Store settings in memory | - If a M is not displayed, press the A key. - Rotate the dial to select the desired memory. - Press the A key to enter VFO mode. - Set up all radio parameters: frequency, offset, tone, etc. - Press and hold the FUNCTION button, then press the A key. Release the FUNCTION button. |

Standard CERT Channels

The following frequencies should be stored in the memory of CERT radios

| CHNL | Frequency (MHz) and offset | PL Tone (Hz) | Remarks |
| --- | --- | --- | --- |
| 1 | 147.015 + | 103.5 | Resource Repeater |
| 2 | 147.015 |  | Repeater output frequency. Use when repeater is fails. |
| 3 | 146.940 - | 123.0 | Tactical Repeater. (Primary channel for CERT out-of-district communications). |
| 4 | 146.940 |  | Repeater output frequency. Use when repeater is fails. |
| 5 | 145.700 |  | ARES simplex frequency. |
| 6 | 145.030 |  | Packet |
| 7 | 146.505 |  | CERT Simplex #1 (Use for in-district communications.) |
| 8 | 147.450 |  | CERT Simplex #2 (Use for in-district communications.) |
| 9 | 146.520 |  | National Simplex Calling Frequency |
| 10 | 162.550 |  | Weather, Emergency Alerts |