DXCC with its accompanying DAT file is a program orientated

toward the Ham Radio Operator.  Its prime function is to give a

Bearing from one location to another so that one knows the proper

direction to aim a directive antenna.

The DAT file is written in ASCII (plain text) and can be modified

with any Text Editor or Word Processor which can save in the

ASCII format.  Simply follow the conventions of either spaces or

commas as shown in the present file.  While discussing the DAT

file it should be noted that a limit of 350 countries and/or

prefixes are allowed.  It is no longer necessary to maintain a

specific case.  The program now accepts both Upper and Lower

Case for entries.

Upon Initial start-up, be sure and begin with the Menu Item which

calls for Initialization.  This set up provides for input for

your location as well as printer codes and may be changed as

desired.

The first item on the Main Menu will take you to a secondary Menu

from which you may search by Specific Prefix (it must match

EXACTLY the listing in the Data file).  When found, the

Prefix/Country Name, Coordinates and Bearing will be indicated.

If you are not sure of the "listed" prefix,then try using the

Menu Item for "Partial" prefix.  This too is discriminatory, if

you think the prefix begins with UA then you may enter U or UA.

If you were looking for England, whose prefix is "G" entering a G

should find items such as GJ (Jersey Isles), GC (Channel Isles)

but NOT G for England, it would have appeared under a Specific

Search.  If all fails then try a Partial Country Name, in the

case of England, use, for example, ENGL and let the program

search. Upon finding the listing with one of these two searches

you may want to enter the EXACT listing under Search by Specific

Prefix and find the Bearing.

Returning to the Main Menu you will see two items which DO

require a Printer.  They are Print Country List and Print Dat

File.  I've found that a list of Countries with Bearings given

FROM a foreign Ham's location is always received with a nice

Thank you note.  Of course you only need to run this list from

your location once, or perhaps after a spill of coffee.  The Data

list is simply a formatted listing of the Data file.

Bearings between two locations is probably the most useful part

of the program.  You may find Bearing and Distance between any

two selected locations (First Option is from your initialized

locale).  All that is needed for this are the coordinates in

Degrees and Minutes.  One caveat, if you want Sunrise and Sunset

for this target, be sure and answer "No" when asked if hard copy

is desired (you may always print the screen).

Run Sunrise / Sunset listing is really a rather boring process,

it was put into the program simply to make additional use of the

Sunrise/Sunset math procedures.  You may enjoy viewing it once,

its much easier to pick up this information by running Bearings

between two locations.  If you want your figures, then run from

any imaginary locale with imaginary figures TO your actual

Latitude and Longitude.  This will provide local figures.  There

are a few spots which, during the year, have no Sunrise or

Sunset.  These will become self evident.

The math is quite accurate, certainly good enough for the

purpose; But, the computer can not divide by zero.  If you enter

two figures which would fall into this range, you should get an

error message prompting you to change one of the numerals

slightly.  This detail has been circumvented within the Printing

program by the addition of a tiny mathematical factor...In other

words, a number such as .0001 is added or subtracted in order to

keep the printout running.  It was quite frustrating to have the

printer quit after four pages due to a "divide by zero" error.

The change shouldn't affect the bearing appreciably.

Please remember to begin by choosing Initialization, using

Degrees and Minutes.  If your searches consistantly turn up

blank, then be sure to switch to Upper Case.

I hope the program runs satisfactorily for you and enjoy....

NF4A