Antenna Plotting Program

PLOT.EXE will plot far-field antenna radiation patterns directly on

your graphics screen.  The patterns may be generated by the MN general-purpose

antenna analysis program or by the special-purpose YO Yagi Optimizer program.

Azimuth and elevation patterns are plotted, using either polar or rectangular

coordinates.  PLOT will work with a Color Graphics Adapter, Enhanced Graphics

Adapter, or Hercules Graphics Card.  You can execute PLOT by itself to view

plots made previously, or you can use the P command while running MN, or the H

command while running YO.  These commands automatically generate a plot file

for the antenna under analysis, call PLOT to display the plot, and then return

to MN or YO when you are done.

If you are using an EGA or HGC, a second plot file may be selected for

pattern comparison.  The new plot is drawn into the second graphics buffer, and

the screens may be switched back and forth instantly with a single key stroke,

without having to wait for the plots to be redrawn.  This permits a precise

instantaneous pattern comparison, revealing effects of small changes in an

antenna design that would not be noticed in a side by side comparison of

printed plots.  Either of the antenna plots may be replaced by a third, and so

on, allowing multiway comparisons two at a time.  This feature is not available

for the CGA because it lacks a second graphics buffer.

Running PLOT

To run PLOT do the following, according to your display type:

HGC:	The HERCULES.COM graphics driver must be loaded before

plotting.  To do this simply execute HERCULES.  This needs

to be done only once per boot, so HERCULES may be put in

your AUTOEXEC.BAT file for convenience.

CGA:	Load GRAFTABL and GRAPHICS once per boot (or put them in

your AUTOEXEC.BAT file).  These are standard DOS programs.

They will improve the CGA graphics characters and allow

you to transfer the CGA plots to your printer.

EGA:	The EGA requires no preliminaries.

When executed, PLOT automatically determines which type of graphics

display card is present, and then configures itself accordingly.  The PLOT

features vary according to the display type.

You may give the name of the plot file on the command line when

invoking PLOT, otherwise you will be prompted for it.  You need not type the

.PLT extension, it will be provided automatically.

The following keys are active in various modes during the plot:

<Enter>	Switch between azimuth and elevation plots

P		Switch between polar and rectangular

coordinates

L		Switch the polar plot scale between log dB

and linear dB

Y		Change the rectangular plot Y-axis gain cutoff

N		Normalize the plot of the lower-gain antenna

to the gain scale of the higher-gain antenna;

pressing again reverts to normal

A		Select another antenna and turn off plot

compare mode

C		Select another antenna and use plot compare

mode (not for CGA)

<Space> 	Switch between two comparison plots

(not for CGA)

1-6		Change display colors (EGA only)

S		Save display colors (EGA only)

<PrtSc>	Print screen (Press <Shift><PrtSc> or

<Shift><F10>)

<Esc>		Exit the plot program

F1		Activate the help panel, an abbreviated

version of this list

The help panel only lists keys which are active at the time the F1 key

is pressed.  For example, the Y key is only listed when in rectangular mode.

Similarly the N, S, C, <Space>, and 1-6 keys are never listed for the CGA,

since the CGA hardware cannot handle these functions.

The A command will turn off compare mode, enabling you to examine the

patterns of single antennas more rapidly.  Also note that the C command can be

used repeatedly to compare multiple antennas two at a time.

Whenever you are prompted for a plot filename you may press <Enter> to

obtain a listing of the available plot files.

Polar Plots

The azimuth polar plot is displayed with the MN +X direction toward the

top of the screen, and the +Y direction to the left.  Zero azimuth angle is at

the top of the screen, 90 degrees is to the left, 180 is at the bottom, and 270

is to the right.  Elevation plots have the +Z direction (overhead) at the top

of the screen, and the +X axis to the right.  Unidirectional antennas, if aimed

in the +X direction, will have their major lobe pointing upward on the azimuth

plot, and toward the right on the elevation plot.  This arrangement allows you

to distinguish azimuth plots from elevation plots at a glance.

You may select either of two radial scales for a polar plot. The

standard ARRL log dB scale causes lower level sidelobes to be compressed toward

the center of the pattern, emphasizing the major lobe shape.  Almost all

antenna patterns in amateur radio publications use this scale, so it provides a

way to compare the antennas you develop using MN to existing designs.  It also

produces patterns having familiar shapes.  The center of the plot is minus

infinity dB (no signal) with this scale, but there is little plot area below

-40 dB.  The other polar scale is a linear dB scale, with -50 dB at the center

of the plot, and much more area for signals between -20 and -50 dB than the log

dB scale.  This scale is useful for examining small sidelobes which are hard to

see in a log dB plot.

The dots forming the radial lines (the ones which are not multiples of

30 degrees) are spaced every 2 dB.  The dots forming the circles are spaced at

multiples of 1 degree.  These calibrations allow you to read directivity values

off the antenna pattern with good accuracy.  Using them you may never need to

use the MN D command to generate tabulated pattern data in a RUN file.

If the antenna is in free-space PLOT will draw a 360 degree elevation

polar plot.  If it is over ground only the upper 180 degree hemisphere is

shown.

To save time, MN normally generates data only for 0 to 180 degrees in

azimuth, and 0 to 180 degrees (the overhead half plane) in elevation.  YO

always generates such data.  In this case PLOT assumes symmetry around the X

axis, and draws the right half of the azimuth plot from the data for the left

half.  Similarly, for free-space antennas the lower half of the elevation plot

simply duplicates the pattern of the upper half.  However, for asymmetrical

antennas you can force MN to generate data for the entire 360 degree plane by

using "360" as a parameter when entering the G command to MN.  Both azimuth and

elevation data will be generated for 360 degrees (elevation data will cover 180

degrees if the antenna is not in free-space).

The polar plots will be perfectly circular on a monitor adjusted for

normal aspect ratio.  If they look elliptical you should adjust your vertical

height control.  This control is located on the rear panel of most monitors.

Rectangular Plots

Rectangular or X-Y plots may be selected with the P command.  This

coordinate system reveals small sidelobe detail even better than the linear dB

polar plot, but the overall shape of the pattern is lost.  The X-axis is

azimuth or elevation angle from 0 to 180 or 360 degrees, and the Y-axis is the

antenna response in dB using a linear scale.  The Y-axis cutoff, or lower limit

in dB, may be changed using the Y command.  Because this parameter is always a

negative number, you may enter it without the minus sign for convenience.  The

Y-axis cutoff is constrained between -1 and -100 dB.  The Y-axis is easiest to

interpret when the cutoff value is a multiple of 10 degrees, but you may enter

any value and the scale and grid will be drawn correctly.  The horizontal grid

lines have dots every 2 or 4 degrees.  The vertical grid lines have dots every

.02, .1, .25, .5, 1, or 2 dB depending on the Y-axis cutoff value.  The X-axis

is 180 degrees wide unless the plot file contains independent data for the

whole 360 degree plane.

Plotting While in MN

MN generates a plot file whenever you enter the P command, and then

calls PLOT.  The root name of the plot file will be the same as that of the

current antenna file, with the extension .PLT.  When experimenting with an

antenna you may wish to save more than one plot file, either for later

examination or for immediate comparison using the plot compare feature.  To

generate a plot file with a new name simply append the name as a parameter to

the P command.  The .PLT extension need not be typed as MN will add it

automatically.

If you are only interested in viewing the azimuth response you need not

wait for the elevation data generation to complete after entering the P

command.  Simply press <Esc> to abort the elevation search, and the azimuth

plot will appear.  The plot file will only contain azimuth data, but will

otherwise plot normally.  You may later enter P, wait for the elevation search

to complete, and MN will add the elevation data to the existing plot file.

Adding the parameter "360" to the G command will cause the azimuth and

elevation data to be generated for the entire 360 degree plane.  This is useful

for plotting the response of asymmetrical antennas, such as wire antennas

erected to take advantage of randomly positioned tall trees.  A subsequent P

command, after a "G 360", will generate the plot file, and call PLOT.

Some highly-directive antennas will plot better if data is generated

every 1 degree instead of the usual 2 degrees.  To cause this use the parameter

"1" with the G command.  This parameter may be used at the same time as the

"360" parameter.  Parameter order and spacing are not important as MN just

looks for a "1" and a "360" somewhere on the G command line.  These parameters

are not available with the P command to avoid confusion with the P parameter

that renames plot files.  To plot with the "360" and "1" parameters, do G first

with the parameters, then P.

Plot data is generated with a dynamic range of 100 dB and with a

resolution of .01 dB.  The elevation angle is annotated on azimuth plots only

if it is nonzero, and the same holds for the azimuth angle on elevation plots.

Plotting While in YO

YO generates a plot file whenever the H key is pressed, and then calls

PLOT to display the pattern.  YO generates free-space plots with 1 degree

resolution for Yagis having 15 or more elements, and plots with 2 degree

resolution for smaller antennas.

DOS Environment Variables for PLOT

You may tell PLOT to display gains in dBi rather than dBd by using the

following DOS command:

SET DB=dBi

Any combination of upper and lower case letters can be used.

To specify a directory to be used as a plot library do the following:

SET PLTLIB=Directory

PLOT will list the plot files in this directory and those in the

current directory when started without a filename on the command line.  You may

load a plot file from the plot library by simply entering the filename, the

directory need not be given.  This also applies if you specify the plot

filename as a command line parameter when invoking PLOT.  For more information

on SET commands see MN.DOC or YO.DOC.

EGA Colors

Keys 1 through 6 will change the screen colors while you are viewing

the EGA display.  After selecting the colors you prefer, you may save the

changes by pressing the S key.  The color codes will be saved in the file

COLORS.EGA.  When you next run PLOT the saved color set will be read back in.

If COLORS.EGA is not found PLOT will use bright white on a black background for

everything.

The color keys are:

1	Background color

2	Antenna pattern color

3	Grid color

4	Grid label color

5	Annotation color

6	Help panel color

Making Hardcopies of Plot Screens

You may press <Shift><PrtSc> to print the current screen.  On some

keyboards PLOT may not recognize the key marked PrtSc.  If this is the case you

may use <Shift><F10> instead.  It is normal for the polar plots to come out

somewhat elliptical on paper.  For the EGA and HGC, if you start the first plot

at the top of the page, a second plot may be put on the same page without any

manual realignment of the printhead.  Simply call up the second plot and press

<Shift><PrtSc> again.  This is a nice way to save both the azimuth and

elevation plots for a single antenna, or to compare two antennas.  PLOT does

not respond to <Shift><PrtSc> or <Shift><F10> with the CGA, or perform any

printing function.  Instead, DOS will intercept <Shift><PrtSc> and print your

plot if GRAPHICS was loaded before starting PLOT.