SMITTY.DOC -- Page 1

SMITTY Documentation -- Transmission-Line Calculations

By Jerry Hall, K1TD     Rev. 3.01, February 1988

SMITTY is a unique program for doing transmission line

calculations, as you would do them on a Smith Chart.  But no knowledge

of a Smith Chart is required to use the program.  SMITTY lets you

calculate input or terminating impedances, as most other Smith Chart

programs do.  In addition, SMITTY lets you calculate line Zo,

electrical length, attenuation and velocity factor from measurements

with open- and short-circuit terminations.  SMITTY also allows you to

design stub matches and series section matches, make admittance-

impedance conversions, series-parallel conversions, and SWR

calculations.

SMITTY.COM is written in Turbo Pascal and compiled for the IBM PC

and compatibles.  It uses no graphics, nor is a math coprocessor

required.  Three files make up the complete program, SMITTY.COM (the

program file), SMITTY.DOC (this file), and SMITTY.DEF.  SMITTY.DEF

contains values defining the number of decimal digits SMITTY.COM

displays during calculations.  More about this file later.

SMITTY.COM and the accompanying files are offered as copyrighted

software, but permission is hereby granted to copy the program and pass

it along to others at no personal gain.  Be sure to include all three

files.

REGISTRATION

No registration fee for SMITTY is requested or expected.  I wrote

this program to satisfy my own needs, but I am pleased to share it with

any other user who finds it to be of value.

If you find the program useful, I do request that you register

your copy.  Just send me your name and mailing address, and mention

what version of SMITTY you are using.  Registered users will be advised

of any later program releases.

GENERAL INFORMATION

There are no restrictions in SMITTY for data that is practical.

For example, you may use 0 + j0 for an impedance value (a short

circuit) or 1/4-wavelength for a line length, etc.  (HINT: Should you

wish to specify an open-circuit termination, use 1E+09 + j0.)  However,

impractical data such as zero for a frequency or for a line impedance

may create divide by zero errors, causing the program to abort.

SMITTY is menu driven.  All menu options are selected merely by

pressing a letter key; upper or lower case is accepted.  The letter-key

selections are itemized in the menu, as well as being highlighted in

the menu description of the function.  From the main menu, the screen

will clear and the opening screen for the selection you've made will be

SMITTY.DOC -- Page 2

displayed.  Some menu items cover two procedures, such as converting an

impedance to an admittance, or vice versa.  At the opening screen of

these procedures you'll be prompted for which calculation you wish to

perform.  Again, and throughout the program, responses to all prompts

other than for data entry are made by pressing a single key.

During program execution you'll be queried with prompts for

various program options.  One example is taking line attenuation into

account, with a Y or N response expected.  Default options are shown

for each of these prompts, an N in this case.  The default may be

selected by pressing the ENTER key (as well as by pressing the letter

key for the default).  Pressing any other keys (in this case, anything

but Y, N, or ENTER) does nothing.

In some procedures you will be asked to enter reactance or

susceptance values at the keyboard.  Be sure to enter negative values

for capacitive reactance and for inductive susceptance.

You may exit the program gracefully from the main menu by pressing

Q (quit).  You may also abort the program at any time by pressing Ctrl-

C.

SMITTY FEATURES

Probably the most common use of the Smith Chart is to determine

the load impedance on a transmission line based on a known input

impedance, and vice versa.  SMITTY does indeed support such

calculations.  These may be made with actual R + jX values or with

normalized values.  A similar calculation is to determine the load

admittance, G + jB, based on a known input admittance, or vice versa.

SMITTY supports these calculations as well.  The choice to work with

impedance or admittance data is made after you select "toward load" or

"toward generator" at the main menu.  Once you've made that choice, say

for impedance, you will continue to work with impedance values until

you return to the main menu.

Frequency Correction

During the execution of some procedures, Smitty requires reactance

data in ohms.  The data to be entered at the keyboard may be the result

of impedance bridge measurements.  Many bridges, such as a General

Radio 1606-A, give a reactance dial reading that must be divided by the

measurement frequency in megahertz to get the ohmic reactance value. In

these procedures you may enter the dial values from such a bridge, and

allow the program to make the frequency correction. A message similar

to the beginning lines of this paragraph will first be displayed, and

after the data is entered you'll be prompted about correcting

reactances for frequency.  Unless you enter bridge dial readings,

you'll want to choose the default response, N(o).

SMITTY.DOC -- Page 3

Line Length and Attenuation

In SMITTY there are several options for entering line lengths.

You may enter electrical lengths directly as either degrees, radians,

or wavelengths.  Or you may enter a physical line length in feet and

SMITTY will calculate and display the electrical length.  In this case,

additional data SMITTY requires for the calculation are the velocity

factor of the line and the frequency.  Velocity factors are always

entered as a decimal fraction, and frequencies in megahertz.  (These

are clearly indicated at the data prompts.)

Various options also exist for taking line attenuation into

account during calculations.  You may indicate a lossless line (no

attenuation), or you may enter attenuation either in decibels or

nepers.

Other Capabilities of SMITTY

SMITTY lets you calculate line Zo, electrical length, attenuation

and velocity factor from measurements with open- and short-circuit

terminations.  The basis for this procedure is contained in Chapter 28

of The ARRL Antenna Book, 15th Edition, 1988.  If you are not familiar

with this technique and want to try the program feature, use these data

values.  These values are the results of actual measurements on a

length of foam-dielectric cable.

Open-circuit readings:  85 + j1280/freq

Short-circuit readings:  4.8 - j80/freq

Frequency: 7.14 MHz

Line length: 100 feet

SMITTY allows you to design stub matches.  You may specify a line

impedance for the stub that is different from the main transmission

line.  Note that for some loads, such as 40 +/- j20 with a 50-ohm line,

one of the two possible solutions occurs with the stub placed 0.0

wavelength from the load.  This is not a program error, but is a valid

solution meaning to shunt the load itself with a matching stub.  Such a

situation exists any time the normalized conductance of the load is 1.0

siemen.  Determining stub positions is an iterative procedure, and

calculations for some loads take more time than for others . . .

typically one second or less for most loads.  Calculations taking the

longest, up to a few seconds, are for cases when the unmatched load

creates a high SWR on the main line.

You may also design series section matches with SMITTY.  Details

for this match are contained in the chapter on matching the line to the

load in any recent edition of The ARRL Antenna Book.  This form of

matching is not widely used, but is more efficient than a stub match.

A 1/4-wavelength transformer is a special case of the series section

match.  Just as the 1/4-wavelength section of line having an

intermediate impedance can match two unlike impedances, a series

section of the proper length and almost any impedance, at the proper

SMITTY.DOC -- Page 4

position in the main line, can match two unlike impedances.  The

advantage of the series section is that it may be a standard type of

line, and the load may contain reactance as well as resistance.

As presently written, no provisions are made in SMITTY for taking

line losses into account for stub matching or for series section

matching.  I have investigated this and find it does make a slight

difference in the outcome of calculations when line attenuation is

considered.  A later version of the program may include this

capability.

With SMITTY, admittances may be calculated from impedances, and

vice versa.  Similarly, series equivalent impedances may be converted

to parallel equivalents, and vice versa.  These are direct conversion

processes, with no data required for line length, attenuation, etc.  If

the data you enter is normalized, the results will be normalized, and

if the data you enter is unnormalized, the results will be

unnormalized.  (Never mind that these procedures may ask if the data is

normalized; your response is used only to determine how many decimal

places to display.)

SMITTY also lets you calculate standing wave ratios or SWR values.

This is a simple procedure that can be done on a 4-function calculator.

But I find that doing it quickly in the computer often beats digging

out the equation (which I can never seem to remember), and then doing a

couple of dozen keystrokes on a calculator.

SMITTY.DEF

SMITTY.DEF is an ASCII text file containing values that define how

many decimal places SMITTY displays during program calculations.  You

can change these values to meet your requirements (any value in the

range from 0 to 9) with EDLIN.COM or your favorite word processor.  The

results are rounded (not truncated) to the number of digits displayed.

Just be sure to treat SMITTY.DEF as an ASCII text file, and do not

change the order in which the values appear.  The values included in

SMITTY.DEF as originally distributed are as follows.

Resistance and reactance -- 3

Resistance and reactance, normalized -- 5

Conductance and susceptance -- 6

Degrees -- 3

Wavelengths -- 5

Decibels (attenuation) -- 2

Standing wave ratio (SWR) -- 3

Feet (line lengths) -- 2

Velocity factor -- 2

Voltage reflection coefficient -- 4

SMITTY.DOC -- Page 5

PROGRAM ERRORS

Although I think the program is completely debugged, it is

possible that undetected errors exist.  Please let me know if you

experience a problem.  But do not confuse program errors with data

input errors.  As mentioned earlier, impractical data will create

divide by zero errors.  The program will abort and a message such as

this one will be displayed:

Run-time error 02, PC=50E7

The 02 run-time error code signifies a divide by zero error.

If you run into an actual program error causing it to abort,

please immediately jot down from the screen display the exact entries

you made for data and the complete error code line.  (Or better yet,

immediately do a print-screen operation to a printer.)  Do this before

you clear the screen or start SMITTY again.  Send me this information,

and I'll get back to you.

During initial development of the program I carefully checked the

results of all procedures against those from manual Smith Chart

calculations, or against manual calculations if the Smith Chart was not

required.  I believe you will not encounter calculation errors.

I hope you find SMITTY to be a useful tool for solving your

transmission-line problems.  I'd appreciate comments and suggestions

for program improvements.  With many thanks,

Jerry Hall, K1TD, 181 Brimfield Rd., Wethersfield, CT 06109

February 14, 1988

################################################################################################