IONSOUND HDX (TM) by W1FM

VERSION 2.20

SKYWAVE PROPAGATION PREDICTION SOFTWARE

FOR AMATEUR, PROFESSIONAL, AND MILITARY APPLICATIONS

USER'S MANUAL

COPYRIGHT 1994 BY JACOB HANDWERKER / W1FM

SKYWAVE TECHNOLOGIES

ALL RIGHTS RESERVED

The author has made every effort to ensure that this program is

correct and accurate. However, no expressed or implied warranty or

guarantee of any kind with respect to its accuracy or effectiveness is

made. The author will therefore not be liable for incidental or otherwise

consequential damages, either direct or indirect, in connection with

furnishing of, or the performance of, or as a result of the use of this

program. The author does not warrant that the functions of the software

will meet your needs or that it will operate error-free and uninterrupted.

IONSOUND HDX is a trademark of Jacob Handwerker / W1FM

SkyWave Technologies

***** Acknowledgments *****

The outstanding propagation work of R. Fricker, BBC External Services,

U.K., and D. Van Troyen, A. Van de Capelle, and A. Deknuydt, in Belgium is

hereby acknowledged.

Grateful appreciation is expressed to R. Dean Straw / N6BV, ARRL

Senior Assistant Technical Editor, for his professionalism in his

assistance with this manual, and for his many helpful suggestions resulting

from the review of the IONSOUND HDX software program.

INTRODUCTION

1. Ionospheric Propagation Background

Radio waves can be classified according to various types of

propagation. These propagation types are ionospheric, tropospheric, or

ground waves. Ionospheric, also known as skywave, propagation provides the

major portion of the overall radiation that leaves an antenna at some

elevation angle above the horizontal plane. Much of the short and

long-distance communications below 30 MHz depends on the bending or

refraction of the transmitted wave in the earth's ionosphere which are

regions of ionization caused by the sun's ultraviolet radiation and lying

about 60 to 200 miles above the earth's surface.

The useful regions of ionization are the E layer (at about 70 miles in

height for maximum ionization) and the F layer (lying at about 175 miles in

height at night). During the daylight hours, the F layer splits into two

distinguishable parts: F1 (lying at a height of about 140 miles) and F2

(lying at a height of about 200 miles). After sunset the F1 and F2 layers

recombine again into a single F layer. During daylight, a lower layer of

ionization known as the D layer exists in proportion to the sun's height,

peaking at local noon and largely dissipating after sunset. This lower

layer primarily acts to absorb energy in the low end of the High Frequency

(HF) band. The F layer ionization regions are primarily responsible for

long distance communications, sometimes in conjunction with the E layer in

a variety of mixed propagation modes.

Vertical incidence ionospheric sounding devices are used to determine

the virtual height of an ionospheric layer at various frequencies by

beaming energy upward and measuring the time delay required for the round

trip. The critical frequency for a vertical incidence sounder is the

maximum frequency above which no energy is returned to earth for a given

layer. An ionogram is a graphic representation of such sounding and usually

depicts the height of the layer (or the time delay) as a function of the

sounding frequency, along with the intensity of the return signal. An

oblique sounding device may require the cooperation of a corresponding

receiving device at a distant point in order to depict received energy

which has been transmitted at incidence angles less than 90 degrees in

elevation; it may also make use of backscatter techniques to assess the

propagation path. Devices such as these can then be used to assess (in real

time) the propagation path frequencies  which can be supported, up to and

including the Maximum Usable Frequency (MUF).

As an adjunct to this Users Manual, it is recommended that other

sources of information concerning HF propagation prediction and related

antenna theory be consulted since this operating manual is not meant to be

a comprehensive tutorial on the theoretical aspects of these subjects. A

bibliography of several of these source materials is shown at the end of

this manual.

2. IONSOUND HDX Overview

IONSOUND HDX Version 2.20 is a very sophisticated ionospheric

propagation prediction program for frequencies between 1.8 MHz and 30 MHz.

IONSOUND HDX is a member of the IONSOUND family of programs which have been

evolving for a number of years. Geographical regions corresponding to those

shown in ARRL's QST magazine "How's DX?" column can primarily be chosen

from the TX and RX location menus along with several others not found in

QST.

IONSOUND HDX has been designed with user friendliness in mind and is

entirely menu-driven, with prompting for various user inputs to the

program. It should be emphasized that a comprehensive understanding of

propagation phenomena and the technical terms associated with the

scientific forecasting of propagation is helpful, but not necessary, to

become skilled in the use of IONSOUND HDX. The goal of the program is to

produce an easy-to-interpret tabular prediction of radio frequency (RF)

link performance between two locations on the earth's surface.

Technical jargon and output detail has been minimized to essential

elements in the interest of simplicity, without a sacrifice in overall

performance of the program or its presentation display capabilities. To

simplify matters, default inputs have been provided. An explanation of the

use of these menus and screens will be provided in this manual, but the

program should be largely self explanatory. Once the operator has

customized IONSOUND HDX to suit his/her particular needs, the information

is saved to disk as a set of defaults. When the program is started, the

operator need only hit the <Enter> key several times to accept the custom

defaults and then make a propagation prediction.

3. General Requirements

IONSOUND HDX is designed for use with IBM or IBM-compatible personal

computers. The program operates with or without an 8087, 80287, or 80387

math coprocessor. It will automatically take advantage of the coprocessor

if it finds it. However, if at all possible, a coprocessor should be

utilized, due to the mathematically intensive nature of the calculations

performed in the propagation prediction process. Processing times can

become lengthy without a coprocessor; in fact, a coprocessor will usually

speed up operation by a factor of 15 or even more. Note that the 80486DX

and the Pentium processors have the coprocessor built-in, while 80486SX

versions do not. If you intend to do antenna modeling and propagation

predictions, an investment in a numeric coprocessor is worthwhile.

A personal computer with 640 kilobytes of RAM is desirable, along with

DOS version 2.11 or greater. For hard copy printout, a printer supporting

IBM Graphics is recommended.

4. Printing IONSOUND HDX Operator Manual

You may print out this Operators Manual. First, make sure your printer

is on-line, then type the following:

TYPE ION_HDX.DOC > PRN   <Enter>  or  PRINTDOC   <Enter>

5. Starting IONSOUND HDX

To start IONSOUND HDX type the following:

ION_HDX   <Enter>

For convenience use the batch file:

ION    <Enter>

Following the start-up of IONSOUND HDX, the program will prompt the

user, in a step-by-step fashion, with several screens prompting user

responses.

6. General, Menus and Screens

All entries, such as for YES/NO (Y/N) selections, can be made in

either lower case or upper case. Default conditions for most of the menus

and screens are shown by a notation such as:  <DEFAULT= #> or <Y> or <N> or

<C/Q>.

Default= # is the option number which will result if the enter key is

pressed instead of actually inputting a number value. Likewise, Y or N

defaults indicate YES or NO, respectively.

When a <C/Q> option is encountered, the default is C (continue);

typing Q indicates "Quit" back to the Main Menu.

7. Display Color Selection

There are eight possible color combinations for the display text and

background. The program comes up in black and white unless you choose

another combination. Caution: a background color other than black will

cause a black/white monitor to be unreadable!

8. Transmit and Receive Location

The selection menu for transmit (TX) and receive (RX) locations each

consist of 21 choices. Choices 1-14 allow selection of predefined locations

corresponding to those shown in QST magazine's "How's DX?" column,

published monthly by the American Radio Relay League (ARRL). Choices 15

through 21 are for additional predefined locations not covered in "How's

DX?"

These selections make it easy to compute IONSOUND HDX predictions for

comparison with the Highest Possible Frequency (HPF), Maximum Usable

Frequency (MUF), and the Frequency of Optimum Transmission (FOT)

predictions derived from U.S. Department of Commerce, National

Telecommunications and Information Administration (NTIA) IONCAP program as

found in QST.

[Note: Although the "How's DX?" list is limited, it can be

successfully used to predict propagation performance between many other

locations which are near those shown in Table 1.]

Table 1

Expanded List of QST "How's DX?" TX/RX Locations

Choice  Location          Latitude  Longitude  Nearest City

1    Alaska              61.00     150.00    Anchorage

2    Australia          -33.87    -151.22    Sydney

3    Central Asia        28.50     -77.50    New Delhi, India

4    U.S. East Coast     39.00      77.00    Washington, DC

5    Eastern Europe      50.50     -30.50    Kiev, Ukraine

6    Hawaii              21.33     157.80    Honolulu

7    Japan               35.75    -139.80    Tokyo

8    U.S. Midwest        39.00      95.00    Kansas City, KS

9    Caribbean           18.50      66.00    San Juan, Puerto Rico

10    South America      -25.00      57.50    Asuncion, Paraguay

11    South Pacific      -14.33     170.70    Pago Pago, Am. Samoa

12    Southern Africa    -15.50     -28.00    Lusaka, Zambia

13    U.S. West Coast     38.00     122.00    San Francisco, CA

14    Western Europe      51.50       0.20    London, England

15    Central America     15.00      90.00    Guatemala City

16    East Mediterranean  31.50     -35.00    Jerusalem, Israel

17    Indian Ocean        -6.50    -107.00    Djakarta, Indonesia

18    U.S. Northeast      42.35      71.05    Boston, MA

19    U.S. Northwest      47.50     122.50    Seattle, WA

20    U.S. Southeast      30.25      81.50    Jacksonville, FL

21    U.S. Southwest      33.50     112.00    Phoenix, AZ

Latitude and longitude values are given in decimal degree format.

Positive values of latitude (+) are north of the Equator; negative values

(-) of latitude are south of the Equator. Positive values of longitude (+)

are west of Greenwich, UK; negative values of longitude (-) are east of

Greenwich.

See QST Magazine, December 1990, Technical Correspondence, Pages

58-59, "Propagation Predictions and Personal Computers" for a discussion of

how these locations are used in conjunction with sunspot numbers and

minimum elevation angle requirements to derive IONCAP predictions for QST

Magazine's "How's DX?" column.

9. Short/Long Path Selection

Selection of Short <S> or Long <L> path gives an opportunity to choose

either the shortest or the longest great circle path from the transmitting

to the receiving location. The default for this selection is the short or S

path. IONSOUND HDX is designed to support only direct paths; skew paths

that are not on great circles are not supported. Following the selection of

either short or long path, the distance in kilometers, statute miles, and

nautical miles from the transmitter to the receiver is provided by the

program.

Also shown is the front/back (F/B) bearing in degrees (eg, 315 / 135).

The front value is the bearing (or heading) direction from the transmitter

toward the receiver. The positive value of bearing indicates the clockwise

number of degrees offset heading from True North (0 degrees) which a

radiated signal will follow on a great circle path from transmitter

location to receiver location. The back value is the direction opposite (or

180 degrees away) from the transmitter-to-receiver direction.

10. Receiver Noise

Receiver noise code can be independently selected for the transmitter

(TX) location and the receiver (RX) location. Since link predictions are

always made for the path from the transmitter to the receiver, it makes a

difference in predicted performance when the two locations are 'swapped'

and the TX receiver noise code is not the same as the RX receiver noise

code. Swapping of the TX and RX locations can easily be done from the Main

Menu. When this 'SWAP' function is exercised, the respective noise codes

are interchanged for prediction purposes, along with the latitudes,

longitudes and location descriptions. A choice of three receiver noise

codes can be inputted by the user. These choices are CITY, RESIDENTIAL, or

RURAL noise. This selection is used in determining the received

signal-to-noise ratio.

The selection of receiver noise code should be made by considering the

geographic location of the TX location receiver and the RX location

receiver in relation to city, residential or rural surroundings.  Choosing

city noise results in more noise at the receiver than residential noise.

Likewise, residential noise is less than city noise but more than rural

noise. The received noise power density, also varies as a function of

frequency at the receive end of the RF link. Lower frequencies have greater

ambient noise background levels than higher frequencies. The actual receive

noise power (expressed in Watts) depends upon the receiver bandwidth. The

default for choosing a TX or RX location receiver noise code is residential

noise.

11. Antenna/Gain Selection

The Transmit and Receive Antenna Selection Menu allows the operator to

choose the antenna for both the transmitter and receiver locations. The

selections offered for transmit/receive antennas represent typical

candidate configurations for predicting propagation performance. Each is a

mathematic model whose gain varies as a function of the elevation angle.

Please note that the overall response of each antenna selection is a

generic, theoretical response, since real-world effects for an individual

location (such as local terrain, other antennas, or nearby power lines)

cannot be included.

Table 2

TX/RX Antenna Gains vs. Elevation Angle

Takeoff Dipole  Vertical Yagi-Uda Log/Rhom Curtain  Isotropic

Angle

D        V       Y       L         C         G

(deg)   (dB)     (dB)    (dB)    (dB)      (dB)      (dB)

1     -9.37    -3.15    4.89   -4.28     11.72   -40 to +40

5     -2.40    -1.21    6.82    2.68     18.68   -40 to +40

10     0.54     1.11    9.14    5.57     21.57   -40 to +40

15     2.19     2.06   10.10    7.13     23.13   -40 to +40

20     3.28     2.16   10.19    8.11     24.11   -40 to +40

25     4.04     1.90    9.93    8.71     24.71   -40 to +40

30     4.58     1.51    9.54    9.05     25.05   -40 to +40

35     4.93     1.02    9.06    9.16     25.16   -40 to +40

40     5.13     0.44    8.48    9.07     25.07   -40 to +40

45     5.20    -0.25    7.78    8.79     24.79   -40 to +40

50     5.13    -1.08    6.95    8.31     24.31   -40 to +40

55     4.93    -2.07    5.96    7.62     23.61   -40 to +40

60     4.58    -3.26    4.77    6.66     22.66   -40 to +40

65     4.04    -4.72    3.32    5.40     21.40   -40 to +40

70     3.28    -6.50    1.53    3.72     19.72   -40 to +40

75     2.19    -8.62   -0.59    1.41     17.41   -40 to +40

80     0.54   -11.33   -3.29   -1.97     14.03   -40 to +40

85    -2.40   -16.41   -8.37   -7.91      8.09   -40 to +40

89    -9.37   -29.99  -21.96  -21.86     -5.86   -40 to +40

Notes on IONSOUND HDX Antennas:

D=Dipole          Horizontal or Vertical Dipole, approx. 3/8 wave high

V=Vertical        Vertical Monopole, ground-mounted

Y=Yagi-Uda        Yagi-Uda Array, approximately 3/4 wave high

L=Log/Rhom        Log Periodic or Rhombic Array, approx. 1/2 wave high

C=Curtain         Curtain Array, wide elevation coverage

I=Isotropic       0 dBi, constant gain at all angles

G=Isotropic Gain  -40 to +40 dBi, constant gain at all angles

The Yagi-Uda (Y) in IONSOUND HDX emulates a Yagi mounted approximately

at 3/4 wavelength above ground. It has a peak gain of +10 dBi (that is,

referenced to an isotropic radiator in free space) at 15 degrees and has

essentially no output at very high elevation angles. Most amateurs select

the IONSOUND HDX Yagi model for predictions in the HF bands above 14 MHz,

or even 7 MHz if they have a 40-meter Yagi. [Note: Many use the Yagi even

for 3.5 MHz just to see how the predictions come out for those lucky hams

who do have 80-meter Yagis!]

The Vertical Monopole (V) selection emulates the behavior of a

ground-mounted vertical antenna over real earth ground. It has a peak gain

of 2 dBi at an elevation angle of 30 degrees, with essentially no output

near 0 degrees or at very high elevation angles. The upward-tilted

elevation pattern for the vertical monopole is broad and usable for

low/medium launch-angle coverage.

The Horizontal or Vertical Dipole (D) selection emulates a dipole

mounted approximately 3/8 wavelengths over ground, with a peak gain of +5

dBi at 45 degrees elevation. The upward-tilted elevation pattern is broad

and usable for all-around elevation coverage. Many amateurs use this

IONSOUND HDX dipole selection for the lower HF bands, mostly on 1.8 and 3.5

MHz.

The variable gain Log-Periodic and Rhombic (L) selection has been

weighted to provide gain ranging from +7 dBi at 1.8 MHz to +16 dBi gain at

30 MHz.  The maximum gain is maintained at an angle of approximately 30

degrees above the horizon, again with essentially no output near 0 degrees

or at very high elevation angles. This pattern emulates a very large

multi-band horizontal Log-Periodic or a terminated Rhombic antenna. At each

frequency the height of the antenna is approximately one-half wavelength

above ground.

The variable gain Curtain (C) Array antenna selection has been

weighted to provide a variable peak gain over an isotropic radiator ranging

from approximately +23 dBi at 1.8 MHz to +28 dBi gain at 30 MHz.  The

maximum gain is maintained at an angle of approximately 30 degrees above

the horizon. Of course, most 160-meter operators have a hard time achieving

any gain at 1.8 MHz, so this curtain antenna provides an upper bound on

what is imaginable for antenna gain on all frequencies. In other words, if

the band doesn't open up for this antenna, nothing will make HF

communication possible!

Selection of 'Choose Your Own Gain' (G) provides an opportunity to

pick an Isotropic Gain antenna between -40 to +40 dBi. An isotropic

radiator is an ideal antenna that radiates uniformly in all directions. The

weighting function for this choice provides the same gain at all elevation

angles, allowing the program to pick out all possible propagation modes on

a theoretical basis, with virtually no limitations due to the use of real

antennas over real ground. Most of the time the lowest possible elevation

angles are predicted when a high-gain isotropic antenna is used, even on

the low frequencies.

The selection of any particular antenna or isotropic gain value will

cause the program to utilize this gain value for all frequencies. If a

particular antenna is suitable at some frequencies but not at others, the

program should be rerun with the correct antenna selection if more accurate

or realistic results are desired.

[Note: The user can use selection 14 from the Main Menu to show the

influence of electrical height on an antenna's major lobe and null

characteristics and the resulting single hop E and F layer distances.]

12. Receiver Bandwidth

The selection of a receiver bandwidth is used to determine the noise

power used into the calculation of signal-to-noise (S/N) ratio. This entry

must be greater than 0 Hz and should be consistent with the type of

communications activity being predicted. A typical value for single

sideband (SSB) voice communication is 3000 Hz. For Morse code (CW)

operation, a value of 500 Hz is typical. For AM, a value of 6000 Hz is

adequate. A default value of 3000 Hz is selected if the <Enter> key is hit

without a numeric value entered.

For direct comparison with IONCAP S/N predictions, a normalized 1 Hz

bandwidth can be used, since that is what IONCAP uses internally.

13. Required S/N Ratio

The selection of a required Signal-to-Noise (S/N) ratio determines the

threshold level of signal quality on which the propagation prediction is

based. Typically, 10 dB or more S/N ratio is required for minimum voice

communications capabilities in a 3 KHz (typical) bandwidth. In case of

severe interference, or fading conditions due to multipath ionospheric

effects, this value should be made higher. The required S/N ratio input by

the operator is used to determine the availability of the link (ie, %N S/N

Availability). As the required minimum S/N value is raised, the RF link is

less likely to be able to support the requirement.

Therefore, you should usually choose the absolute minimum S/N that is

needed in order to assess the %N (S/N Availability percentage) and the %T

(Total Reliability percentage) of the link. The %P (Path Availability

percentage) of the link is independent of the minimum required S/N ratio,

indicating instead that the path is open for some level of communication.

14. Transmitter Power

The selection of transmitter power represents the amount of power (in

kilowatts) delivered to the selected antenna. For example, to designate 100

watts delivered to the antenna, the entry would be made as 0.1 (i.e., 1/10

kilowatt). Transmitter power must be entered as a value greater than 0.

Increasing or decreasing the amount of power has a direct bearing on the

received S/N ratio and thus affects %N S/N Availability and %T Total Link

Reliability. Thus, a 10 dB increase in signal power results in a 10 dB

increase in received S/N ratio. The default selection value is 1 kW.

[Note: Feedline and other losses to the antenna should be considered

in the selection of transmitter power, since this value represents the

amount of power actually delivered to a matched antenna.]

15. Sunspot Number (SSN) or Solar Flux Number (SFN)

The level of solar activity influences ionospheric propagation.

IONSOUND HDX accepts either SSN (Smoothed Sunspot Number) or SFN (Solar

Flux Number) values. The program uses these values for computation of D, E,

and F layer absorption effects on transmitted signals in the ionosphere.

The SSN is based upon a statistically smoothed set of observations of

sunspots and clusters of sunspots. The SSN can be obtained from

publications such as QST (published by the ARRL) or from CQ Magazine. The

SFN is based upon a 2800 MHz measurement of solar noise and is broadcast

hourly on broadcast services such as WWV. Solar flux data is also available

on most packet clusters. If real-time indications of solar activity are

utilized, either SSN or SFN, running-averages should be kept and used as

input to IONSOUND HDX. Robust predictions may involve 5, 10, 15 or 30 day

running averages, while longer-term averages may be 6 months or longer.

Prior to actual entry of SSN or SFN, a choice is presented for

selection of using either SSN or SFN. To pick use of SSN an S should be

entered; to pick use of SFN, an F should be entered.  The default for this

selection is use of the SSN.

For SSN input, a value greater than 0 must be entered. For SFN input,

a value greater than or equal to 63.75. If SSN is entered, IONSOUND HDX

computes the equivalent SFN. Likewise, it computes and displays SSN if SFN

is used.

[Note: Sunspot data can also be obtained from the "Solar Indices

Bulletin", National Geophysical Data Center, Boulder, Colorado. See

Appendix for a discussion of National Bureau of Standards (NBS) forecasts

and prediction availability via radio broadcasts and on-line

telephone/modem services.]

[CAUTION: Following SSN/SFN entry, any manually entered changes to the

F-layer height or the E-layer height should be carefully considered since

program derived values will be overridden. In general, knowledge of

vertical height from ionospheric soundings is useful and may be used if

available.]

16. Minimum Elevation Angle

The operator may enter a minimum elevation angle. This is useful if

the horizon towards the desired target location is blocked by hills or

other obstructions. Selecting a higher minimum angle precludes unrealistic

low-order modes from being used in the computations.

Following the elevation angle selection, the program computes the

lowest-order F layer propagation mode (showing the number of hops), the

calculated takeoff angle, and the unabsorbed isotropic receiver power

density and field strength available at the distant receiver at the oblique

critical frequency for this mode. Additional elevation angles may be tried

if desired.

With each minimum elevation angle the program finds the corresponding

F layer hops, power density and field strength. Finally, after you have

decided on a minimum elevation angle (or choose 0 degrees by default), the

program will proceed.

17. Choosing Prediction Frequencies

The menu for selection of prediction frequencies presents a variety of

choices. In all cases, the entry of any frequency is a MHz value.

Selection 1 allows entry of up to nine separate frequencies in the 1.8

MHz to 30 MHz range. The prediction order will be in the same sequence as

the frequencies are entered.

Selection 2 allows entry of a range of frequencies defined by the

lowest frequency (greater than or equal to 1.8 MHz), a frequency increment

(greater than 0), and a highest frequency (less than or equal to 30 MHz). A

number must be entered for each prompt, or the program will simply cycle

back to the first prompt. If the frequency increment chosen is too small,

resulting in more than nine frequencies, the upper frequency limit will be

truncated in order to limit the total number of frequencies to nine.

If selection 2 is chosen and a previously defined range of frequencies

already exists, the program will prompt the user whether to keep this

previous range of frequencies by typing Y or N. The default for this choice

is <Y> so that the program can continue with this previously defined range

by simply hitting the <Enter> key.

Selection 3 allows a predefined subset of all 9 HF amateur band

frequencies (based on U.S.A. Allocations) currently available in the 1.8-30

MHz range. The frequencies are chosen such that there is one representative

frequency from each band. [Note: Technically the 1.80 MHz frequency lies in

the Medium Frequency (MF) band which is in the range 0.3 MHz to 3 MHz.] The

All HF Amateur Band predefined frequencies are:

1.8, 3.5, 7.0, 10.1, 14.0, 18.1, 21.0, 24.9, 28.0 MHz.

Selection 4 allows a predefined subset of 5 high-band HF amateur band

frequencies (based on U.S.A. Allocations) currently available in the 14-30

MHz range. The frequencies are chosen such that there is one representative

frequency from each band. The High-Band HF Amateur frequencies are:

14.0, 18.1, 21.0, 24.9, 28.0 MHz.

Selection 5 will automatically load prestored frequencies from the

file ION_FREQ.DAT. Up to nine frequencies, covering the range 1.8-30 MHz,

can be prestored in the file. This file can be automatically modified by

the user from within the program. It can be used to store frequency net

lists or other favorite sets of frequency information.

The default selection for the Frequency Menu is <3> which picks the 9

HF amateur band frequencies to be used for prediction purposes. The default

selection is also obtained by hitting the <Enter> key.

18. Choosing Prediction Months

The Month Selection Menu for selection of prediction months presents a

variety of choices.

If a selection entry between 1 and 12 is made, this entry will then

represent a single prediction month. For example, an entry of 3 represents

the month of March; 12 represents December.

If selection 13 is made, all 12 months in sequence starting from

January and ending with December will be used for prediction purposes.

If selection 14 is made, the program will prompt you for the total

number of months (between 1 and 12) for which you want predictions.

Following the entry of the number of months, the program then prompts you

for each month in the sequence which you care to use for prediction

purposes.

If selection 15 is made for entering an interval of months, the

program will prompt you for the starting month, an integer increment value,

and then the ending month. The program will then list the months

corresponding to this selected interval and will ask you if you wish to

change the range of months selected. If the month range interval is not

acceptable to you, type Y to change the range. If the range is acceptable,

then type N, the default, to proceed. Should the increment of months or

range be inconsistent or inappropriate, the program will ask you to

re-enter the month range.

If selection 16 is made then the user has an opportunity to change the

default month to be used in the selection process. When first executing,

the default month is set to the present month. Select a new default month

by entering a value from 1 to 12. The new default month will then be used

for all subsequent propagation predictions. The setting of the default

month does not preclude using any other month or months or month intervals

when this menu is subsequently accessed.

Selection 17 from the Month Selection Menu allows a return to the Main

Menu of the IONSOUND HDX program.

19. Choosing Prediction Times

The operator uses the Time Selection Menu to choose propagation

prediction times.

If 0 or <Enter> is selected, IONSOUND HDX computes a 24 Hour Summary

Table for presentation to the computer screen. A maximum of 8 unique

parameters may be chosen, in any order, for these predictions.

Selections from 1 to 24 compute predictions for a single point in

time. The hour and the minutes are entered in Universal Coordinated Time

(UTC), using a number between 1.00 and 24.00. The digit (or digits) to the

left of the decimal point correspond to the hour; the digits to the right

of the decimal point correspond to the minutes (i.e., 12.35 corresponds to

12 hours and 35 minutes, UTC).

Selection 25 chooses every full hour from 1 to 24 for the prediction

process.

Selection 26 allows entry of particular times of your own choosing.

The user is prompted for the number of individual times, up to a maximum of

50. Each individual time is then entered one at a time following prompts.

Selection 27 allows an interval of time values to be selected. The

starting time is entered, then the time increment (which must be greater

than 0), and finally the ending time. As a simplification, the time moment

selected for the interval should be rounded to the nearest 15 minutes. If a

very small time increment is selected such that the total number of

individual times exceeds 50, a message will appear indicating that the

total number of time moments has been truncated to 50.

Following a continuation prompt indicating hit <Enter> to continue,

the individual times in the overall time interval selected will appear on

the screen. A prompt by the program will then ask whether you wish to

change these times. If you want to change these times type Y; if these

times are acceptable, type N. The default value for changing these times is

<N> so that the program can continue by simply hitting the <Enter> key.

[Note: If selection 27 is chosen by the user and a previously defined

interval of time exists, the program will prompt whether you wish to use

the previous time interval. The default for keeping the previously defined

time interval is <Y> so that the program can continue by simply hitting the

<Enter> key.]

Selection 28 of the Time Selection Menu allows the user to return to

the Main Menu.

20. Choosing Prediction Modes

The Mode Selection Menu for choosing prediction modes presents a

variety of choices, mainly for advanced users of IONSOUND HDX. These choice

can greatly influence the propagation prediction process. At the beginning

of the Mode Selection Menu, the lowest-order predicted F layer mode is

displayed. Selecting a value of N from 1 to 10 causes the program to

automatically seek other propagating modes supported by the ionosphere (for

both the E layer and F layer) in addition to the value of the lowest order

F layer mode.

Selection of N = 1 (the default value) will cause the mode searching

algorithm to consider at least 1 hop for the minimum number of F layer

hops. Selecting N = 2 will cause mode searching to consider at least 2

hops. Likewise, further increasing the value of N selected will cause the

algorithm to search out an ever-increasing complexity of E layer and F

layer hop combinations, up to the maximum value of N = 10. As the value of

selected N is increased, the prediction time will also increase

accordingly.

[Note: The mode searching algorithm is a complex process, since the

program also considers mixed (i.e., combined E and F layer) modes of

propagation. If at any time and at any frequency the lowest calculated F

layer mode is blocked by the E layer, the program will seek mixed modes

having the same number of hops, except that an E layer hop will replace one

of the F layer hops. If this mode does not appear to propagate, another try

is then made but with one more F layer hop than the original. If this mode

in turn does not propagate, then a mixed mode at this increased number of

hops is tried, except that one or two E layer hops are substituted in

succession. The types of attempts at finding a propagating mode are

continued in this fashion until all modes have been exhausted, up to and

including two more hops than the starting number determined by the lowest F

layer mode.]

Selection 11 allows the user to enumerate which E layer, F layer or

combined E and F layer hop modes the program should be forced to consider.

Following this selection, the user is asked to input the number of modes to

predict, up to a maximum of value of 10. Prompting for the desired number

of modes takes place through individual entry of each separate E layer and

F layer hop mode combination desired. To input a given mode, the value of

the hop corresponding to the F layer mode is entered first, followed by a

comma and then the value of the hop corresponding to the E layer mode. For

example, to enter a mode corresponding to 3 hops using the F layer and 1

hop using the E layer a value of 3,1 is entered.

Selection 12 allows the user to force the program to consider a single

E layer propagation mode between the transmitter and the receiver. This

one-hop E layer prediction can be useful when it becomes possible for E

layer propagation to result in a higher MUF than the F layer mode.

Selection 13 from the Mode Selection Menu allows the user to return to

the Main Menu of the IONSOUND HDX program.

21. Printing

Make sure that your printer is powered up and on-line before

attempting to print anything. The most common usage of IONSOUND HDX is

showing 24-hour prediction screens. These may be captured to the printer by

the use of <Shift PrintScreen>. Two screens may be printed on a single

sheet of paper. Most printers will require that you take them off-line and

force a form feed in order to eject a printed page of paper.

[Note: As an alternative to printing on paper, various file capture

utilities may be utilized. An example of such a computer program utility is

PRN2FILE.COM and its documentation PRN2FILE.DOC which is available from

Ziff-Davis Publishing Co., 1 Park Avenue, New York, NY 10016. Download of

PRN2FILE.COM from PC-Magnet, an online service of PC-Magazine is also

available. Call 1-800-346-3247 for closest access point.]

Bibliography

Bandwidth and Signal-to-Noise Ratios in Complete Systems, CCIR

Report 195, Vol. III, ITU, Geneva, 1963.

Bixby C., and Morris, J., "The Art and Science of DXing," QST,

Jan 1979, pp 11-14.

CCIR 1986, "A Set of Simplified HF Antenna Patterns for Planning Purposes,"

Report 1062, International Telecommunications Union, Geneva

CCIR 1988, "Available Microcomputer-Based HF Radio Propagation Prediction

Procedures,"  IWP 6/1 Doc 320, CCIR Secretariat, International

Telecommunications Union, Geneva

Davies K., "Ionospheric Radio," Blaisdell Publishing Co., Waltham,

Massachusetts, 1969.

Hall J., K1TD, "Propagation Predictions and Personal Computers,"

QST, Dec 1990, pp 58-59.

Jacobs, G., Propagation: "Do-it-Yourself Forecasting," CQ, Oct

1990, pp 108-112.

Johnson R.C., and Jasik, H., Antenna Engineering Handbook (2nd

Ed.), McGraw-Hill Book Co., New York, 1984.

Maslin N.M., "Assessment of HF Communications Reliability",

AGARD Conference Proceedings No. 263: Special Topics in HF

Propagation, AGARD-CP-263, 1979.

Reference Data for Radio Engineers, Howard W. Sams, Inc., 1972.

Schwartz M., Information Transmission, Modulation, and Noise,

McGraw-Hill Book Co., New York, 1959

Solar Indices Bulletin, National Geophysical Data Center,

Boulder, Colorado

Sumner D., "Chart Your Way to Better DX," QST, Jan 1977, pp

58-60.

The ARRL Antenna Book, 16th Edition, ARRL, Inc., 1988

The ARRL Operating Manual, ARRL, Inc., 1987.

The ARRL Handbook, 1994, ARRL, Inc.

Thrane E.V., "State of the Art of Modeling and Prediction in HF

Propagation,"  AGARD Lecture Series No. 127: Modern HF

Communications, AGARD-LS-127, 1983.

White E., "Those Propagation Charts," How's DX:, QST, Apr 1983,

pp 63-64.

Glossary of Terms

ARRL          American Radio Relay League

BBC           British Broadcasting Corporation

BRNG          bearing

CCIR          International Radio Consultative Committee

CW            continuous wave, Morse code

dB            decibel

dBuV          dB signal level with respect to 1 microvolt

DBUVM         dB field strength with respect to 1 microvolt/meter

(dBuV/m)

dBW           dB power with respect to 1 Watt

DBWM          dB power density with respect to 1 Watt/meter squared

(dBW/m^2)

DOS           disk operating system

ELE           elevation angle

F/B           front/back

FOE           E Layer critical vertical incidence frequency

FOF           F Layer critical vertical incidence frequency

FOT           optimum working frequency (usually below MUF)

FREQ          frequency

Ham           amateur radio operator

HF            high frequency

HPF           highest possible frequency

Hz            hertz (unit of frequency)

IONCAP        Ionospheric Communications Analysis and Prediction

Program

L PATH        long path

LUF           lowest useful frequency (usually limited by absorption

and noise)

MCFO          maximum critical oblique frequency

MCFV          maximum critical vertical frequency

MHz           megaHertz

MUF           maximum useable frequency (for a particular layer and

distance)

NOAA          National Oceanographic and Atmospheric Administration

(U.S.)

NPW           Noise Power in dB-Watts (decibels above or below 1

watt)

NTIA          National Telecommunications and Information

Administration (U.S.)

%N            signal-to-noise availability N, expressed in percent (%)

[percentage of days of the month that the signal-to-noise

ratio meets or exceeds the minimum signal-to-noise ratio]

%P            propagation path availability P, expressed in percent (%)

[percentage of days of the month that the predicted

propagation path will be available]

%T            total link reliability N x P, expressed in percent (%)

[represents the numeric product of signal-to-noise

availability and propagation path availability, and is

expressed as a percentage value signifying overall link

quality]

RX            receiver

S/N or SNR    signal-to-noise ratio in decibels

S PATH        short path

SBRNG         switched bearing (long path bearing, 180 degrees

opposite BRNG)

SESC          Space Environmental Services Center, NOAA, Boulder, CO

(U.S.)

SSN           smoothed sunspot number

SFN           solar flux number (measured at 2800 MHz)

SVM           signal voltage in dB-Microvolts (dBuV)

SWL           shortwave listener

TX            transmitter

UTC           Universal Coordinated Time

VHF           very high frequency

WWV           A radio station of the National Bureau of Standards

(U.S.)

Appendix

NATIONAL BUREAU OF STANDARDS (NBS) SERVICES

The U.S. National Bureau of Standards (NBS) broadcasts the latest

geomagnetic (Ap and K indices) and 2800 MHz solar flux level number (SFN),

as well as short-term forecasts of expected conditions on Radio Station

WWV, simultaneously at 18 minutes past each hour on 2.5, 5, 10, 15, and 20

MHz. These transmissions originate from Ft. Collins, CO. Radio Station

WWVH, located in Hawaii, broadcasts Geophysical Alerts at 45 minutes past

the hour on 2.5, 5, 10 and 15 MHz. WWV and WWVH information is updated

every 3 hours starting at 0000 UTC.

The on-duty forecaster at the NOAA Space Environmental Services Center

(SESC) in Boulder, CO is also able to provide Alert data by calling

303-497-3171. This information is also available, free of charge, by

calling NOAA's SESC at 303-497-3235. The SESC also provides a free on-line,

menu-driven modem bulletin board service at 303-497-5000, 24 hours a day,

for access to propagation data, solar reports, solar and geomagnetic data,

and MUF predictions. Modem access is at 300, 1200, or 2400 baud, with a

standard protocol of 8-bit data word, 1 stop bit, and no parity.