pic_c

Release 1.0

Ownership

This software is owned by Collin Brendemuehl/DPC.  It is not "public domain".  You may register your copy with DPC by sending $25.00 (US) to:

DPC

Attn: PIC C

103 S Broadway Suite 101

Edmond OK 73034

Registered users will receive updates to the compiler as they become available.

Source code to the compiler is available for the sum of $150.00 (US).  With the source code the user may make those enhancements desired or needed.

Introduction

The PIC micro-controllers from Microchip have, for the type of work that I do, proved themselves very useful.  It is easy to use and has a simple instruction set.  But as a C programmer, I don't always like to work with multiple assembler formats, one for each micro-controller.  There exists too much to do and not enough hours in the day.

Each system has its own unique characteristics that need to be remembered.  While that can be done, I'd rather not take the time and clutter my mind with unnecessary information.  Rather, I'd prefer to continue programming in C and forget the rest.  (That's what reference manuals are for!)

I purchased a Parallax development system for my projects, but soon became frustrated at not having a C compiler available.  Hence, a few hours programming and "Voila!", I can now do my PIC programming in C.  The listings include the compiler along with some sample code.  In the sample code you'll see some of the characteristics of the PIC that make it unique.

First, a PIC can only call 2 levels of subroutines.  If you call 3, you'll NEVER return to the first.  That can be hair-raising to have happen.  But having a C compiler can help you control that problem.

The "for", "while" and "if" operations make use of the "CJxx" (compare and jump if false) instructions within the Parallax instruction set.  I use these instead of the "CSxx" (compare and skip if false) to allow more space for inserting code for customization.

Supported items include:

for()

if()

while()

=

== !=

<= <

>= >

&=

|=

^=

<<

>>

char

Virtually no "lint" operations are performed.  This allows the user to execute an assignment to a pre-defined register, such as 'C'.

Syntax Restrictions:

The simplicity of this compiler makes it useful for simple projects.  Inherent in this is some limitation on the nature of a C statement.  You will notice in the example given that no math is allowed in the "rvalue" part of a statement ( to the right of an assignment ).  This made creation of the compiler simpler and also more efficient.

Complex conditions are not allowed.  Only a single lvalue, a comparison, and a single rvalue are allowed.

The compiler generates no errors.  For instance, if a com-ment trailing "*/" is ommitted, the compiler will just keep on running and your system will be "hung".

The compiler will only read 500 lines of code for a program.

You may add in-line assembler by using the "#A" directive.  Place the #A before each line containing a line of assembler.  This line will be output as-is to the output file.

Future Enhancements

Over the next year I have the following enhancements in mind to make to the compiler:

1.	"for" and "while" loop enhancement

(Currently the loop is clean but inefficient)

2.	Data passing when calling a function

(Single data element in W)

3.	Data returns from a function

(Single data element in W)

You may have more enhancements that you wish to see.  Please mail your suggestions.

Warranty

No warranty is made as to the effectiveness or quality of this software package.