*************************************************************************

PC-PAKRATT II

RELEASE NOTES

************ Jul. 22, 1991 ******************** ver. 5.1 ****************

This file contains information not found in the manual and

includes information about the latest release of

PC-Pakratt II.

UPGRADING FROM PC-PAKRATT

There is NO backward compatibility to the original version

of PC-Pakratt.  Any macros and message files used with PC-

Pakratt will have to be recreated for PC-Pakratt II.  You

may wish to create a new sub-directory for PC-Pakratt II so

as to keep your PC-Pakratt files for reference.  Since PC-

Pakratt II is started with the command pp, any batch files

that you may have created for PC-Pakratt will have to be

changed for PC-Pakratt II.

PK-232 EPROM REVISION DATE DISPLAYED

When PC-Pakratt II is starting up, the program will read the

release date of the firmware in the PK-232 and display the

results in the upper right corner of the initialization

screen.

UNKNOWN COMMAND ERRORS

To disable "Unknown Command" errors, see page 25 in the

PC-Pakratt II Operating Manual or the -[MODE] SWITCH

section below.

-[MODE] SWITCH, PAGE 6

If you want to have PC-Pakratt II boot straight into a

communications mode, (for example, typing pp -packet will

cause PC-Pakratt II to boot straight into the packet mode),

then you may want PC-Pakratt II to ignore any unknown

commands.  PC-Pakratt II can be configured to ignore any

unknown commands by setting the "Unknown Command Errors"

option to be disabled.  Start from the opening menu and

select "PC-Pakratt Parameters".  From there set "Unknown

Command Errors" to be disabled.  Be sure to save this change

in PC-Pakratt II with the "Save PC-Pakratt Configuration"

option in the "Files Menu".  See page 25, "Turning Off

Unknown Command Errors" in the PC-Pakratt II manual.

ADDED COMMAND LINE SWITCHES, PAGE 6

Additional command line switches have been added.  They

are -bw and -color, with -color being the default.  If you

are working with a computer which uses an LCD or Monochrome

screen, then -bw can be added to the command line when

starting PC-Pakratt II.

For use with the Fast Initialization feature, two command

line switches have been added.  The -i will force parameter

upload and -f will prevent it.  See "FAST INITIALIZATION"

and the Host Mode on Exit dicussion in the section titled

THE COMMUNICATIONS MENU, PAGE 23 below.

Added a -S#### command line switch to specify the amount of

RAM that is too be set aside for the scroll back buffer.

See "Scroll Back Buffer, Page 27" below.

Added new command line switch, -MAIN. This switch will by-

pass the opening screen, initialize the TNC and stop at the

Communications menu.

CHOOSING A COM PORT, PAGE 7

When first running the program, PC-Pakratt II will prompt

for the COM port to use.  Once the COM port is specified,

PC-Pakratt II will now prompt for the port's IRQ & address.

The default values will be shown in the window.  The user

will also be prompted for the address and IRQ number when

changing COM ports from the PcPakratt Parameters menu.  The

IRQ & COM port address for all 4 ports are now stored in the

.CFG file.

Since the IRQ and address numbers are now user specified

when first running PC-Pakratt II and when changing the com

port in the PCPakratt Parameter menu, the entire discussion

of modifying the .cfg file can now be ignored.

CGA SNOW PROBLEMS

Some CGA video cards have a tendency to produce a "snow

like" pattern on the screen.  A new selection is available

in the PcPakratt Parameter screen called Video that is

designed to address this problem.  Video can be toggled

between "Direct" and "BIOS Calls".  PC-Pakratt II will

detect the type of video card used and if a CGA card is

found, then PC-Pakratt II will set Video to "BIOS Calls",

otherwise Video will be set to "Direct".  This may also help

with reported non-compatible CGA cards.

MOUSE NOTES

If you have a Logitech Bus Mouse, and you find that the

cursor disappears about half way down the screen, then you

may need a newer version of MOUSE.COM.  We have found

problems with MOUSE.COM files dated in September 1987.

Logitech is a trademark of LOGITECH Inc.

CONFIGURATION MENU, PAGE 12

The option to "Set COLOURS" has been removed.  The changing

of colors is now done from the "Communications Menu" from

the selection "Change Colours".

The Setup Screens are now available in the Configuration

Menu.  Various operating parameters can be set up from this

menu.

CBELL/PACKET SETUP SCREEN, PAGE 13

Removed CBELL from the Packet Setup Screen.  Replaced CBELL

with CONALARM.  CONALARM toggles between OFF, SOFT (3 bells)

and LOUD (Obnoxious siren).

In AMTOR, the state of CONALARM is automatically checked. If

LOUD or SOFT, PC-Pakratt II turns on CBELL so that links

made to 7 char SELCAL stations are announced.  CBELL is

turned off when not in AMTOR.

AMTOR/BAUDOT/ASCII/MORSE SETUP SCREEN, PAGE 16

The command XMIT OK is an ALL MODE command and has been

added to this setup screen.

New parameter XLENGTH added (specific to PC-Pakratt II).

XLENGTH specifies the maximum number of characters that can

be typed before the data is sent to the PK-232.  Only active

in AMTOR/BAUDOT/ASCII/MORSE.

FILES MENU, PAGE 18

The nomenclature for the first two items in the "Files Menu"

have been changed.  "Save Configuration" has been changed to

now read "Save PcPakratt Configuration" and "Save Parameter

Defaults" has been changed to now read "Save PK-232

Parameters".

The Files Menu now allows descriptions to be given to the

Message and Note files.  The names will be displayed in the

respective menus when uploading, viewing and editing files.

The message/notes descriptions are saved in PKDEF_.CFG

SPECIFYING A TEXT EDITOR, PAGE 20

PC-Pakratt II now includes a text editor called PKEDIT.  The

two files necessary to run PKEDIT, PKEDIT.EXE and PKEDIT.HLP

were included in the PC-Pakratt II disks that you received.

The default text editor that is specified in PC-Pakratt II

is PKEDIT.  PC-Pakratt II assumes that the two files,

PKEDIT.EXE and PKEDIT.HLP are in the same directory that PC-

Pakratt II is in.  Help screens for PKEDIT are accessed with

the <F1> key.

THE COMMUNICATIONS MENU, PAGE 23

Host Mode on Exit:  Added a selection to the PCPakratt

Parameter Screen called Host Mode on Exit.  When enabled,

(default), Host Mode on Exit will leave the PK-232 in the

Host Mode when leaving PC-Pakratt II.  When disabled, Host

Mode on Exit will place the PK-232 into the normal command

mode.  When starting PC-Pakratt II, the PK-232 must be in

the host mode for Fast Initialization to work.  The setting

for Host Mode on Exit is saved with the Save PcPakratt

Parameters setting in the Files Menu.

TNC Time:  Added a selection called TNC Time.  This

specifies whether GMT or Local time will be sent to the PK-

232s internal clock.  It is the PK-232s internal clock that

time stamps data.

PCPAKRATT PARAMETERS MENU, PAGE 25

The option "Rx Window (43 line mode)" now reads "Rx Window

(High Res mode).   When the High Res Mode is enabled, (high

res is enabled with the -h command line), PC-Pakratt II will

go to the 43 line mode if an EGA video card is detected and

to the 50 line mode with a VGA card.

If VGA is detected, the maximum number of lines in the Rx

window can be 44 otherwise an EGA is assumed and the maximum

of lines will be 37.

SCROLL BACK BUFFER, PAGE 27

The ability to set the size of scroll back buffer in the

PCPakratt Parameter screen has been removed, (to make room

for other features such as Video and TNC Time).  With

version 5.1, PC-Pakratt II will attempt to set aside about

64K of RAM for the scroll back buffer when the program is

started.  If there is insufficient RAM available, PC-Pakratt

II will subtract 4K from the amount of RAM it sets aside for

the scroll back buffer and tries again.  If this fails, PC-

Pakratt II subtracts another 4K and tries again.  This

continues until the amount of RAM that the scroll back buffer

can use is matched by the amount of RAM available in the PC.

A new command line switch has been added that forces PC-

Pakratt II to set aside a specified amount of RAM.

The switch is -S####  where #### is a number IN HEX betwwen

1000 to FF00.  Do NOT type a $ charachter in front the

number that you want to enter.  If you specify FF00, then

PC-Pakratt II will set aside 64K of RAM for the scroll back

buffer.  A setting of 1000 will set aside 4K of RAM.  If

there is not enough RAM in your PC to run PKEDIT from within

PC-Pakratt II, you can free up RAM by decreasing the scroll

back buffer requirements.

The percentage of the scroll back buffer that has been used

is now displayed on the screen when in any mode.  To clear

the scroll back buffer, press the <CTRL><PgUp> key.  The

percentage of buffer used will return to 0% when cleared.

SAVE/RESTORE PAKMAIL, PAGES 27 and 40

The discussion about "Save/Restore Mail" does not take into

account the July 1990 firmware release.  Starting with the

July 1990 firmware release, the internal battery in the PK-

232 CAN BE LEFT INSTALLED.  The internal battery will now

back up PakMail messages in the PK-232.  We suggest that

you connect the battery jumper (JP-1), and disable

Save/Restore Mail.

If "Save/Restore Mail" is enabled and the PakMail mailbox is

empty, the file PAKMAIL.MES will not be created.

The restore PakMail messages to disk function has been

changed making the second to the last sentence on page 40

incorrect.  If "Save/Restore Mail" is enabled, message

restore from disk will occur only if the mailbox has been

found empty during program startup.  Restoring PakMail

messages can be still invoked from the PakMail menu in the

Packet Communications Screen (F2 Key).

SENDING A FILE, PAGE 32

PC-Pakratt II has been enhanced to speed up file transfers.

Data is sent to the PK-232 in 256 byte chunks.  A 2K file

now takes less than 10 seconds to send to the PK-232.

SAVING SCROLL BACK BUFFER TO DISK, PAGE 33

A new entry is available when in the scroll back screen.

The F2 key will now save the entire contents of the scroll

back buffer to disk.  If the scroll back buffer should fill,

the first 25% will be discarded to make room for more

incoming data.

DEFINING MACRO KEYS, PAGE 34

The Macro Menu now allows the macros to be named.  A new menu

item has been added to the Macro Menu.  The name of the

macro appears in Macro Menu.  The macro names are saved in

a default macro file.  Macros can be executed from the Macro

Menu.

Macro routines have been enhanced to accept functions keys,

ALT-keys and control characters.  The passthru char (^p) is

no longer need to to place control characters in the macro.

Control chars are shown in reverse video, function & ALT

keys are shown in brackets; e.g. [F1], [aF2], [aS].

The scroll lock key, when toggled on, allows the cursor,

HOME, ESC, BS and INS keys to be placed in the macro.  When

scroll lock is off, these keys operate normally.

ESC quits the macro editor without saving the macro (in

memory).  HOME copies the macro into memory (but not to

disk).  INS toggles on/off the insert character mode.  CTRL-

PGUP clears the macro.  The up/down/left/right keys mode the

cursor about the screen.

MULTIPLE CONNECTS IN PACKET

The SHIFT-F9 key now toggles the multiple channel display

mode in Packet.  When the Channel Only display is on, the

channel number in the status window is shown in reverse

video.  Only data sent to that channel will be displayed.

Monitored text data will also be displayed.

When the channel is changed, the RX window is updated with

data only for that channel.  Likewise, when the scroll back

buffer is used, only the data on that channel will be shown.

If data is received on a channel that is not currently being

viewed, a visual indication of the channel number will be

presented on screen.  The following is an example of what

appears when new data has been received but not yet viewed

on channel one.

============= ESC to Exit MM Chs:1 ============== 000 % ==

When the SHIFT-F9 key is pressed again, all data will be

shown in the RX window and in the scroll back buffer.

F10 KEY IN PACKET, PAGE 36

The operation of the F10 key in packet has been enhanced

so that when switching between VHF and HF, the HBAUD

parameter will now also change.  The HBAUD setting for VHF

will be 1200.  The HBAUD setting for HF will be 300.  The

F9 key can change the value of HBAUD independently.

AUTO-CONNECT, PAGE 38

The contents of the Auto-Connect string will now be

displayed on the screen.  When you click the mouse on the

desired Auto-Connect string, the appropriate connect

instruction is sent to the PK-232.

PACKET PAKMAIL OPERATION, PAGE 39

When in PakMail, the last number entered (READ,KILL and

EDIT) is now remembered and is displayed in the window when

you again Read, Kill or Edit a message.

TRANSMIT ECHO IN PACKET

An additional option has been added to the PcPakratt

Parameters Menu called "Packet Echo".  When this feature is

enabled, transmitted packets will be echoed to the receive

buffer part of the screen.  Packet Echo is disabled and

enabled in the PcPakratt Parameters menu.  The default is

disabled.  Remember to save any changes you make before

leaving PC-Pakratt II by selecting "Save PcPakratt

Parameters" in the Files Menu.

FAST INITIALIZATION

An additional option has been added to the PcPakratt

Parameters Menu called "Fast Initialization".  When enabled,

PK-232 parameters will NOT be uploaded from the program disk

to the PK-232 when starting PC-Pakratt II.  This will

significantly reduce the amount of time required to boot PC-

Pakratt II.  The default setting has Fast Initialization

disabled.

When starting PC-Pakratt II, the status of Fast

Initialization will be assessed, and if enabled, PC-Pakratt

II will then determine if the PK-232 is still in the host

mode.  If the PK-232 is found to be in the host mode, then

the PK-232 parameters that are stored on disk will NOT be

uploaded to the PK-232.

Two additional command line switches have been added that

affect Fast Initialization.  Starting PC-Pakratt II with a

pp -f will force PC-Pakratt II to initialize in the "Fast"

mode regardless of the setting of Fast Initialization.

Starting the program with a pp -i will force PC-Pakratt II

to upload from disk, all PK-232 parameters.  Also see the

Host Mode on Exit dicussion in the section titled THE

COMMUNICATIONS MENU, PAGE 23 above.

If PC-Pakratt II will be restoring PakMail messages from

disk, Fast Initialization will be disabled.  See

"SAVE/RESTORE PAKMAIL", PAGES 27 and 40 above.

BATCH FILE EXAMPLE, PAGE 41

The last line in the batch file example is in error.  The

line "CD\PC-Pakratt II" would not work.  DOS only allows up

to eight characters and no spaces.  The recommended

directory for PC-Pakratt II is PCRATT2.  Therefore, the last

line of the example batch file should read "CD\PCRATT2".

If you run the CORRECTED batch file, (see "Batch File

Example, Page 48" below), then PK-FAX and it's assorted

files DO NOT need to reside in the same directory as PC-

Pakratt II.

AMTOR MAILBOX OPERATION

PC-Pakratt II now supports the AMTOR mailbox functions that

are found in the PK-232 EPROMS dated July 22, 1991 and

later.  In AMTOR, F2 executes MDCHECK as in Packet.  The

following keys work both in Packet and AMTOR (note keys have

been reassigned in Packet):  F7 toggles MDMON, SHIFT-F7

toggles TMAIL and MAILDROP (in Packet) and ALT-F7 executes

FREE.

INPUT EDITING

When an input is required by PC-Pakratt II, the previous

value for the input will be displayed.  This is done so

that if a minor change is needed (e.g. to add a -1 to the

end of a call sign), one can simply use the cursor keys to

move to the point of change and type it in (without re-

typing the entire value).  If the new value is completely

different than the old value and is shorter as well, the

"left-over" characters do not need to be deleted.  Example:

replacing WA3ABC with N7AA will leave N7AABC on the screen,

but only the new value will be accepted, and the additional

letters "BC" do not need to be deleted.

PC-PAKRATT II BINARY FILE TRANSFER

PC-Pakratt II now has its own binary file transfer routine

for use in the packet mode.  The binary transfer routine is

invoked with an ALT-F while in the packet screen.  The

transfer protocol is compatible with YAPP.

YAPP BINARY FILE TRANSFER

When using YAPP as your binary file transfer program, two

changes will have to be made to the initialization string in

YAPP.  The following is a sample of the last few lines from

a modified initialization string.

CTRL-A OHOOFF CTRL-W

mon on

conok on

*** EOF  {end of commands sent to TNC on program start}

conok off

mon off

host on

*** EOF  {end of commands sent to TNC on program end}

The CTRL-A and the CTRL-W above mean that the control-a and

control-w chrachter must be inserted into the YAPP file.  Do

NOT type CTRL-A, it will not work.

Note:  Due to internal chracterisitcs of YAPP, it will

"dump" stations already connected.

Before YAPP starts sending commands to the PK-232, the host

mode will have to be turned off.  In the above example, the

first line performs this function.  When leaving YAPP, the

last command sent to the PK-232 must be the host on command

as seen above.  Permission to print the YAPP initialization

screen courtesy of Jeff Jacobson.

BATCH FILE EXAMPLE, PAGE 48

The second and third line of the example batch file are in

error.  The second line should read PKFAX.  The line

"CD\PC-Pakratt II" would not work.  DOS only allows up to

eight characters and no spaces.  The recommended directory

for PC-Pakratt II is PCRATT2.  Therefore, the last line of

the example batch file should read "CD\PCRATT2".

CLEARING THE TRANSMIT BUFFER

Pressing an ALT-T to clear the on screen transmit buffer

will now clear the any data in the PK-232 transmit buffer

as the on screen display.

RETURN TO RECEIVE MODE

When in the BAUDOT, FEC, ASCII and CW mode, inserting a

CTRL-D at the end of transmitted text will force the PK-232

to return to the receive mode when the CTRL-D is

encountered.  To embed a CTRL-D in the transmit buffer,

simply press and hold the CTRL key followed by the letter

D.  This same procedure works in both the transmit buffer

and when writing macros.

Please be advised that a CTRL-D imbeded in text that is

being sent via AMTOR ARQ/SITOR Mode B will cause the link to

be dropped when this charachter is reached.

CLEARING MFROM, MTO, DFROM AND CFROM CALLSIGNS

To remove a callsign from one of these parameter lists, move

the cusor to the desired callsign, press the spacebar then

press the RETURN key.

MS-DOS 5.00 INFORMATION

PC-Pakratt II with Fax has been tested and found to be

compatible with MS-DOS 5.0.  Both programs also work well

with IBM DOS 4.01.  IBM is a Registered Trademark of

International Business Machines Corporation.  MS-DOS and

Microsoft are Registered Trademarks of Microsoft

Corporation.

PS/2 MODEL 50Z USERS

If your IBM PS/2 Model 50Z was manufactured prior to

September 1988, your computer may be in need of a new System

Control Board.  Contact your IBM Dealer or AEA.  PS/2 is a

trademark of International Business Machines Corporation.

LEADING EDGE MODEL D USERS

Some customers have reported problems with PC-Pakratt II when

used on machines with a BIOS version lower than 3.00.  A

Leading Edge Model D may need a BIOS version of 3.00 or

later to work with PC-Pakratt II.

If the serial port on your computer uses a DB-9 type of pin

configuration, a DB-25 - DB-9 adapter will be required.  A

DB-25 to DB-9 adapter that is available from your computer

dealer should be acceptable.  Be sure to note the specific

gender that is required for your serial board.

PRINTING FILES WITH PK-FAX

When printing the contents of the buffer, PK-FAX will print

to the printer that is attached to the computer as described

in the PK-FAX configuration screen.  PK-FAX will not print

to the parallel port on the "Y" cable you received with the

PK-232.

PC-PAINTBRUSH FILES

Any picture created with PC-Paintbrush will have to be

created with PC-Paintbrush configured in the "two-color"

mode.  Run your PC-Paintbrush install program and select the

appropriate two-color mode for the video display used on your

computer.  You must reset the computer before the new

selections become active.  PC-Paintbrush is a trademark of

ZSoft Corporation.

MANUALS

When a conflict occurs between the PK-232 Users Manual and

either the PC-Pakratt II or PK-FAX manual, use the

information in the software manual!

RS-232 CABLES

PC-Pakratt II requires pins 2, 3, and 7.  PK-FAX requires pins

2, 3, 5, and 7.  If you build an RS-232 cable, be sure to

use pins 2, 3, 5, and 7.  The cable supplied with the PK-232

works with both PC-Pakratt II and PK-FAX.

UPDATE INFORMATION

Update announcements of PC-Pakratt II and the PK-232, are

mailed to our users via the information that we receive from

the warranty cards.  Please return your warranty card.

WINDOWS 3.0 APPLICATION NOTES

PC-Pakratt II has been found to behave itself when run under

Windows 3.0 under the following conditions.

o  Must run the 386 Enhanced Mode.  We have had no

experience running PC-Pakratt II in the "Real" or

"Standard" mode.

o  PC-Pakratt II can run as a window.  When creating

your PIF file, make sure that the box labeled

"Background" under "Execution" has been checked

and that there is an "X" in the box.

Windows is a trademark of Microsoft and 386 is a

trademark of Intel Corporation.

PK-FAX ENHANCEMENTS

Added support for the COM3 and COM4 serial ports.  The

options lists for the "Printer Configuration/Printer port"

menu and the "PK-232 Configuration/PK-232 serial port" menu

now include COM3 and COM4.  The user may use the "COM3/COM4

configuration" menu under "PK-232 Configuration" to alter

COM3 or COM4 parameters such as IRQ bit, vector and base

register.

The printer control codes may now be customized for printers we

haven't covered specifically in PK-FAX.  Here is how to do

it:

1. Under "Printer Configuration," set "A. Printer type" to

the type of printer whose control codes come closest to the

printer being set up.  Then set "A. Printer type" to Custom.

2. Select "B. Printer graphics density" and choose the

desired dot density.  The density is expressed in dots per

inch, which assumes a printing width of 8 inches.  Some

printers have a usable width narrower than 8 inches.  The

user should divide the desired number of dots by 8 and

select that number.

3. Select "I. Printer control codes" and change the

appropriate codes.  The codes must be entered in

hexadecimal, not decimal or ASCII characters.  The changed

codes stay in effect as long as "Printer type" is set to

Custom.

The Printer Control Codes menu contains the following items:

A. Set graphics mode               Sent each line before the

image data.

B. Set line spacing                Sent once at the start of

the image.

C. Do line feed                    Sent each line after the

image data.

D. Reset printer                   Sent once at the end of

the image.

E. Vertical dots                   Shows number of vertical

dots and their

orientation.  Set the

number with the "Printer

data bits" command under

the "Printer

Configuration" menu.

F. Horizontal dots                 Shows number of

horizontal dots.  Set

the number with the

"Printer graphics

density" command under

the "Printer

Configuration" menu.

Example:  The Tandy DMP-132 is not covered in PK-FAX.  Set

"Printer type" to Radio Shack and then Custom.  Then set

"Printer graphics density" to 60 dpi to yield 480 dots

across the paper.  Select "Printer control codes" and set

the following:

A. Set graphics mode      12

C. Do line feed           0A

D. Reset printer          1E 0D 0A

It's important to set "Printer type" to the correct printer

before going to Custom.  Some printer types are handled as

special cases in PK-FAX:

HP LaserJet & ThinkJet receive dot data in a horizontal

format rather than vertically as most other printers do.  In

higher densities horizontal lines need to be received twice.

Radio Shack and Gorilla type printers need to have the

unused bit 7 of the data byte set to 1.

Okidata printers need to receive twice any data bytes

that happen to be equal to 3.

Toshiba printers must have the 8 data bits expanded out to

24 bits.

TECHNICAL SUPPORT

To obtain support for this or any other AEA product:

IN THE U.S. AND CANADA:

AEA, Inc.

P.O. Box C2160

Lynnwood, WA  98036-0918

tel: (206) 775-7373

fax: (206) 775-2340

IN EUROPE:

ICS Electronics

Unit V, Rudford Industrial Estate

Ford, Arundel, West Sussex

England  BN180BD

tel: 0903 731101

Technical Support is also available via the CompuServe BBS.

Type GO HAMNET at any prompt.  CompuServe ID # 76702,1013.

NOTE:  To get a hard copy of these notes on your printer,

print the README.DOC file and not this batch file.

#

#t a hard copy of these notes on your printer,

print th