PSATNC.COM

==========

This Packet driver allows you to do packet radio, either 1200 baud AFSK

or 9600 baud FSK according to G3RUH. It runs with sound cards containing Analog

Device's PSA Chipset.

First run either TPAT1200 or TPAT9600 and set there the audio parameters

(which audio input to use and the audio levels) and the channel parameters

(p-persistence, tx-delay and so on). These programs save a file containing

the customized DSP firmware (these files are usually named FIRM1200.DSP or

FIRM9600.DSP)

Then start the driver, with the command line:

PSATNC packet_int dsp_file [int_num] [io_base]

packet_int is the interrupt number your application program will communicate

with this driver (usually 0x60 or 0x61)

The DSP file is either FIRM1200.DSP or FIRM9600.DSP.

The int_num is one interrupt number of the following ones which is not used

by any other PC component: 3,5,7,9,10,11,12

The base_addr is the address at which the sound card can be found, this is

usually either 0x220 or 0x240.