CHAT server setup details (Version 4 BPQ)

-----------------------------------------

Getting started

---------------

To run the chat server you must have already installed the G8BPQ

version 4 node software.

Next set up a file which at the moment contains a one line logon

message. The file is

CHAT.CNF

Configure a window in DV which must be at least 22K in size. The

program requires 2 parameter values. The first is the starting

stream number and the second is the number of streams you wish

the chat node to manage. The maximum number is 10. When you open

the DV window all the streams will be disconnected and the messages

written to the screen.

SYSOP commands

--------------

From the local terminal you can see all the messages being sent

through the server and also talk to any or all of the users.

When you type it will be displayed in the bottom area of the

sysop screen. Any line entered here which is not preceded by a

/     will be sent out to all users as a message labelled

[SYSOP] xxxxxxxx

If you wish to communicate with one particular user type

/n xxxxxx     where n is the stream number to which the

user is connected. The message will appear on the users screen as

<*SYSOP*> xxxxxxxx

The command  /c  will sent a polite message to all users then

disconnect them before closing the DV window.

The command  /q  will close it all down quickly.

User Commands

=============

The user commands are explained by the help list which is sent in

response to a   /H   command from the user.