Installation of WFBB 7.00g & Upgrade 7.00i

Please Note: WFBB 7.00 should be excluded from your Anti Virus Program immediately after Installation. Some A/V Programs think the WFBB Configuration.EXE is a Trojan Horse and will remove the Configuration Program. This may be true with All A/V Programs.

Important: The Following Syntax Must Be Edited In The PIF and Housekeeping to work correctly.

Note: Please Refer to Environment1, 2 & 3.PNGS.

Right Click on My Computer. Left Click Properties, Advanced Tab, Environment Variables Button. In System Variables Window, High Light PATH, Then Press Edit Button. At the End of the Existing Syntax String add the Following Syntax:

;C:\fbb;C:\fbb\bin;C:\fbb\system

Click OK, OK, OK Restart Computer.

From the WFBB 7.00g Folder Run the FBB Install.exe. You may be prompted several times to ether Add/Over Write. Please answer Yes

For All. WFBB should be Installed at the C: Root Directory. If Installed in a different Path or Drive, you will need to Edit the Paths in the, FBB\INIT.SRV and EPURMESS.INI, FBB\BIN\RUNFBB.BAT Files And Environment Variable Syntax String.

At the end of the Installation, the Configuration Program will start.

Fill in the necessary Information using Next to Page,

From Page 5 (Default Options at first connection), to Page 9.

(Forwarding) can for the most part be left as is.

On Page 9 Please set as Follows to be compatible with other BBS.

Systems:

Note: See Included PNG:

FBB Forwarding

FBB Compressed

X-Protocol Forwarding

Do-Not use checksum

MBL/RLI message Header

Page 10 (Miscellaneous) Remove from Test Mode

Page 14 (Time Information) Set Difference with UTC hours

Page 16 (Communication Configuration) Set as Follows:

See Included PNG:

Com 1 Winsock, Hex Address as Telnet Port Number, 6300=189c Hex

Com 2 None  You Can Add Other Telnet Ports Here

Com 3 None  You Can Add Other Telnet Ports Here

Com 8 TFWIN

File & Save

Next

Page 17 (Ports Configuration)

See Included PNG:

Port 1 WA8DED  THIS WILL BE YOUR BPQ32 PORT:

COM 8

MUX= Number of Ports allocated in BPQCFG.TXT including AXIP

Users

The Remaining Settings set for the use of BPQ

PORT 2 TCPIP

COM 1

Remaining Setting User Defined, Do Not Have RD-ONLY Checked.

File & Save

Exit

You will be Prompted to restart the Computer Yes.

After Restarting.

Copy or Move the Included AMSAT File to the System\FWD Folder. Please Note: No Extension can be added to the AMSAT File.

Copy or Move the TFWIN.DLL, bpqded32.dll, C_Filter.DLL And RUNFBB.BAT to The WFBB\BIN Folder REPLACE ALL, You may Edit the RUNFBB.BAT to send a Daily Maintenance and Housekeeping report to your BBS, Right Click on RUNFBB.BAT, Edit, Remove the rem only from The:

**Addition** Section

rem    echo SP Add BBS Call Sign Here

rem    echo /EX

**Maintenance** section

rem    echo SP  Add BBS Call Sign Here

rem    ECHO /EX

At this point with BPQ32 Installed, Start WFBB.

From the All Programs, FBB for Windows, WinFbb, BPQ should start with WFBB. Correct any Errors If Any Exit WFBB. You will see BPQ32 Hold In Memory Window. You will need to close this window to exit BPQ32.

Copy or Move from the 7.00I Upgrade Folder WFBB.EXE to The WFBB\BIN Folder REPLACE ALL.

This Completes the Installation of WFBB 7.00G and 7.00I Upgrade.

For WFBB 7.00E, only the 7.00I Upgrade is needed, Run WFBB 16 Bit Only, 32 Bit 7.00E is Not Y2K Compatible

Written by Michael Melnik N9PMO 30Jan2009

For assistance contact me at n9pmo@wi.rr.com

For additional Downloads see: www.n9pmo.com