FlexNet-Dokumentation           SNDDIAG.EXE       (C)1996 Tom Sailer HB9JNX

Stand: 15.4.96

Ident: SNDDIAG

Diagnose-Utility fuer die SoundBlaster- und WSS-Treiber

Dieses Utility soll helfen, die SoundBlaster- und WSS-Treiber einfacher

in Betrieb nehmen zu koennen. Die Einstellung der korrekten Audio-Parameter

erwies sich in der Vergangenheit als ein schwieriges Unterfangen.

Dieses Utility kann erst gestartet werden, wenn sich FlexNet mit einem

Soundkartentreiber im Speicher befindet und der Soundkartentreiber laeuft.

Also FSET MODE ... nicht vergessen!

Dieses Utility kann in einem oszilloskopaehnlichen Diagramm das Eingangssignal

anzeigen. Diese Anzeige eignet sich dazu, den Eingangspegel der Karte einzu-

stellen. Das Eingangssignal sollte das Diagramm etwa zu 75% aussteuern.

Die Audioparameter der Soundkarte koennen entweder mit der zur Soundkarte

gelieferten Utilities oder mit dem Utility SNDMIXER gesetzt werden.

Das Augendiagramm (Demodulatorausgang) kann zur Ueberpruefung der Filterein-

stellung sowie der Qualitaetskontrolle der Kombination Soundkarte/Transceiver

benutzt werden.

Dieses Utility braucht eine EGA/VGA- oder Herkules-kompatible Grafikkarte.

---------------------------------------------------------------------------

FlexNet documentation           SNDDIAG.EXE       (C)1996 Tom Sailer HB9JNX

Date: 15.4.96

Ident: SNDDIAG

Diagnostic utility for the SoundBlaster and WSS drivers

It proved to be a rather difficult task to correctly set the audio parameters

of the SoundBlaster and WSS drivers. This utility should remedy the situation.

This utility must be started after FlexNet with a sound card driver is

in memory and started. The sound card driver must be running, i.e. do not

forget to run FSET MODE ....

An oscilloscope like diagram of the input signal at the sound card's input

should help to adjust the audio level.  The height of the trace should be

about 75% of the diagram's height. The audio parameters of the soundcard

may be set either with the utilities that were included with your sound card

or the utility SNDMIXER.

An eye diagram (demodulator output) may be used to verify the filter settings

as well as to check the quality of the combination sound card/transceiver.

This utility runs only on computers with an EGA/VGA- or Hercules-compatible

graphics card.

Autor:  Tom Sailer

Weinbergstrasse 76

CH-8408 Winterthur

Packet: HB9JNX @ HB9W.CHE.EU

EMail:  sailer@ips.id.ethz.ch