.  PTCT is a Terminal program for the PTC pactor controller from SCS.

On one hand it is a program wich make full use of function keys

F1 to F10 and combiations of keys to give all the commands to the

PTC as: AM ,PT ,L 1 a.s.o.

On the other hand you can use it as a dumb terminal program.

All the commands to the PTC are given then with the normal text keys

and the most of the function keys don't work.

This is the best way to learn the commands of the PTC.

The program is written in Turbo Pascal and many of the routines

of DL5FBD are used.

.  When the program is running there are 2 windows on the screen

The upper window is the receive screen and the lower window is the

command and transmit screen.

.  A status line gives information about the status of the PTC.

.  If you have the Firmware version lower than 2.0  set local echo on

with  "ECHO 1".

Description of the function keys F1 .. F10.

===========================================

Esc  Before typing a command, use yhe ESC key.

F1   Help

Here all the function keys and the combinations keys are shown.

F2   Switch to Pactor standby. When connected in pactor to a station

this key disconnects. F2 also stops calling a station.

F3   Switch to Amtor standby.When connected in amtor to a station

this key disconnects. F3 also stops calling a station.

F4   Switch to RTTY Baudot.

F5   Call a station in Pactor or Amtor.

There is a extra window on the screen for the call of the station

you want to connect to.

F6   Toggles listen mode for pactor and amtor.

F7   Toggles Unproto(100 Baud) for pactor

and Fec for amtor and Transmit in rtty.

F8   Calling CQ in pactor, amtor or rtty.

F9   Shows all the parameters of the ptc. The SHOW command.

F10   Switch to dumb terminal.

Use it if you want to change parameters of the PTC or

when you want to read from or to write to your mini-

mailbox of the PTC.

Description of Combination keys.

================================

ALT-F1  Toggle a help line between Rx and Tx screen.

ALT-F2  Shows the pactor commands of the ptc (When on pactor STB).

ALT-F3  Shows the amtor commands of the ptc (When on amtor STB).

CTRL-F2 Hard disconnect for PACTOR. If you loose the link

with your QSO partner you can force to go to STB

PACTOR with this combination. Don't use it if the

link is good for your QSO partner sends REQ's

till his 'timeout' time.

(F2 is the normal disconnect key for PACTOR)

CTRL-F3 Hard disconnect for AMTOR. If you loose the link

with your QSO partner you can force to go to STB

AMTOR with this combination. Don't use it if the

link is good for your QSO partner sends REQ's

till his 'timeout' time.

(F3 is the normal disconnect key for AMTOR)

ALT-F5  Call a station in Pactor or Amtor from a list.

There is an extra window on the screen with the calls that are

in the file PTCT.CAL. The maximum number of calls in this file

is 10. If there are more the rest is ignored.

You must restart the program if you create a new list.

Don't forget the return after the last call in the list.

ALT-F7 Toggles Unproto with 200 Baud.

ALT-A   Use a text editor. The editor must first be installed

with ALT-C and the 'k' option.

ALT-B   Show the scrollback buffer. All received data from

and transmitted data to the ptc is stored (in memory) here.

ALT-E   Ending of reading the scrollback buffer. (Also does Esc)

ALT-C   Configuration.

Description follows.

ALT-F   Write all received data in a file.

ALT-I   To write additional information (i.e Name and QTH) in the

disklog when disklog is ON and when you are connected.

ALT-K   Make the call of your QSO partner known to the program.

This is ONLY necessary if the QSO partner DID CONNECT YOU.

If you connected, the call is already known to the program.

You need this callsign if you want to use CTRL-END. and

for the disklog.

ALT-L   Write the PACTOR logbook of the PTC on the screen.

ALT-O   Goto DOS and back to PTCT with EXIT.

ALT-P   Print a file.

ALT-R   Clear the receive screen.

ALT-T   Clear the transmit screen and the transmit buffer.

ALT-V   View a diskfile.

ALT-W   Toggles Wordout. With 'Wordout on' you can correct a typing

error before the word is transmitted. The backspace Char is

not transmitted when Wordout is ON.

ALT-X   Ending the program.

ALT-Y   Clears the Scrollback buffer.

The start screen.

=================

������ 26.02.1993 ��� 17:57 ������ Receive ��������������  PA0NC �����������ͻ

�                                                                            �

�                                                                            �

�==  Pactor  V.2.0  ==                                                       �

�(C)   SCS-GmbH   1992                                                       �

�                                                                            �

�32768 Bytes good RAM /  20750 Bytes free                                    �

�                                                                            �

�<System-Level: 1>  ROM-Call: PA0NC                                          �

�                                                                            �

�*** PACTOR ACTIVE >>>                                                       �

�                                                                            �

�*** STBY >>                                                                 �

�                                                                            �

�                                                                            �

�cmd:                                                                        �

�                                                                            �

����������������������������� Commands & Transmit ��������������������������͹

�                                                                            �

�                                                                            �

�                                                                            �

�                                                                            �

�                                                                            �

����������������������������������������������������������������������������ͼ

F1=Help  PACTOR    STB            IGNOR         WO Off  9600Bd�COM2�PTCT V2.60

The lower line on the screen is the status line. It gives important

information of the status of the PTC. The status line is devided in

10 fields.

Field 1: This is a hint to use the F1 key.

Field 2: The mode pactor, amtor or rtty is displayed here.

Field 3: STB, LISTEN, UNPROTO, FEC, CALLING or CONNECT or baudrate in Rtty.

Field 4: The call of the calling or connected station(only when YOU called).

Field 5: PHASE, IDLE, ERROR,TRAF,DATA or REQ.

Field 6: TRANSM. or RECEIV.

Field 7: WO On or WO Off  (the ALT-W key)

Field 8,9,10: information about serial parameters and program.

The statusline when calling a station in pactor.

================================================

F1=Help  PACTOR  CALLING  DF0THW  PHASE  TRANSM. WO On   9600Bd�COM1�PTCT V2.4

The statusline when connected to a station in pactor.

=====================================================

F1=Help  PACTOR  CONNECT  DF0THW  IDLE   RECEIV. WO On   9600Bd�COM1�PTCT V2.4

The statusline when transmitting FEC in amtor.

==============================================

F1=Help  AMTOR     FEC            IDLE   TRANSM. WO On   9600Bd�COM1�PTCT V2.4

The statusline when transmitting UNPROTO in pactor.

===================================================

F1=Help  PACTOR  UNPROTO                 TRANSM. WO On   9600Bd�COM1�PTCT V2.4

The installation of new parameters( the ALT-C key).

===================================================

If yoy want the standard build-in parameters to become active then

delete or rename the file PTCT.INI

Below is the window as it appears when ALT-C is pressed.

�����������������������������������������������������������������������������ͻ�

��               PTCT V2.6  Installation of new parameters                    ��

��                                                                            ��

��    COMx-Hardware   Serial parameters      Screen         Delay times       ��

��                                                                            ��

��    COM  Adr  IRQ  * Baudrate  :9600      Change          Display/ms:1000   ��

��                   * Wordlength:8         COLORS          Char /ms  :10     ��

��   * 1  $3F8   4   * Parity    :n         BIOS/RAM:r      Lines /ms :25     ��

��   * 2  $2F8   3   * Stopbits  :1         CGAWrite:n      Blank /min:10     ��

��   * 3  $3E8   4                          Line len:67                       ��

��   * 4  $2E8   3     ScrollBackbuffer     TxWindow:5      Warning Signals   ��

��   * V24-COMx: 1     Bytes :8192          VgaMode :n      Bell/Morse:m      ��

��                                                          Frequency :500    ��

��     DiskLog         Editor                                                 ��

��     ON              Q.EXE                 Save           Station calls     ��

��                                          PTCT.INI        Call      :PA0NC  ��

��                     AAB                                  Selcall   :PANC   ��

̺                     OFF                                                    ��

�� * = change in PTCT.INI                                                     ��

�� Choose one of the blinking characters.                                     ��

��                                       Stop with  ESC.                      ��

�� Your choice :                                                              ��

�����������������������������������������������������������������������������ͼ�

������������������������������������������������������������������#�����������ͼ

F1=Help  PACTOR    STB            IGNOR          WO Off 9600Bd�COM3� PTCT V2.6

D     is for installing the disklog feature. See below.

E     is for installing an editor. See below.

A     is for Automatic Answer Back (AAB) in Amtor.See Below.

O     is for user defined colors. Try it. WARNING! THE COLORS FOR RX AND

TX SCREEN MUST BE DIFFERENT.

R     is for video output via BIOS or RAM.

G     is for older CGA screens.

I     is wraplength.

W     is height of TX window.

V     is for VGA mode.

S     is for saving the new parameter file.

Y     is for the displaytime of error windows.

H,N   is for delay time for each character and for linefeeds.If the

program works fine,leave them unchanged.

K     is the Blank time.

B     the reserved bytes of the Scroll back buffer.

M     When switching to different modes with the F keys you can hear

a morse sign, a single beep or nothing.

F     is for choosing the frequency of the warning signals.

c     is your Callsign.

L     is your SelCall.

End or

DEL   Change the transmit direction of the PTC.

In PACTOR the program sends a CTRL-Y and in AMTOR a +?

CTRL+END  Transmits the sentence  ' yourcall de mycall '

Changing the com port.

======================

1.  The very first character in the PTCT.INI file is the com. port.

You can change it to 2 or 3 if you want to use com port 2 or 3 with

your own editor. The IRQ and Adress are changed automaticaly if you

use standards.

Calling CQ

==========

When calling CQ with FEC or Unproto you can transmit a CQ text with

the F8 key. There must be 3 CQ files in the same directory as

PTCT.EXE,  named  PTCT.CQA for Amtor, PTCT.CQP for Pactor and

PTCT.CQB for Baudot rtty.

Sample files are in this package. Always start this files with a blank

line.

CHANGE THE CALLSIGN TO YOUR OWN CALLSIGN !

Using The personal Mailbox of the PTC.

=====================================

If you want to use the mailbox system of the PTC switch to 'Dumb

terminal' with  F10. You can use now the commands Dir, Read, Write

Send and Delete. All with the restrictions decribed in the manual

of the PTC.

If you change the mode from pactor to amtor a.s.o when in 'dumb

terminal' the newmode is not known to the program. When back in

the full terminal mode use the F2 key then.

If you want to write a file in the memory of the mailbox do as follows:

First write a textfile with your own editor( ALT-A).

Strike F10 (dumb terminal).

After the prompt cmd:  type:

W test  (the filename you want to write in the MB memory).

type:  ALT-S   (Give the name of the diskfile).

The textfile now goes from disk to the MB memory.

If you want to write on disk a file wich is in the MB memory do as follows:

Strike F10.

After the prompt Cmd: Type: R test (name of the MB memory file).

After the reading strike F10 again.

Strike ALT-B (scroll back buffer).

Strike ALT-F (save the scroll back buffer on disk).

Strike ALT-B for going back to the program.

Now you can edit the diskfile with ALT-A.

The editor.

===========

With ALT-A you can use your favourable texteditor.

The name of the editor must be installed with the ALT-C command

and then  k.  The editor must be in the same directory as PTCT.EXE is,

or the full path and filename must be given with the ALT-C and then k,

or the path to the editor must be given in the autoexec.bat file.

If you don't want to use an editor answer  NONE  at the  k

option in the ALT-C screen.The first character of the editorname

must not be a  N !

The helpline.

=============

When on pactor STB or amtor STB you can toggle a helpline with the

most important function keys at the place where normaly the separating

line between the RX en TX screen is. This can be usefull if you are

using the program for the first time.

The command for this feature is ALT-F1.

Another time ALT-F1 and the helpline disappears. This also happens

when the dumb terminal (F10) is switched on.

The AAB feature.

================

There is a automatic AAB function in this software.

If PTCT receives a $ (dollar sign) then it sends

QRA mycall myselcall. Both your call and selcall come from

the PTC. Some stations and most aplink stations send

a dollar sign as a 'Who Are You' sign (WRU).

Your PTC then answers with your call and selcall.

If you don't need this feature switch it OFF with ALT-C and

the l option.

WARNING !  SWITCH AAB to OFF WHEN RECEIVING  7PLUS FILES !

WARNING !  SWITCH REMOTE OFF WHEN RECEIVING 7PLUS FILES !

The WORDOUT and LINEWRAP features.

==================================

On PACTOR the wordout is automaticaly switched off while most pactor

stations do accept the backspace character. So if you typed a mistake,

type the backspace key and then the right key.

On AMTOR , UNPROTO,FEC and RTTY the wordout is automaticaly switched

to on. In all modes you can change wordout with ALT-W.

If you are typing on the keyboard it is not neccessary to type a hard

return at the end of a line. After X characters there is an automatic

return and linefeed at the end of a word. A return is still always

possible at a shorter line. (X=installed wraplength)

The logbook possibility.

========================

The PTC has the posibility of a short logbook for the last 16 PACTOR

contacts. This program has the possibility to write on disk the

contacts you made even in PACTOR and in AMTOR. If disklog in

the ALT-C screen is set to OFF the PTC's logbook is used.

If you set the disklog to ON the logbook for the contacts is

automaticaly saved on disk.

There is no end of the logfile (perhaps your diskcapacity ?).

So it is better to rename after some time the file PTCT.LOG and

to start with a new one.

Only when you have a link with another station you can give some

additional information (name and QTH) to the logfile PTCT.LOG. Use ALT-I.

Use it when you are receiving. There is no lost of information of

receiving text during the time you are using ALT-I.

The maximum number of characters you can add is 35. Don't pass the End of

the ALT-I window.

If you use ALT-A you can even edit the log file.

I hope you enjoy this program.

Best 73 from Roel, PA0NC @ PI8DAZ.