FlexNet driver documentation    KISS.EXE         (C)1994 G. Jost DK7WJ

Translation: Tom Sailer HB9JNX

Date: 10/94

Ident: KISS

Driver for KISS communications over serial interfaces

WARNING: KISS.EXE should only be used to attach another computer. It is not

able to drive a KISS TNC. Parameters such as TxDelay and full/half duplex

are not forwarded. The only use of this driver is PC to PC or PC to RMNC

coupling.

16550 support (FIFO mode) is implemented.

The KISS baudrate must be specified with the mode command.

The RMNC KISS CRC mode is supported and activated with the mode parameter 'c'.

All other mode parameters are ignored.

COM1-4 may be specified directly at driver start. Other port addresses or

IRQ numbers are optional. The address and the interrupt number may be

specified either decimal or hexadecimal prefixed with '0x'.

Description with "KISS /?".