FlexNet documentation           SNDDIAG.EXE       (C)1996 Tom Sailer HB9JNX

Date: 15.4.96

Ident: SNDDIAG

Diagnostic utility for the SoundBlaster and WSS drivers

It proved to be a rather difficult task to correctly set the audio parameters

of the SoundBlaster and WSS drivers. This utility should remedy the situation.

This utility must be started after FlexNet with a sound card driver is

in memory and started. The sound card driver must be running, i.e. do not

forget to run FSET MODE ....

An oscilloscope like diagram of the input signal at the sound card's input

should help to adjust the audio level.  The height of the trace should be

about 75% of the diagram's height. This utility currently cannot set the

parameters of the driver. With SoundBlaster cards the utilities included

with your sound card must be used to set the audio levels.

An eye diagram (demodulator output) may be used to verify the filter settings

as well as to check the quality of the combination sound card/transceiver.

This utility runs only on computers with an EGA/VGA- or Hercules-compatible

graphics card.

Autor:  Tom Sailer

Weinbergstrasse 76

CH-8408 Winterthur

Packet: HB9JNX @ HB9W.CHE.EU

EMail:  sailer@ips.id.ethz.ch